| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 259R1 (WP 1/15) |
|  |  | English only Original: English |
| Question(s): | 1/15 | Geneva, 28 September - 09 October 2009 |
| TEMPORARY DOCUMENT |  |  |
| Source: | Editors, G.hntreq |  |
| Title: | Draft Recommendation G.9971 (G.hntreq) “Requirements of transport functions in IP home network” |  |

This document is the latest version of G.hntreq based on the discussion of C545, TD257(WP1) and LS received in Q1/SG15 meeting on September 30 and October 2.

This document will be distributed by LS for further comment.

Q.1/15 expects Draft Recommendation G.9971 to be offered for consent to SG 15 at its June 2010 meeting.

Draft Recommendation G.9971 (G.hntreq) “Requirements of transport functions in IP home network”

Summary

TBD

Keywords

TBD

# 1	Scope

This recommendation, G.hntreq, specifies the functional requirements of transport functions in an IP home network based on the generic architecture described by ITU-T Recommendation G.9970 “Generic home network transport architecture”. As popularity of broadband services such as FTTH, VDSL and DOCSIS 3.0 increases, the home network will support various services and connections of many types of end devices. Once home network can be connected to access network seamlessly, the fault, performance, transfer capability, addressing and security for the home network should be managed the same as those for the access network by network operators.  Note that although NT/ONT, which terminates access network, is also studied in this recommendation from the view point of the home network management, it is out of scope that devices are directly connected to NT/ONT without AGW.  Moreover, it is assumed that Ethernet service is provided at WAN side of NT/ONT.

While ITU-T Recommendation G.9960 specifies the physical layer, such as transceiver, and the mapping between physical layer and Layer 2 protocols, G.hntreq studies the subject of IP and Ethernet Bridging technologies, which does not overlap with the scope of G.9960.

[Editor’s Note:  As draft Recommendations G.9961, G.9962 and G.9965 progress in Q.4/15 they will be added to this reference appropriately in the final version of G.hntreq.

This recommendation first clarifies the position of the home network in end-to-end transport networks as well as the home network functional architecture and then provides the functional requirements for the transport capabilities of some key components in the home network, such as Access Gateway (AGW), IP Terminal and so on. Moreover, it mentions some other functional requirements, such as QoS control, management and security in order that the operators can provides their services safely as far as IP Terminal.

# 2	References

[1] ITU-T Rec. G.9970, Generic home network transport architecture, 2009

[2] ITU-T Rec. H.622, A generic Home Network architecture with support for multimedia services, 2008

[3] ITU-T Rec. G.9960, Next generation Home Networking transceivers, 2009

[4] HGI, Home gateway technical requirement, Version 1.0, 2008

[5] Broadband Forum TR-069, CPE WAN Management Protocol v1.1, 2007

[6] ITU-T Rec.Y.2111, Resource and admission control functions in Next Generation Networks, 2008

[7] ITU-T Rec. Y.2001, General overview of NGN , 2004

[8] ITU-T Rec. Y.1540, Internet protocol data communication service - IP packet transfer and availability performance parameters, 2007

[10] ITU-T Rec. Y.1541, Network performance objectives for IP-based services, 2006

[11] ITU-T Rec. Y.1563, Ethernet frame transfer and availability performance, 2009

[12] ITU-T Rec. Y.2091, Terms and definitions for Next Generation Networks, 2008

[13] ITU-T Rec. Y.1223, Interworking guidelines for transporting assured IP flows, 2008

[14] ITU-T Rec. I.371, Traffic control and congestion control in B-ISDN, 2004

[15] Broadband Forum TR-124, Functional Requirements for Broadband Residential Gateway

Devices, Version 1.0, 2006

[16] Metro Ethernet Forum MEF10.1, Ethernet Services Attributes Phase 2, 2006

[17] Metro Ethernet Forum MEF10.1.1, Amendment to Ethernet Services Attributes Phase 2, 2009

[18] IEEE Std. 802.1AB-REV/draft 6.0, Station and Media Access Control Connectivity Discovery, 2009

[19] IEEE Std. 802.1Qat, Stream Reservation Protocol (SRP), 2007

[20] UPnP Forum, Device Architecture 1.0, 2008

# 3	Definitions

3.1	AGW (Access Gateway): AGW associates access network with IP home network and handles IP related protocol packets for these networks. It is equal to IGD (Internet Gateway Device)/RG (Residential Gateway) in Broadband Forum or Home Gateway in HGI by its definition in this recommendation.   Note that it is different from the terminologies used in Y.2091: its “access gateway” means edge router, while its “residential gateway” means terminal adaptor.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

AGW: 	Access Gateway

FTTH: 	Fibre to The Home

ICMP:	Internet Control Message Protocol

IP: 	Internet Protocol

IPCP:	Internet Protocol Control Protocol

IPv6CP: IPv6 Control Protocol

LA:	Local Agent

LLDP: 	Link Layer Discovery Protocol

LM:	Local Manager

L2F:	Layer 2 Forwarding

L3F:	Layer 3 Forwarding

L1T:	Layer 1 Termination

L2T:	Layer 2 Termination

L3T:	Layer 3 Termination

MAC:	Media Access Control

NAT:	Network Address Translation

NATP:	Network Address/Port Translation

NAT-PT: NAT- Protocol Translation

NGN: 	Next Generation Network

NSP:	Network Service Provider

NT: 	Network Termination

ONT:	Optical Network Termination

PD:	Prefix Delegation

PPP:	Point-to-Point Protocol

QoS:	Quality of Service

RA:	Remote Agent

RM:	Remote Manager

RMS: 	Remote Management Server

RSVP:	Resource Reservation Protocol

SIP:	Session Initiation Protocol

SLAAC: StateLess Address AutoConfiguration

# 5	Conventions

LAN (Local Area Network): It represents home network side of AGW.  Ethernet is normally used as layer 2 interface.

WAN (Wide Area Network): It represents the access network side of AGW. Ethernet is normally used as layer 2 interface.

# 6. The position of Home Network in end-to-end networks

This clause describes the Home Network to be managed in end-to-end transport networks as shown in Figure 6-1.   The end-to-end network consists of Home Network, AGW, Access Networks, Core Network and some application servers.

# Figure 6-1 - Home Network to be managed in end-to-end networks

The Primary Terminal is the end-point where the service from the NSP is terminated, according to H.622 [2].  That is, the end-to-end services are provided between two Primary terminals or between each application server and each Primary Terminal.  The AGW associates IP Home Network with Access Network, which is terminated by NT/ONT. Moreover, the Primary Terminal defined in Application layer normally associates with the IP Home Network, in which case the Primary Terminal is called IP Terminal in Transport layer. Therefore, IP Terminal, IP Home Network, AGW, Access Network including NT/ONT, Core Network and their relevant Control network should be managed by the NSPs.  In order to achieve this, this recommendation specifies the functions of each device related to IP Home Network, such as IP Terminal, Ethernet Switch, AGW as well as NT/ONT in more detail, compared to ITU-T G.9970.  Note that it is out of scope that IP Terminal is directly connected to NT/ONT without AGW.  Moreover, it is assumed that Ethernet is used at WAN side of NT/ONT in this recommendation.

# 7. Typical IP Home Network configurations

This clause describes configurations of the IP Home Network to be studied in this recommendation.   Figure 7-1 shows two kinds of IP Home Network configuration.  Configuration 1 shows that one Ethernet Switch either aggregates the traffic from multiple IP Terminals to AGW or switches the traffic between two IP Terminals.  Note that multiple Ethernet Switches can be put in cascade between IP Terminal and AGW.  On the other hand, Configuration 2 shows that each IP Terminal is directly connected to AGW.  Although these figures shows the separate type AGW, this recommendation also studies the aggregate type AGW that combines NT/ONT.

# Figure 7-1 - Typical IP Home Network configurations

# 8. IP Home Network functional architecture

Key devices can be defined by using following several functional components.

- Layer 1 Termination (L1T): Termination functions of Physical layer, such as Ethernet PHY.

- Layer 2 Termination (L2T): Termination functions of Ethernet port, such as MAC address assignment.

- Layer 2 Forwarding (L2F): Ethernet bridging functions using MAC forwarding table as well as L2 QoS processing, such as L2/L2 QoS mapping, which will be discussed in clause 10.  Note that L2F of Ethernet Switch also contains L2/L2 mapping function between Ethernet and wireless within LAN for example.

- Layer 3 Termination (L3T): Termination functions of IP port, such as IP address assignment.

- Layer 3 Forwarding (L3F): IP routing functions using IP routing table as well as L3 QoS processing, such as L3/L3 and L3/L2 QoS mappings, which will be discussed in clause 10.

By using these functional components, IP Terminal can be composed of L1T, L2F, L2T, L3F and L3T, as shown in Figure 8-1 (A).  On the other hand, Ethernet Switch and NT/ONT can be composed of L1T and L2F, as shown in Figure 8-1 (B).

# Figure 8-1 - Functional components of IP Terminal, Ethernet Switch and NT/ONT

It is complicated to show transport layer functions of AGW in one figure, because there are various kinds of AGW.  As shown in Figure 8-2, there are three fundamental types of separate type AGW, which does not combine NT/ONT.

- NAT/NAPT type: Source or destination IP address of an IP packet is changed after forwarded in AGW in accordance with NAT/NAPT.  Note that a MAC address in Layer 2 is terminated in each port.

- IP Router type: An IP packet is just forwarded in AGW without changing its source and destination IP address. Note that a MAC address in Layer 2 is terminated in each port.

- Ethernet Switch type: An Ethernet packet is just forwarded in AGW without changing its source and destination MAC addresses.

# Figure 8-2 - Functional components of separate type AGW

On the other hand, Figure 8-3 shows three types of aggregate type AGW combining NT/ONT.  Each type in Figure 8-3 corresponds to that in Figure 8-2.  For example, Figure 8-3 (B) shows that an IP packet received at WAN-side of NT/ONT is forwarded in Layer 2 and then is forwarded in Layer 3 to LAN-side of AGW.   Moreover, Figure 8-3 (C) case, Ethernet Switch type AGW combined with NT/ONT, shows that IP packets directly flow from WAN to LAN or from LAN to WAN.  Although this case is similar to only NT/ONT case without AGW, some security function is required for Figure 8-3 (C) case in AGW.

# Figure 8-3 - Functional components of aggregate type AGW

In contrast to the above WAN-side features of AGW, three types of LAN-side physical ports can be identified for AGW, as shown in Figure 8-4. Although Figure 8-4 depicts the configuration of LAN-side ports for NAT/NAPT separate type AGW, it is also applied to its corresponding aggregate type AGW as well as IP Router separate/aggregate type AGWs.  In each type, an IP packet from each LAN-side port can be forwarded to WAN-side port in Layer 3.  However, the following distinctions can be found in each type.  Note that each type takes an example that AGW has three LAN-side physical ports.

(X) Hub type: As LAN-side ports are physically connected with each other, an Ethernet packet from each LAN-side port is broadcast to all the LAN-side ports.

(Y) Switch type: As LAN-side ports are connected with each other through Layer 2 Forwarding (L2F), an Ethernet packet from each LAN-side port is transported to the target LAN-side port based on its MAC forwarding table.

(Z) Multiple segments type: As LAN-side ports are connected with each other through Layer 3 Forwarding (L3F), an IP packet from each LAN-side port is routed to the target LAN-side port based on its IP routing table.

# Figure 8-4 - LAN side physical ports of NAT/NAPT separate type AGW

In order to help understanding, Figure 8-5 shows three key devices, such as IP Terminal, Ethernet Switch and NAT/NAPT aggregate type AGW with Switch type LAN ports as well as their relationship with IP Home Network and Ethernet Home Network.

# Figure 8-5 - An example of home network functional architecture

To summarize, the following transport layer functional requirements can be identified for the IP Home Network key devices, such as IP Terminal, Ethernet Switch, AGW and NT/ONT.

R8-1: IP Terminal requires transport layer 1, 2 and 3 functions, as shown in Figure 8-1 (A).

R8-2: Ethernet Switch and NT/ONT requires transport layer 1 and 2 functions, as shown in Figure 8-1 (B).

R8-3: Separate type AGW, which is one of three types shown in Figure 8-2, requires corresponding transport layer functions.

R8-4: Aggregate type AGW, which is one of three types shown in Figure 8-3, requires corresponding transport layer functions.

R8-5: Separate/aggregate type AGWs may have LAN-side transport layer functions shown in Figure 8-4.

# Figure 8-6 - Functional architecture of separate type AGW

In addition to the transport layer 1, 2 and 3 functions described above, some associated functions, such as QoS control, management and security, are also needed.  It is illustrated in Figure 8-6, which shows separate type AGW as an example.   Note that as this recommendation focuses on transport layer 2 and 3, its scope of QoS control functions is only on layer 2 and 3.  Moreover, although management and security functions are related to transport layer 1, 2 and 3 as well as the functions above layer 4, this recommendation handles only layer 2 and 3 aspects for them.   Accordingly, the following requirement can be identified.

R8-5: Layer 2 and 3 of each device associated with IP Home Network, such as IP Terminal, Ethernet Switch, AGW and NT/ONT, require QoS control, management and security when needed.  Figure 8-6 illustrates this for separate type AGW as an example.

Before discussing these three issues, it is better to clarify IP network configuration in access and home network.  Clause 9 will describe IP configuration and its assignment in IP home network, considering possible services.  Based on it, clause 10 will discuss QoS control for each key device, such as IP Terminal, Ethernet Switch, AGW and NT/ONT.  As the same, clause 11 will discuss the remote and local management, while clause 12 will discuss the security aspect.

# 9. IP network configuration and IP address assignment

This clause describes IP network configuration as well as how to assign IP addresses for each IP network configuration.  IP network configuration in IP Home Network can be categorized based on how IP connection is established in the access network and the home network by the End User or NSP.  Note that this clause describes only IP address assignment that is necessary for transporting IP packets, which is described in clause 8.  Some other IP addresses necessary for loopback or management purposes will be described in clause 11.

According to its overview described in Clause 8.3 of ITU-T Recommendation G.9970, the following three fundamental cases can be identified.

- Native IPv4/IPv6 connection to AGW (cf. Figure 9-1): The upper figure shows that a native IPv4 address is assigned at WAN-side port of AGW by using NSP’s DHCPv4, while local IPv4 addresses are assigned at AGW’s LAN-side port and at IP Terminal by using end-user’s DHCPv4.  The lower figure shows that NSP’s DHCPv6-PD (Prefix Delegation) provides IPv6 prefix for home network, while IPv6 addresses are assigned at IP Terminal by using end-user’s DHCPv6 or SLAAC (StateLess Address AutoConfiguration) based on the provided IPv6 prefix.   NAT/NAPT type AGW is used for the former, while IP Router type AGW is used for the latter.

# Figure 9-1 - Native IPv4/IPv6 connection to AGW

- IPv4/IPv6 PPP connection to AGW (cf. Figure 9-2): The upper figure shows that an IPv4 address of IPv4 PPP is assigned at WAN-side port of AGW by using NSP’s IPCP, while local IPv4 addresses are assigned at AGW’s LAN-side port and at IP Terminal by using end-user’s DHCPv4 as same as the above (A).  The lower figure shows that an unnumbered IPv6 PPP is established at WAN-side port of AGW by using NSP’s IPv6CP.  Moreover, NSP’s DHCPv6-PD provides IPv6 prefix for home network, while IPv6 addresses are assigned at IP Terminal by using end-user’s DHCPv6 or SLAAC based on the provided IPv6 prefix.  As the same, NAT/NAPT type AGW is used for the former, while IP Router type AGW is used for the latter.

# Figure 9-2 - IPv4/IPv6 PPP connection to AGW

- IPv4/IPv6 PPP to IP Terminal (cf. Figure 9-3): The upper figure shows that an IPv4 address of IPv4 PPP is assigned at IP Terminal by using NSP’s IPCP.  As the same, the lower figure shows that an IPv6 address of IPv6 PPP is assigned at IP Terminal by using NSP’s IPv6CP as well as either SLAAC or HDCPv6.   Ethernet Switch type AGW is used for both cases.

# Figure 9-3 - IPv4/IPv6 PPP connection to IP Terminal

Moreover, there are some other types of IP network configuration, which can be derived from the fundamental three cases described above.  The first case is IPv6 over IPv4 where IPv6 Terminal wants to communicate with WAN-side IPv6 servers through IPv4 WAN.  Figure 9-4 shows this in more detail that an IPv4 address of IPv4 PPP is assigned at WAN-side port of AGW first, and then NSP’s DHCPv6-PD over L2TP of this IPv4 provides IPv6 prefix for home network, while IPv6 addresses are assigned at IP Terminal by using end-user’s DHCPv6 or SLAAC based on the provided IPv6 prefix.  IPv6 IP Terminal can communicate with IPv6 servers in WAN by using IPv6 over IPv4 L2TP.   The second case is IPv6/v4 NAT-PT (Protocol Translation) where IPv4 Terminal wants to communicate with IPv6 application servers in WAN.  Figure 9-5 shows this in more detail that an IPv4 address of IPv4 PPP is assigned at WAN-side port of AGW by using NSP’s IPCP, while local IPv4 addresses are assigned at AGW’s LAN-side port and at IP Terminal by using end-user’s DHCPv4.   Moreover, as an IPv6 address over IPv4 L2TP is assigned at WAN-side port of AGW, IPv4 IP Terminal can communicate with IPv6 application servers in WAN by using this IPv6 over IPv4 L2TP.    IP Router type AGW is used for Figure 9-4, while NAT/NAPT type AGW is used for Figure 9-5.

# Figure 9-4 - IPv6 connection over v4

# Figure 9-5 - IPv4/IPv6 NAT-PT

# 10. QoS control

# This clause describes the architecture for QoS control in IP Home Network, and then lists the functional requirements for AGW as well as other devices. QoS control can be discussed not only for IP and Ethernet layers but also for other layer 2 protocols, e.g., IEEE802.11, and lower layer protocols. Therefore, QoS mapping should be specified between each two layers.  However, as the scope of this recommendation is IP and Ethernet layers, it only discusses the QoS mapping between these two layers, as shown in Figure 10-1.  QoS mappings other than this should refer to other standard documents, such as DLNA, UPnP and ITU-T Recommendation G.9960.

# Figure 10-1 - The scope of QoS control

# 10.1 Two kinds of QoS guarantee services

# There are two kinds of QoS guarantee services: one is that the QoS is guaranteed in end-to-end along the communication path between users or between a user and an application server in WAN, the other is that the QoS is guaranteed by hop-by-hop in the network. The former, the end-to-end QoS guarantee service, is realized by standardised parameterized QoS control protocols, such as SIP and RSVP for IP and IEEE802.1Qat for Ethernet, as well as admission control.  The parameterized QoS control protocols guarantee end-to-end QoS after the communication path is admitted by admission control.  Note that end-to-end QoS is guaranteed irrespective of network types, network configurations and the implementation of each Network Element.  On the other hand, the latter, the hop-by-hop QoS guarantee service, is realized by standardised prioritised QoS control protocols, such as Diffserv for IP and VLAN service for Ethernet. Note that as the prioritized QoS control protocols guarantee only hop-by-hop QoS in the network, QoS cannot be guaranteed in end-to-end of the network.

# 10.2 Three device types for QoS control

# QoS control is performed in Ethernet Switch, NT/ONT and IP Terminal as well as various kinds of AGW in home network.  These devices are categorized into three in terms of QoS control in each layer, as shown in Figure 10-2.

# Layer 3 QoS control device: Only IP packet QoS control is performed at L3F.  Examples of this type are the following separate type AGWs: (A) NAT/NAPT type and (B) IP Router type in Figure 8-2 as well as (X) Hub type and (Z) Multiple segments type in Figure 8-4.

- Layers 2 and 3 QoS control device: Both Ethernet and IP QoS control is performed at L2F and L3F, respectively. Examples of this type are (A) IP Terminal in Figure 8-1 as well as (Y) Switch type of separate type AGW in Figure 8-4.  In addition to these, this type of device also includes all the aggregate type AGWs that correspond to separate type AGWs described in the above Layer 3 QoS control device.

- Layer 2 control device: Only Ethernet QoS control is performed at L2F.  Examples of this type are (B) Ethernet Switch or NT/ONT in Figure 8-1. In addition to these, this type of device also includes (C) Switch type of separate type AGW in Figure 8-2 as well as the aggregate type AGW corresponding to this separate type AGW.

# Figure 10-2 - Three device types for QoS control

# 10.3 Home network QoS control for each QoS service

# How to perform QoS control in the home network is categorized into the following three types, depending on the QoS services provided by WAN and the QoS control supported by LAN.

# QoS service type 1: Best effort services are provided by WAN, while LAN supports non-QoS control.  In such a case, there is no way to provide for QoS guarantee services between two IP Terminals.

- QoS service type 2: QoS guarantee services are provided by WAN.  LAN also supports QoS control, but WAN cannot control it.  Although QoS control is terminated at AGW, QoS guarantee services can be provided between two IP Terminals if using QoS mapping at AGWs.  Note that QoS mapping is for L2/L2 or L3/L3.  This type of service corresponds to “NGN Release 1 model”, which is specified by ITU-T Recommendation Y.2001.

- QoS service type 3: QoS guarantee services are provided by WAN.  Moreover, LAN supports QoS control, which WAN can also control.  Accordingly, QoS guarantee services can be terminated at IP Terminal.  AGW, IP Terminal and other devices in the home network will cooperatively perform QoS guarantee control for the LAN-WAN traffic to use QoS guarantee services in WAN, taking into account the traffic within the LAN. This type corresponds to “NGN Release 2 model” providing end-to-end NGN services, which is also specified by ITU-T Recommendation Y.2001.

Note that QoS guarantee services mean either end-to-end QoS guarantee service realized by parameterized QoS or hop-by-hop QoS guarantee service realized by prioritized QoS.

# Figure 10-3 – Relationship between QoS service types and QoS control

# 10.4 Parameters for QoS control

# This section describes two kinds of parameters used for QoS control: one is traffic descriptors characterizing user traffic flows; the other is QoS parameters representing quality objectives for the end-to-end QoS guarantee services based on SLA.

# Recommended traffic descriptors are listed as follows. Although these parameters have been specified for Ethernet in Metro Ethernet Forum Specification MEF10.1, these can be also applied to IP.

CIR: Committed Information Rate

CBS: Committed Burst Size

EIR: Excess Information Rate

EBS: Excess Burst Size

Recommended QoS parameters can be identified in some ITU-T recommendation.  Y.1541 specifies QoS parameters for IP as follows.

IPTD: IP packet Transfer Delay

IPDV: IP packet Delay Variation

IPLR: IP packet Loss Ratio

IPER: IP packet Error Ratio

On the other hand, Y.1563 specifies QoS parameters for Ethernet as follows.

FTD: Frame Transfer Delay

FDV: Frame Delay Variation

FLR: Frame Loss Ratio

FER: Frame Error Ratio

# 10.5 Functional components for QoS control

# Based on I.371, this section describes functional components for QoS control as follows.  Figure 10-4 shows a functional architecture for QoS control, where queuing block contains the functional components, such as (4), (5), and (6). Note that it is symmetry from LAN to WAN and from WAN to LAN in general. Traffic descriptors in the previous section are used in the following (1), (5), (6) and (7), while QoS parameters are used in the following (7).  Both traffic descriptors and QoS parameters from the control and/or management planes are first received by equipment manager, which then transfers them to admission control as well as sets them onto policing, bandwidth control and traffic shaping for the end-to-end QoS guarantee services.  Moreover, as a special case, the equipment manager also sets traffic descriptors onto bandwidth control and traffic shaping for the hop-by-hop QoS guarantee services.

# Policing: Policing function detects unexpected traffic and protects network from these traffic flows. Some traffic descriptors for Ethernet and/or IP are provisioned for detecting such traffic flows before user communication is initiated. This function can be located at ingress of AGW LAN/WAN sides for example.

- Classification: Classification function recognizes type of each traffic flow and assigns various “priority” to each. This recognition is performed according to values of specific fields in Ethernet frame or IP packet or both of them. This function can be located at ingress of AGW LAN/WAN sides for example.

- Marking: Marking function writes “priority” in each Ethernet frame or IP packet according to the result of “classification”. Otherwise, a specific “priority” value is provisioned.  This function can be located at ingress of AGW LAN/WAN sides for example.

- Priority control: Priority control function controls input of traffic flows to queues according to the priority level either assigned by “Classification” or written by “Marking”.  SP (Strict Priority) is a typical mechanism for this function.

- Bandwidth control: Bandwidth control function controls output of traffic flows from stored queues in order to guarantee minimum bandwidth based on CIR and CBS of traffic descriptors.  Bandwidth control function assigns bandwidth for traffic flows by using scheduling mechanisms, such as WFQ (Weighted Fair Queuing), WRR (Weighted Round Robin), DRR (Deficit Round Robin) and so on.

- Traffic shaping: Traffic shaping function also controls output of traffic flows from stored queues, similar to bandwidth control.  However, the purposes of this traffic shaping function are the restriction of traffic flows below the maximum transfer rate as well as the reduction of delay variation in every traffic flow.

- Admission control: Admission control function manages and judges whether to accept or reject each managed traffic flow for the end-to-end QoS guarantee services based on the traffic descriptors and QoS parameters before user communication is initiated.  One example is the SIP case. The management plane provisions QoS parameters onto the admission control via equipment manager. After the admission control receives the traffic descriptors from control plane via equipment manager, it judges to accept the traffic flow by comparing the received traffic descriptors with the already-provisioned QoS parameters as well as analysing whether or not the network resources can be used via another control plane.  In case of acceptance, the admission control sends its acceptance message to the policing and queuing.  Another example is RSVP case.  After the admission control receives the traffic descriptors and QoS parameters from control plane via equipment manager, it judges to accept the traffic flow by analysing whether or not the network resources can be used by using another control plane.   In case of acceptance, the admission control sends its acceptance message to the policing and queuing. Judgement algorithms in admission control are out of scope in this recommendation.

- Traffic measurement: Based on Y.1540, traffic measurement function counts incoming and outgoing traffic to evaluate offered traffic load. Several counting units can be specified, such as the number of bytes, the number of Ethernet frame or IP packets.  Examples of counting mechanisms are jumping window and sliding window.

# Figure 10-4 - Functional architecture for QoS control

# All of these functional components are not always required for each device.  The following section 10.6 will describe functional components necessary for each QoS service type in section 10.3.  Note that as it is only traffic measurement for QoS service type 1 “Best effort service” to require, section 10.6 focuses on QoS service type 2 and 3.

# 10.6 Typical QoS control functional components of each device for QoS service type 2 and 3

# This section identifies QoS control functional components necessary for each device, which is categorized as device type (a), (b) and (c) in section 10.2.  Required functional components depend on QoS service type 2 and 3, as described in section 10.3.  Each QoS service type 2 and 3 is divided into two. One is end-to-end QoS guarantee service, which is realized by using parameterized QoS control protocol. The other is hop-by-hop QoS guarantee service, which is realized by using prioritized QoS control protocol.

Table 10-1 shows control functional components for QoS service type 2 in case of (A) end-to-end QoS guarantee service as well as in case of (B) hop-by-hop QoS guarantee service.  Moreover, Table 10-2 shows control functional components for QoS service type 3 in case of (A) end-to-end QoS guarantee service as well as in case of (B) hop-by-hop QoS guarantee service.  Note that Table 10-1 and Table 10-2 show only separate type AGWs.  However, QoS control functional components necessary for aggregate type AGWs can be easily derived from these tables.  For example, both aggregate type AGW (a) and (b) in Figure 8-3 correspond to separate type AGW (b) in Figure 8-2, while aggregate type AGW (c) in Figure 8-3 correspond to separate type AGW (c) in Figure 8-2.

# Table 10-1 Control functional components for QoS service type 2

# (A) In case of end-to-end QoS guarantee service

| Device | Separate type AGW |  |  | NT/ONT |
| --- | --- | --- | --- | --- |
| Device type | (a) | (b) | (c) | (c) |
| Policing | R | R for L3; O for L2 | R | R |
| Classification | R | R for L3; O for L2 | R | R |
| Marking | O* | O* | O* | O* |
| Priority control | R | R for L3; O for L2 | R | R |
| Bandwidth control | R | R for L3; O for L2 | R | R |
| Traffic shaping | R | R for L3; O for L2 | R | R |
| Admission control | R | R for L3; O for L2 | R | R |
| Traffic measurement | O*** | O*** | O*** | O*** |

# (B) In case of hop-by-hop QoS guarantee service

| Device | Separate type AGW |  |  | NT/ONT |
| --- | --- | --- | --- | --- |
| Device type | (a) | (b) | (c) | (c) |
| Policing | N/A | N/A | N/A | N/A |
| Classification | R | R for L3; O for L2 | R | R |
| Marking | O* | O* | O* | O* |
| Priority control | R | R for L3; O for L2 | R | R |
| Bandwidth control | O** | O** | O** | O** |
| Traffic shaping | O** | O** | O** | O** |
| Admission control | N/A | N/A | N/A | N/A |
| Traffic measurement | O*** | O*** | O*** | O*** |

# R: Recommended

O: Optional

N/A: Not Applicable

# Note

(*) In case that specific value in Ethernet MAC and/or IP header has to be overwritten, the support of this functional component is recommended.

(**) In case that transfer bandwidth has to be limited and/or surplus bandwidth has to be shared with already provisioned traffic flows, the support of these functional components is recommended.

(***) In case that traffic flows have to be measured for traffic management, the support of this functional component is recommended. This function should not affect user traffic flows.

# Table 10-2 Control functional components for QoS service type 3

# (A) In case of end-to-end QoS guarantee service

| Device | IP Terminal | Ethernet Switch | Separate type AGW |  |  | NT/ONT |
| --- | --- | --- | --- | --- | --- | --- |
| Device type | (b) | (c) | (a) | (b) | (c) | (c) |
| Policing | N/A | R | R | R for L3; O for L2 | R | R |
| Classification | R for L3; O for L2 | R | R | R for L3; O for L2 | R | R |
| Marking | O* | O* | O* | O* | O* | O* |
| Priority control | R for L3; O for L2 | R | R | R for L3; O for L2 | R | R |
| Bandwidth control | R for L3; O** for L2 | R | R | R for L3; O** for L2 | R | R |
| Traffic shaping | R for L3; O** for L2 | R | R | R for L3; O** for L2 | R | R |
| Admission control | R for L3; O for L2 | R | R | R for L3; O for L2 | R | R |
| Traffic measurement | O*** | O*** | O*** | O*** | O*** | O*** |

# (B) In case of hop-by-hop QoS guarantee service

| Device | IP Terminal | Ethernet Switch | Separate type AGW |  |  | NT/ONT |
| --- | --- | --- | --- | --- | --- | --- |
| Device type | (b) | (c) | (a) | (b) | (c) | (c) |
| Policing | N/A | N/A | N/A | N/A | N/A | N/A |
| Classification | R for L3; O for L2 | R | R | R for L3; O for L2 | R | R |
| Marking | O* | O* | O* | O* | O* | O* |
| Priority control | R for L3; O for L2 | R | R | R for L3; O for L2 | R | R |
| Bandwidth control | O** | O** | O** | O** | O** | O** |
| Traffic shaping | O** | O** | O** | O** | O** | O** |
| Admission control | N/A | N/A | N/A | N/A | N/A | N/A |
| Traffic measurement | O*** | O*** | O*** | O*** | O*** | O*** |

# R: Recommended

O: Optional

N/A: Not Applicable

# Note

(*) In case that specific value in Ethernet MAC and/or IP header has to be overwritten, the support of this functional component is recommended.

(**) In case that transfer bandwidth has to be limited and/or surplus bandwidth has to be shared with already provisioned traffic flows, the support of these functional components is recommended.

(***) In case that traffic flows have to be measured for traffic management, the support of this functional component is recommended. This function should not affect user traffic flows.

# 10.7 QoS control requirements for QoS service type 2

As the first step, this recommendation discusses QoS control requirements focusing on QoS service type 2 (NGN release 1), considering the previous sections.   The following requirement can be identified first.

R10-1 AGW and NT/ONT should support L2 QoS control and/or L3 QoS control for NGN Release 1 services.  Table 10-1 can be referred to for the required QoS control functional components for each device.

At least L3 QoS control is required for offering L3 services.  However, L3/L2 QoS mapping should be performed in L3F, in case that L2 QoS control is provided.  Therefore, the following requirements can be identified.

R10-2: AGW should support L3 QoS control in case that network provides only L3 QoS control for NGN Release 1 services.

R10-3: In case that network provides both L3 and L2 QoS control for NGN Release 1 services, AGW should support L3 and L2 QoS control including L3/L2 QoS mapping, while NT/ONT should support L2 QoS control.

L2/L2 and/or L3/L3 QoS mapping should be performed in AGW when providing QoS guarantee services to IP Terminal in NGN Release 1.   As QoS guarantee services are divided into end-to-end QoS guarantee and hop-by-hop services, they are discussed separately as follows.

Figure 10-5 shows an example for realizing hop-by-hop QoS guarantee service. Note that WAN and LAN management plane may correspond to remote manager and local manger, respectively, which will be described in clause 11.  Note that QoS mapping, such as mapping between LAN/WAN markings, is required in QoS control functional components other than admission control.  Following scenario can be found.

- LAN management plane sets proprietary LAN marking specifications onto AGW.

- After retrieving LAN marking specifications, WAN management plane sets the mapping between proprietary LAN and standardized WAN markings onto QoS control of AGW.

- QoS control components of IP Terminal sends control messages containing proprietary marking #l.

- QoS control components of AGW maps the received marking #l to the corresponding standardized marking #w in WAN and then send control messages containing marking #w to WAN.

# Figure 10-5 – A realization of hop-by-hop QoS guarantee services in QoS service type 2

Figure 10-6 shows an example for realizing end-to-end QoS guarantee services. As same as hop-by-hop QoS service scenario, the mapping between proprietary LAN and standardized WAN markings is needed.  Moreover, QoS mapping, such as mapping of LAN/WAN traffic descriptors and mapping of LAN/WAN markings, as well as QoS parameters are required in QoS control functional components, such as admission control, policing, bandwidth control and traffic shaping. Following scenario can be found.  Note that the following scenario only describes the one other than marking mapping.

- LAN management plane sets proprietary LAN traffic descriptor specifications onto AGW.

- After retrieving LAN traffic descriptor specifications, WAN management plane sets the mapping between proprietary LAN and standardized WAN traffic descriptors onto admission control of AGW.  It also sets its QoS parameters.

- Admission control of IP Terminal sends control messages containing proprietary traffic descriptor #l.

- Admission control of AGW maps the received traffic descriptor #l to the corresponding standardized traffic descriptor #w in WAN and then sends control messages containing traffic descriptor #w to WAN.

# Figure 10-6 - A realization of end-to-end QoS guarantee services in QoS service type 2

# 11. Home network management

# 11.1  Home network management architecture

# Clause 8.2 in ITU-T Recommendation G.9970 shows two kinds of management schemes by Remote Management Server (RMS).  Scheme A is that RMS directly manages home network end devices, such as IP Terminal, Non-IP GW and Non-IP Terminal. On the other hand, Scheme B is that RMS manages such devices through AGW.   Scheme A will be used when IPv6 Terminal is supplied by NSP in future, while Scheme B will be used when some management protocols for home network, such as UPnP, are used by the end users.  Therefore, this recommendation describes the requirements for home network management architecture focusing on Scheme B.  Although Scheme B addresses that AGW manages various kinds of home network end devices, this recommendation studies management of IP Terminal as well as devices composing IP Home Network, such as Ethernet Switch.  It is shown in Figure 11-1.

# Figure 11-1 - Remote end device management configuration

# Scheme B has two kinds of management interfaces: one is the management interface between RMS and AGW, the other is the one between AGW and each device in IP Home Network.  Figure 11-2 shows the functional management architecture for Scheme B.  Note that this figure shows the case that the NAT/NAPT aggregate type AGW with Switch type LAN ports. The definition of each functional component can be described below:

- Local Agent (LA): According to the commands from Local Manager, Local L2 Agent manages L2F of Ethernet Switch, while Local L3 Agent manages IP Terminal and LAN-side parts of AGW.

- Local Manager (LM): Management application functions for the IP Home Network, which interworks with Local Agent.

- Remote Agent (RA): According to the commands from Remote Manager, it manages WAN-side parts of AGW.  Moreover, it also interworks with Local Manager in order to manage IP Home Network.

- Remote Manager (RM): Management application functions in RMS, which interworks with Remote Agent in the IP Home Network.

# This recommendation assumes the following Management applications for Local Manager:

- Configuration management applications

  - To show L3 (IP) network topology in IP Home Network.

  - To show L2 (Ethernet) network topology in IP Home Network.

  - To set, get and show the management information of each device in IP Home Network.

  - To upgrade the function of each device in IP Home Network.

  - To get specific information of each device in IP Home Network, such as URLs for logging data.

  - To reset or initialize each device in IP Home Network.

- Fault management applications

  - To check whether the management interface is set up properly or not.

  - To check whether each device in IP Home Network is set up properly or not.

- Performance management applications

  - To check the network performance of the IP Home Network.

# The following clause 11.2 describes the requirements for the RM-RA management interface, while clause 11.3 describes the requirements for the LM-LA management interface. Note that LM can reside in any device in IP Home Network, although Figure 11-2 depicts the case that it resides in AGW. Moreover, there will be the case of only Local M/A without Remote M/A in order to manage IP Home Network locally.

# Figure 11-2 - An example of functional management architecture for Scheme B

# 11.2 Requirements for RM-RA management interface

Requirements for the management interface between RM and RA can be discussed from two aspects. One is requirements for the management interface itself, while the other is the requirements for managing the WAN-side transport layer of AGW.  As this interface is already discussed in BBF TR-069 and relevant documents, this recommendation describes only high level requirements.

The following requirements can be identified for the management interface.

R11-R1: The standardized protocols should be used for the RM-RA management interface.  TR-069 is recommended.

R11-R2: IP address should be assigned to each RM and RA.

R11-R3: It should be checked whether the RM-RA management interface protocol is set up properly or not, when needed.

As shown in Figure 8-2 of clause 8, NAT/NAPT type AGW has L3T, while IP Router type AGW does not have L3T.   However, for the purpose of localizing the fault of the AGW WAN-side, IP address should be assigned to the WAN-side port of AGW. As described in clause 9, we have to keep in mind that there are several kinds of IP connection, such as native IPv4/v6, PPP, L2TP and so on. Therefore, the following requirements can be identified for managing transport layer of AGW.

R11-R4: RA should notify RM of its managing IP address of AGW WAN-side.  Note that IP address should be assigned to each IPv4/v6, PPP, L2TP connection at AGW WAN-side.

Moreover, as described at the end of clause 10, the following requirements can be identified for managing home network when providing NGN Release 1 services.

R11-R5: RA should notify RM of its managing LAN traffic descriptors specifications in case of providing QoS guarantee services to IP Terminal in end-to-end QoS guarantee services of NGN release 1.

R11-R6: RA should receive L2/L2 and/or L3/L3 QoS mapping, such as mapping between LAN/ WAN traffic descriptors, as well as QoS parameters in case of providing QoS guarantee services to IP Terminal in end-to-end QoS guarantee services of NGN release 1.

R11-R7: RA should notify RM of its managing marking specifications in case of providing QoS guarantee services to IP Terminal in hop-by-hop QoS guarantee services of NGN release 1.

R11-R8: RA should receive L2/L2 and/or L3/L3 QoS mapping, such as mapping between LAN/WAN marking in case of providing QoS guarantee services to IP Terminal in hop-by-hop QoS guarantee services of NGN release 1.

# 11.3 Requirements for LM-LA management interface

Requirements for the management interface between LM and LA can be discussed from two aspects. One is requirements for management interface itself, while the other is the requirements for managing transport layer of the IP Home Network.

# The following requirements can be identified for the former.

R11-L1: The standardized protocols should be used for the LM-LA management interface.  The candidate protocols are ICMP, UPnP or LLDP.

R11-L2: It should be checked whether the LM-LA management interface protocol is set up properly or not, when needed.

# As shown in Figure 8-2, NAT/NAPT type AGW has L3T, while IP Router type AGW does not have L3T.   However, for the purpose of localizing the fault of the AGW LAN-side, IP address should be assigned to each LAN-side port of AGW. Moreover, we must keep in mind that IP address and so on should be also assigned statically although clause 9 described in such a way that they are assigned automatically.   Therefore, management requirements for the latter can be categorized into the following three categories.

# Configuration management

R11-C1: Local L3 Agent can notify LM of its managing IP address.

R11-C2: Local L3 Agent can notify LM of its managing MAC address.

R11-C3: Local L2 Agent can notify LM of its managing MAC forwarding table.

R11-C4: Local L3 Agent can notify LM of the management information of its managing device, such as the product class, the manufacture’s name, the device model name and the device model number.

R11-C5: Local L2 Agent can notify LM of the management information of its managing device, such as the product class, the manufacture’s name, the device model name and the device model number.

R11-C6: Local L3 Agent can perform setting of IP address based on the commands from LM, when static address setting is needed for its managing device.

R11-C7: It should be checked whether the transport layer of two devices is set up properly or not, when needed.

R11-C8: Local L3 Agent can reply back the management information of its managing device based on the command from LM.

R11-C9: Local L3 Agent can reply back URLs for log of its managing device based on the command from LM.

R11-C10: Local L3 Agent can perform the reset or initialization of its managing device based on the command from LM.

- Fault management

R11-F1: Local L3 Agent can notify LM of the status of its managing device periodically or when the device is in abnormal state.

R11-F2: Local L2 Agent can notify LM of the status of its managing device periodically or when the device is in abnormal state.

- Performance management

R11-P1: Local L3 Agent can reply back the number of packets per port or flow and so on device based on the command from LM.

# 12. Security management

Although security management is categorized into authentication/authorization, encryption and defence, this recommendation discusses requirements for encryption and defence relevant to home network transport layer 2 and 3.

- Encryption: The type of encryption and the key exchange protocol are relevant to this recommendation.

- Defence: Packet filtering methods to prevent suspicious traffic from/to the AGW are relevant to this recommendation.

# Appendix I: Relationship among home network standards

Figure 1 shows which standardization bodies should be taken into account in order to study this recommendation.  As this recommendation focuses on the architecture, QoS and management aspects of layer 2 and 3 in the home network, Broadband Forum, HGI, IEEE and UPnP are especially the ones to be coordinated with.

# Figure 1/Appendix I - Major standardization bodies related to home network

Moreover, Figure 2 shows the position of this recommendation within ITU-T.

- Layer 1 and layer 2 specifications of home network are being studied in SG9 and SG15, while this recommendation discusses the architecture and high level requirements of layer 2 and 3 of home network on the basis of G.9970.  As the layer 2 of this recommendation focuses only on MAC Switch including VLAN, there is no overlap with other recommendations, although some cooperation may be needed.

- As this recommendation assumes only Ethernet for the Access network, the relevant access network recommendations are G.983 and so on.

- As this recommendation handles QoS in transport layer, NGN and QoS recommendations in SG13 and in SG12, respectively, are referred to.  On the other hand, SG16 is studying home network architecture as well as QoS in application layer.

# Figure 2/Appendix I - Relationship among ITU-T SGs for home network

_________________