| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | TD 691 Rev.1 (PLEN/15) |
|  |  | English only Original: English |
| Question(s): | 9/15 | 10-21 September 2012 |
| TD |  |  |
| Source: | Editor G.8021/Y.1341 |  |
| Title: | Draft Amendment 1 to Recommendation ITU-T G.8021/Y.1341 (for Consent, September 2012) |  |

# Introduction

This document contains the draft Amendment 1 to Recommendation ITU-T G.8021/Y.1341, which is based on the agreement at the Xi’an meeting in May 2012. Following items are incorporated:

- WD26 (AL):

  - TI_TimeStamp1 is added to clauses 9.2.1 (ETHx_FT), 9.2.2 (ETHG_FT)  and 9.4.1 (ETHDe_FT).

  - NOTE for DM primitives (i.e. pB_FD, pB_FDV, pF_FD, pF_FDV, pN_FD, pN_FDV) is added to Table 9-4 (ETHx_FT_Sk) and Table 9-y+1 (ETHG_FT_Sk).

- WD9 (Multi company)

  - The specification of CSF in clause 8.1.16 is updated.

- C2072 (AL)

  - MI_LM_Intermediate_Request is added to clauses c8.1.9.1, c8.1.9.2, Figures 8-33 and 8-34

  - MI_DM_Intermediate_Request is added to clauses 8.1.10.1, c8.1.10.2, Figures 8-41 and 8-42.

  - MI_1DM_Intermediate_Request is added to clauses c8.1.11.1, c8.1.11.5, Figures 8-49 and 8-54 (On-demand 1DM)

  - MI_SL_Intermediate_Request is added to clauses 8.1.14.1, c8.1.14.2, Figures 8-xx and 8-xx+2

  - MI_1SL_Intermediate_Request is added to clauses 8.1.15.1, c8.1.15.5, Figures 8-xx+10 and 8-xx+17

  - All MI_XX_Intermediate Request are added to Table 9-11 and Fig 9-33 (ETHDe_FT_So), and Table 9-12 and Fig 9-35 (ETHDe_FT_Sk)

- C2115 (NEC)

  - The specification of CSF with DCI in clause 8.1.16 is updated.

  - MI_CSFdciEnable signal is added to Tables 9-5, 9-7, 9-9 and Figures 9-19, 9-25, 9-29.

# Reference

[1]	Recommendation ITU-T G.8021/Y.1341 (May 2012), “Characteristics of Ethernet transport network equipment functional blocks”

_______________

Recommendation ITU-T G.8021/Y.1341

Characteristics of Ethernet transport network equipment functional blocks

Amendment 1

Summary

Amendment 1 to Recommendation ITU-T G.8021/Y.1341 (2012) presents the support of the intermediate retrieval of on-demand performance measurement results, and enhancements concerning the modelling of delay measurement and client signal failure functions.

History

| Edition | Recommendation | Approval | Study Group |  |
| --- | --- | --- | --- | --- |
| 1.0 | ITU-T G.8021/Y.1341 | 2004-08-22 | 15 |  |
| 1.1 | ITU-T G.8021/Y.1341 (2004) Amend. 1 | 2006-06-06 | 15 |  |
| 2.0 | ITU-T G.8021/Y.1341 | 2007-12-22 | 15 |  |
| 2.1 | ITU-T G.8021/Y.1341 (2007) Amend. 1 | 2009-01-13 | 15 |  |
| 2.2 | ITU-T G.8021/Y.1341 (2007) Amend. 2 | 2010-02-22 | 15 |  |
| 3.0 | ITU-T G.8021/Y.1341 | 2010-10-22 | 15 |  |
| 3.1 | ITU-T G.8021/Y.1341 (2010) Amend. 1 | 2011-07-22 | 15 |  |
| 4.0 | ITU-T G.8021/Y.1341 | 2012-05-07 | 15 |  |
| 4.1 | ITU-T G.8021/Y.1341 (2010) Amend. 1 | 2012-xx-xx | 15 |  |

# Update clause 8.1.9

### 8.1.9	Loss Measurement (LM) Processes

#### 8.1.9.1	Overview

Figure 8-33 shows the different processes inside MEPs and MIPs that are involved in the on-demand Loss Measurement Protocol.

The MEP On-demand OAM Source insertion process is defined in clause 9.4.1.1, the MEP On-demand OAM Sink extraction process in clause 9.4.1.2, the MIP On-demand OAM Sink Extraction process in clause 9.4.2.2, and the MIP On-demand OAM Source insertion process in clause 9.4.2.1. In summary, they insert and extract ETH_CI OAM signals into and from the stream of ETH_CI_D Traffic Units together with the complementing P and D signals going through a MEP and MIP; the extraction is based on MEL and Opcode. Furthermore, the insertion process inserts the correct MEL and SA values into the OAM Traffic Units.

Figure 8-33 – Overview of Processes involved with on-demand Loss Measurement

The on-demand LM control process controls the on-demand LM protocol. The protocol is activated upon receipt of the MI_LM_Start(DA,P,Period) signal and remains activated until the MI_LM_Terminate signal is received.

The result is communicated via the MI_LM_Result(N_TF, N_LF, F_TF, F_LF) signal when the process is terminated by the MI_LM_Terminate signal or when an intermediate result is requested via the MI_LM_Intermediate_Request signal. If the on-demand LM control process activates the multiple monitoring on different CoS levels simultaneously, each result is independently managed per CoS level.

The LMM Generation process generates an LMM Traffic Unit that passes transparently through MIPs, but that will be processed by the LMM Reception Process in MEPs. The LMR Generation Process generates an LMR Traffic Unit in response to the receipt of an LMM Traffic Unit. The LMR Reception process receives and processes the LMR Traffic Units.

----

#### 8.1.9.2	LM Control Process

The behaviour of the on-demand LM Control Process is defined in Figure 8-34.

Figure 8-34 – On-demand LM Control Behaviour

Upon receipt of the MI_LM_Start(DA,P,Period), the LM protocol is started. Every Period the generation of an LMM frame is triggered (using the LMM(DA,P,0) signal), until the MI_LM_Terminate signal is received.

The received counters are used to count the near end and far end transmitted and lost frames. This result is reported using the MI_LM_Result(N_TF, N_LF, F_TF, F_LF) signal after the receipt of the MI_LM_Terminate signal or of the MI_LM_Intermediate_Request signal.

# Update clause 8.1.10

### 8.1.10	Delay Measurement (DM) Processes

#### 8.1.10.1	Overview

Figure 8-41 shows the different processes inside MEPs and MIPs that are involved in the on-demand Delay Measurement Protocol.

The MEP On-demand OAM Source insertion process is defined in clause 9.4.1.1, the MEP On-demand OAM Sink extraction process in clause 9.4.1.2, the MIP On-demand OAM Sink Extraction process in clause 9.4.2.2, and the MIP On-demand OAM Source insertion process in clause 9.4.2.1. In summary, they insert and extract ETH_CI OAM signals into and from the stream of ETH_CI_D Traffic Units and the complementing P and D signals going through a MEP and MIP; the extraction is based on MEL and Opcode. Furthermore, the insertion process inserts the correct MEL and SA values into the OAM Traffic Units.

Figure 8-41 – Overview of Processes involved with on-demand Delay Measurement

The on-demand DM control process controls the on-demand DM protocol. The protocol is activated upon receipt of the MI_DM_Start(DA,P,Test ID,Length,Period) signal and remains activated until the MI_DM_Terminate signal is received. The result is communicated via the MI_DM_Result(count, B_FD[], F_FD[] ,N_FD[]) signal when the process is terminated by the MI_DM_Terminate signal or when an intermediate result is requested via the MI_DM_Intermediate_Request signal. If the on-demand DM control process activates the multiple monitoring on different CoS levels simultaneously, each result is independently managed per CoS level. Optional Test ID TLV can be utilized to distinguish each measurement if multiple measurements are simultaneously activated in an ME. If the protocol is used in multipoint-to-multipoint environments, the multicast class 1 address can be used for DA and the test result is independently managed per peer node.

The DMM generation process generates DMM Traffic Units that pass through MIPs transparently, but are received and processed by DMM Reception processes in MEPs. The DMR Generation process may generate a DMR Traffic Unit in response. This DMR Traffic Unit also passes transparently through MIPs, but is received and processed by DMR Reception processes in MEPs.

At the Source MEP side, the DMM generation process stamps the value of the Local Time to the TxTimeStampf field in the DMM message when the first bit of the frame is transmitted. Note well that at the sink MEP side, the DMM reception process stamps the value of the Local Time to the RxTimeStampf field in the DMM message when the last bit of the frame is received.

The DMR generation and reception process stamps with the same way as the DMM generation and reception process.

----

#### 8.1.10.2	DM Control Process

The behaviour of the on-demand DM Control Process is defined in Figure 8-42.

Figure 8-42 – On-demand DM Control Behaviour

Upon receipt of the MI_DM_Start(DA,P,Test ID,Length,Period), the DM protocol is started. Every Period the generation of a DMM frame is triggered (using the DMM(DA,P,0,Test ID TLV,TLV) signal), until the MI_DM_Terminate signal is received. The TLV field of the DMM frames can have two types of TLVs. The first one is the Test ID TLV, which is optionally used for a discriminator of each test and the value ‘Test ID’ is included in the TLV. The second one is the Data TLV, which is determined by the Generate(Length) function. Generate(Length) generates a Data TLV with length ‘Length’ of arbitrary bit pattern to be included in the DMM frame.

Upon receipt of a DMR Traffic Unit the Delay value recorded by this particular DMR Traffic Unit is calculated. This result is reported using the MI_DM_Result(count, B_FD[], F_FD[] ,N_FD[]) signal after the receipt of the MI_DM_Terminate signal or of the MI_DM_Intermediate_Request signal. Note that the measurements of F_FD and N_FD are not supported by peer MEP if both TxTimeStampb and TxTimeStampf are zero.

# Update clause 8.1.11

### 8.1.11	One Way Delay Measurement (1DM) Processes

#### 8.1.11.1	Overview

Figure 8-49 shows the different processes inside MEPs and MIPs that are involved in the on-demand One Way Delay Measurement Protocol.

The MEP On-demand OAM Source insertion process is defined in clause 9.4.1.1, the MEP On-demand OAM Sink extraction process in clause 9.4.1.2, and the MIP On-demand OAM Sink Extraction process in clause 9.4.2.2. In summary, they insert and extract ETH_CI OAM signals into and from the stream of ETH_CI_D Traffic Units and the complementing P and DE signals going through a MEP and MIP; the extraction is based on MEL and Opcode. Furthermore, the insertion process inserts the correct MEL and SA values into the OAM Traffic Units.

Figure 8-49 – Overview of Processes involved with on-demand One Way Delay Measurement

The on-demand 1DM protocol is controlled by the on-demand 1DM Control_So and 1DM Control_Sk processes. The on-demand 1DM Control_So process triggers the generation of 1DM Traffic Units upon the receipt of an MI_1DM_Start(DA,P,Test ID,Length,Period) signal. The on-demand 1DM Control_Sk process processes the information from received 1DM Traffic Units after receiving the MI_1DM_Start(SA,Test ID) signal The result is communicated by the sink MEP when the on-demand 1DM Control_Sk process is terminated  by the MI_1DM_Terminate signal or when an intermediate result is requested via the MI_1DM_Intermediate_Request signal..

The 1DM generation process generates 1DM messages that pass transparently through MIPs and are received and processed by the 1DM Reception Process in MEPs.

At the Source MEP side, the 1DM generation process stamps the value of the Local Time to the TxTimeStampf field in the 1DM message when the first bit of the frame is transmitted. Note well that at the sink MEP side, the 1DM reception process records the value of the Local Time when the last bit of the frame is received.

----

#### 8.1.11.5	1DM Control_Sk Process

Figure 8-54 shows the behaviour of the on-demand 1DM Control_Sk process. The MI_1DM_Start(SA) signal starts the processing of 1DM messages coming from a MEP with SA as MAC address. The protocol runs until the receipt of the MI_1DM_Terminate signal.

While running the process processes the received 1DM(rSA,TxTimeStampf,RxTimef,rTestID) information. First the rSA is compared with the SA from the MI_1DM_Start (SA) signal. If the rSA is not equal to this SA, the information is ignored. Next the rTestID is compared with the TestID from the MI_1DM_Start (Test ID) signal. If the MI_1DM_Start (Test ID) signal is configured and rTestID is available but both values are different, the information is ignored. Otherwise the Delay from the single received 1DM Traffic Unit is calculated. This result is reported using the MI_1DM_Result(count, N_FD[]) signal after the receipt of the MI_1DM_Terminate signal or of the MI_1DM_Intermediate_Request signal.

Figure 8-54 – On-demand 1DM Control_Sk Process

# Update clause 8.1.14

### 8.1.14	Synthetic Loss measurement (SL) Processes

#### 8.1.14.1	Overview

Figure 8-xx shows the different processes inside MEPs and MIPs that are involved in the on-demand synthetic loss measurement protocol.

The MEP On-demand OAM insertion process is defined in clause 9.4.1.1, the MEP OAM on-demand extraction process in clause 9.4.1.2, the MIP OAM extraction process in clause 9.4.2.1, and the MIP OAM insertion process in clause 9.4.2.2. In summary, they insert and extract ETH_CI OAM signals into and from the stream of ETH_CI_D traffic units and the complementing P and D signals going through a MEP and MIP; the extraction is based on MEL and Opcode. Furthermore, the insertion process inserts the correct MEL and SA values into the OAM traffic units.

Figure 8-xx – Overview of processes involved with on-demand synthetic loss measurement protocol

The SL protocol is controlled by the on-demand SL Control process.

The On-demand SL Control process is activated upon receipt of the MI_SL_Start(DA,P,Test_ID,Length,Period) signal and remains activated until the MI_SL_Terminate signal is received. The measured synthetic loss values are output via the MI_SL_Result(N_TF,N_LF,F_TF,F_LF) signal when the process is terminated by the MI_SL_Terminate signal or when an intermediate result is requested via the MI_SL_Intermediate_Request signal.

The SLM generation process generates SLM traffic units that pass through MIPs transparently, but are received and processed by SLM reception processes in MEPs. The SLR generation process may generate an SLR traffic unit in response. This SLR traffic unit also passes transparently through MIPs, but is received and processed by SLR reception processes in MEPs.

----

#### 8.1.14.2	SL Control Process

The behaviour of the on-demand SL Control process is defined in Figure 8-xx+2. There are multiple instances of the on-demand SL Control process, each handling an independent stream of SLM frames.

Figure 8-xx+2 – On-demand SL Control behaviour

Upon receipt of the MI_SL_Start(DA,P,Test ID,Length,Period), the SL protocol is started. Every designated ‘period’ the generation of an SLM frame is triggered (using the SLM(DA,P,MEP_ID,Test_ID,TxFCl,TLV) signal), until the MI_SL_Terminate signal is received. The MEP_ID is the MI_MEP_ID of the MEP itself. The TLV field of the SLM frames is determined by the Generate(Length) function. Generate(Length) generates a Data TLV with length ‘Length’ of arbitrary bit pattern, as described in section 8.1.8.2. If the Length is 0, the TLV is set to NULL.

Upon receipt of an SLR traffic unit, the received counter values are used to count the near-end and far-end transmitted and lost synthetic frames. This result is reported using the MI_SL_Result(N_TF,N_LF,F_TF,F_LF) signal after the receipt of the MI_SL_Terminate signal or of the MI_SL_Intermediate_Request signal.

# Update clause 8.1.15

### 8.1.15	One Way Synthetic Loss measurement (1SL) Processes

#### 8.1.15.1	Overview

Figure 8-xx+10 shows the different processes inside MEPs and MIPs that are involved in the on-demand One Way Synthetic Loss measurement protocol.

The MEP On-demand OAM source insertion process is defined in clause 9.4.1.1, the MEP On-demand OAM sink extraction process in clause 9.4.1.2, the MIP On-demand OAM sink extraction process in clause 9.4.2.2. In summary, they insert and extract ETH_CI OAM signals into and from the stream of ETH_CI_D traffic units and the complementing P and DE signals going through a MEP and MIP; the extraction is based on MEL and Opcode. Furthermore, the insertion process inserts the correct MEL and SA values into the OAM traffic units.

Figure 8-xx+10 – Overview of processes involved with on-demand One Way Synthetic Loss measurement

The on-demand 1SL protocol is controlled by the on-demand 1SL Control_So and 1SL Control_Sk processes. The on-demand 1SL Control_So process triggers the generation of 1SL Traffic Units upon the receipt of an MI_1SL_Start(DA,P, Test ID,Length,Period) signal. The on-demand 1SL Control_Sk process processes the information from received 1SL Traffic Units after receiving the MI_1SL_Start(SA,Test ID) signal. The result is communicated by the sink MEP when the process is terminated by the MI_1SL_Terminate signal or when an intermediate result is requested via the MI_1SL_Intermediate_Request signal.

The 1SL generation process generates 1SL messages that pass transparently through MIPs and are received and processed by the 1SL Reception Process in MEPs.

----

#### 8.1.15.5	1SL Control_Sk Process

Figure 8-xx+17 shows the behaviour of the on-demand 1SL Control_Sk process. The MI_1SL_Start(SA,Test ID) signal starts the processing of 1SL messages coming from a MEP with SA as MAC address. The protocol runs until the receipt of the MI_1SL_Terminate signal.

While running the process processes the received 1SL(rSA, rMEP_ID, rTest_ID, TxTCf, RxTCl) information. First the rSA is compared with the SA from the MI_1SL_Start (SA,Test ID) signal. If the rSA is not equal to this SA, the information is ignored. Next the rTestID is compared with the TestID from the MI_1SL_Start (SA,Test_ID) signal. If the Test_ID signal is configured and rTest_ID is available but both values are different, the information is ignored. Otherwise the loss from the single received 1SL Traffic Unit is calculated. This result is reported using the MI_1SL_Result(N_TF, N_LF) signal after the receipt of the MI_1SL_Terminate signal or of the MI_1SL_Intermediate_Request signal.

Figure 8-xx+17 – On-demand 1SL Control_Sk Process

# Update clause 8.1.16

### 8.1.16	CSF Insert Process

Figure 8-zz – CSF Insert process

Figure 8-zz shows the CSF Insert Process Symbol and Figure 8-zz+1 defines the behaviour. If any of the aCSF-RDI, aCSF-FDI or aCSF-LOS signals are true, the CSF Insert process continuously generates ETH_CI traffic units where the ETH_CI_D signal contains the CSF signal until the aCSF  traffic unit. This continues until the condition no longer holds, ie all of aCSF-RDI, aCSF-FDI and aCSF-LOS are false. At this point, CSF traffic unit(s) with DCI (Defect Clear Information) are generated indicating that the defect has been cleared if MI_CSFdciEnable = True.

NOTE – Figure 8-zz+1 shows a case where a single CSF traffic unit with DCI is generated. However, the detail transmission condition (eg. transmission period, the number of traffic unit) is out of scope of this recommendation.

The generated CSF traffic units are inserted in the incoming stream, i.e., the output stream contains the incoming traffic units and the generated CSF traffic units.

Figure 8-zz+1 – CSF Insert behaviour

If exactly one of aCSF-RDI, aCSF-FDI and aCSF-LOS are set, the getCSFType() function returns RDI, FDI or LOS as appropriate.  The behaviour of getCSFType() when more than one of the conditions are set is for further study.

NOTE - As described in [ITU-T Y.1731], triggering CSF is client and application specific.  Ideally all clients and applications should ensure that at most one of the conditions is set at any given time.

# Update clauses 9.2 and 9.4

#### 9.2.1.1	ETHx Flow Termination source function (ETHx_FT_So)

Interfaces

Table 9-3 – ETHx_FT_So interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE ETH_RP: ETH_RI_CC_RxFCl ETH_RI_CC_TxFCf ETH_RI_CC_RDI ETH_RI_CC_Blk ETH_RI_LMM(OAM,P,DE) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) [1...MLM] ETH_RI_DMM(OAM,P,DE) ETH_RI_DMR(rSA,TxTimeStampf, RxTimeStampf,TxTimeStampb,RxTimeb, rTestID)[1...MDM] ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR(rMEP_ID,rTest_ID, TxFCf, TxFCb) [1...MSL] ETH_TP: ETHx_FT_So_TI_TimeStampl ETHx_FT_So_MP: ETHx_FT_So_MI_MEL ETHx_FT_So_MI_MEP_MAC ETHx_FT_So_MI_CC_Enable ETHx_FT_So_MI_LMC_Enable ETHx_FT_So_MI_MEG_ID ETHx_FT_So_MI_MEP_ID ETHx_FT_So_MI_CC_Period ETHx_FT_So_MI_CC_Pri ETHx_FT_So_MI_LML_Enable[1...MLM] ETHx_FT_So_MI_LM_MAC_DA[1...MLM] ETHx_FT_So_MI_LM_Period[1...MLM] ETHx_FT_So_MI_LM_Pri[1...MLM] ETHx_FT_So_MI_DM_Enable[1...MDM] ETHx_FT_So_MI_DM_MAC_DA[1...MDM] ETHx_FT_So_MI_DM_Test_ID[1...MDM] ETHx_FT_So_MI_DM_Length[1...MDM] ETHx_FT_So_MI_DM_Period[1...MDM] ETHx_FT_So_MI_DM_Pri[1...MDM] ETHx_FT_So_MI_1DM_Enable[1...M1DM] ETHx_FT_So_MI_1DM_MAC_DA[1...M1DM] ETHx_FT_So_MI_1DM_Test_ID[1...M1DM] ETHx_FT_So_MI_1DM_Length[1...M1DM] ETHx_FT_So_MI_1DM_Period[1...M1DM] ETHx_FT_So_MI_1DM_Pri[1...M1DM] ETHx_FT_So_MI_SL_Enable[1...MSL] ETHx_FT_So_MI_SL_MAC_DA[1...MSL] ETHx_FT_So_MI_SL_Test_ID[1...MSL] ETHx_FT_So_MI_SL_Length[1...MSL] ETHx_FT_So_MI_SL_Period[1...MSL] ETHx_FT_So_MI_SL_Pri[1...MSL] ETHx_FT_So_MI_1SL_Enable[1...M1SL] ETHx_FT_So_MI_1SL_MAC_DA[1...M1SL] ETHx_FT_So_MI_1SL_Test_ID[1...M1SL] ETHx_FT_So_MI_1SL_Length[1...M1SL] ETHx_FT_So_MI_1SL_Period[1...M1SL] ETHx_FT_So_MI_1SL_Pri[1...M1SL] | ETH_FP: ETH_CI_D ETH_CI_P ETH_CI_DE ETH_RP: ETH_RI_LM_Result(N_TF,N_LF,F_TF,F_LF) [1...MLM] ETH_RI_DM_Result(B_FD,F_FD,N_FD) [1...MDM] ETH_RI_SL_Result(N_TF,N_LF,F_TF,F_LF) [1...MSL] |

Processes

Figure 9-14 – ETHx_FT_So Process

#### 9.2.1.2	ETHx Flow Termination sink function (ETHx_FT_Sk)

Interfaces

Table 9-4 – ETHx_FT_Sk interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D ETH_CI_P ETH_CI_DE ETH_CI_SSF ETH_RP: ETH_RI_LM_Result( N_TF,N_LF,F_TF,F_LF) [1...MLM] ETH_RI_DM_Result( B_FD,F_FD,N_FD) [1...MDM] ETH_RI_SL_Result(N_TF,N_LF,F_TF,F_LF) [1...MSL] ETH_TP: ETHx_FT_Sk_TI_TimeStampl ETHx_FT_Sk_MP: ETHx_FT_Sk_MI_CC_Enable ETHx_FT_Sk_MI_LMC_Enable ETHx_FT_Sk_MI_1Second ETHx_FT_Sk_MI_LM_DEGM ETHx_FT_Sk_MI_LM_M ETHx_FT_Sk_MI_LM_DEGTHR ETHx_FT_Sk_MI_LM_TFMIN ETHx_FT_Sk_MI_MEL ETHx_FT_Sk_MI_MEG_ID ETHx_FT_Sk_MI_PeerMEP_ID[i] ETHx_FT_Sk_MI_CC_Period ETHx_FT_Sk_MI_CC_Pri ETHx_FT_Sk_MI_GetSvdCCM ETHx_FT_Sk_MI_1DM_Enable[1...M1DM] ETHx_FT_Sk_MI_1DM_MAC_SA[1...M1DM] ETHx_FT_Sk_MI_1DM_Test_ID[1...M1DM] ETHx_FT_Sk_MI_1SL_Enable[1...M1SL] ETHx_FT_Sk_MI_1SL_MAC_SA[1...M1SL] ETHx_FT_Sk_MI_1SL_Test_ID[1...M1SL] | ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE ETH_AI_TSF ETH_AI_TSD ETH_AI_AIS ETH_RP: ETH_RI_CC_RxFCl ETH_RI_CC_TxFCf ETH_RI_CC_RDI ETH_RI_CC_Blk ETH_RI_LMM(OAM,P,DE) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) [1...MLM] ETH_RI_DMM(OAM,P,DE) ETH_RI_DMR(rSA,TxTimeStampf, RxTimeStampf,TxTimeStampb,RxTimeb, rTestID) [1...MDM] ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR(rMEP_ID,rTest_ID,TxFCf, TxFCb) [1...MSL] ETHx_FT_Sk_MP: ETHx_FT_Sk_MI_cLOC[i] ETHx_FT_Sk_MI_cUNL ETHx_FT_Sk_MI_cMMG ETHx_FT_Sk_MI_cUNM ETHx_FT_Sk_MI_cDEG ETHx_FT_Sk_MI_cUNP ETHx_FT_Sk_MI_cUNPr ETHx_FT_Sk_MI_cRDI ETHx_FT_Sk_MI_cSSF ETHx_FT_Sk_MI_cLCK ETHx_FT_Sk_MI_pN_TF ETHx_FT_Sk_MI_pN_LF ETHx_FT_Sk_MI_pF_TF ETHx_FT_Sk_MI_pF_LF ETHx_FT_Sk_MI_pF_DS ETHx_FT_Sk_MI_pN_DS ETHx_FT_Sk_MI_pB_FD ETHx_FT_Sk_MI_pB_FDV ETHx_FT_Sk_MI_pF_FD ETHx_FT_Sk_MI_pF_FDV ETHx_FT_Sk_MI_pN_FD ETHx_FT_Sk_MI_pN_FDV ETHx_FT_Sk_MI_SvdCCM |

NOTE - If the delay measurement message rate is smaller than one second, there will be more than one set of primitive values (i.e. pB_FD, pB_FDV, pF_FD, pF_FDV, pN_FD, pN_FDV) for some 1-second period. If the delay measurement message rate is larger than one second, there will be no set of primitive values for some 1-second period.

Processes

Figure 9-17 – ETHx_FT_Sk Process

#### ETH Group Flow Termination source function (ETHG_FT_So)

Interfaces

Table 9-y – ETHG_FT_So interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_AP: ETH_AI_D[1...M] ETH_AI_P[1...M] ETH_AI_DE[1...M] ETH_RP: ETH_RI_CC_RxFCl ETH_RI_CC_TxFCf ETH_RI_CC_RDI ETH_RI_CC_Blk ETH_RI_LMM(OAM,P,DE) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) [1...MLM] ETH_RI_DMM(OAM,P,DE) ETH_RI_DMR(rSA,TxTimeStampf, RxTimeStampf,TxTimeStampb,RxTimeb, rTestID) [1...MDM] ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR(rMEP_ID,rTest_ID, TxFCf, TxFCb) [1...MSL] ETH_TP: ETHG_FT_So_TI_TimeStampl ETHG_FT_So_MP: ETHG_FT_So_MI_MEL ETHG_FT_So_MI_MEP_MAC ETHG_FT_So_MI_CC_Enable ETHG_FT_So_MI_LMC_Enable ETHG_FT_So_MI_MEG_ID ETHG_FT_So_MI_MEP_ID ETHG_FT_So_MI_CC_Period ETHG_FT_So_MI_CC_Pri ETHG_FT_So_MI_LML_Enable[1...MLM] ETHG_FT_So_MI_LM_MAC_DA[1...MLM] ETHG_FT_So_MI_LM_Period[1...MLM] ETHG_FT_So_MI_LM_Pri [1...MLM] ETHG_FT_So_MI_DM_Enable [1...MDM] ETHG_FT_So_MI_DM_MAC_DA [1...MDM] ETHG_FT_So_MI_DM_Test_ID [1...MDM] ETHG_FT_So_MI_DM_Length [1...MDM] ETHG_FT_So_MI_DM_Period [1...MDM] ETHG_FT_So_MI_DM_Pri [1...MDM] ETHG_FT_So_MI_1DM_Enable [1...M1DM] ETHG_FT_So_MI_1DM_MAC_DA [1...M1DM] ETHG_FT_So_MI_1DM_Test_ID [1...M1DM] ETHG_FT_So_MI_1DM_Length [1...M1DM] ETHG_FT_So_MI_1DM_Period [1...M1DM] ETHG_FT_So_MI_1DM_Pri [1...M1DM] ETHG_FT_So_MI_SL_Enable [1...MSL] ETHG_FT_So_MI_SL_MAC_DA [1...MSL] ETHG_FT_So_MI_SL_Test_ID [1...MSL] ETHG_FT_So_MI_SL_Length [1...MSL] ETHG_FT_So_MI_SL_Period [1...MSL] ETHG_FT_So_MI_SL_Pri [1...MSL] ETHG_FT_So_MI_1SL_Enable [1...M1SL] ETHG_FT_So_MI_1SL_MAC_DA [1...M1SL] ETHG_FT_So_MI_1SL_Test_ID [1...M1SL] ETHG_FT_So_MI_1SL_Length [1...M1SL] ETHG_FT_So_MI_1SL_Period [1...M1SL] ETHG_FT_So_MI_1SL_Pri [1...M1SL] | ETH_FP: ETH_CI_D[1...M] ETH_CI_P[1...M] ETH_CI_DE[1...M] ETH_RP: ETH_RI_LM_Result(N_TF,N_LF,F_TF,F_LF) [1...MLM] ETH_RI_DM_Result(B_FD,F_FD,N_FD) [1...MDM] ETH_RI_SL_Result(N_TF,N_LF,F_TF,F_LF) [1...MSL] |

Processes

Figure 9-x+1 – ETHG_FT_So Process

#### 9.2.2.2	ETH Group Flow Termination sink function (ETHG_FT_Sk)

Interfaces

Table 9-y+1 – ETHG_FT_Sk interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D[1...M] ETH_CI_P[1...M] ETH_CI_DE[1...M] ETH_CI_SSF ETH_RP: ETH_RI_LM_Result( N_TF,N_LF,F_TF,F_LF) [1...MLM] ETH_RI_DM_Result( B_FD,F_FD,N_FD) [1...MDM] ETH_RI_SL_Result( N_TF,N_LF,F_TF,F_LF) [1...MSL] ETH_TP: ETHG_FT_Sk_TI_TimeStampl ETHG_FT_Sk_MP: ETHG_FT_Sk_MI_CC_Enable ETHG_FT_Sk_MI_LMC_Enable ETHG_FT_Sk_MI_1Second ETHG_FT_Sk_MI_LM_DEGM ETHG_FT_Sk_MI_LM_M ETHG_FT_Sk_MI_LM_DEGTHR ETHG_FT_Sk_MI_LM_TFMIN ETHG_FT_Sk_MI_MEL ETHG_FT_Sk_MI_MEG_ID ETHG_FT_Sk_MI_PeerMEP_ID[i] ETHG_FT_Sk_MI_CC_Period ETHG_FT_Sk_MI_CC_Pri ETHG_FT_Sk_MI_GetSvdCCM ETHG_FT_Sk_MI_1DM_Enable [1...M1DM] ETHG_FT_Sk_MI_1DM_MAC_SA [1...M1DM] ETHG_FT_Sk_MI_1DM_Test_ID [1...M1DM] ETHG_FT_Sk_MI_1SL_Enable [1...M1SL] ETHG_FT_Sk_MI_1SL_MAC_SA [1...M1SL] ETHG_FT_Sk_MI_1SL_Test_ID [1...M1SL] | ETH_AP: ETH_AI_D[1...M] ETH_AI_P[1...M] ETH_AI_DE[1...M] ETH_AI_TSF ETH_AI_TSD ETH_AI_AIS ETH_RP: ETH_RI_CC_RxFCl ETH_RI_CC_TxFCf ETH_RI_CC_RDI ETH_RI_CC_Blk ETH_RI_LMM(OAM,P,DE) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) [1...MLM] ETH_RI_DMM(OAM,P,DE) ETH_RI_DMR(rSA,TxTimeStampf, RxTimeStampf,TxTimeStampb,RxTimeb, rTestID) [1...MDM] ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR(rMEP_ID,rTest_ID,TxFCf, TxFCb) [1...MSL] ETHG_FT_Sk_MP: ETHG_FT_Sk_MI_cLOC[i] ETHG_FT_Sk_MI_cUNL ETHG_FT_Sk_MI_cMMG ETHG_FT_Sk_MI_cUNM ETHG_FT_Sk_MI_cDEG ETHG_FT_Sk_MI_cUNP ETHG_FT_Sk_MI_cUNPr ETHG_FT_Sk_MI_cRDI ETHG_FT_Sk_MI_cSSF ETHG_FT_Sk_MI_cLCK ETHG_FT_Sk_MI_pN_TF ETHG_FT_Sk_MI_pN_LF ETHG_FT_Sk_MI_pF_TF ETHG_FT_Sk_MI_pF_LF ETHG_FT_Sk_MI_pF_DS ETHG_FT_Sk_MI_pN_DS ETHG_FT_Sk_MI_pB_FD ETHG_FT_Sk_MI_pB_FDV ETHG_FT_Sk_MI_pF_FD ETHG_FT_Sk_MI_pF_FDV ETHG_FT_Sk_MI_pN_FD ETHG_FT_Sk_MI_pN_FDV ETHG_FT_Sk_MI_SvdCCM |

NOTE - If the delay measurement message rate is smaller than one second, there will be more than one set of primitive values (i.e. pB_FD, pB_FDV, pF_FD, pF_FDV, pN_FD, pN_FDV) for some 1-second period. If the delay measurement message rate is larger than one second, there will be no set of primitive values for some 1-second period.

Processes

Figure 9-x+3 – ETHG_FT_Sk Process

#### 9.3.2.1	ETH to ETH adaptation source function (ETHx/ETH_A_So)

Interfaces

Table 9-5 – ETHx/ETH_A_So interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D ETH_CI_P ETH_CI_DE ETH_CI_APS ETH_CI_SSF ETH_CI_SSFrdi ETH_CI_SSFfdi ETHx/ETH_A_So_MP: ETHx/ETH_A_So_MI_Active ETHx/ETH_A_So_MI_MEP_MAC ETHx/ETH_A_So_MI_Client_MEL ETHx/ETH_A_So_MI_LCK_Period ETHx/ETH_A_So_MI_LCK_Pri ETHx/ETH_A_So_MI_Admin_State ETHx/ETH_A_So_MI_MEL ETHx/ETH_A_So_MI_APS_Pri ETHx/ETH_A_So_MI_CSF_Period ETHx/ETH_A_So_MI_CSF_Pri ETHx/ETH_A_So_MI_CSF_Enable ETHx/ETH_A_So_MI_CSFrdifdiEnable ETHx/ETH_A_So_MI_CSFdciEnable | ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE |

Processes

Figure 9-19 – ETHx/ETH_A_So process

#### 9.3.3.1	ETH to ETH multiplexing adaptation source function (ETHx/ETH-m_A_So)

Interfaces

Table 9-7 – ETHx/ETH-m_A_So interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D[1...M] ETH_CI_P[1...M] ETH_CI_DE[1...M] ETH_CI_SSF[1] ETH_CI_SSFrdi[1] ETH_CI_SSFfdi[1] ETH_TFP: ETH_CI_D ETH_CI_P ETH_CI_DE ETHx/ETH-m_A_So_MP: ETHx/ETH-m_A_So_MI_Active ETHx/ETH-m_A_So_MI_MEP_MAC ETHx/ETH-m_A_So_MI_Client_MEL[1…M] ETHx/ETH-m_A_So_MI_LCK_Period[1…M] ETHx/ETH-m_A_So_MI_LCK_Pri[1…M] ETHx/ETH-m_A_So_MI_Admin_State ETHx/ETH-m_A_So_MI_VLAN_Config[1...M] ETHx/ETH-m_A_So_MI_Etype ETHx/ETH-m_A_So_MI_PCP_Config ETHx/ETH-m_A_So_MI_MEL ETHx/ETH-m_A_So_MI_CSF_Period ETHx/ETH-m_A_So_MI_CSF_Pri ETHx/ETH-m_A_So_MI_CSF_Enable ETHx/ETH-m_A_So_MI_CSFrdifdiEnable ETHx/ETH-m_A_So_MI_CSFdciEnable | ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE ETHF_PP: ETH_PI_D ETH_PI_P ETH_PI_DE ETHTF_PP: ETH_PI_D ETH_PI_P ETH_PI_DE |

Processes

Figure 9-25 – ETHx/ETH-m_A_So process

#### 9.3.4.1	ETH Group to ETH adaptation source function (ETHG/ETH_A_So)

Interfaces

Table 9-9 – ETHG/ETH_A_So Interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D[1…M] ETH_CI_P[1…M] ETH_CI_DE[1…M] ETH_CI_APS ETH_CI_SSF[1] ETH_CI_SSFrdi[1] ETH_CI_SSFfdi[1] ETHG/ETH_A_So_MP: ETHG/ETH_A_So_MI_Active ETHG/ETH_A_So_MI_MEP_MAC ETHG/ETH_A_So_MI_Client_MEL[1..M] ETHG/ETH_A_So_MI_LCK_Period[1…M] ETHG/ETH_A_So_MI_LCK_Pri[1…M] ETHG/ETH_A_So_MI_Admin_State ETHG/ETH_A_So_MI_MEL ETHG/ETH_A_So_MI_APS_Pri ETHG/ETH_A_So_MI_CSF_Period ETHG/ETH_A_So_MI_CSF_Pri ETHG/ETH_A_So_MI_CSF_Enable ETHG/ETH_A_So_MI_CSFrdifdiEnable ETHG/ETH_A_So_MI_CSFdciEnable | ETH_AP: ETH_AI_D[1...M] ETH_AI_P[1...M] ETH_AI_DE[1...M] |

Processes

Figure 9-29 – ETHG/ETH_A_So process

#### 9.4.1.1	ETH Diagnostic Flow Termination Source Function for MEPs (ETHDe_FT_So)

Interfaces

Table 9-11 – ETHDe_FT_So interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE ETH_RP: ETH_RI_LMM(D,P,DE) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) ETH_RI_LBM(D,P,DE) ETH_RI_LBR(SA,rTLV,TID) ETH_RI_DMM(D,P,DE) ETH_RI_DMR(rSA,TxTimeStampf,RxTimeStampf, TxTimeStampb,RxTimeb,rTestID) ETH_RI_LTM(D,P,DE) ETH_RI_LTR(SA,TTL,TID,TLV) ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR(rMEP_ID,rTest_ID,TxFCf,TxFCb) ETH_TP: ETHDe_FT_So_TI_TimeStampl ETHDe_FT_So_MP: ETHDe_FT_So_MI_LM_Start(DA,P,Period) ETHDe_FT_So_MI_LM_Intermediate_Request ETHDe_FT_So_MI_LM_Terminate ETHDe_FT_So_MI_LB_Discover( P) ETHDe_FT_So_MI_LB_Series(DA,DE,P,N, Length, Period) ETHDe_FT_So_MI_LB_Test (DA,DE,P,Pattern, Length, Period) ETHDe_FT_So_MI_LB_Test_Terminate ETHDe_FT_So_MI_DM_Start(DA,P,Test ID,Length,Period) ETHDe_FT_So_MI_DM_Intermediate_Request ETHDe_FT_So_MI_DM_Terminate ETHDe_FT_So_MI_1DM_Start(DA,P,Test ID,Length,Period) ETHDe_FT_So_MI_1DM_Terminate ETHDe_FT_So_MI_TST(DA,DE,P,Pattern, Length, Period) ETHDe_FT_So_MI_TST_Terminate ETHDe_FT_So_MI_LT(TA,TTL.P) ETHDe_FT_So_MI_MEP_MAC ETHDe_FT_So_MI_MEL ETHDe_FT_So_MI_MEP_ID ETHDe_FT_So_MI_SL_Start(DA,P,Test_ID,Length,Period) ETHDe_FT_So_MI_SL_Intermediate_Request ETHDe_FT_So_MI_SL_Terminate ETHDe_FT_So_MI_1SL_Start( DA,P,Test_ID,Length,Period) ETHDe_FT_So_MI_1SL_Terminate | ETH_FP: ETH_CI_D ETH_CI_P ETH_CI_DE ETHDe_FT_So_MP: ETHDe_FT_So_MI_LM_Result( N_TF, N_LF, F_TF, F_LF) ETHDe_FT_So_MI_LB_Discover_Result(MACs) ETHDe_FT_So_MI_LB_Series_Result(REC,ERR,OO) ETHDe_FT_So_MI_LB_Test_Result (Sent, REC, CRC, BER, OO) ETHDe_FT_So_MI_DM_Result(count,B_FD[],F_FD[],N_FD[]) ETHDe_FT_So_MI_TST_Result(Sent) ETHDe_FT_So_MI_LT_Results(Results) ETHDe_FT_So_MI_SL_Result(N_TF,N_LF,F_TF,F_LF) |

Processes

Figure 9-33 – ETHDe_FT_So Process

#### 9.4.1.2	ETH Diagnostic Flow Termination Sink Function for MEPs (ETHDe_FT_Sk)

Interfaces

Table 9-12 – ETHDe_FT_Sk interfaces

| Inputs | Outputs |
| --- | --- |
| ETH_FP: ETH_CI_D ETH_CI_P ETH_CI_DE ETHDe_FT_Sk_MP: ETHDe_FT_Sk_MI_MEL ETHDe_FT_Sk_MI_MEP_MAC ETHDe_FT_Sk_MI_1DM_Start(SA,Test ID) ETHDe_FT_Sk_MI_1DM_Intermediate_Request ETHDe_FT_Sk_MI_1DM_Terminate ETHDe_FT_Sk_MI_TST_Start(SA,Pattern) ETHDe_FT_Sk_MI_1SL_Intermediate_Request ETHDe_FT_Sk_MI_TST_Terminate ETHDe_FT_Sk_MI_1SL_Start( SA,MEP ID, Test ID) ETHDe_FT_Sk_MI_1SL_Terminate ETH_TP: ETHDe_FT_Sk_TI_TimeStampl | ETH_AP: ETH_AI_D ETH_AI_P ETH_AI_DE ETH _RP: ETH_RI_LMM(D,P,DE) ETH_RI_LMR(TxFCf,RxFCb,TxFCb,RxFCl) ETH_RI_LMR(rSA,TxFCf,RxFCf,TxFCb,RxFCl) ETH_RI_LBM(D,P,DE) ETH_RI_LBR(SA,rTLV,TID) ETH_RI_DMM(D,P,DE) ETH_RI_DMR( rSA,TxTimestampf,RxTimeStampf, TxTimeStampb,RxTimeb,rTest ID) ETH_RI_LTM(D,P,DE) ETH_RI_LTR(SA,TTL,TID,TLV) ETH_RI_SLM(OAM,P,DE,TxFCb) ETH_RI_SLR( rMEP_ID,rTest_ID,TxFCf,TxFCb) ETHDe_FT_Sk_MP: ETHDe_FT_Sk_MI_1DM_Result( count,N_FD[]) ETHDe_FT_Sk_MI_TST_Result( REC,CRC,BER,OO) ETHDe_FT_Sk_MI_1SL_Result(N_TF,N_LF) |

Processes

Figure 9-35 – ETHDe_FT_Sk processes

____________________