| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 619 R1 (PLEN) |
|  |  | English only Original: English |
| Question(s): | 14/15 | Geneva, 19-30 September 2016 |
| TD |  |  |
| Source: | Editor G.8052/Y.1346 |  |
| Title: | Draft revised Recommendation ITU-T G.8052/Y.1346 (v1.11) for consent |  |

This document is the editor version of G.8052/Y.1346 “Protocol-neutral management information model for the Ethernet transport capable network element” v1.11.

# Document history:

| Version | Document | Description |
| --- | --- | --- |
| 1.01 | WD18 (9/2013) | - Initial version to G.8052 (7/2013) |
| 1.02 | WD26 (10/2013) Nuremberg | - Update the model as decided per the WD12 (10/2013) discussion and as shown in WD12r1 - Replace the inserted HTML file and Data Dictionary with hyperlinks. |
| 1.03 | WD11 (9/2014) Shanghai | - Align with editorial updates done by the TSB editor on the published version of G.8052 (8/2013) - Add additional abbreviations - In the description of ETH_CrossConnection object, delete the editor note. - In the description of ETH_PortGroup, change “See G.8021 Amd2 section 9.1.1” to “See G.8021 section 9.1.1”. - In the description of the onDemandPerformanceData attribute of the OnDemand1WayMeasurementJobControlSink object class, change OS to management system. - In the description of the III.3.43.1.2 portRefList attribute of the SetCrossConnectionData data type, remove the double question marks. - Regenerate the data dictionary, include back into the main document |
| 1.04 | TD246/3 (11/2014) Geneva | - Add JPEG images of the respective UML diagrams next to the figures for the purpose easier viewing, i.e. the images/diagrams can be zoomed in/out independent of the document text. - Note that in the 9/2014 Shanghai report, it was said that the editor version of G.8052 v1.04 will be produced in WD11r1 to align with G.8051 v3.04. It has been verified that the only update in G.8051 v3.04 is to add the missing MI signal ETHDe_FT_Sk_MI_1SL_Result (N_TF,N_LF) to Table 8-5/G.8051. This MI has been already modelled in G.8052 (see the “OnDemand1SLPerformanceData” data type in Figure 7-8/G.8052) and thus has not impact on the existing information model in G.8052 v1.03. - Update the “Ethernet Adaptation Modelling Class Diagram” in v1.03 RSA file to - Re-add the missing CsfRdiFdiEnableSource_Pac as shown in current Figure 6-12 - add «M.3160» TrailTerminationPointSource as shown in current Figure 6-12 - add «M.3160» TrailTerminationPointSink as shown in current Figure 6-12 - Add a new subclause 6.4.4 “Ethernet Adaptation Modelling”, which summarize the previous 3 subclause. This 6.4.4 only contains Fig.6-12 with new caption “Ethernet Adaptation Modelling” - There is a UML diagram “Threshold Profile Control” in the HTML but not used in the document. Figure added. Need surrounding text to describe the usage of the control. |
| 1.05 | TD246R1/3 (12/2014) Geneva | Updates made: - Modeled ETH-ED in Tables 9-y & 9-y+1/G.8021: new classes MccCtp (MEP_MAC, MEL), MccCtpSource (Mcc_Pri, MEP_Id, EDM_Enable, EDM_Period, EDM_Duration), MccCtpSink (EDM_Received: Read only), MccCtpBidirectional - Modeled ETH-BN in Tables 9-3 & 9-5/G.8021: MepSink has new attribute bandwidthReport (SA, PortId, nominalBw, currentBw) - Added Priority to OnDemand1wayMeasurementJobControlSink for modelling ETHDe_FT_Sk_MI_1DM_Start(SA,Test ID,P), per C1056, and also updated Figure 7-8 On-Demand Measurement - Provide the links for the G.8052 v1.05 model RSA file, Papyrus file, HTML file, HTML file, and Data Dictionary To discuss: Done on 2015.07.02 - To discuss the requirement and model for archiving the received EDM at the sink side - Contribution asked - Why OnDemand1wayMeasurementJobControlSink is not subclass from OnDemanMeasurementJobControl - 2015.07.02 SG15 meeting: Verified. The answer is that OnDemand1wayMeasurementJobControlSource and OnDemand2WayMeasurementJobControl have more common attributes than OnDemand1wayMeasurementJobControlSink. So define OnDemand1wayMeasurementJobControlSource and OnDemand2WayMeasurementJobControl as subclass of the abstract superclass OnDemanMeasurementJobControl and leave OnDemand1wayMeasurementJobControlSink by itself. |
| 1.06 | WD18 (3/2015) Ottawa | Updates made: - Replaced the model files (RSA), HTML files, and DD files with the ones using the Open Model Profile in Clause 8 and Appendix III. Note: The Papyrus model file not replaced yet |
| 1.07 | WD18r1 (3/3015) Ottawa TD372/WP3 Geneva (7/2015) | Updates made: - Update Table 6-1 per Table 6-1 in WD10r1 To do - Update Figure 6-1 per Table 1 of WD10r1 to align with Figure 1-1/G.8021 - To update the model for the new MIs and changes of MIs - Done: Because of the discussion of WD07r4, need to consider 1way/2wayMeasurementJobControlSource/Sink refactoring for single/dual-endedControl for 1way/2way measurement - See v1.08 below |
| 1.08 | TD372r1/3 | Modeled the following missing MIs - ETHx_FT_Sk_MI_1DM_Pri[1…M1DM] - Added multi-valued (1..*) attribute 1DmPriority to MepSink - ETHG_FT_Sk_MI_1DM_Pri [1...M1DM] - Added multi-valued (1..*) attribute 1DmPriority to MepSink - ETHx_FT_Sk_MI_MEP_MAC - No need to add because it is inherited from the Mep supercalss - ETHG_FT_Sk_MI_MEP_MAC - No need to add because it is inherited from the Mep supercalss Modeled the missing parameter P in - ETHDe_FT_Sk_MI_1DM_Start(SA, P, Test_ID) - Added input parameter priority to the operation establishOnDemandDualEndedMeasurementJobControlSink Checked that the following two MIs are already modelled. Yes, modelled in the MepSink object class - ETHx_FT_Sk_MI_BW_Report(SA, PortID, NominalBW, CurrentBW) - ETHG_FT_Sk_MI_BW_Report(SA, PortID, NominalBW, CurrentBW) Re-factored 1way/2wayMeasurementJobControlSource/Sink to better indicate single/dual-ended and 1Way/2Way measurement: - MO: OnDemand[1WayDualEnded]MeasurementJobControlSource/Sink - MO: OnDemand[2WaySingleEnded]MeasurementJobControlSource/Sink - Operation: establishOnDemand/Proactive[1WayDualEnded]MeasurementJobSource/Sink - Operation: establishOnDemand/Proactive[2WaySingleEnded]MeasurementJobSource/Sink - Enumeration: 1WayOamType:1DM, 1SL, LM_CCM (i.e., add LM_CCM) - Enumeration: 2WayOamType:LM_LMM, DM, SL (i.e., remove LM_CCM) - MO: ProActive[1WayDualEnded]MeasurementJobControlSource/Sink - MO: ProActive [2WaySingleEnded]MeasurementJobControlSource/Sink - And updated the corresponding description To do: - Update Figure 6-1 per Table 1 of WD10r1 to align with Figure 1-1/G.8021 - Regenerate Figures 7-8 (on-demand) and 7-13 (Pro Active) from HTML (because of the re-factoring of the measurement job control) |
| 1.09 | WD40 (10/2015) Wuhan = TD512/3 (2/2016) | Updates - Migrate the model profile from Luna to the Mars version of Eclipse - Update object class description for grouping attributes according to the transport processes - Note: The model file v1.09 = onf2015.422_G.8052_v1.08.01_SS.00 To-do - To generate the GenDoc Data Dictionary and replace the RSA Data Dictionary - To drop the RSA HTML - To update the model per the discussion of WD06 for single/dual-ended measurement job control - To model the new MI for BNM Reception process - Old to-do lists |
| 1.10 | WD16 (2/2016) = TD512R1/3 (2/2016) | Updates - Reference G.8001 for the definitions of Traffic conditioning function and Traffic shaping function - Move to Papyrus 1.1.3 and based on OpenModelProfile 0.2.0 - Imported the core model from G.7711 v1.02 into G.8052 v1.10 - Normalized SN, SNC, PG to FD, FC, FcSwitch respectively. In particular the following object classes - ETH_CrossConnection, ETH_SubNetworkConnectionProtectionGroup - Generate the GenDoc Data Dictionary and replace the RSA Data Dictionary - Dropped the RSA HTML - Replaced the figures in clauses 6 and7 for normalizing SN, SNC, PG to FD, FC, and FcSwitch - Replaced the content of Appendix I (UML Modeling Guidelines) with pointer to Annex A of G.7711 To-do - Update Figures 5-8 & 5-9 from <<ituClass>> to <<openModelClass>> - Update Figure 6-1 (see Editor Note for instruction) Done in v1.10.01 - To update the model per the discussion of WD06 (10/2015) for single/dual-ended measurement job control - To model the new MI for BNM Reception process - Old to-do lists |
| 1.11 | WD16 (9/2016) = TD619R1/P (9/2016) | Updates - Accept the track changes in v1.10 - Items 2 through 46 in C1970 - Item 1 in C1970 - In Figure 7-30 regading NE1 rplNeighbourNode, correct typo Port 41 to Port 12 - In Figure 6-1, removed ODUkP-X-L_AP per C2009 |

# Contents

1.	Scope	6

2.	References	8

3.	Definitions	10

3.1.	Terms defined elsewhere:	10

3.2.	Terms defined in this Recommendation	11

4.	Abbreviations and acronyms	11

5.	Conventions	14

5.1.	Conventions defined elsewhere:	14

5.2.	Conventions defined in this Recommendation	16

6.	Overview of the Model	18

6.1.	ETH Sub-Layer	19

6.2.	Basic Configuration Structure	22

6.3.	Location (Topology) of OAM Functions	25

6.4.	MI Groupings and Mapping	27

7.	Modelling of Ethernet Functions	42

7.1.	OAM Compound Functions	42

7.2.	TCS Function	59

7.3.	Performance Monitoring	60

7.4.	Ethernet Multiplexing	62

7.5.	Connection Function	63

7.6.	LAG Management Modelling	70

7.7.	PhysicalSubsystem Modelling	72

8.	UML Model file	73

Appendix I UML Modelling Guidelines	75

Appendix II Mapping of G.8021 Atomic Functions to G.8052 Model Artefacts	92

Appendix III UML Model Data Dictionary	93

# List of figures

Figure 1-1 – Scope of G.8052 Interface	21

Figure 2-1 – Structure of References	24

Figure 5-1 – Figure 9-63/G.8021 – ETH NCM MEP compound functions	27

Figure 5-2 – NCM MEP at the boundary of a layer network	27

Figure 5-3 – Figure 9-64/G.8021 – ETH TCM MEP compound functions	28

Figure 5-4 – Figure 9-65/G.8021 – ETH MIP compound functions	28

Figure 5-5 – Figure 9-66/G.8021 – ETH MIP compound functions	29

Figure 5-6 – Figure 5-3B/G.806 Amd1 – Diagrammatic convention for Traffic Conditioning & Shaping function	29

Figure 5-7 – MEPs and MIPs along a Maintenance Entity (ME)	29

Figure 5-8: Illustrative Diagram for Ethernet server Adaptation modelling	30

Figure 5-9: Illustrative Diagram for Non-Ethernet server Adaptation modelling	31

Figure 6-1 – Over view of Object Class mapping to G.8021 Atomic Functions	31

Figure 6-2 – ETH sublayering amended with G.8052 object classes	33

Figure 6-3: ETH Sublayer Object Instance Diagram	34

Figure 6-4 – Basic Configuration Structure Object Diagram	35

Figure 6-5 – MEP infrastructure modelling object diagram	36

Figure 6-6 – CTP Inheritance Class Diagram	36

Figure 6-7 – TTP Inheritance Class Diagram	37

Figure 6-8 – ETH CTP Pool Class Diagram	38

Figure 6-9 – Illustration of positionSequence and codirectional attributes	39

Figure 6-10 – Modelling of an IrDI Example	39

Figure 6-11 – Modelling of an IaDI Example	40

Figure 6-12: Modelling of Non-Ethernet server Adaptation MI groupings	54

Figure 7-1 – Figure II.1/Y.1731 – Example MEG level assignment for shared MEG levels	55

Figure 7-2 – Mandatory and optional MEPs	56

Figure 7-3 – MEP Class Diagram	57

Figure 7-4 – MEP on-demand diagnostic function	58

Figure 7-5 – 2-way On-demand Measurement	59

Figure 7-6 – 1-way On-demand Measurement Control Source	60

Figure 7-7 – 1-way On-demand Measurement Control Sink	60

Figure 7-8 – On-demand Measurement Class Diagram	61

Figure 7-9 – MEP pro-active measurement function	62

Figure 7-10 – 2-way Pro-active Measurement	64

Figure 7-11 – 1-way Pro-active Measurement Control Source	65

Figure 7-12 – 1-way Pro-active Measurement Control Sink	66

Figure 7-13 – Pro-active Measurement	67

Figure 7-14 – MEP configuration function	68

Figure 7-15 – MEP configuration	69

Figure 7-16 – MIP configuration parameters	70

Figure 7-17 – "half MIP" compound function	71

Figure 7-18 – MIP/half MIP configuration function	71

Figure 7-19 – MIP class diagram	72

Figure 7-20 – TCS/GTCS configuration function	72

Figure 7-21 – TCS Containment Class Diagram	73

Figure 7-22 – Frame Loss Measurement mechanisms	74

Figure 7-23 – Frame Delay Measurement mechanisms	75

Figure 7-24 – Ethernet multiplexing configuration function	76

Figure 7-25 – Ethernet multiplexing configuration	76

Figure 7-26 – Connection Fragment Class Diagram	77

Figure 7-27 – Linear Protection Class Diagram	79

Figure 7-28 – Generic Model of Ethernet Ring Protection Function	80

Figure 7-29 – Ring Protection Class Diagram	81

Figure 7-30 – Ring Protection: RPL parameters	82

Figure 7-31 – R-APS Control Process Type	83

Figure 7-32 –Generic Model of Ethernet Link Aggregation	84

Figure 7-33 – LAG Management Class Diagram	85

Figure 7-34 – PhysicalSubsystem modelling overview	86

Figure I.2-1 – Profiling Object Classes Using Conditional Inheritance	90

Figure I.2-2 – Profiling Object Classes Using Conditional Packages	90

Figure I.2-3 – Profiling Object Classes Using (Conditional) Data Types	91

Figure I.2-4 – XOR Modelling	92

Figure I.2-5 – Choice Modelling	92

Figure I.3-1: Additional object class properties	94

Figure I.3-2: Additional attribute properties	96

Figure I.3-3: Additional association property	97

Figure I.3-4: Generic exceptions	98

Figure I.3-5: Common operations exceptions	99

Figure I.3-6: Additional operation properties	100

Figure I.3-7: Additional parameter properties	101

Figure I.3-8: Basic data types	101

Figure I.3-9: Additional data type properties	103

Figure I.3-10: Conditional Package	103

Figure I.3-11: ITU-T Protocol Neutral Model Profile	105

# List of tables

Table 5-1 – Colour Code convention	29

Table 6-1 – MI Groupings of the Ethernet MAC layer (ETH) functions	40

Table 6-2 – Derived MI Groupings of the Ethernet MAC layer (ETH) functions	46

Table 6-3 – MI Groupings of the Ethernet PHY Layer (ETYn) functions	47

Table 6-4 – MI Groupings of the Non-Ethernet server to ETH adaptation functions	48

Table 6-5 – Derived MI Groupings of the Non-Ethernet server Adaptation functions	52

Table 7-1 – On-demand measurement and Maintenance MI list	57

Table 7-2 – Pro-active measurement MI list	61

Table 7-3 – MEP configuration MI list	67

Table 7-4 – MIP configuration MI list	70

Table 7-5 – TCS configuration MI list	71

Table 7-6 – Mapping of Multiplexing related MI to G.8052 artefacts	75

Table 7-7 – Mapping of Connection Function related MI to G.8052 artefacts	76

Table I.1-1 – UML artefact naming conventions	87

Table I.3-1 – Additional UML artefact properties defined in complex stereotypes	103

Table II-1 –Mapping between G.8021 Ethernet Atomic Functions and UML Model Artifacts	105

# Draft revised Recommendation G.8052/Y.1346

Protocol-neutral management information model for the Ethernet transport capable network element

# Summary

Recommendation G.8052/Y.1346 contains the protocol neutral UML information model for Ethernet Transport NE Management. The model is based on the Ethernet equipment functions specified in Recommendation ITU-T G.8021/Y.1341, generic management requirements in Recommendation ITU-T G.7710/Y.1701, and Ethernet specific management requirements in Recommendation ITU-T G.8051/Y.1345.

The 2016 revision of this Recommendation has change the UML modeling tool from RSA to open source Papyrus tool, update the G.8052 information model to align with the G.7711 v2.0 Core information model, drop subclassing the TP classes from M.3160, and support the additional management requirements in Recommendation ITU-T G.8051.

# Keywords

Information Model, Protocol-neutral, Carrier Ethernet, Transport Resource, UML

# Introduction

This Recommendation contains the object classes for the Ethernet Transport NE management. It includes the Termination Points (TP), Maintenance Entity Group (MEG) End Point (MEP), MEG Intermediate Point (MIP), Traffic Conditioning & Shaping (TCS), Loss Measurement (LM), Delay Measurement (DM), and the general Performance Monitoring (PM) Current Data (CD) and History Data (HD).

The TP, MEP, MIP, LM, DM, and TCS object classes support the configuration and fault management functions as specified in Recommendation [ITU-T G.8051].

The Ethernet TPs are modeled as subclasses of the generic GlobalClass defined in Recommendation [ITU-T G.7711] and for extending LTP and LP classes of [ITU-T G.7711].

The Ethernet general PM CD and HD object classes are modeled as subclasses of the generic Current Data and History Data defined in Recommendation [ITU-T Q.822].

The Ethernet general CD and HD object classes support only the Quality of Service (QoS) directly related PM parameters, i.e. Severely Errored Second (SES) and Unavailable Second (UAS), for Service Level Agreement (SLA) verification. The additional PM object classes for supporting Loss Measurement (LM) and Delay Measurement (DM) monitoring uses the general CD and HD object classes as super classes.

The object model defined in this Recommendation is protocol-neutral with respect to management protocols. The model could be used as the base for further defining the information model for any specific management protocol.

The model in this Recommendation has been specified using the Open Source UML modelling tool “Papyrus”.

# Draft revised Recommendation G.8052/Y.1346

Protocol-neutral management information model for the Ethernet transport capable network element

# Scope

This Recommendation provides a management-protocol-neutral information model for managing network elements in the Ethernet transport network as defined in Recommendation [ITU-T G.8010]. It identifies the telecommunications management network (TMN) managed entities required for the management of Ethernet transport network elements. These entities are relevant to information exchanged across standardized interfaces defined in the [ITU-T M.3010] TMN architecture. The management-protocol-neutral information model should be used as the base for defining management-protocol-specific information models, for example, XML (Web Service or Netconf/Yang) information model, CORBA IDL model, and SNMP MIB.

The information model defined in this Recommendation is an augmentation to the generic code model specified in Recommendation [ITU-T G.7711] for managing Ethernet transport resources. The core information model defined in [ITU-T G.7711] can be used as the base for the extension of Ethernet-specific information models.

The specific mapping of the management-protocol-neutral model into management-protocol-specific model is the decision of the management-protocol-specific solution design. For example, an object class defined in this Recommendation may be mapped into multiple tables in a SNMP MIB. Protocol-specific solutions and their mapping from the protocol-neutral model will be described in other Recommendations and is out of scope of this Recommendation.

This Recommendation applies to Ethernet transport network elements and those systems in the TMN that manage such network elements. Functional capabilities of Ethernet transport equipment are defined in Recommendation [ITU-T G.8021], and requirements of the management of Ethernet transport equipment are provided in Recommendation [ITU-T G.7710/Y.1701] and Recommendation [ITU-T G.8051]. The information model specified in this Recommendation applies to the EMS-NE interface, as shown in Figure 1-1, specifically for managing the Ethernet functional capabilities of the NE.

Figure 1-1 – Scope of G.8052 Interface

There are several different perspectives from which management information may be defined for management purposes. The network element viewpoint is concerned with the information that is required to manage a network element. This refers to information required to manage the network element function and the physical aspects of the network element. This Recommendation addresses only the network element view of Ethernet transport network management.

The management-protocol-neutral information model specified in this Recommendation consists of a set of transport-technology-specific managed object classes, i.e., Ethernet-specific managed object classes. These Ethernet-specific managed object classes are inherited from the generic managed object classes defined in other ITU-T Recommendation such as M.3160, including Managed Element, Termination Point and its subclasses, Subnetwork, and Subnetwork Connection. Because of object class inheritance, the Ethernet management information model also inherits the generic object management capabilities, such as object creation/deletion, notification of object creation/deletion, attribute value retrieval/modification, notification of attribute/state value change, scoped and filtered retrieval of object instances, and abortion of outstanding operations. The description of these generic object management capabilities is provided in other ITU-T Recommendations, such as the M.3700 series, and therefore is outside the scope of this Recommendation.

The object classes defined in this Recommendation cover the areas of fault management, configuration management, and performance management.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T G.7710/Y.1701]	ITU-T Recommendation G.7710/Y.1701 (10/2012), Common equipment management function requirements, plus Amendment 1 (9/2016)

[ITU-T G.7711]	ITU-T Recommendation G.7711/Y.1702 (9/2016), Generic protocol-neutral information model for transport resources

[ITU-T G.8001]	ITU-T Recommendation G.8001 (4/2016), Terms and definitions for Ethernet frames over transport

[ITU-T G.8010]	ITU-T Recommendation G.8010 (02/2004), Architecture of Ethernet layer networks, plus Amendment 1 (5/2006) and Amendment 2 (7/2010)

[ITU-T G.806]	ITU-T Recommendation G.806 (02/2012; Characteristics of Transport Equipment – Description Methodology and Generic Functionality

[ITU-T G.8013]	ITU-T Recommendation G.8013/Y.1731 (8/2015), OAM functions and mechanisms for Ethernet based networks

[ITU-T G.8021]	ITU-T Recommendation G.8021/Y.1341 (4/2015), Characteristics of Ethernet transport network equipment functional blocks

[ITU-T G.8021.1]	ITU-T Recommendation G.8021.1/Y.1341.1 (10/2012), Types and characteristics of Ethernet transport network equipment

[ITU-T G.8031]	ITU-T Recommendation G.8031 (1/2015), Ethernet Linear Protection Switching

[ITU-T G.8032]	ITU-T Recommendation G.8032 (8/2015), Ethernet Ring Protection Switching

[ITU-T G.8051]	ITU-T Recommendation G.8051/Y.1345 (8/2015), Management aspects of the Ethernet Transport network element

[ITU-T M.3100]	ITU-T Recommendation M.3100 (04/2005), Generic network information model

[ITU-T M.3160]	ITU-T Recommendation M.3160 (11/2008), Generic Management Information Model – Protocol Neutral, plus Amendment 1 (3/2016)

[ITU-T Q.822]	ITU-T Recommendation Q.822 (04/1994), Stage 1, Stage 2 and Stage 3 Description for the Q3 interface – Performance Management

[IEEE 802.1Q]	IEEE Standard for Local and metropolitan area networks — Media Access Control (MAC) Bridges and Virtual Bridge Local Area Networks (2011), Connectivity Fault Management

[IEEE 802.1AX]	IEEE Standard for Local and metropolitan area networks — Link Aggregation (2008)

[IEEE 802.3]	IEEE 802.3 (2008), IEEE Standard for Information technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements Part 3: Carrier sense multiple access with Collision Detection (CSMA/CD) Access Method and Physical Layer Specifications.

Figure 2-1 – Structure of References

# Definitions

## Terms defined elsewhere:

This Recommendation uses the following terms defined elsewhere:

3.1.1	Dual-Ended [ITU-T G.8001]

3.1.2	maintenance entity (ME) [ITU-T G.8001]

3.1.3	maintenance entity group (MEG) [ITU-T G.8001]

3.1.4	MEG end point (MEP) [ITU-T G.8001]

3.1.5	MEG intermediate point (MIP) [ITU-T G.8001]

3.1.6	On-demand Measurement [ITU-T G.8001]

3.1.7	on-demand monitoring [ITU-T G.8001]

3.1.8	One-way [ITU-T G.8001]

3.1.9	proactive monitoring [ITU-T G.8001]

3.1.10	Pro-active Measurement [ITU-T G.8001]

3.1.11	Ring Protection Link (RPL) [ITU-T G.8001]

3.1.12	Single-Ended [ITU-T G.8001]

3.1.13	traffic conditioning function [ITU-T G.8001]

3.1.14	traffic shaping function [ITU-T G.8001]

3.1.15	Two-way [ITU-T G.8001]

## Terms defined in this Recommendation

This Recommendation defines no additional terms.

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

1DM	Delay Measurement 1-way (using 1DM)

1LM	Loss Measurement 1-way (using 1SL, CCM)

1SL	1-way Synthetic Loss measurement

AIS	Alarm Indication Signal

APS	Automatic Protection Switching

ASN.1	Abstract Syntax Notation One

CBS	Committed Burst Size

CC	Cross Connect

CCM	Continuity Check Message

CD	Current Data

CF	Coupling Flag

CIR	Committed Information Rate

CM	Colour Mode

CMISE	Common Management Information Service Element

CORBA	Common Object Request Broker Architecture

CRC	Cyclic Redundancy Check

CSF	Client Signal Fail

CTP	Connection Termination Point

DEG	Degraded

DM	Delay Measurement 2-way (using DMM/DMR)

DMo	On-Demand Delay Measurement

EBS	Excess Burst Size

EIR	Excess Information Rate

EMS	Element Management System

ETH	Ethernet layer

ETY	Ethernet physical layer

EUI	Extended Unique Identifier

F-Flr	Far-end - Frame loss ratio

FD	Frame Delay

FDV	Frame Delay Variation

FFS	For Further Study

FP	Flow Point

GTCS	Group Traffic Conditioning and Shaping

GUI	Graphical User Interface

HD	History Data

HTML	HyperText Markup Language

IaDI	Intra Domain Interface

IDL	Interface Definition Language

IrDI	Inter Domain Interface

LAG	Link Aggregation Group

LAN	Local Area Network

LB	Loop Back

LBM	Loop Back Message

LBR	Loop Back Reply

LCC	Lower Camel Case

LCK	Lock

LCT	Local Craft Terminal

LLDP	Link Layer Discovery Protocol

LM	Loss Measurement 2-way (using LMM/LMR, SLM/SLR)

LMM	Loss Measurement Message

LMo	On-Demand Loss Measurement

LMR	Loss Measurement Reply

LT	Link Trace

LTM	Link Trace Message

LTR	Link Trace Reply

MAC	Medium Access Control

ME	Maintenance Entity

MEG	Maintenance Entity Group

MEL	Maintenance Entity Level

MEP	MEG End Point

MI	Management Information

MIB	Management Information Base

MIP	MEG Intermediate Point

MMRP	Multiple MAC Registration Protocol

MTU	Maximum Transmission Unit

MVRP	Multiple VLAN Registration Protocol

N-Flr	Near-end - Frame loss ratio

NA	Not Applicable

NCM	Network Connection Monitoring

NE	Network Element

NMS	Network Management System

OAM	Operation, Administration & Maintenance

PDU	Protocol Data Unit

PM	Performance Monitoring

PTN	Packet Transport Network

QoS	Quality of Service

RAPS	Ring Automatic Protection Switching

RPL	Ring Protection Link

SD	Signal Degrade

SDL	Specification and Description Language

SES	Severely Errored Second

SLA	Service Level Agreement

SLM	Synthetic Loss Message

SLR	Synthetic Loss Reply

SMS	Service Management System

SNCP	SubNetwork Connection Protection

SNMP	Simple Network Management Protocol

SSL	Server Signal Fail

TCM	Tandem Connection Monitoring

TCS	Traffic Conditioning & Shaping

tfFlr	total far-end Frame loss ratio

tfLf	total far-end Lost frames

tfTf	total far-end Transmitted frames

TLV	Type-Length-Value

TMN	Telecommunications Management Network

tnFlr	total near-end Frame loss ratio

tnLf	total near-end Lost frames

tnTf	total near-end Transmitted frames

TP	Termination Point

TST	Test

TTL	Time To Live

TTP	Trail Termination Point

UAS	UnAvailable Second

UCC	Upper Camel Case

UML	Unified Modelling Language

UTC	Coordinated Universal Time

VLAN	Virtual Local Area Network

XML	eXtensible Markup Language

# Conventions

## Information modelling conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.1.

### UML modelling conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.1.

### Model Artefact Lifecycle Stereotypes conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.2.

### Forwarding entity terminology conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.3.

### Conditional package conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.4.

### Pictorial diagram conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.5.

## Equipment Function Conventions

### Maintenance Entity Group End Point (MEP) [ITU-T G.8021]

MEG End Points (MEPs) terminate Maintenance Entities (MEs) which can span the end-to-end Network Connection or a portion of the Network Connection defined as a Tandem Connection.

The diagrammatic convention for Network Connection Monitoring MEP (NCM MEP) compound functions is shown in Figure 5-1:

Figure 5-1 – Figure 9-63/G.8021 – ETH NCM MEP compound functions

Note: The ETH/ETH atomic function defined in [ITU-T G.8021] is only used within the optional TCM MEP compound function (i.e., not "stand alone") whereas the ETH/ETH-m atomic function defined in [ITU-T G.8021] is only used at the layer boundary (i.e., "stand alone" and never be a part of a MEP), regardless of the number of client signals (even in case of only one signal when there is no multiplexing).

Figure 5-2 – NCM MEP at the boundary of a layer network

The diagrammatic convention for Tandem Connection Monitoring MEP (TCM MEP) compound functions is shown in Figure 5-3:

Figure 5-3 – Figure 9-64/G.8021 – ETH TCM MEP compound functions

### Maintenance Entity Group Intermediate Point (MIP) [ITU-T G.8021]

The diagrammatic convention for MIP compound functions is shown in Figure 5-4:

Figure 5-4 – Figure 9-65/G.8021 – ETH MIP compound functions

The diagrammatic convention for half MIP compound functions is shown in Figure 5-5:

Figure 5-5 – Figure 9-66/G.8021 – ETH MIP compound functions

### Traffic Conditioning & Shaping function (TCS) [ITU-T G.806]

The diagrammatic convention for a TCS function is shown in Figure 5-6:

Figure 5-6 – Figure 5-3B/G.806 Amd1 – Diagrammatic convention for Traffic Conditioning & Shaping function

### MEPs and MIPs along an individual ME

: The diagrammatic convention for MEPs and MIPs along an individual ME as shown in Figure 5-7:

Figure 5-7 – MEPs and MIPs along a Maintenance Entity (ME)

Note that the ME can span the whole end-to-end Network Connection or a portion of it called a Tandem Connection.

## Conventions defined in this Recommendation

This Recommendation uses the following conventions:

### Colour Code convention

The following "colour code" is used in this document:

Table 5-1 – Colour Code convention

| "colour code" | G.8052 Object Class |
| --- | --- |
|  | MaintenanceEntityGroupEndPoint |
|  | ETH_TrailTerminationPoint |
|  | ETH_ConnectionTerminationPoint |
|  | OnDemandMeasurementJob |
|  | ProActiveMeasurementJob |
|  | MaintenanceJob |
|  | TerminationPointPool |
|  | specific highlighting |
|  | not in scope |

### Modelling convention for Adaptation Functions

Every adaptation function has a MI_Active parameter. This is not modelled since it will always be active in the Ethernet technology.

#### Ethernet Server Adaptation Modelling

Figure 5-8: Illustrative Diagram for Ethernet server Adaptation modelling

#### Non-Ethernet Server Adaptation Modelling

Figure 5-9: Illustrative Diagram for Non-Ethernet server Adaptation modelling

# Overview of the Model

Figure 6-1 below shows the mapping between the object classes and the Ethernet atomic functions defined in Figure 1-1/G.8021/Y.1341.

Figure 6-1 – Over view of Object Class mapping to G.8021 Atomic Functions

(based on Figure 1-1/G.8021/Y.1341)

Figure 6-1A below shows the containment relationship among the Ethernet object classes defined in this Recommendation. The containment relationship reflects the lifecycle dependency between object instances.

Figure 6-1A –Ethernet Object Class Containment Relationship

Figure 6-1B below shows the relationship of the Ethernet object classes to the GlobalClass and LocalClass objects defined in the Recommendation [ITU-T G.7711] Core Model for inheriting the uuid or the localIdList attributes. See Recommendation [ITU-T G.7711] Annex C for how the identifier attributes uuid and localIdList are used for identifying object instance.

Figure 6-1B –Ethernet Object Class Identifier

## ETH Sub-Layer

Figure 6-2 (based on Figure 9-1/G.8021/Y.1341) shows the sub-layer structure within the Ethernet layer. The G.8052 managed object classes have been added to the figure. The corresponding (high level) object instance diagram is contained in Figure 6-3.

Figure 6-2 – ETH sublayering amended with G.8052 object classes

(based on Figure 9-1/G.8021/Y.1341)

Figure 6-3: ETH Sublayer Object Instance Diagram

The basic configuration structure which shows the modelling of the Ethernet flow through the network element is contained in Figure 6-4.

Figure 6-4 – Basic Configuration Structure Object Diagram

## Basic Configuration Structure

The basic structure and high-level view of the object classes can be seen in the following figures.

All "auxiliary" object classes (e.g., MEP, MIP, TCS) will be attached to these basic object classes as necessary. This is shown in Figure 6-5 below.

Figure 6-5 – MEP infrastructure modelling object diagram

Figure 6-6 – CTP Inheritance Class Diagram

Figure 6-7 – TTP Inheritance Class Diagram

Figure 6-8 – ETH CTP Pool Class Diagram

## Location (Topology) of OAM Functions

The OAM functions will be modelled as auxiliary objects attached to the basic object classes (TrailTerminationPoint and ConnectionTerminationPoint); see also section 6.2. From monitoring point of view the direction and the order of the OAM functions TCM MEP, MIP and TCS within the signal flow of the ConnectionTerminationPoint (CTP) is important. This is reflected in the model by two attributes: codirectional and positionSequence.

The direction of the auxiliary object identifies if the sink part of the OAM function handle the same signal direction as the sink part of the containing CTP. This is modelled using a Boolean typed codirectional attribute in the axiliary object. TRUE identifies that the sink part of the OAM function and the sink part of the containing CTP handle the same signal direction (i.e., the TCM MEP, MIP and TCS is facing the server layer of the containing CTP). FALSE identifies that the sink part of the OAM function and the sink part of the containing CTP handle the opposite signal direction.

The order determines the processing sequence all auxiliary objects contained within the same CTP. This is modelled using the positionSequence attribute in the containing CTP. The ordered listing of the containing auxiliary objects starts from the server side of the CTP.

Figure 6-9 illustrates the positionSequence and codirectional attributes.

Note that this is a fictitious example trying to show many possible locations in one figure.

Figure 6-9 – Illustration of positionSequence and codirectional attributes

Realistic examples are shown in the following two figures. These examples are based on two interface types IaDI and IrDI. Note that the assignment of the maintenance entity levels <7 in the figures are arbitrary.

Figure 6-10 – Modelling of an IrDI Example

Figure 6-11 – Modelling of an IaDI Example

## MI Groupings and Mapping

This section lists (from G.8021) all atomic functions together with their MI. It is structured according to the sections in G.8021:
6.4.1	Ethernet MAC layer (ETH) and Mapping
6.4.2	Ethernet PHY Layer (ETYn) and Mapping
6.4.3	Non-Ethernet server to ETH adaptation and Mapping.

Conventions used in the tables:

- Atomic functions having the same list of MI are grouped.

- Fault and Performance MIs are not shown.

- Complementary atomic functions are shown with a green background.

- MI of adaptation functions are identified as "client layer related" (blue) or "server layer related" (red).

### Ethernet MAC layer (ETH) and Mapping

Table 6-1 – MI Groupings of the Ethernet MAC layer (ETH) functions

| Symbol | Management Information | G.8052 artefact |
| --- | --- | --- |
| Ethernet MAC layer (ETH) termination source functions |  |  |
|  | MI_MEL MI_MEP_MAC MI_CC_Enable MI_LM_Enable MI_MEG_ID MI_MEP_ID MI_CC_Period MI_CC_Pri MI_DM_Enable MI_DM_MAC_DA MI_DM_Length MI_DM_Period MI_DM_Pri MI_1DM_Enable[1...M1DM] MI_1DM_MAC_DA[1...M1DM] MI_1DM_Test_ID[1...M1DM] MI_1DM_Length[1...M1DM] MI_1DM_Period[1...M1DM] MI_1DM_Pri[1...M1DM] MI_SL_Enable[1...MSL] MI_SL_MAC_DA[1...MSL] MI_SL_Test_ID[1...MSL] MI_SL_Length[1...MSL] MI_SL_Period[1...MSL] MI_SL_Pri[1...MSL] MI_1SL_Enable[1...M1SL] MI_1SL_MAC_DA[1...M1SL] MI_1SL_Test_ID[1...M1SL] MI_1SL_Length[1...M1SL] MI_1SL_Period[1...M1SL] MI_1SL_Pri[1...M1SL] |  |
|  | MI_MEL MI_MEP_MAC MI_CC_Enable MI_LM_Enable MI_MEG_ID MI_MEP_ID MI_CC_Period MI_CC_Pri MI_LML_Enable[1...MLM] MI_LM_MAC_DA[1...MLM] MI_LM_Period[1...MLM] MI_LM_Pri [1...MLM] MI_DM_Enable [1...MDM] MI_DM_MAC_DA [1...MDM] MI_DM_Test_ID [1...MDM] MI_DM_Length [1...MDM] MI_DM_Period [1...MDM] MI_DM_Pri [1...MDM] MI_1DM_Enable [1...M1DM] MI_1DM_MAC_DA [1...M1DM] MI_1DM_Test_ID [1...M1DM] MI_1DM_Length [1...M1DM] MI_1DM_Period [1...M1DM] MI_1DM_Pri [1...M1DM] MI_SL_Enable [1...MSL] MI_SL_MAC_DA [1...MSL] MI_SL_Test_ID [1...MSL] MI_SL_Length [1...MSL] MI_SL_Period [1...MSL] MI_SL_Pri [1...MSL] MI_1SL_Enable [1...M1SL] MI_1SL_MAC_DA [1...M1SL] MI_1SL_Test_ID [1...M1SL] MI_1SL_Length [1...M1SL] MI_1SL_Period [1...M1SL] MI_1SL_Pri [1...M1SL] |  |
|  | MI_LM_Start(DA,P,Period) MI_LM_Terminate MI_LB_Discover(P) MI_LB_Series(DA,DE,P,N, Length, Period) MI_LB_Test(DA,DE,P,Pattern, Length, Period) MI_LB_Test_Terminate MI_DM_Start(DA,P, Length,Period) MI_DM_Terminate MI_1DM_Start(DA,P, Length,Period) MI_1DM_Terminate MI_TST(DA,DE,P,Pattern, Length, Period) MI_TST_Terminate MI_LT(TA,TTL,P) MI_MEP_MAC MI_MEL MI_MEP_ID MI_LM_Pri MI_SL_Start(DA,P,Length,Period,Type) ETHDe_FT_So_MI_SL_Terminate ETH_De_FT_So_MI_SL_Enable ETHDe_FT_So_MI_SL_Intermediate_Request ETHDe_FT_So_MI_SL_Terminate ETHDe_FT_So_MI_1SL_Start(DA, P,Test_ID,Length, Period) ETHDe_FT_So_MI_1SL_Terminate |  |
|  | MI_MEL MI_MIP_MAC |  |
|  | no MI |  |
| Ethernet MAC layer (ETH) termination sink functions |  |  |
|  | MI_CC_Enable MI_LM_Enable MI_1Second MI_LM_DEGM MI_LM_M MI_LM_DEGTHR MI_LM_TFMIN MI_MEL MI_MEG_ID MI_PeerMEP_ID[i] MI_CC_Period MI_CC_Pri MI_GetSvdCCM MI_1DM_Enable MI_1DM_MAC_SA ETHx_FT_Sk_MI_1SL_Test_ID[1...M1SL] ETHx_FT_Sk_MI_BW_Report(SA, PortID, NominalBW, CurrentBW) |  |
|  | MI_CC_Enable MI_LM_Enable MI_1Second MI_LM_DEGM MI_LM_M MI_LM_DEGTHR MI_LM_TFMIN MI_MEL MI_MEG_ID MI_PeerMEP_ID[i] MI_CC_Period MI_CC_Pri MI_GetSvdCCM MI_1DM_Enable [1...M1DM] MI_1DM_MAC_SA [1...M1DM] MI_1DM_Test_ID [1...M1DM] MI_1SL_Enable [1...M1SL] MI_1SL_MAC_SA [1...M1SL] MI_1SL_Test_ID [1...M1SL] MI_BW_Report(SA, PortID, NominalBW, CurrentBW) |  |
|  | MI_Active MI_LM_Pri MI_MEL MI_MEP_MAC MI_1DM_Start(SA, Test_ID) MI_1DM_Intermediate_Request MI_1DM_Terminate MI_TST_Start(SA,Pattern) MI_TST_Terminate MI_1SL_Start( SA,MEP_ID, Test_ID) MI_1SL_Intermediate_Request MI_1SL_Terminate MI_1DM_Result(count, N_FD[]) MI_TST_Result(REC,CRC,BER,OO) MI_1SL_Result(N_TF,N_LF) |  |
|  | MI_MEL MI_MIP_MAC |  |
|  | MI_SSF_Reported |  |
| Ethernet MAC layer (ETH) adaptation source functions |  |  |
|  | MI_Active MI_MEP_MAC MI_Client_MEL[1..M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] MI_Admin_State MI_MEL MI_APS_Pri MI_CSF_Period MI_CSF_Pri MI_CSF_Enable MI_CSFrdifdiEnable MI_CSFdciEnable |  |
|  | MI_Active MI_MEP_MAC MI_Client_MEL[1..M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] MI_Admin_State MI_MEL MI_APS_Pri MI_CSF_Period MI_CSF_Pri MI_CSF_Enable MI_CSFrdifdiEnable MI_CSFdciEnable |  |
| ETHx/ETHG has exactly the same MI as ETHx/ETH-m. | MI_Active MI_MEP_MAC MI_Client_MEL[1…M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] MI_Admin_State MI_Vlan_Config[1…M] MI_Etype MI_PCP_Config MI_MEL MI_CSF_Period MI_CSF_Pri MI_CSF_Enable MI_CSFrdifdiEnable MI_CSFdciEnable |  |
|  | no MI |  |
|  | MI_Active MI_MEL MI_RAPS_Pri MI_MIP_MAC |  |
|  | MI_Active MI_TxPauseEnable MI_Agg[1..Na]_AP_List MI_AggPort[1..Np]_ActorAdmin_State |  |
|  | MI_Active |  |
|  | MI_Active MI_MEL MI_MEP_MAC MI_MCC_Pri MI_MEP_ID MI_EDM_Enable MI_EDM_Period MI_EDM_Duration |  |
| Ethernet MAC layer (ETH) adaptation sink functions |  |  |
|  | MI_Active MI_MEP_MAC MI_Client_MEL MI_LCK_Period MI_LCK_Pri MI_Admin_State MI_AIS_Period MI_AIS_Pri MI_MEL MI_CSF_Reported MI_CSFrdifdiEnable |  |
|  | MI_Active MI_MEP_MAC MI_Client_MEL[1..M] MI_LCK_Period[1..M] MI_LCK_Pri[1..M] MI_Admin_State MI_AIS_Period[1..M] MI_AIS_Pri[1..M] MI_MEL |  |
| ETHx/ETHG exactly the same as ETHx/ETH-m. | MI_Active MI_Admin_State MI_MEP_MAC MI_Client_MEL[1…M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] MI_AIS_Period[1…M] MI_AIS_Pri[1…M] MI_VLAN_Config[1…M] MI_P_Regenerate MI_PVID MI_PCP_Config MI_Etype MI_MEL MI_Frametype_Config MI_Filter_Config |  |
|  | no MI |  |
|  | MI_Active MI_RAPS_MEL |  |
|  | MI_Active MI_PLLThr[1..Na] |  |
|  | MI_Active MI_FilterConfig |  |
|  | MI_Active MI_MEP_MAC MI_MEL MI_EDM_received (MEP_ID, Duration) |  |
| Ethernet MAC layer (ETH) traffic conditioning source functions |  |  |
|  | MI_Prio_Config MI_Queue_Config[] MI_Sched_Config |  |
|  | MI_Prio_Config[] MI_Queue_Config[][] MI_Sched_Config |  |
| Ethernet MAC layer (ETH) traffic conditioning sink functions |  |  |
|  | MI_Prio_Config MI_Cond_Config[] |  |
| ETH Group Traffic Conditioning Function (ETH_GTCS_Sk) ETH_GTCS_Sk function can be modelled by multiple ETH_TCS_Sk functions |  |  |

Based on the groupings above the following packages have been identified.

Table 6-2 – Derived MI Groupings of the Ethernet MAC layer (ETH) functions

| MI groupings | Package name |
| --- | --- |
| TTP MI source grouping |  |
| MI_Active MI_MEP_MAC MI_Admin_State MI_MEL | EthServerTtpSourcePackage |
| MI_Etype MI_PCP_Config | EthServerTtpEtypeSourcePackage |
| MI_APS_Pri | EthServerTtpApsSourcePackage |
| TTP MI sink grouping |  |
| MI_Active MI_MEL MI_Admin_State MI_MEP_MAC | EthServerTtpSinkPackage |
| MI_P_Regenerate MI_PVID MI_PCP_Config MI_Etype MI_Frametype_Config MI_Filter_Config | EthServerTtpEtypeSinkPackage |
| CTP MI source grouping |  |
| MI_Client_MEL[1..M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] | EthServerSourcePackage |
| MI_Vlan_Config[1…M] | EthServerVlanConfigSourcePackage |
| CTP MI sink grouping |  |
| MI_Client_MEL[1…M] MI_LCK_Period[1…M] MI_LCK_Pri[1…M] MI_AIS_Period[1…M] MI_AIS_Pri[1…M] MI_VLAN_Config[1…M] | EthServerSinkPackage |

### Ethernet PHY Layer (ETYn) and Mapping

Table 6-3 – MI Groupings of the Ethernet PHY Layer (ETYn) functions

| Symbol | Management Information | Managed Object Class |
| --- | --- | --- |
| Ethernet PHY Layer (ETYn) termination source functions |  |  |
|  | MI_FTSEnable |  |
| Ethernet PHY Layer (ETYn) termination sink functions |  |  |
|  | no MI |  |
| Ethernet PHY Layer (ETYn) adaptation source functions |  |  |
|  | MI_Active MI_TxPauseEnable |  |
| ETY3 to ETC3 adaptation source functions |  |  |
|  | MI_Active |  |
| ETY4 to Ethernet PP-OS adaptation source function |  |  |
|  | MI_Active |  |
| Ethernet PHY Layer (ETYn) adaptation sink functions |  |  |
|  | MI_Active MI_FilterConfig MI_MAC_Length |  |
| ETY3 to ETC3 adaptation sink functions |  |  |
|  | MI_Active |  |
| ETY4 to Ethernet PP-OS adaptation sink function |  |  |
|  | MI_Active |  |

### Non-Ethernet server to ETH adaptation and Mapping

Table 6-4 – MI Groupings of the Non-Ethernet server to ETH adaptation functions

| Symbol | Management Information | Managed Object Class |
| --- | --- | --- |
| Non-Ethernet server to ETH adaptation source functions |  |  |
|  | MI_Active MI_CSFEnable MI_CSFrdifdiEnable |  |
|  | MI_Active MI_CSFEnable | ? |
| Non-Ethernet server to ETH adaptation sink functions |  |  |
|  | MI_Active MI_FilterConfig MI_CSF_Reported MI_MAC_Length MI_CSFrdifdiEnable MI_AcSL MI_AcEXI MI_AcUPI ODUkP/ETH_A_Sk: MI_AcPT MI_AcEXI MI_AcUPI ODUkP-X-L/ETH_A_Sk: MI_AcVcPT MI_AcEXI MI_AcUPI |  |
|  | MI_Active MI_CSF_Reported Sn-X/ETC3_A_Sk: MI_AcSL MI_AcEXI MI_AcPFI MI_AcUPI ODU2P/ETHPP-OS_A_Sk: MI_AcPT MI_AcEXI MI_AcUPI | ? |

Based on the groupings above the following packages have been identified.

Table 6-5 – Derived MI Groupings of the Non-Ethernet server Adaptation functions

| MI groupings | Package name |
| --- | --- |
| MI source grouping |  |
| MI_CSFrdifdiEnable | CsfRdiFdiEnableSource_Package |
| MI_CSFEnable | CsfEnableSource_Package |
| MI sink grouping |  |
| MI_FilterConfig MI_MAC_Length MI_CSFrdifdiEnable | CsfRdiFdiEnableSink_Package |
| MI_CSF_Reported | CsfReportedSink_Package |

### Ethernet Adaptation Modelling

Figure 6-12 below summarizes the mdoeling in the above 3 subclauses.

Figure 6-12: Ethernet Adaptation Modelling

# Modelling of Ethernet Functions

## OAM Compound Functions

OAM is done in the network by creating Maintenance Entities (ME). In multipoint services multiple MEs are grouped together forming a Maintenance Entities Group (MEG); see definitions in section 7 (Ethernet network management) of [ITU-T G.8010].

Each MEG is terminated by a set of MEG End Points (MEPs). It is also possible to perform OAM functions on a MEG by MEG Intermediate Points (MIPs) which allow a limited set of OAM functions along the MEs.

MEL Dependencies between layers

From architecture point of view each layer has its own 8 Maintenance Entity Levels (MELs) which are independent from the MELs of its client and server layer.
But it is also possible that two layers share the same 8 MELs.
In these cases the management has to make sure that the MELs of both layers do not overlap.

Management of MEPs and MIPs

Maintenance Entities (ME) are monitored using MEPs and MIPs; see following figure from Y.1731.

Figure 7-1 – Figure II.1/Y.1731 – Example MEG level assignment for shared MEG levels

It is assumed that the management systems "north" of the G.8052 interface are responsible for establishing the necessary configuration, like the example shown above. The management systems involved may include a service management system (SMS) of the service provider and the network management system (NMS) and element management system (EMS) of the operators. The EMS/NE interface has to provide means (operations) to create all the necessary MEPs and MIPs separately.
For example, the 5 MEPs and 2 MIPs in network element 5 above are created manually via the EMS/NE interface.

### MEP Compound Function

There are two different types of MEP compound functions:

- mandatory NCM MEPs at the boundary of a layer network, monitoring a network connection

- optional TCM MEPs in the middle of a layer network, monitoring a tandem connection.

Figure 7-2 – Mandatory and optional MEPs

From management point of view a MEP has the following constraints:

- A MEP can be unidirectional or bidirectional; unidirectional MEPs have a limited set of OAM functionality.

- A MEP belongs to one, and only one, MEL.
MEPs terminate MEGs and each MEG is associated to one MEL.

- A MEP is addressed by one, and only one, MAC address.
The MAC address (or more precise the EUI-48) is bound to a physical subsystem and one physical subsystem can hold thousands of MEP functions, and all those MEP functions share in such case one MAC address.

The MEP compound function supports three applications which are organised in "jobs":

- On-demand measurement job

- Pro-active measurement job

- Maintenance job.

Figure 7-3 – MEP Class Diagram

Note: This diagram is the lower portion of Figure 7-15

Note: The MEP object is further described later on  in Figure 7-8, Figure 7-13 and Figure 7-15 below describing the individual segments of the MEP model.

The Management Information (MI) of the compound functions (defined in G.8021) needs to be mapped to G.8052 artefacts. The following sections list all the MIs defined for the MEP compound function in tables and associate them to applications (coloured background). The corresponding part of the model is shown below the table.

#### MEP on-demand Diagnostic Function

The MEP on-demand diagnostic function exists in NCM and TCM MEPs.

Figure 7-4 – MEP on-demand diagnostic function

Note: The ETH/<client>_A atomic function is for further study.

G.8021 defines the following Management Information (MI) for on-demand measurements and maintenance:

Table 7-1 – On-demand measurement and Maintenance MI list

| Functionality | G.8021 ETHDe_FT_So_MI | G.8021 ETHDe_FT_Sk_MI |
| --- | --- | --- |
| General identifiers | MEL | MEL |
|  | MEP_MAC | MEP_MAC |
|  | MEP_ID |  |
| 2-way on-demand loss of frame measurement | LM_Start(DA,P,Period) |  |
|  | LM_Terminate |  |
|  | LM_Result( N_TF, N_LF, F_TF, F_LF) |  |
| 1-way on-demand synthetic loss of frame measurement | 1SL_Start(DA,P,Test_ID,Length,Period) | 1SL_Start(SA,MEP ID, Test ID) |
|  | 1SL_Terminate | 1SL_Terminate |
|  |  | 1SL_Result(N_TF,N_LF) |
| 2-way on-demand synthetic loss of frame measurement | SL_Start(DA,P,Test_ID,Length,Period) |  |
|  | SL_Terminate |  |
|  | SL_Result(N_TF,N_LF,F_TF,F_LF) |  |
| 1-way on-demand frame delay measurement | 1DM_Start(DA,P Test ID,Length,Period) | 1DM_Start(SA,Test ID) |
|  | 1DM_Terminate | 1DM_Terminate |
|  |  | 1DM_Result(count,N_FD[]) |
| 2-way on-demand frame delay measurement | DM_Start(DA,P,Test ID,Length,Period) |  |
|  | DM_Terminate |  |
|  | DM_Result(count,B_FD[],F_FD[],N_FD[]) |  |
| On-demand loop back | LB_Discover(P) |  |
|  | LB_Series(DA,DE,P,N, Length, Period) |  |
|  | LB_Test(DA,DE,P,Pattern, Length, Period) |  |
|  | LB_Test_Terminate |  |
|  | LB_Discover_Result(MACs) |  |
|  | LB_Series_Result(REC, OO) |  |
|  | LB_Test_Result(Sent, REC, CRC, BER, OO) |  |
| On-demand test | TST(DA,DE,P,Pattern, Length, Period) | TST_Start(SA,Pattern) |
|  | TST_Terminate | TST_Terminate |
|  | TST_Result(Sent) | TST_Result(REC,CRC,BER,OO) |
| On-demand link trace | LT(TA,TTL.P) |  |
|  | LT_Results(Results) |  |

Figure 7-5, Figure 7-6 and Figure 7-7 provide three sequence diagrams describing the message sequences related to the on-demand measurements.

Figure 7-5 – 2-way On-demand Measurement

Figure 7-6 – 1-way On-demand Measurement Control Source

Figure 7-7 – 1-way On-demand Measurement Control Sink

Figure 7-8 – On-demand Measurement Class Diagram

#### MEP pro-active Measurement Function

The MEP pro-active measurement function exists in NCM and TCM MEPs.

Figure 7-9 – MEP pro-active measurement function

Note: The ETH/<client>_A atomic function is for further study.

G.8021 defines the following Management Information (MI) for pro-active measurements:

Table 7-2 – Pro-active measurement MI list

| Functionality | G.8021 ETH(G)_FT_So_MI | G.8021 ETH(G)_FT_Sk_MI |
| --- | --- | --- |
| General identifiers | MEL | MEL |
|  | MEG_ID | MEG_ID |
|  | MEP_ID | PeerMEP_ID[i] |
|  | MEP_MAC |  |
| Continuity check | CC_Enable | CC_Enable |
|  | CC_Period | CC_Period |
|  | CC_Pri | CC_Pri |
| Last CCM triggering a fault received |  | GetSvdCCM |
|  |  | (out) SvdCCM |
| 1-way pro-active loss of frame measurement using CCM | LMC_Enable | LMC_Enable |
|  |  | LM_DEGM |
|  |  | LM_M |
|  |  | LM_DEGTHR |
|  |  | LM_TFMIN |
| 1-way pro-active loss of frame measurement using LMM/LMR | LML_Enable |  |
|  | LM_MAC_DA |  |
|  | LM_Period |  |
|  | LM_Pri |  |
| 1-way pro-active synthetic loss of frame measurement | 1SL_Enable | 1SL_Enable |
|  | 1SL_MAC_DA | 1SL_MAC_SA |
|  | 1SL_Test_ID | 1SL_MEP_ID |
|  | 1SL_Length | 1SL_Test_ID |
|  | 1SL_Period |  |
|  | 1SL_Pri |  |
| 2-way pro-active synthetic loss of frame measurement | SL_Enable |  |
|  | SL_MAC_DA |  |
|  | SL_Test_ID |  |
|  | SL_Length |  |
|  | SL_Period |  |
|  | SL_Pri |  |
| 1-way pro-active frame delay measurement | 1DM_Enable | 1DM_Enable |
|  | 1DM_MAC_DA | 1DM_MAC_SA |
|  | 1DM_Test_ID | 1DM_Test_ID |
|  | 1DM_Length |  |
|  | 1DM_Period |  |
|  | 1DM_Pri |  |
| 2-way pro-active frame delay measurement | DM_Enable |  |
|  | DM_MAC_DA |  |
|  | DM_Test_ID |  |
|  | DM_Length |  |
|  | DM_Period |  |
|  | DM_Pri |  |
| Fault cause list |  | cLOC[i], cUNL, cMMG, cUNM, cDEG, cUNP, cUNPr, cRDI, cSSF, cLCK |
| Performance primitive list |  | pN_TF, pN_LF, pF_TF, pF_LF, pF_DS, pN_DS, pB_FD, pB_FDV, pF_FD, pF_FDV, pN_FD, pN_FDV |

Figure 7-10, Figure 7-11 and Figure 7-12 provide three sequence diagrams describing the message sequences related to the pro-active measurements.

Figure 7-10 – 2-way Pro-active Measurement

Figure 7-11 – 1-way Pro-active Measurement Control Source

Figure 7-12 – 1-way Pro-active Measurement Control Sink

Figure 7-13 – Pro-active Measurement

#### MEP Configuration Function

The MEP configuration function exists in NCM and TCM MEPs.

Figure 7-14 – MEP configuration function

Note: The ETH/<client>_A atomic function is for further study.

G.8021 defines the following Management Information (MI) for configuring a MEP:

Table 7-3 – MEP configuration MI list

| Functionality | G.8021 ETH(G)/ETH_A_So_MI + ETHx/ETH-m_A_So_MI | G.8021 ETH(G)/ETH_A_Sk_MI + ETHx/ETH-m_A_Sk_MI |
| --- | --- | --- |
| General identifiers | MEP_MAC | MEP_MAC |
|  | MEL | MEL |
|  | Client_MEL[1..M] | Client_MEL[1…M] |
| Lock | Admin_State | Admin_State |
|  | LCK_Period[1..M] | LCK_Period[1…M] |
|  | LCK_Pri[1..M] | LCK_Pri[1…M] |
| Alarm indication signal |  | AIS_Period[1…M] |
|  |  | AIS_Pri[1…M] |
| Client signal fail | CSF_Enable | CSF_Reported |
|  | CSFrdifdiEnable | CSFrdifdiEnable |
|  | CSF_Period | cCSF |
|  | CSF_Pri |  |
|  | CSFdciEnable |  |
| Automatic protection switching | APS_Pri |  |

Note:
APS_Pri MI not in ETHx/ETH-m_A since APS is only used for SNCP which is not possible at the layer boundary.
Red enhancements only in ETHG/ETH_A and ETHx/ETH-m_A.
ETHx/ETHG_A has the same MI as ETHx/ETH-m_A but without CSF. This is because ETHx/ETHG_A is on hold for the time being.

Figure 7-15 – MEP configuration

### MIP Compound Function

From management point of view a MIP has the following constraints:

- A MIP is always bidirectional; i.e., can only be used in bidirectional connections in the carrier Ethernet environment. Note that a unidirectional MIP is possible in the Enterprise environment as defined in [IEEE 802.1Q].

- A MIP belongs to one, and only one, MEL.
MIPs are associated to a specific MEG and a MEG is associated to one MEL.

- A MIP is addressed by one, and only one, MAC address.
The MAC address (or more precise the EUI-48) is bound to a physical subsystem and one physical subsystem can hold thousands of MIP functions, and all those MIP functions share in such case one MAC address.

- A MIP compound function can be divided in two "half MIP" functions; see Figure 7-17.
This is necessary because some current implementations are not able to support full MIP functions (i.e., half MIP functions need to be modelled).

Figure 7-16 – MIP configuration parameters

Figure 7-17 – "half MIP" compound function

The identification of the "half MIPs" from management point of view is modelled by a Boolean attribute "isFullMip". Note that the up-half MIP is not supported in the model.

The Management Information (MI) of the MIP compound function (defined in [ITU-T G.8021]) is mapped to the MipBidirectional object class.

Figure 7-18 – MIP/half MIP configuration function

[ITU-T G.8021] defines the following Management Information (MI) for configuring a MIP:

Table 7-4 – MIP configuration MI list

| Functionality | ETHDi_FT_So | ETHDi_FT_Sk |
| --- | --- | --- |
| General identifiers | MEL | MEL |
|  | MIP_MAC | MIP_MAC |

Figure 7-19 – MIP class diagram

## TCS Function

This section maps the (G)TCS function related MIs to the corresponding object classes.

Figure 7-20 – TCS/GTCS configuration function

Table 7-5 – TCS configuration MI list

| Functionality | ETH_(G)TCS_So_MI | ETH_(G)TCS_Sk_MI |
| --- | --- | --- |
| General identifiers | Prio_Config[1..M] | Prio_Config[1..M]* |
| Queing | Queue_Config[][1..M] |  |
| Scheduling | Sched_Config |  |
| Conditioning |  | Cond_Config[][1..M]* |

Notes:
Red enhancements only in GTCS.
* ETH Group Traffic, traffic conditioning process is performed per flow point but there is no correlation between each process. Therefore, an ETH_GTCS_Sk function can be modelled by multiple ETH_TCS_Sk functions and no specific function is defined in this recommendation.

Figure 7-21 – TCS Containment Class Diagram

## Performance Monitoring

Performance Monitoring allows measurement of different performance parameters like Frame Loss Ratio, Frame Delay and Frame Delay Variation. The mechanisms are defined in [ITU-T G.8013/Y.1731].

Figure 7-22 – PM Measurement Job Containment Relationship

### Loss Measurement

The Frame Loss Measurement (LM) provides performance data that is based on the lost frames between the ingress and the egress of a Maintenance Entity (ME); i.e., between two maintenance Group End Points (MEPs).

LM using CMM or LMM is restricted to MEGs which have only a single ME.

Six LM functions are defined:

-	2-way on-demand LM

-	1-way on-demand synthetic LM

-	2-way on-demand synthetic LM

-	1-way pro-active LM.

-	1-way pro-active synthetic LM

-	2-way pro-active synthetic LM

The single-ended on-demand LM function is managed only at the source MEP. The sink MEP does not need any management.

The dual-ended pro-active LM function is managed at source and sink MEP.

Figure 7-22A shows the information flows within the [ITU-T G.8021] atomic functions.

Figure 7-22A – Frame Loss Measurement mechanisms

### Delay Measurement

The Frame Delay Measurement (DM) provides performance data that is based on the delay of the frames between the ingress and the egress of a Maintenance Entity (ME); i.e., between two Maintenance Group End Points (MEPs).

Four DM functions are defined:

-	1-way on-demand DM

-	2-way (round-trip) on-demand DM.

-	1-way pro-active DM

-	2-way (round-trip) pro-active DM.

The 1-way DM function is started at the source MEP and enabled at the sink MEP.

The 2-way DM function is managed only at the source MEP. The sink MEP does not need any management.

Figure 7-22B shows the information flows within the [ITU-T G.8021] atomic functions.

Figure 7-22B – Frame Delay Measurement mechanisms

### Threshold Profile Control

<Editor Note: Need surrounding text to describe the use of the threshold profile control mechanism.>

Figure 7-23 – Thrshold Profile Control mechanisms

## Ethernet Multiplexing

This section maps the Ethernet multiplexing related MIs to the corresponding object classes.

The Ethernet multiplexing configuration function exists only in ETH TTP and ETH CTP.

Figure 7-24 – Ethernet multiplexing configuration function

Note: The ETH/<client>_A atomic function is for further study.

G.8021 defines the following Management Information (MI) for configuring the Ethernet multiplexing function:

Table 7-6 – Mapping of Multiplexing related MI to G.8052 artefacts

| Functionality | G.8021 ETHx/ETH-m_A_So_MI | G.8021 ETHx/ETH-m_A_Sk_MI |
| --- | --- | --- |
| TTP associated MI | Etype | Etype |
|  | PCP_Config | PCP_Config |
|  |  | P_Regenerate |
|  |  | PVID |
|  |  | Frametype_Config |
|  |  | Filter_Config |
| CTP associated MI | VLAN_Config[1...M] | VLAN_Config[1...M] |

Note:
ETHx/ETHG_A has the same MI as ETHx/ETH-m_A. ETHx/ETHG_A is on hold for the time being.

Figure 7-25 – Ethernet multiplexing configuration

## Connection Function

This section maps the connection function related MIs to the corresponding object classes.

Figure 7-26 – Connection Fra gment Class Diagram

Table 7-7 – Mapping of Connection Function related MI to G.8052 artefacts

| Functionality | ETH_C_MI | G.8052 |
| --- | --- | --- |
| General connection management | Create_FF |  |
|  | Modify_FF |  |
|  | Delete_FF |  |
| Individual connection management | ETH_C_MP per flow forwarding process: |  |
|  | FF_Set_PortIds |  |
|  | FF_ConnectionType |  |
|  | FF_Flush_Learned |  |
|  | FF_Flush_Config |  |
|  | FF_Group_Default |  |
|  | FF_ETH_FF |  |
|  | FF_Ageing |  |
|  | FF_Learning |  |
|  | FF_STP_Learning_State[i] |  |
| SNCP configuration | ETH_C_MP per SNC/S protection process: |  |
|  | PS_WorkingPortId |  |
|  | PS_ProtectionPortId |  |
|  | PS_ProtType |  |
|  | PS_OperType |  |
|  | PS_HoTime |  |
|  | PS_WTR |  |
|  | PS_ExtCMD |  |
|  | PS_BridgeType |  |
|  | PS_SD_Protection |  |
| RAPS configuration | ETH_C_MP per Ring protection process: |  |
|  | RAPS_RPL_Owner_Node |  |
|  | RAPS_RPL_Neighbour_Node |  |
|  | RAPS_Propagate_TC[1…M] |  |
|  | RAPS_Compatible_Version |  |
|  | RAPS_Revertive |  |
|  | RAPS_Sub_Ring_Without_Virtual_Channel |  |
|  | RAPS_HoTime |  |
|  | RAPS_WTR |  |
|  | RAPS_GuardTime |  |
|  | RAPS_ExtCMD |  |
|  | RAPS_RingID |  |

### Linear Protection Function

The Ethernet linear protection function is defined in [ITU-T G.8031]. The related "Management Information" is listed in [ITU-T G.8021] and further clarified in [ITU-T G.8031].

This function is modelled by the ETH_SNCP_Group object class.

Figure 7-27 – Linear Protection Class Diagram

Note: This diagram is the lower left portion of Figure 7-26

### Ring Protection Function

The Ethernet ring protection function is defined in [ITU-T G.8032]. The related "Management Information" is listed in [ITU-T G.8021] and further clarified in [ITU-T G.8032].

Each Ring Protection Control Process is modelled by an ETH_RAPS_Group object class.

Figure 7-28 – Generic Model of Ethernet Ring Protection Function

(based on Figure 9-4/G.8032/Y.1344)

Figure 7-29 – Ring Protection Class Diagram

Figure 7-30 – Ring Protection: RPL parameters

Network Elements without interconnection have one Ring Protection Control Process. Network Elements with interconnection have one Ring Protection Control Process per ring interconnected. The type of control process is modelled by the controlProcessType attribute. For an example see Figure 7-31 below.

Figure 7-31 – R-APS Control Process Type

## LAG Management Modelling

The management of Link Aggregation Groups (LAGs) is based on the generic model shown in Figure 7-32 and on the specifications from [IEEE 802.1AX].

Figure 7-32 –Generic Model of Ethernet Link Aggregation

(based on Figure 9-52/G.8021/Y.1341)

The corresponding Object Classes are contained in Figure 7-33.

Figure 7-33 – LAG Management Class Diagram

## PhysicalSubsystem Modelling

The PhysicalSubsystem object class groups a set of atomic functions within the ManagedElement. It is used for the configuration of all properties (e.g., the EUI-48 identifier (MAC address)) that are common to all object instances (and their containing object instances) grouped by the PhysicalSubsystem.

Even though these common properties are defined in the PhysicalSubsystem, they will be also part of the individual object classes, but read-only if the PhysicalSubsystem is supported and read-write if the PhysicalSubsystem is not supported.

Figure 7-34 – Physical Subsystem modelling overview

# UML Model file

This section contains the information model files and the companion profile file specified using the “Papyrus” modelling tool.

- G.8052 Model File:

, which is also available on the IFA at https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.8052/G.8052_v2.00/G.8052_v2.00_PAP.zip.

The G.8052_v1.10_PAP.zip file contains the following four files:

  - .project,

  - CoreModel.di,

  - CoreModel.notation

  - CoreModel.uml

In order to use this G.8052 model, one needs to also install the Open Model Profile v0.2.2 and G.7711 v2.0

- OpenModelProfile File:  https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/OpenModelProfile/OpenModelProfile_v0.2.2.zip.

The openmodelprofile-0.2.0.zip file consists of four files:

  - .project,

  - OpenModel_Profile.profile.di

  - OpenModel_Profile.profile.notation

  - OpenModel_Profile.profile.uml

The OpenModelProfile defines the UML model meta constructs (i.e. basic building blocks, such class, attribute, association, etc.) to be use to build information models. Using the same OpenModelProfile is essential step before achieving common IM.

- G.7711 v2.0 File: https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.7711/G.7711_v2.00/G.7711_v2.00_PAP.zip

NOTE –  Papyrus is an open source modeling tool. In order to view and further extend or modify the information model, one will need to install the open source Eclipse software and the Papyrus tool. The installation guide for Eclipse and Papyrus can be found at the following link:

https://www.eclipse.org/papyrus/updates/index.php.

# UML Modelling Guidelines

(This appendix does not form an integral part of this Recommendation)

See Recommendation [ITU-T G.7711] Annex A

NOTE – The UML Modeling Guidelines specified in the 7/2013 version of this Recommendation has been enhanced and normatively specified in Annex A of Recommendation {ITU-T G.7711}.

# Mapping of G.8021 Atomic Functions to G.8052 Model Artefacts

(This appendix does not form an integral part of this Recommendation)

This appendix provides further detailed mapping between the G.8021 atomic functions and the G.8052 UML model artifacts. Note that in some cases a 1:1 mapping is not possible.

Table II-1 –Mapping between G.8021 Ethernet Atomic Functions and UML Model Artifacts

| G.8021 section | G.8021 atomic function | G.8052 model artefact | Note |
| --- | --- | --- | --- |

For Further Study

# UML Model Data Dictionary

(This appendix does not form an integral part of this Recommendation)

This appendix contains the details of the Ethernet NE management-protocol-neutral information model, including the description and properties of the object classes and their attributes and operations. This detailed information is generated automatically from a modelling tool, which was used to develop the information model in UML notation.

, which is also available on the IFA at https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.8052/G.8052_v2.00/G.8052_v2.00_DD.zip.

___________

The linkage of the Ethernet-specific object classes to the generic object classes is specified in details in Appendix I of this Recommendation.