| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 15 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 620 R1 (PLEN/15) |
|  |  | English only Original: English |
| Question(s): | 14/15 | Geneva, 19-30 September 2016 |
| TD |  |  |
| Source: | Editors G.8152/Y.1375 |  |
| Title: | Draft revised Recommendation ITU-T G.8152/Y.1375 (v0.17) for consent |  |

This document is the editor version of G.8152/Y.1375 “Protocol-neutral management information model for the MPLS-TP network element” v0.17.

Introduction

The protocol-neutral management information model for the MPLS-TP network element (draft new Recommendation G.8152) is based on the structure used in Recommendation G.8052 for Ethernet.

# Document history:

| Version | Document | Description |
| --- | --- | --- |
| 0.01 | WD18 (06/2012) | - Initial draft |
| 0.02 | WD18r1 (06/2012) | - References to G.8010 replaced by references to G.8101 - Versions of references updated - MIs have been updated based on the latest G.8121 contained in wd05r1 (06/2012) - The sentence providing the relationship to IETF has been added to the Summary and Scope sections; according to SG15 report R30 - The Scope section has been updated based on cd120r1 (Ethernet virtual meetings) - Figure 6 1 updated according the editors instructions - Answers from Q9 regarding the open issues identified in the document have been addedInitial draft |
| 0.03 | WD12 (09/2012) | - All revision marks from version 0.02 have been accepted - References to the modelling conventions and model artefact properties added - Enhanced the scope of the Recommendation to cover also G.8121.1 and G.8121.2 - Tbd: Integrate the MI Grouping Packages proposed in C 2187 |
| 0.04 | WD18 (07/2013) | - All revision marks from version 0.03 have been accepted - High level class diagrams added in new section Error! Reference source not found. |
| 0.05 | WD05 (09/2013) | This version is an update to TD91/3 (7/2013) - All revision marks from version 0.04 have been accepted - High level class diagrams for on-demand (Error! Reference source not found.) and pro-active (Error! Reference source not found.) measurements have been enhanced. |
| 0.05r1 | WD05r1 (09/2013) | - Incorporate the new structure as proposed in WD13 (9/2013 Shenzhen) |
| 0.06r1 | WD05r2 (09/2013) | - Incorporated new figures from model updated at Shenzhen meeting - Included new graphics in instructions for HTML file - Included zip file of v0.06 in HTML format - Included zip file of v0.06 in RSA format |
| 0.06r2 | WD05r (10/2013) | - Fixed Diagrams |
| 0.07 | WD28 (10/2013 ) Nuremberg | - Per WD16 discussion, description text for MEP and MEP bidirectional were updated - Add attributes to the MEP abstract class for the common MI signals of MTDe_TT_So and MTDe_TT_Sk - Import the M.3160 Managed Element and Termination Pont super classes - Replace the insertion of HTML file and Data Dictionary with hyper links |
| 0.08 | TD140r1/WP3 (4/2014) | - Changes in version 0.07 accepted - References section updated - MEP and MIP compound function diagrams updated - High Level TP Containment Class Diagram Figure 6-2 updated - TP Inheritance Class Diagram Figure 6-3 updated - Attributes added to MEP object classes and MEP Class Diagram Figure 6-5 updated - MI information in Table 7-1, Table 7-3, Table 7-4, Table 7-5, Table 7-6 updated based on G.8121 (11/2013) - New Table 6-3 on Derived MI Groupings of the MPLS-TP functions added - New Figure 6-9 on Modelling of non-MPLS-TP server adaptation MI groupings added - Modelling of SCC/MCC access added in new section 7.6 - Table 7-1, Table 7-2 and Table 7-4 enhanced with the MI from G.8121.1 and G.8121.2 - GenericCurrentData object class from G.gim has been integrated to the model |
| 0.09 | WD12 (8/2014) | - Added Data Dictionary |
| 0.10 | TD247/3 (11/2014) | - Update per editing instruction in Shanghai meeting report section 7.2, except WD16r1 clause 1.14 (MT TTP) - Figure 6-3: delete the operations enableGal and disableGal from MT_TTPBi. Reason: For Session and LSP the Gal is always enabled and should not be changed; for Channel (i.e. PW), Gal enable vs disable is indirectly determined by MI_CW_Enable, MI_CW_Enable = TRUE implies MI_Gal_Enable = True and MI_CW_Enable = FALSE implies MI_Gal_Enable = False, and therefore MI_Gal_Enable should not be changed independently. - Figure 6-6: change * to 0..1 for MIP Bi; change the operation getAllContainedMips to getContainedMip (double check with Bernd) - Add attribute CW_Enable to MT_TTP, applicable when MplsTpLayer=Channel, default=N/A (i.e. nullable) - Define MT_NE subclassed from M.3160 ME, with MEId=CC:ICC:NodeId - Update per WD20 proposals in WD06r1, WD15r1, WD16r1, WD19r1, and WD20, except WD16r1 on Data Dictionary 1.14 (MT_TTP) - Per VM3 decision, incorporate CD03 to add tables in clauses 6.3.2 – 6.3.5 and update table in 6.3.6 - Per VM3 decision, incorporate CD04 to add Table 7-1 in clause 7.1 on OAM supporting mechanism - Add JPEG images of the respective UML diagrams next to the figures for the purpose easier viewing, i.e. the images/diagrams can be zoomed in/out independent of the document text. To Do: - Resolve WD16r1(Shanghai) clause 1.14 (MT TTP): Need to resolve the list of existing attributes in the HTML and the list of attributes in WD16r1 comment - Update according to editing/drafting instruction from the virtual meeting minute (CD01r5). - Update Figures 5-6 and 5-7 (from EoT to MPLS-TP) |
| 0.11 | TD247R1/3 (12/2014) | Updated per C921 - Add abbreviations Updated per TD248/3 (VM meeting report) - In the model and Figure 6-8, move AdminState from MEP_So and MEP_Sk to the MEP abstract class - Remove the default value for aisOamTool and lckOamTool from object classes MT_ConnectionTerminationPointSink and MaintenanceEntityGroupEndPointSink - Remove sourceAddress parameter from the model establish[On-demand¦Proactive]1way measurement Job sink operation, test responder start, linkTraceResult of the MEP Sink object - In the Data Dictionary 1.20.2.2.3 and 1.20.2.2.4, replace “on-demand” by “proactive” in the description - MEP Sink MO, Operation establishProActive1WayMeasurementJobSink operation, output parameter measurementJob, change the data type from OnDemand1WayMeasurementJobControlSink to ProActive1WayMeasurementJobControlSink - MEP Source object (1.22), operation testInitiatorStart (1.22.2.8), has input parameters per MTDe_TT_So_MI_1TH_Start (CoS,Length,Period) and no output parameters. The operation testInitiatorTerminate (1.22.2.9) have no input parameter, one output parameter per MTDe_TT_So_MI_1TH_Result (Sent). Note that the management system at the responder MEP should be informed (by OOB) about the value of this output parameter in order to calculate the throughput. - MEP Sink object (1,21), operation testResponderStart (1.21.2.4) has no input parameter and no output parameter. The operation testResponderTerminate (1.21.2.5) has no input parameter, has output parameters per MTDe_TT_Sk_MI_1TH_Result (REC,CRC,BER,OO). Note that the management system at the responder MEP should be informed (by OOB) about the value of this output parameter in order to calculate the throughput. Updated per editing instruction from discussion on C848 - Updated Figure 6-5 for MEP and added Figure 6-6b (MIP with dot1 & dot2 subclasses) Provide the links for the draft G.8152 v0.11 RSA file, Papyrus file, HTML file, and Data Dictionary file. To do: - To update Tables 7-2, 7-3, 7-4, and 7-5 - Further modelling per discussion on C847 |
| 0.12 | WD19 (3/2015) Ottawa | Updates made: - Replaced the model files (RSA), HTML files, and DD files with the ones using the Open Model Profile in Clause 8 and Appendix III. Note: The Papyrus model file not replaced yet |
| 0.13 | WD19r1 (3/2015) Ottawa TD374/3 (7/2015) Geneva | Updates made: - Update clause 5.1 as proposed in WD08 with the modification that the Note will be below Figure 5-2. - Reference G.8013 for single/dual-ended protocol and one/two way measurement |
| 0.14 | TD374r1/3 | Updates made: - Per C1442: change the default value for the protectionType attribute to 1_FOR_1_BIDIRECTIONAL_WITH_APC. This default value applies to both SNC and trail protection types of MPLS-TP linear PS. - Re-factored 1way/2wayMeasurementJobControlSource/Sink to better indicate single/dual-ended and 1Way/2Way measurement: - MO: OnDemand[1WayDualEnded]MeasurementJobControlSource/Sink - MO: OnDemand[2WaySingleEnded]MeasurementJobControlSource/Sink - Operation: establishOnDemand [1WayDualEnded]MeasurementJobSource/Sink - Operation: establishOnDemand [2WaySingleEnded]MeasurementJobSource/Sink - Enumeration: 1WayOamType:1DM, 1SL, LM_CCM (i.e., add LM_CCM) - Enumeration: 2WayOamType:LM_LMM, DM, SL (i.e., remove LM_CCM) - Operation: establishProactive[1WayDualEnded]MeasurementJobSource/SinkG8113Dot1/2 - Operation: establishProactive[2WaySingleEnded]MeasurementJobSource/SinkG8113Dot1/2 - MO: ProActive[1WayDualEnded]MeasurementJobControlSource/SinkG8113Dot1/2 - MO: ProActive [2WaySingleEnded]MeasurementJobControlSource/SinkG8113Dot1/2 - And updated the corresponding description |
| 0.15 | WD15r1 (10/2015) Wuhan =TD514/3 (2/2016) | Updates done - Migrate the model profile from Luna to the Mars version of Eclipse - Note: Model G.8152 v0.15 = v0.14 + profile update To-do - To generate the GenDoc Data Dictionary and replace the RSA Data Dictionary - To drop the RSA HTML - To update object class description for grouping attributes according to the transport processes - To update the model per the discussion of WD06 for single/dual-ended measurement job control - To complete the description of the object classes, attributes, operations, parameters, and data type To do from v0.10: - Resolve WD16r1(Shanghai) clause 1.14 (MT TTP): Need to resolve the list of existing attributes in the HTML and the list of attributes in WD16r1 comment - Update according to editing/drafting instruction from the virtual meeting minute (CD01r5). - Update Figures 5-6 and 5-7 (from EoT to MPLS-TP) To do from v0.11: - To update Tables 7-2, 7-3, 7-4, and 7-5 - Further modelling per discussion on C847 |
| 0.16 | WD18 (2/2016) = TD514R1/3 (2/2016) | Update - Move to Papyrus 1.1.3 and based on OpenModelProfile 0.2.0 - Imported the core model from G.7711 v1.02 into G.8052 v1.10 - Normalized SN, SNC, PG to FD, FC, FcSwitch respectively. In particular the following object classes - MT_CrossConnection, MT_SubNetworkConnectionProtectionGroup - Generate the GenDoc Data Dictionary and replace the RSA Data Dictionary - Dropped the RSA HTML - Replaced the figures in clauses 6 and7 for normalizing SN, SNC, PG to FD, FC, and FcSwitch To do - Update Figures 5-6 & 5-7 from <<ituClass>> to <<openModelClass>> - To do from v0.10: - Resolve WD16r1(Shanghai) clause 1.14 (MT TTP): Need to resolve the list of existing attributes in the HTML and the list of attributes in WD16r1 comment - Update according to editing/drafting instruction from the virtual meeting minute (CD01r5). - Update Figures 5-6 and 5-7 (from EoT to MPLS-TP) - To do from v0.11: - To update Tables 7-2, 7-3, 7-4, and 7-5 - Further modelling per discussion on C847 - To update object class description for grouping attributes according to the transport processes - To update the model per the discussion of WD06 for single/dual-ended measurement job control - To complete the description of the object classes, attributes, operations, parameters, and data type |
| 0.17 | WD18 (9/2016) = TD620r1/P (9/2016) for consent | Updates - Accept all tracked changes - Removed galEnable from Mep - Moved adminState from Mep to MepSource - Define new attribute cvSeries in Mep without data type, inherited by MepSourceDotOne and MepSourceDotTwo, with CvSeriesDotOne and CvSeriesDotTwo data type respectively - Define new attribute and new operations: - Mep: 1ThOamTool, 1ThStart (), 1ThTerminate () - MepSourceDot1: 1ThStart (CoS,Pattern, Length,Period), 1ThTerminate (Send) - MepSinkDot1: 1ThStart (Pattern), 1ThTerminate (Rec, Crc, Ber, OO); number of received, Crc error, Ber error, out of order error - MepSourceDot2: 1ThStart (), 1ThTerminate () - MepSinkDot2: 1ThStart (), 1ThTerminate () - Deleted the operations having “?” as experimental, such as those in MepSource (linkTrace?, loopbackDiscover?, loopbackTest?, loopbackSeries?, loopbackTerminate?, testInitiatorStart?, and testInitiatorTeminate?) and MepSink (). - Removed the attributes from the MT TTP which are duplicated with those in the MEP. - Datatype 1WayOamType add values 1LM and 1SL - Marked all the measurement related classes as preliminary - Update clauses 3 and 4 per agreement on the C1929 discussion - In Figure 6-1, removed ODUkP-X-L_AP/MT per C2009 - Delete PhysicalSubsystem class - Add TP_Pac (from G.8052), being superclass of MT TTP - Deleted M.3160 - Editing instructions in TD613/3 (G.8152 virtual meeting) - Clone the Core model GlobalClass, LocalClass, and NetworkElement |

# Content

Document history:	2

Content	9

List of figures	12

List of tables	12

Summary	14

Keywords	14

Introduction	14

1	Scope	15

Figure 1-1 – Scope of G.8152 Interface	16

2	References	17

Figure 2-1 – Structure of References	18

3	Definitions	18

3.1	Terms defined elsewhere	18

3.2	Terms defined in this Recommendation	19

4	Abbreviations and acronyms	19

5	Conventions	21

5.1	Conventions defined elsewhere	21

5.1.1	Maintenance Entity Group End Point (MEP) [ITU-T G.8121]	21

Figure 5-1 –– Figure 9-25/G.8121 – MT NCM MEP compound functions	22

Figure 5-2 – Figure 9-26/G.8121 – MT TCM MEP compound functions	22

5.1.2	Maintenance Entity Group Intermediate Point (MIP) [ITU-T G.8121]	22

Figure 5-3 – Figure 9-27/G.8121 – MT MIP compound functions	23

Figure 5-4 – Figure 9-28/G.8121 – MT Half MIP compound functions	23

5.1.3	MEPs and MIPs along a Maintenance Entity	23

Figure 5-5 – MEPs and MIPs along a Maintenance Entity (ME)	23

5.2	Conventions defined in this Recommendation	24

5.2.1	Colour Code convention	24

Table 5-1 – Colour Code convention	24

5.2.2	Modelling convention for Adaptation Functions	24

6	Overview of the Model	25

Figure 6-1 – Over view of Object Class mapping to G.8121 Atomic Functions	25

6.1	MPLS-TP Sub-Layer	26

6.2	Basic Configuration Structure	26

6.2.1	High Level TP Containment Class Diagram	26

Figure 6-2 – High Level TP Containment Class Diagram	26

6.2.2	TP Inheritance Class Diagram	26

Figure 6-3 – TP Inheritance Class Diagram	27

6.2.3	Connection Fragment Class Diagram	28

Figure 6-4 – Connection Fragment Class Diagram	28

Editor Note: To be aligned with the CIM Core Model	28

6.2.4	MEP Class Diagram	28

Figure 6-5 – MEP Class Diagram	30

6.2.5	MIP Class Diagram	30

Figure 6-6 – MIP Class Diagram	30

Figure 6-6a – MIP and Dot1 & Dot2 Class Diagram	31

6.2.6	On Demand Measurement Class Diagram	31

Figure 6-7 – On Demand Measurement Class Diagram	34

6.2.7	Pro Active Measurement Class Diagram	34

Figure 6-8 – Pro Active Measurement and Generic Class Diagram	35

6.3	MI Grouping and Mapping	36

6.3.1	MPLS-TP Connection function (MT_C)	36

Table 6-1 – MI Groupings of the MPLS-TP Connection function	36

6.3.2	MPLS-TP Termination function (MT_TT)	36

Table 6-2 – MI Groupings of the MPLS-TP Termination function	36

6.3.3	MPLS-TP to MPLS-TP adaptation function (MT/MT_A)	40

Table 6-3 – MI Groupings of the	40

6.3.4	MT Diagnostic Functions (MTDe and MTDi)	42

Table 6-4 – MI Groupings of the	42

6.3.5	MPLS-TP to Non-MPLS-TP client adaptation functions	46

Table 6-5 – MI Groupings of the MPLS-TP to Non-MPLS-TP client adaptation functions	46

6.3.6	Non-MPLS-TP Server to MPLS-TP adaptation functions	48

Table 6-6 – MI Groupings of the Non-MPLS-TP Server to MPLS-TP adaptation functions	48

Table 6-7 – Derived MI Groupings of the MPLS-TP functions	52

Figure 6-9 – Modelling of non-MPLS-TP server adaptation MI groupings	54

7	Modelling of MPLS-TP functions	54

7.1	OAM compound functions	54

Table 7-1 – OAM Capability Support	55

7.1.1	MEP compound function	56

Figure 7-2 – Mandatory and optional MEPs	56

Figure 7-3 – MEP Class Diagram	57

Table 7-1 – On-demand measurement and Maintenance MI list	58

Table 7-2 – Pro-active measurement MI list	63

Table 7-3 – MEP configuration MI list	69

7.1.2	MIP compound function	70

Figure 7-10 – MIP configuration parameters	71

Figure 7-11 – "half MIP" compound function	72

Table 7-4 – MIP configuration MI list	74

7.2	Fault Management	75

7.3	Performance Monitoring	75

7.3.1	Loss Measurement	75

7.3.2	Delay Measurement	75

7.4	MPLS-TP Multiplexing	76

Table 7-5 – Mapping of Multiplexing related MI to G.8152 artefacts	76

7.5	Connection Function	76

Table 7-6 – Mapping of Connection Function related MI to G.8152 artefacts	77

7.5.1	Linear Protection Function	77

7.6	SCC/MCC Access Function	78

8	UML Model File	80

Bibliography	85

# List of figures

Figure 1-1 – Scope of G.8152 Interface	9

Figure 2-1 – Structure of References	12

Figure 5-1 –MT NCM MEP compound functions	15

Figure 5-2 – MT TCM MEP compound functions	16

Figure 5-3 MT MIP compound functions	16

Figure 5-4 – MT Half MIP compound functions	17

Figure 5-5 – MEPs and MIPs along a Maintenance Entity (ME)	17

Figure 5-6: Illustrative Diagram for MPLS-TP server Adaptation modelling	18

Figure 5-7: Illustrative Diagram for Non-MPLS-TP server Adaptation modelling	19

Figure 6-1 – Over view of Object Class mapping to G.8121 Atomic Functions	19

Figure 6-2 – High Level TP Containment Class Diagram	20

Figure 6-3 – TP Inheritance Class Diagram	21

Figure 6-4 – Connection Fragment Class Diagram	22

Figure 6-5 – MEP Class Diagram	23

Figure 6-6 – MIP Class Diagram	24

Figure 6-7 – On Demand Measurement Class Diagram	26

Figure 6-8 – Pro Active Measurement Class Diagram	26

Figure 6-9 – Modelling of non-MPLS-TP server adaptation MI groupings	33

Figure 7-1 – OAM Capabilities Structure	34

Figure 7-2 – Mandatory and optional MEPs	34

Figure 7-3 – MEP Class Diagram	35

Figure 7-4 – MEP on-demand diagnostic function	36

Figure 7-5 – On-demand Measurement and Maintenance	37

Figure 7-6 – MEP pro-active measurement function	38

Figure 7-7 – Pro-active Measurement	40

Figure 7-8 – MEP configuration function	40

Figure 7-9 – MEP configuration	41

Figure 7-10 – MIP configuration parameters	42

Figure 7-11 – "half MIP" compound function	42

Figure 7-12 – MIP/half MIP configuration function	43

Figure 7-13 – MIP class diagram	43

Figure 7-14 – Ethernet multiplexing configuration function	45

Figure 7-15 – Ethernet multiplexing configuration	45

Figure 7-16 – MT/SCC_A function, MT/MCC_A function, and MT/client_A function	47

Figure 7-17 – SCC / MCC Access Class Diagram	48

# List of tables

Table 5-1 – Colour Code convention	17

Table 6-1 – MI Groupings of the MPLS-TP to Non-MPLS-TP client adaptation functions	27

Table 6-2 – MI Groupings of the Non-MPLS-TP Server to MPLS-TP adaptation functions	29

Table 6-3 – Derived MI Groupings of the MPLS-TP functions	32

Table 7-1 – On-demand measurement and Maintenance MI list	36

Table 7-2 – Pro-active measurement MI list	38

Table 7-3 – MEP configuration MI list	40

Table 7-4 – MIP configuration MI list	43

Table 7-5 – Mapping of Multiplexing related MI to G.8152 artefacts	45

Table 7-6 – Mapping of Connection Function related MI to G.8152 artefacts	45

Table II-1 –Mapping between G.8121 Ethernet Atomic Functions and UML Model Artifacts	52

Draft revised Recommendation G.8152/Y.1375

Protocol-neutral management information model for the MPLS-TP network element

# Summary

This Recommendation contains the protocol neutral UML model for MPLS-TP NE Management.

This Recommendation provides a representation of the MPLS-TP technology using the methodologies that have been used for other transport technologies (e.g. SDH, OTN and Ethernet).

# Keywords

Information Model, Protocol-neutral, MPLS-TP, Transport Resource, UML

# Introduction

This Recommendation contains the object classes for the MPLS-TP NE management. It includes the Termination Points (TP), Maintenance Entity Group (MEG) End Point (MEP), MEG Intermediate Point (MIP), Traffic Conditioning & Shaping (TCS), Loss Measurement (LM), Delay Measurement (DM), and the general Performance Monitoring (PM) Current Data (CD) and History Data (HD).

The TP, MEP, MIP, LM, DM, and TCS object classes support the configuration and fault management functions as specified in Recommendation [ITU-T G.8151].

The MPLS-TP TPs are modeled as subclasses of the generic GlobalClass defined in Recommendation [ITU-T G.7711] and extending the LTP and LP classes of [ITU-T G.7711].

The MPLS-TP general PM CD and HD object classes are modelled as subclasses of the generic Current Data and History Data defined in Recommendation [ITU-T Q.822].

The MPLS-TP general CD and HD object classes support only the Quality of Service (QoS) directly related PM parameters, i.e. Severely Errored Second (SES) and Unavailable Second (UAS), for Service Level Agreement (SLA) verification. The additional PM object classes for supporting Loss Measurement (LM) and Delay Measurement (DM) monitoring uses the general CD and HD object classes as super classes.

The object model defined in this Recommendation is protocol-neutral with respect to management protocols. The model could be used as the base for further defining the information model for any specific management protocol.

The model in this Recommendation has been specified using the Open Source UML modelling tool “Papyrus”.

Draft revised Recommendation G.8152/Y.1375

Protocol-neutral management information model for the MPLS-TP network element

# Scope

This Recommendation provides a management protocol-neutral information model for managing network elements in the MPLS-TP transport network as defined in Recommendation [ITU-T G.8110.1/Y.1370.1]. It identifies the telecommunications management network (TMN) managed entities required for the management of MPLS-TP transport network elements. These entities are relevant to information exchanged across standardized interfaces defined in the [ITU-T M.3010] TMN architecture. The management protocol-neutral information model should be used as the base for defining management protocol-specific information models, for example XML (Web Service or Netconf/Yang) information model, CORBA IDL model, and SNMP MIB.

The information model defined in this Recommendation is an augmentation to the generic code model specified in Recommendation [ITU-T G.7711] for managing Ethernet transport resources. The core information model defined in [ITU-T G.7711] can be used as the base for the extension of Ethernet-specific information models.

The specific mapping of the management protocol-neutral model into management protocol-specific model is the decision of the management protocol-specific solution design. For example, an object class defined in this Recommendation may be mapped into multiple tables in a SNMP MIB. Protocol-specific solutions and their mapping from the protocol-neutral model may be described in other Recommendations and is out of scope of this Recommendation.

This Recommendation applies to MPLS-TP transport network elements and those systems in the TMN that manage such network elements. Functional capabilities of MPLS-TP transport equipment are defined in Recommendations [ITU-T G.8121/Y.1381], [ITU-T G.8121.1/Y.1381.1], [ITU-T G.8121.2/Y.1381.2] and requirements of the management of MPLS-TP transport equipment are provided in Recommendation [ITU-T G.7710/Y.1701] and Recommendation [ITU-T G.8151/Y.1374]. The information model specified in this Recommendation applies to the EMS-NE interface, as shown in Figure 1-1, specifically for managing the MPLS-TP functional capabilities of the NE.

Figure 1-1 – Scope of G.8152 Interface

The object classes defined in this Recommendation cover the areas of fault management, configuration management, and performance management.

There are several different perspectives from which management information may be defined for management purposes. The network element viewpoint is concerned with the information that is required to manage a network element. This refers to information required to manage the network element function and the physical aspects of the network element. This Recommendation addresses only the network element view of Ethernet transport network management.

The management-protocol-neutral information model specified in this Recommendation consists of a set of transport-technology-specific managed object classes, i.e., MPLS-TP-specific managed object classes. These MPLS-TP-specific managed object classes are inherited from the generic managed object classes defined in other ITU-T Recommendation such as [ITU-T M.3160], including Managed Element, Termination Point and its subclasses, Subnetwork, and Subnetwork Connection . Because of object class inheritance, the MPLS-TP management information model also inherits the generic object management capabilities, such as object creation/deletion, notification of object creation/deletion, attribute value retrieval/modification, notification of attribute/state value change, scoped and filtered retrieval of object instances, and abortion of outstanding operations. The description of these generic object management capabilities is provided in other ITU-T Recommendations, such as the M.3700 series, and therefore is outside the scope of this Recommendation.

The object classes defined in this Recommendation cover the areas of fault management, configuration management, and performance management.

This Recommendation provides a representation of the MPLS-TP technology using the methodologies that have been used for other transport technologies (e.g. SDH, OTN and Ethernet).

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T G.7710/Y.1701]	ITU-T Recommendation G.7710/Y.1701 (02/2012), Common equipment management function requirements, plus Amendment 1 (9/2016)

[ITU-T G.7711]	ITU-T Recommendation G.7711/Y.1702 (9/2016), Generic protocol-neutral information model for transport resources

[ITU-T G.8052/Y.1346]	ITU-T Recommendation G.8052/Y.1346 (9/2016), “Protocol-neutral management information model for the Ethernet Transport network element”

[ITU-T G.8101/Y.1355]	ITU-T Recommendation G.8101/Y.1355 (1/2015), Terms and definitions for MPLS transport profile

[ITU-T G.8110.1/Y.1370.1]	ITU-T Recommendation G.8110.1/Y.1370.1 (12/2011), Architecture of MPLS Transport Profile (MPLS-TP) layer network

[ITU-T G.8113.1/Y.1372.1]	ITU-T Recommendation G.8113.1/Y.1372.1 (4/2016), Operations, Administration and Maintenance mechanism for MPLS-TP in Packet Transport Network (PTN)

[ITU-T G.8113.2/Y.1372.2]	ITU-T Recommendation G.8113.2/Y.1372.2 (8/2015), Operations, administration and maintenance mechanisms for MPLS-TP networks using the tools defined for MPLS

[ITU-T G.8121/Y.1381]	ITU-T Recommendation G.8121/Y.1381 (4/2016), Characteristics of MPLS-TP equipment functional blocks

[ITU-T G.8121.1/Y.1381.1]	ITU-T Recommendation G.8121.1/Y.1381.1 (4/2016), Characteristics of MPLS-TP equipment functional blocks supporting G.8113.1/Y.1373.1

[ITU-T G.8121.2/Y.1381.2]	ITU-T Recommendation G.8121.2/Y.1381.2 (4/2016), Characteristics of MPLS-TP equipment functional blocks supporting G.8113.2/Y.1373.2

[ITU-T G.8131/Y.1382]	ITU-T Recommendation G.8131/Y.1382 (07/2014), Linear protection switching for MPLS transport profile (MPLS-TP), plus Amendment 1 (4/2016)

[ITU-T G.8151/Y.1374]	ITU-T Recommendation G.8151/Y.1374 (1/2015), Management aspects of the MPLS-TP network element, plus Amendment 1 (4/2016)

[ITU-T M.3010]	ITU-T Recommendation M.3010 (02/2000), Principles for a telecommunications management network

[ITU-T M.3160]	ITU-T Recommendation M.3160 (11/2008), Generic Management Information Model – Protocol Neutral

[ITU-T Q.822]	ITU-T Recommendation Q.822 (04/1994), Stage 1, Stage 2 and Stage 3 Description for the Q3 interface – Performance Management

Figure 2-1 – Structure of References

# Definitions

## Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

### Terms defined in [ITU-T G.8001]

Maintenance entity (ME)

Maintenance entity group (MEG)

Maintenance entity group end point (MEP) compound source function

Maintenance entity group intermediate point (MIP) compound function

On-demand monitoring

Proactive monitoring

Dual-ended

One-way

Single-ended

Two-way

## Terms defined in this Recommendation

This Recommendation defines the following terms:

# Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

1DM	One-way Delay Measurement

1DMo	On-demand one-way Delay Measurement

1DMp	Proactive one-way Delay Measurement

1SL	One-way Synthetic Loss Measurement

1SLp	Proactive one-way Synthetic Loss Measurement

AIS  	Alarm Indication Signal

APS  	Automatic Protection Switching

CCM  	Continuity Check Message

CD	Current Data

CII	Common Interworking Indicator

CSF  	Client Signal Fail

CTP  	Connection Termination Point

CW	Control Word

DEG           Degraded

DM	Delay Measurement

DMo           On-demand Delay Measurement

DMp	Proactive Delay Measurement

DP	Data Plane

ECC	Embedded Communication Channel

EMS	Element Management System

ETH	Ethernet MAC layer network

G-ACh	Generic Associated Channel

GAL	Generic Associated Channel (G-ACh) Label

GFP	Generic Framing Procedure

GUI	Graphical User Interface

HD	History Data

HTML	HyperText Markup Language

LB  	Loop Back

LBM 	Loop Back Message

LBR  	Loop Back Reply

LCK 	Locked

LCT	Local Craft Terminal

LM	Loss Measurement

LMM          Loss Measurement Message

LMo  	On-demand Loss Measurement

LMp	Proactive Loss Measurement

LMR           Loss Measurement Reply

LOC  	Loss Of Continuity

LSP	Label Switched Path

LT  	Link Trace

LTM           LinkTrace Message

LTR  	Link Trace Reply

MAC 	Medium Access Control

MCC	Management Communication Channel

ME	Maintenance Entity

MEG	Maintenance Entity Group

MEP	Maintenance entity group End Point

MI	Management Information

MIB	Management Information Base

MIP	Maintenance entity group Intermediate Point

MPLS	Multi-Protocol Label Switching

MPLS-TP	Multi-Protocol Label Switching – Transport Profile

MS-PW	Multi-Segment Pseudowire

MT	Multi-Protocol Label Switching – Transport Profile

MTD	MPLS-TP Diagnostic function

MTDi	MPLS-TP Diagnostic function within MTx MIP

MTS	MPLS-TP Section

NC	Network Connection

NCM  	Network Connection Monitoring

NE	Network Element

NMS	Network Management System

OAM	Operation, Administration and Maintenance

PHB	Per Hop Behaviour

PM	Performance Monitoring

PRI	Priority

PSC	PHB Scheduling Class

PSN	Packet Switched Network

PTN	Packet Transport Network

PW	Pseudo-Wire

QoS	Quality of Service

RDI  	Remote Defect Indication

SCC	Signalling Communication Channel

SCC Type	Signalling Communication Channel Type

SES	Severely Errored Second

Sk	Sink

SLA	Service Level Agreement

SL	Synthetic Loss Measurement

SLp	Proactive Synthetic Loss Measurement

SLo	On-demand Synthetic Loss Measurement

SN	Sub-Network

SNC	Sub-Network Connection

SNCP	Sub-Network Connection Protection

SNMP	Simple Network Management Protocol

So	Source

SQ	SeQuence

SSF  	Server Signal Fail

TC	Traffic Class

TCM	Tandem Connection Monitoring

TCS	Traffic Conditioning & Shaping

TH	Throughput

TMN	Telecommunications Management Network

TP	Termination Point

TSNUM	Tributary Slot Number

TT	Trail Termination

TTL	Time-To-Live

TTP	Trail Termination Point

UAS	UnAvailable Second

UML	Unified Modelling Language

# Conventions

## Information modelling conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.1.

### UML modelling conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.1.

### Model Artefact Lifecycle Stereotypes conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.2.

### Forwarding entity terminology conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.3.

### Conditional package conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.4.

### Pictorial diagram conventions

See ITU-T Rec.G.7711/Y.1702 clause 5.5.

## Equipment Function Conventions

### Maintenance Entity Group End Point (MEP) [ITU-T G.8121]

MEG End Points (MEPs) terminate Maintenance Entities (MEs) which can span the end-to-end Network Connection or a portion of the Network Connection defined as a Tandem Connection.

The diagrammatic convention for Network Connection Monitoring MEP (NCM MEP) compound functions is shown in Figure 5-1:

Figure 5-1 –– Figure 9-25/G.8121 – MT NCM MEP compound functions

The diagrammatic convention for Tandem Connection Monitoring MEP (TCM MEP) compound functions is shown in Figure 5-2:

Figure 5-2 – Figure 9-26/G.8121 – MT TCM MEP compound functions

Note: Unlike the Ethernet technology, the same MT/MT atomic function defined in [ITU-T G.8121] can be used either within the optional TCM MEP (i.e., not "stand alone") or at the layer boundary (i.e., "stand alone" and not be a part of a MEP), regardless of the number of client signals (even in case of only one signal when there is no multiplexing).

### Maintenance Entity Group Intermediate Point (MIP) [ITU-T G.8121]

The diagrammatic convention for MIP compound functions is shown in Figure 5-3:

Figure 5-3 – Figure 9-27/G.8121 – MT MIP compound functions

The diagrammatic convention for half MIP compound functions is shown in Figure 5-4:

Figure 5-4 – Figure 9-28/G.8121 – MT Half MIP compound functions

### MEPs and MIPs along a Maintenance Entity

The diagrammatic convention for MEPs and MIPs along an individual ME as shown in Figure 5-5:

Figure 5-5 – MEPs and MIPs along a Maintenance Entity (ME)

Note that the ME can span the whole end-to-end Network Connection or a portion of it called a Tandem Connection.

## Conventions defined in this Recommendation

This Recommendation uses the following conventions:

### Colour Code convention

The following "colour code" is used in this document:

Table 5-1 – Colour Code convention

| "colour code" | G.8152 Object Class |
| --- | --- |
|  | MaintenanceEntityGroupEndPoint |
|  | MT_TrailTerminationPoint |
|  | MT_ConnectionTerminationPoint |
|  | OnDemandMeasurementJob |
|  | ProActiveMeasurementJob |
|  | MaintenanceJob |
|  | TerminationPointPool |
|  | specific highlighting |
|  | not in scope |

### Modelling convention for Adaptation Functions

Every adaptation function has a MI_Active parameter. This is not modelled since it will always be active in the MPLS-TP technology.

#### MPLS-TP Server Adaptation Modelling

Figure 5-6: Illustrative Diagram for MPLS-TP server Adaptation modelling

#### Non-MPLS-TP Server Adaptation Modelling

Figure 5-7: Illustrative Diagram for Non-MPLS-TP server Adaptation modelling

# Overview of the Model

Figure 6-1 below shows the mapping between the object classes and the MPLS-TP atomic functions defined in Figure 1/G.8121/Y.1381.

Figure 6-1 – Over view of Object Class mapping to G.8121 Atomic Functions

(based on Figure 1/G.8121/Y.1381)

Figure 6-1A below shows the containment relationship among the MPLS-TP object classes defined in this Recommendation. The containment relationship reflects the lifecycle dependency between object instances.

Figure 6-1A –MPLS-TP Object Class Containment Relationship

Figure 6-1B below shows the relationship of the MPLS-TP object classes to the GlobalClass and LocalClass objects defined in the Recommendation [ITU-T G.7711] Core Model for inheriting the uuid or the localIdList attributes. See Recommendation [ITU-T G.7711] Annex C for how the identifier attributes uuid and localIdList are used for identifying object instance.

Figure 6-1B –MPLS-TP Object Class Identifier

## Basic Configuration Structure

This section contains the initial high level class diagrams. Once completed, they will be moved to the corresponding sections in the Recommendation.

### High Level TP Containment Class Diagram

Figure 6-2 – High Level TP Containment Class Diagram

### TP Inheritance Class Diagram

Figure 6-3 – TP Inheritance Class Diagram

### Connection Fragment Class Diagram

Figure 6-4 – Connection Fragment Class Diagram

### MEP Class Diagram

Figure 6-5 – MEP Class Diagram

### MIP Class Diagram

Figure 6-6 – MIP Class Diagram

Figure 6-6a – MIP and Dot1 & Dot2 Class Diagram

### On Demand Measurement Class Diagram

Figure 6-7 – On Demand Measurement Class Diagram

### Pro Active Measurement Class Diagram

Figure 6-8 – Pro Active Measurement and Generic Class Diagram

## MI Grouping and Mapping

This section lists (from [ITU-T G.8121/Y.1381]) all atomic functions together with their MI. It is structured according to the clause 9 of G.8121:

Conventions used in the tables:

- Atomic functions having the same list of MI are grouped.

- Fault and Performance MIs are not shown.

- MI of adaptation functions are identified as "client layer related" (blue) or "server layer related" (red).

### MPLS-TP Connection function (MT_C)

Table 6-1 – MI Groupings of the MPLS-TP Connection function

| Symbol | Management Information | Managed Object Class |
| --- | --- | --- |
| MPLS-TP Connection function |  |  |
|  | MI_MatrixControl per matrix connection: MI_ConnectionType MI_Return_CP_ID MI_ConnectionPortIds per SNC pProtection group: MI_PS_WorkingPortId MI_PS_ProtectionPortId MI_PS_ProtType MI_PS_OperType MI_PS_HoTime MI_PS_WTR MI_PS_ExtCMD MI_PS_BridgeType MI_PS_SD_Protection |  |

### MPLS-TP Termination function (MT_TT)

Table 6-2 – MI Groupings of the MPLS-TP Termination function

| Symbol | Management Information | Managed Object Class |
| --- | --- | --- |
| MPLS-TP (MT) trail termination source function |  |  |
|  | MI_GAL_Enable MI_TTLVALUE MI_ MEG_ID MI_ MEP_ID MI_CC_OAM_Tool MI_RDI_OAM_Tool MI_CC_Enable MI_CVp_Enable MI_CC_CoS MI_CC_Period MI_LMp_OAM_Tool MI_LMp_Enable[i] MI_LMp_Period[i] MI_LMp_CoS[i] MI_DMp_OAM_Tool MI_DMp_Enable[i] MI_DMp_Period[i] MI_DMp_Test_ID[i] MI_DMp_CoS[i] MI_DMp_Length[i] MI_1DMp_OAM_Tool MI_1DMp_Enable[i] MI_1DMp_Period[i] MI_1DMp_Test_ID[i] MI_1DMp_Length[i] MI_1DMp_CoS[i] MI_SLp_OAM_Tool MI_SLp_Enable[i] MI_SLp_Period[i] MI_SLp_Test_ID[i] MI_SLp_Length[i] MT_TT_So_MI_SLp_CoS[i] |  |
| G.8121.1 specific | MI_LMC_Enable |  |
| G.8121.2 specific | MI_CCCV_Mode[i] MI_Local_Discr MI_DMp_CopyPad[i] MI_LMp_Test_ID[i] MI_LMp_LMType[1...MLMp] MI_LMp_CountBytes[i] |  |
| MPLS-TP (MT) trail termination sink function |  |  |
|  | MI_GAL_Enable MI_ MEG_ID MI_ PeerMEP_ID MI_ CC_OAM_Tool MI_ RDI_OAM_Tool MI_CC_Enable MI_CVp_Enable MI_CC_Period MI_CC_CoS MI_Get_SvdCC MI_LMp_OAM_Tool MI_LMp_Enable[i] MI_LMp_CoS[i] MI_LM_DEGM MI_LM_M MI_LM_DEGTHR MI_LM_TFMIN MI_ DMp_OAM_Tool MI_DMp_Enable[i] MI_DMp_CoS[i] MI_ 1DMp_OAM_Tool MI_1DMp_Enable[i] MI_1DMp_Test_ID[i] MI_ SLp_OAM_Tool MI_SLp_Enable[i] MI_SLp_CoS[i] MI_AIS_OAM_Tool MI_LCK_OAM_Tool MI_1second |  |
| G.8121.1 specific | MI_LMC_Enable |  |
| G.8121.2 specific | MI_CCCV_Mode[i] MI_Remote_Discr[i] MI_PM_ClearError MI_PM_Responder_Enable |  |

### MPLS-TP to MPLS-TP adaptation function (MT/MT_A)

Table 6-3 – MI Groupings of the

| Symbol | Management Information (blue: client layer related red: server layer related) | Managed Object Class |
| --- | --- | --- |
| MPLS-TP to MPLS-TP adaptation source function |  |  |
|  | MI_Active MI_Admin_State MI_Label[i] MI_LSPType[i] MI_CoS[i] MI_PHB2TCMapping[i] MI_QoSEncodingMode[i] MI_Mode MI_LCK_Period[i] MI_LCK_CoS[i] MI_LCK_OAM_Tool[i] MI_GAL_Enable[i] (Note: should be MI_GAL_Enable set double (Server and clinet)?) MI_APS_CoS MI_APS_OAM_Tool |  |
| MPLS-TP to MPLS-TP adaptation sink function |  |  |
|  | MI_Active MI_AdminState MI_Label[i] MI_LSPType[i] MI_CoS[i] MI_ TC2PHBMapping[i] MI_QoSDecodingMode[i] MI_Mode MI_AIS_Period[i] MI_AIS_CoS[i] MI_AIS_OAM_Tool[i] MI_LCK_Period[i] MI_LCK_CoS[i] MI_LCK_OAM_Tool[i] MI_APS_OAM_Tool MI_GAL_Enable [i] (Note: should be MI_GAL_Enable set double (Server and clinet)?) |  |
| G.8121.2 specific | MI_Local_Defect[i] |  |

### MT Diagnostic Functions (MTDe and MTDi)

Table 6-4 – MI Groupings of the

| Symbol | Management Information | Managed Object Class |
| --- | --- | --- |
| MT Diagnostic trail termination source function for MEP (MTDe_TT_So) |  |  |
|  | MI_ GAL_Enable MI_TTLVALUE MI_CV_OAM_Tool MI_CV_Series () MTDe_TT_So_MI_1TH_OAM_Tool MI_1TH_Start (CoS,Length,Period) MI_1TH_Terminate MI_ LMo_OAM_Tool MI_LMo_Start(CoS,Period) [i] MI_LMo_Terminate[i] MI_ DMo_OAM_Tool MI_DMo_Start (CoS,Test_ID,Length,Period)[i] MI_DMo_Terminate[i] MI_ 1DMo_OAM_Tool MI_1DMo_Start(CoS,Test_ID,Length,Period)[i] MI_1DMo_Terminate[i] MI_ SLo_OAM_Tool MI_SLo_Start (CoS,Test_ID,Length,Period)[i] MI_SLo_Terminate[i] MI_Admin_State MI_Lock_Instruct_Enable MI_DP_Loopback_Enable |  |
| G.8121.1 specific | MI_MEP_ID MI_CV_Series (Target MEP/MIP ID, CoS, N, Length, Period) MI_CV_Test(CoS, Pattern, Length, Period) |  |
| G.8121.2 specific | MI_CV_Series (Session_ID, Count, Period, CoS, Size, ValidateFEC, ValidateReverse, TargetFECStack) MI_CV_Trace (Session_ID, CoS, ValidateFEC, ValidateReverse, TargetFECStack) MI_FEC_Checking MI_Target_FEC MI_Ifnum MI_MTU MI_DMo_Start(CoS, Test_ID, Length, Period, CopyPad)[i] MI_LMo_Start(CoS, Test_ID, Period, LMType,CountBytes)[i] LMDMo_Start(CoS, Test_ID, Length, Period, LMType, CountBytes, CopyPad)[i] MI_LMDMo_Terminate [i] MI_LI_Period MI_LI_MEPID MI_LI_CoS |  |
| MT Diagnostic trail termination sink function for MEP (MTDe_TT_Sk) |  |  |
|  | MI_ GAL_Enable MI_CV_OAM_Tool MI_1TH_OAM_Tool MI_1TH_Start MI_1TH_Terminate MI_ LMo_OAM_Tool MI_ DMo_OAM_Tool MI_ 1DMo_OAM_Tool MI_1DMo_Start(Test_ID)[i] MI_1DMo_Terminate[i] MI_ SLo_OAM_Tool MI_DP_Loopback_Enable |  |
| G.8121.1 specific | MI_MEP_ID MI_1TH_Start(Period) |  |
| G.8121.2 specific | MI_FEC_Checking PM_Responder_Enable |  |
| MT Diagnostic trail termination source function for MIP (MTDi_TT_So) |  |  |
|  | MI_GAL_Enable MI_TTLVALUE MI_MIP_ID MI_CV_OAM_Tool MI_DP_Loopback_Enable |  |
| G.8121.2 specific | MI_Target_FEC MI_Ifnum MI_MTU |  |
| MT Diagnostic trail termination sink function for MIP (MTDi_TT_Sk) |  |  |
|  | MI_GAL_Enable MI_MIP_ID MI_CV_OAM_Tool MI_DP_Loopback_Enable |  |
| G.8121.2 specific | MI_FEC_Checking |  |

### MPLS-TP to Non-MPLS-TP client adaptation functions

Table 6-5 – MI Groupings of the MPLS-TP to Non-MPLS-TP client adaptation functions

| Symbol | Management Information (blue: client layer related red: server layer related) | Managed Object Class |
| --- | --- | --- |
| MPLS-TP to Non-MPLS-TP client adaptation source functions |  |  |
|  | MI_AdminState MI_FCSEnable MI_CWEnable MI_SQUse MI_PRI2PSCMapping MI_MEP_MAC* MI_Client_MEL* MI_LCK_Period* MI_LCK_Pri* MI_MEL* * ETH OAM related |  |
|  | MI_Active MI_ECC_CoS MI_GAL_Enable |  |
|  | MI_Active MI_ECC_CoS MI_GAL_Enable |  |
| MPLS-TP to Non-MPLS-TP client adaptation sink functions |  |  |
|  | MI_Admin_State MI_LCK_Period * MI_LCK_Pri * MI_Client_MEL * MI_MEP_MAC * MI_AIS_Pri * MI_AIS_Period * MI_MEL MI_FCSEnable MI_CWEnable MI_SQUse MI_GAL_Enable MI_CoS2PRIMapping MI_PRI2PSCMapping MI_PSC2CoSMapping * ETH OAM related |  |
|  | MI_Active MI_GAL_Enable |  |
|  | MI_Active MI_GAL_Enable |  |

### Non-MPLS-TP Server to MPLS-TP adaptation functions

Table 6-6 – MI Groupings of the Non-MPLS-TP Server to MPLS-TP adaptation functions

| Symbol | Management Information (blue: client layer related red: server layer related) | Managed Object Class |
| --- | --- | --- |
| Non-MPLS-TP Server to MPLS-TP adaptation source functions |  |  |
|  | MI_Active MI_SCCType (not in ETH/MT) MI_Etype (only in ETH/MT) MI_Label[1…M] MI_LSPType[1…M] MI_CoS[1…M] MI_PHB2TCMapping[1…M] MI_QoSEncodingMode[1…M] MI_Mode[1…M] MI_GAL_Enable[1…M] Only in ODUkP-h/MT: MI_INCREASE MI_DECREASE MI_TSNUM MI_ODUflexRate |  |
| Non-MPLS-TP Server to MPLS-TP adaptation sink functions |  |  |
|  | MI_Active MI_SCCType (not in ETH/MT) MI_Etype (only in ETH/MT) MI_Frame_Type_Config (only in ETH/MT) MI_Label[1…M] MI_LSPType[1…M] MI_CoS[1…M] MI_TC2PHBMapping[1…M] MI_QoSDecodingMode[1…M] MI_Mode MI_LCK_Period[1…M] MI_LCK_CoS[1…M] MI_Admin_State MI_AIS_Period[1…M] MI_AIS_CoS[1…M] MI_GAL_Enable[1…M] MI_LCK_OAM_Tool [1…M] MI_AIS_OAM_Tool[1…M] Only in ODUkP-h/MT: MI_INCREASE MI_DECREASE MI_AcSL MI_AcEXI MI_LastInvalidUPI MI_cPLM MI_cLFD MI_cEXM MI_cUPM (not ETH/MT) |  |
| G.8121.2 specific | MI_Local_Defect[i] |  |

Based on the groupings above the following packages have been identified.

Table 6-7 – Derived MI Groupings of the MPLS-TP functions

| MI groupings | Package/Object class name |
| --- | --- |
| TTP MI source grouping |  |
| MI_Active |  |
| MI_INCREASE MI_DECREASE MI_TSNUM MI_ODUflexRate | HaoCapableTtpSource_Pac |
| TTP MI sink grouping |  |
| MI_Active |  |
| MI_Frame_Type_Config | EthServerTtpSink_Pac |
| MI_INCREASE MI_DECREASE | HaoCapableTtpSink_Pac |
| TTP MI grouping |  |
| MI_SCCType | GfpMapping_Pac |
| MI_Etype | EthServerTtp_Pac |
| MT_CTP MI source grouping |  |
| MI_PHB2TCMapping[1…M] MI_QoSEncodingMode[1…M] MI_APS_OAM_CoS[1…M] | MT_ConnectionTerminationPointSource |
| MT_CTP MI sink grouping |  |
| MI_TC2PHBMapping[1…M] MI_QoSDecodingMode[1…M] MI_LCK_Period[1…M] MI_LCK_CoS[1…M] MI_Admin_State MI_AIS_Period[1…M] MI_AIS_CoS[1…M] MI_LCK_OAM_Tool [1…M] MI_AIS_OAM_Tool[1…M] | MT_ConnectionTerminationPointSink |
| MT_CTP MI grouping |  |
| MI_Label[1…M] MI_LSPType[1…M] MI_CoS[1…M] MI_Mode[1…M] MI_GAL_Enable[1…M] MI_APS_OAM_Tool[1…M] | MT_ConnectionTerminationPoint |

Figure 6-9 – Modelling of non-MPLS-TP server adaptation MI groupings

# Modelling of MPLS-TP functions

## OAM compound functions

OAM is done in the network by creating Maintenance Entities (ME). In multipoint services multiple MEs are grouped together forming a Maintenance Entities Group (MEG); see definitions in section 8 (MPLS-TP OAM Architecture) of [ITU-T G.8110.1/Y.1370.1].

Each MEG is terminated by a set of MEG End Points (MEPs). It is also possible to perform OAM functions on a MEG by MEG Intermediate Points (MIPs) which allow a limited set of OAM functions along the MEs.

Table 7-1 – OAM Capability Support

| OAM function |  |  | OAM mechanism |
| --- | --- | --- | --- |
| Compound function | MEP | Network Connection Monitoring |  |
|  |  | Tandem Connection Monitoring |  |
|  | MIP |  | (on-demand) CV |
|  | TCS |  | -- |
| Proactive Measurement | Loss Measurement | 1-way | CCM (G.8121.1) |
|  |  | 2-way | LM (G.8121, G.8121.2), LMDM (G.8121.2) |
|  |  | 1-way synthetic | -- |
|  |  | 2-way synthetic | SLM (G.8121) LM (G.8121.2) LMDM (G.8121.2) |
|  | Delay Measurement | 1-way | 1DM (G.8121) |
|  |  | 2-way | DM (G.8121, G.8121.2) LMDM (G.8121.2) |
| On-demand Measurement | Loss Measurement | 2-way | LM (G.8121, G.8121.2), LMDM (G.8121.2) |
|  |  | 1-way synthetic | -- |
|  |  | 2-way synthetic | SLM (G.8121) LM (G.8121.2) LMDM (G.8121.2) |
|  | Delay Measurement | 1-way | 1DM (G.8121, G.8121.1) |
|  |  | 2-way | DM (G.8121, G.8121.1, G.8121.2) LMDM (G.8121.2) |
|  | Maintenance | 1-way throughput test | 1TH (G.8121, G.8121.1) |
|  |  | On-demand loop back | CV (G.8121, G.8121.1, G.8121.2) |
|  |  | On-demand link trace | CV (G.8121) CV (G.8121.2) |
| Proactive Fault management | Continuity Check and Connectivity Verification |  | CC/CV (G.8121, G.8121.2) CCM (G.8121.1) |
|  | Remote Defect Indication |  | RDI (G.8121, G.8121.1, G.8121.2) |
|  | Alarm Indication Signal |  | AIS (G.8121, G.8121.1, G.8121.2) |
|  | Locked Signal (Lock Report) |  | LCK (G.8121, G.8121.1, G.8121.2) |
| On-demand Fault management | Connectivity Verification |  | CV (G.8121, G.8121.1, G.8121.2) |
|  | Lock Instruction |  | LKI (G.8121.2) |
|  | Automatic Protection Switching |  | APS (G.8121) |
|  | Management communication channel/ Signalling communication channel |  | MCC/SCC (G.8121) |

Note:

- OAM mechanisms for MEP are all the OAMs defined in G.8121 series. (The exception is APS. NCM MEP does not support APS.)

- OAM mechanism for MIP is (on-demand) CV only

### MEP compound function

There are two different types of MEP compound functions:

- mandatory NCM MEPs at the boundary of a layer network, monitoring a network connection

- optional TCM MEPs in the middle of a layer network, monitoring a tandem connection.

Figure 7-2 – Mandatory and optional MEPs

From management point of view a MEP has the following constraints:

- A MEP can be unidirectional or bidirectional; unidirectional MEPs have a limited set of OAM functionality.

- A MEP belongs to one, and only one, MEL.
MEPs terminate MEGs and each MEG is associated to one MEL.

- A MEP is addressed by one, and only one, MAC address.
The MAC address (or more precise the EUI-48) is bound to a physical subsystem and one physical subsystem can hold thousands of MEP functions, and all those MEP functions share in such case one MAC address.

The MEP compound function supports three applications which are organised in "jobs":

- On-demand measurement job

- Pro-active measurement job

- Maintenance job.

Figure 7-3 – MEP Class Diagram

Also see lower part of Figure 6-5Note: Figure 7-3 is later on divided into Figure 7-5, Figure 7-7 and Figure 7-9 below describing the individual segments of the MEP model.

The Management Information (MI) of the compound functions (defined in [ITU-T G.8121/Y.1381]) needs to be mapped to G.8152 artefacts. The following sections list all the MIs defined for the MEP compound function in tables and associate them to applications (coloured background). The corresponding part of the model is shown below the table.

#### MEP on-demand diagnostic function

The MEP on-demand diagnostic function exists in NCM and TCM MEPs.

Figure 7-4 – MEP on-demand diagnostic function

#### MEP pro-active measurement function

The MEP pro-active measurement function exists in NCM and TCM MEPs.

Figure 7-6 – MEP pro-active measurement function

#### MEP configuration function

The MEP configuration function exists in NCM and TCM MEPs.

Figure 7-8 – MEP configuration function

### MIP compound function

Figure 7-10 – MIP configuration parameters

# Figure 7-11 – "half MIP" compound function

The Management Information (MI) of the MIP compound function (defined in [ITU-T G.8121/Y.1381]) is mapped to the MipBidirectional object class.

Figure 7-12 – MIP/half MIP configuration function

## Fault Management

FFS

## Performance Monitoring

Performance Monitoring allows measurement of different performance parameters like Frame Loss Ratio, Frame Delay and Frame Delay Variation.

### Loss Measurement

The Frame Loss Measurement (LM) provides performance data that is based on the lost frames between the ingress and the egress of a Maintenance Entity (ME); i.e., between two maintenance Group End Points (MEPs).

LM is restricted to MEGs which have only a single ME.

The following LM functions are defined:

-	2-way on-demand LM

-	1-way on-demand synthetic LM

-	2-way on-demand synthetic LM

-	2-way pro-active LM.

-	1-way pro-active synthetic LM

-	2-way pro-active synthetic LM

The single-ended on-demand LM function is managed only at the source MEP. The sink MEP does not need any management.

The dual-ended pro-active LM function is managed at source and sink MEP.

### Delay Measurement

The Frame Delay Measurement (DM) provides performance data that is based on the delay of the frames between the ingress and the egress of a Maintenance Entity (ME); i.e., between two Maintenance Group End Points (MEPs).

The following DM functions are defined:

-	1-way on-demand DM

-	2-way (round-trip) on-demand DM.

-	1-way pro-active DM

-	2-way (round-trip) pro-active DM.

The 1-way DM function is started at the source MEP and enabled at the sink MEP.

The 2-way DM function is managed only at the source MEP. The sink MEP does not need any management.

## MPLS-TP Multiplexing

This section maps the MPLS-TP multiplexing related MIs to the corresponding object classes.

The MPLS-TP multiplexing configuration function exists only in MT TTP and MT CTP.

Figure 7-14 – MPLS-TP multiplexing configuration function

## Connection Function

This section maps the connection function related MIs to the corresponding object classes.

Table 7-6 – Mapping of Connection Function related MI to G.8152 artefacts

| Functionality | MT_C_MI | G.8152 |
| --- | --- | --- |
| General connection management | Create_MC |  |
|  | Modify_MC |  |
|  | Delete_MC |  |
| Individual connection point management | MT_C_MP per input and output connection point |  |
|  | for further study |  |
| Individual connection management | MT_C_MP per matrix connection: |  |
|  | MT_C_MI_ConnectionType |  |
|  | MT_C_MI_Return_CP_ID |  |
|  | MT_C_MI_ConnectionPortIds |  |
| SNCP configuration | MT_C_MP per SNC/S protection process: |  |
|  | for further study |  |

### Linear Protection Function

The MPLS-TP linear protection function is defined in [ITU-T G.8131/Y.1382]. The related "Management Information" is listed in [ITU-T G.8121/Y.1381].

This function is modelled by the MT_SNCP_Group object class.

Figure 7 16 – MPLS-TP Linear Protection

## SCC/MCC Access Function

SCC and MCC can be accessed when the containing LSP is terminated. Each channel is able to transport IPv4, IPv6 and OSI structured signals. The diamonds in Figure 7-16 represent traffic shaping and conditioning functions that may be needed to prevent the SCC/MCC forwarding points from exceeding their committed bandwidth in congestion situations.

Figure 7-16 – MT/SCC_A function, MT/MCC_A function, and MT/client_A function

(copy from Figure 10-5/G.8121/Y.1382)

SCC and MCC access are modelled using the same pattern. Only the bidirectional SCC/MCC termination point can be instantiated. Each termination point contains one CTP for IPv4 access, one CTP for IPv6 access and one CTP for OSI access. These CTPs have the capability to shape and condition the communication signals.

The management of the SCC/MCC access function has been added to the bidirectional MT TTP.

Figure 7-17 below contains the related class diagram.

Figure 7-17 – SCC / MCC Access Class Diagram

<<Editor Note: The embedded JPEG image doesn’t have the EccTrafficShaping MO>>

# UML Model File

This section contains the information model files and the companion profile file specified using the “Papyrus” modelling tool.

- G.8152 Model File:

, which is also available on the IFA at https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.8152/G.8152_v1.00/G.8152_v1.00_PAP.zip.

The G.8152 PAP zip file contains the following four files:

  - .project,

  - CoreModel.di,

  - CoreModel.notation

  - CoreModel.uml

In order to use this G.8152 model, one needs to also install the Open Model Profile v0.2.2 and G.7711 v2.0

- OpenModelProfile File:  https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/OpenModelProfile/OpenModelProfile_v0.2.2.zip.

The openmodelprofile zip file consists of four files:

  - .project,

  - OpenModel_Profile.profile.di

  - OpenModel_Profile.profile.notation

  - OpenModel_Profile.profile.uml

The OpenModelProfile defines the UML model meta constructs (i.e. basic building blocks, such class, attribute, association, etc.) to be use to build information models. Using the same OpenModelProfile is essential step before achieving common IM.

- G.7711 v2.0 File: https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.7711/G.7711_v2.00/G.7711_v2.00_PAP.zip

NOTE –  Papyrus is an open source modeling tool. In order to view and further extend or modify the information model, one will need to install the open source Eclipse software and the Papyrus tool. The installation guide for Eclipse and Papyrus can be found at the following link:

https://www.eclipse.org/papyrus/updates/index.php.

UML Model Data Dictionary

(This appendix does not form an integral part of this Recommendation)

This appendix contains the details of the Ethernet NE management-protocol-neutral information model, including the description and properties of the object classes and their attributes and operations. This detailed information is generated automatically from a modelling tool, which was used to develop the information model in UML notation.

, which is also available on the IFA at https://www.itu.int/ifa/t/2013/sg15/exchange/wp3/q14/G.8152/G.8152_v1.00/G.8152_v1.00_DD.zip.

Mapping of G.8121 Atomic Functions to G.8152 Model Artefacts

(This appendix does not form an integral part of this Recommendation)

This appendix provides further detailed mapping between the G.8121 atomic functions and the G.8152 UML model artifacts. Note that in some cases a 1:1 mapping is not possible.

Table II-1 –Mapping between G.8121 Ethernet Atomic Functions and UML Model Artifacts

| G.8121 section | G.8121 atomic function | G.8152 model artefact | Note |
| --- | --- | --- | --- |

For Further Study

UML Modelling Guidelines

(This appendix does not form an integral part of this Recommendation)

See Recommendation [ITU-T G.7711] Annex A

_________________