3GPP TSG RAN WG3 #97	R3-173354

Berlin, Germany, 21st – 25th August 2017

Title:	Reply to liaison letter from IEEE 802.1

Release:	Rel-15

Work Item:	NR Work Item

Source:	RAN3

To:	IEEE 802.1

Cc:

Contact Person:

#### Name:	Matteo Fiorani

Tel. Number:	+46 725 969 109

E-mail Address:	Matteo <dot> Fiorani <at> Ericsson <dot> Com

Attachments: 	None

1. Overall Description:

RAN3 would like to thank IEEE 802.1 for the updates on the ongoing IEEE P802.1CM Time-Sensitive Networking (TSN) for Fronthaul project. RAN3 deems the work in IEEE P802.1CM highly relevant and it will take P802.1CM and its potential future amendments into consideration.

Regarding the request for information on the transport requirements for the high layer split, RAN3 would like to mention that some transport requirements were discussed during the NR SI phase. These requirements are reported in TR 38.801 for different split options (see Annex A). It should be highlighted that these requirements are not normative, are based on an LTE design, and represent the view in RAN3 at the time TR 38.801 was agreed. Given that RAN3 has selected split option 2 for normative work in Rel-15, the transport requirements for split option 2 are deemed to be the most relevant for the ongoing work in IEEE P802.1CM. Such requirements and their assumptions are reported below. RAN3 would also like to mention that the transport capacity requirements for split option 2 scales with the MIMO layers. RAN3 will inform IEEE 802.1 in case that additional transport requirements for split option 2 will be identified during the NR WI phase.

| Items | Assumption |
| --- | --- |
| Channel Bandwidth | [100MHz(DL/UL)] |
| Modulation | [256QAM(DL/UL)] |
| Number of MIMO layer | [8(DL/UL)] |

| Protocol Split option | Required bandwidth | Max. allowed one-way latency [ms] |
| --- | --- | --- |
| Option 2 | [DL: 4016Mb/s] [UL:3024 Mb/s] | [1.5~10ms] |

2. Actions:

To IEEE 802.1 Working Group.

ACTION: 	IEEE 802.1 Working Group is kindly asked to take the above into account.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#97bis	9-13 October 2017	Prague, Czech Republic

RAN3#98	26 November to 1 December 2017	Reno, USA

Description of the split option