CM4000  – RELEASE NOTES

Management Security Revisions

8 Port Modem (5.2.8) – V90 - Changes from 5.0.37 to 5.2.8

- Digital Pad AT@E reporting is now working correctly; worked in 5.0.26; broken in 5.0.27

- Improvement: A V90 or K56flex LAPM V42bis connection is made. The client continues to initiate rate renegotiations when high speed data is downloaded from the server to client. Continuous fallbacks occur.

- High Power mode register AT:T114=3 is now working; worked in 5.0.26; broken in 5.0.27

- Character * does not parse correctly. The "*" causes a delay.

- Rockwell DTMF detection work around.

- Bell 212 MNP connections when answering fix.

- Addition of 4000 Manager Security. This command must be saved with AT*W.

AT"S0 (default) - allows modem to work the way it used to.

AT"S1 - modem will not respond to DC session or Firmware upgrade commands unless security validation on the manager was successful.

AT"S2 - modem will not respond to any MABP commands except startup and control messages unless security validation on the manager was successful.

- V90 Retrains with USR Courier Rev 7.3.14 sometimes fall back to v.34 fix.

- Incorrect Number Of Bytes Reported in Last Connection field of the 4000 Manager fix.

- The transmit levels for V.90 connection is the value of AT :T51 minus 1.  AT:T51 still defaults to 13.

4 Port Modem (3.3.2) - Changes from 3.2.11 to 3.3.2

- AT: T51 and AT: T52 values are now correctly retained in the stored configuration for analog configurations only.  The AT: T51 & AT: T52 values will be forced to digital defaults if a digital adapter is detected regardless of what is in the stored configuration.

- Dial character * does not parse correctly fix

- Addition of 4000 Manager security. This command must be saved with AT*W.

AT"S0 (default) - allows modem to work the way it used to.

AT"S1 - modem will not respond to DC session or Firmware upgrade commands unless security validation on the manager was successful.

AT"S2 - modem will not respond to any MABP commands except startup and control messages unless security validation on the manager was successful.

- The FAX Class 2.0 +FTI, +FTSI, +FNF, and +FCSI responses are now compliant with the Class 2.0

Standard and now include the opening quote.  Previous revisions only included the trailing quote.

- 4000 Manager  (Version 4.0 Build 25) – Changes from Build 18 to Build 25

- Short call alarms and No Connects alarms will be generated, as a Total amount of Short calls (No Connects), not as Consecutive Short calls (No Connects). (Note: Ring No Answer Alarms are generated as consecutive Ring No Answers)  - FIXED

- The Corrective Actions (AT Commands) do not work when the Short Call Alarm is enabled and a short call is made into a modem. The AT command is not issued to the modem that has received a short call. (Ring No Answer and No Connect Alarms -OK) - FIXED

- Added latest Help file

- Added Security Feature.  If a PRI card has the Security feature enabled, then the manager will display a pop-up menu, and the user must enter the PRI card's password.

- Call Duration Hours bug fix

- TX/RX indicators on Modem property screen continue to flicker when connection is terminated fix

- Start Manager with connection up, last connection will now display No Data Available.

PRI  (Main 2.2.2/Boot 2.2.2) – Changes from 2.1.8 to 2.2.2

- Added support for L2F

- Added Security Feature