RADIUS Support for Connection Profiles

on the Pipeline/Max

Donn Rochette

Marco S. Hyman

27 June 94

Ascend Communications Inc.

1275 Harbor Bay Pkwy

Alameda, CA 94502

(510) 769-6001

1.  Introduction

================

This document describes the features and changes added to the Ascend

Pipeline/Max products in support of remote data access using a RADIUS

server.

RADIUS is the Remote Authentication Dial In User Service; a database

system that accepts requests and returns responses in the form of a UDP

datagram.  In simplified  terms a request is made to a RADIUS server which

in turn replies with a specific data entry for the given user name and

password.

The Ascend Pipeline/Max makes use of this capability to download various

types of data from a remote server.  Initial product releases supports

remote access of a connection profile for a specified user on incoming

calls.

2.  User Interface Updates

==========================

At a high level the additions to the user interface simply allow for

enable and disable of the use of a RADIUS server.  When RADIUS is used to

allow remote access to dynamic configuration parameters any local

configuration  will take precedence.  That is, a local connection profile

for a system will take precedence over a remote entry.

2.1 Ethernet Mod Config

-----------------------

The Ethernet Module Configuration (Mod Config) screen allows the user to

enable support of a RADIUS server.  The menu is displayed when, from the top

level edit menu Ethernet followed by Mod Config entries are selected.  The

menu items supporting RADIUS are the 'Auth=', 'Auth Host #1=',

'Auth Host #2=', 'Auth Host #3=', 'Auth Port=', and 'Auth Key=' fields.

+-----------------------+

| 20-700 Mod Config	|

| Ether options...	|

| WAN options...	|

| SNMP options...	|

| TServ options...	|

| Bridging=Yes/No	|

| Domain Name=Ascend.Com|

| Pri DNS=0.0.0.0	|

| Sec DNS=0.0.0.0	|

| Auth=RADIUS		|

| Auth Host #1=0.0.0.0	|

| Auth Host #2=0.0.0.0	|

| Auth Host #3=0.0.0.0	|

| Auth Port=0		|

| Auth Key=******	|

| Syslog=Yes/No		|

| Log Host=0.0.0.0	|

| Log Facility=		|

|			|

2.1.1 'Auth='

This field contains the remote authentication mode to be used by the

Max/Pipeline when the dial in user can not be locally authenticated.

The options are None, TACACS, RADIUS, and RADIUS/LOGOUT.

2.1.1.1 'Auth=None'

No remote authentication will be performed.

2.1.1.2 Auth=TACACS

Selection of auth type TACACS will cause the Pipeline/Max to request a

remote validation of incoming calls.  Like RADIUS, a remote validation

request is sent to a Tacacs server only if a local connection profile

is not available.  A validation message is sent when the call is

established and when the call is cleared.

2.1.1.3 'Auth=RADIUS'

Selection of RADIUS Auth supports remote validation of incoming calls.

If there is not a local connection profile for the call a remote

validation will be performed by sending a request to a RADIUS server.

If the validation of user name & password is successful a connection

profile will be downloaded.

2.1.1.4 'Auth=RADIUS/LOGOUT'

Selection of Auth with RADIUS/LOGOUT will perform a remote validation

of incoming calls as defined above.  In addition it will send a request

to a RADIUS server when the given session is terminated.

The standard RADIUS server does not support a logout capability.  The user

will need to modify their server to support this capability.  When a

Pipeline/Max is configured for RADIUS/LOGOUT it will send an auth request

message to the RADIUS server with a request code of PW_ACCOUNTING_REQUEST

when a call is being terminated.  Additional user defined functionality can

be added to the RADIUS server to be executed when this logout indication has

been received.  An example of the changes required to be made to a RADIUS

server to support a logout function can be provided by Ascend.  Refer to

section 3.2.1 for more details.

2.1.2 'Auth Host #1='

An IP address for a RADIUS or Tacacs server is specified as an Auth Host.

This is used as the primary or first destination of validation requests.

2.1.3 'Auth Host #2='

An IP address for a RADIUS or Tacacs server is specified as an Auth Host.

This is used as the secondary destination of validation requests.  If

the first host does not respond host #2 will be used as destination.

2.1.4 'Auth Host #3='

An IP address for a RADIUS or Tacacs server is specified as an Auth Host.

This is used as the tertiary destination of validation requests.  if

the first & second hosts do not respond host #3 will be used as destination.

2.1.5 'Auth Port='

This specifies the UDP port number to be used in remote validation

requests.  This port number MUST be the same port number used by the RADIUS

daemon.  By default the value used by the RADIUS daemon is defined in the

file /etc/services on the UNIX host that the daemon is running on.

2.1.6 'Auth Key='

This specifies a RADIUS client password.  It is used only for RADIUS and

RADIUS/LOGOUT authentication types.  This value MUST match the value

assigned in the RADIUS clients file.  Refer to section 3.2.2 below.

3.  Functional Description

==========================

Feature enhancements available with the use of a RADIUS server fall

into distinct categories differentiated by the type of data that is

extracted from the remote host.  There are dynamic data updates that

occur when calls are established and torn down.  There are also

static configuration data elements that are accessed during an

initialization procedure.  These are further defined by the specific

data objects that are updated.  The following descriptions are divided

by specific data object.

3.1 RADIUS Background

---------------------

One of the motivations for providing remote validation and data access

support from a RADIUS server is the ability to expand the amount of

information available for routing and configuration beyond the limits

of local hardware resources.  Data can reside on a remote host and

accessed by transferring it to the Pipeline/Max where it can reside

in temporary storage.  This storage can be freed when the information

is no longer required.

The RADIUS server works by receiving a validation request for a user

with name and password.  The server maintains a user file that describes

each user and password.  Within this user file, on the RADIUS host

machine, is a definition of data attributes and values.  This user

specific data can be of variable types.  We make use of this system by

defining connection information for users that can dial into a Pipeline/

Max.  We can add data definitions that can describe routing specific

information as well as system configuration parameters.

User data accessed through a RADIUS server is composed of an attribute

and a value.  Please refer to the Livingston Documentation for more

complete coverage on this subject.  Attributes are defined in a RADIUS

dictionary file.  A name, integer encoding and data type are assigned

to each attribute.  Ascend has defined a number of attributes that

are specific to the operation of a Pipeline/Max.  The existing attributes

defined in the RADIUS internet draft (draft-ietf-nas-radius-00.txt) can be

used to define a basic connection.  In order for a user to make use of

extended features available with RADIUS they will  need to use the

dictionary defined by Ascend.  They can then define their own user files

which can be accessed remotely from a Pipeline/Max.  Refer to section 3.2.1

below for more details.

3.2 RADIUS Configuration

------------------------

There are a number of files and configuration values that will need

to be set-up in order to allow a Pipeline/Max to function properly with a

RADIUS server.  A RADIUS server makes use of three text files.  These

files serve as the source of data to be returned to clients making

authentication requests.

It would be advisable to start by downloading the Ascend RADIUS files.

This can be done by anonymous FTP from ftp.ascend.com. From the pub/Radius

directory get the file 'radius.tar.Z' in binary mode.  Uncompress and

tar the file.  It un-tars into a directory named radius.  The directory

contains:

README:		A table of contents of the directory

radiusd:		A directory containing the source to an unmodified

radius daemon.

logoutd:		A directory containing the source to a radius

daemon modified to support the logout request

issued by the Ascend Pipeline when so configured.

ascend.dif:		A copy of the patches applied to the files in

radiusd to create logoutd.

clients:		A sample RADIUS clients file

users:		A sample RADIUS users file

dictionary		The RADIUS dictionary.  This dictionary MUST be

used to use the advanced RADIUS features supported

by the Ascend Pipeline.

draft-ietf-nasreq-radius-01.txt

A copy of the latest internet draft describing

RADIUS.

ascend.doc:		This document.

3.2.1 RADIUS Dictionary

The RADIUS dictionary file defines the data attributes used within

the system.  As described in 3.1 above Ascend has added attributes

for use in the Pipeline/Max systems.  The user will need to copy

the Ascend dictionary file to their RADIUS database directory.

Once this is done the RADIUS daemon will need to be restarted.

To do this a sample UNIX command line session as defined below

may be used.  In reality your favorite command sequence can be used

used as long as the daemon is stopped and re-started with a pointer

to the new Ascend dictionary.

To use the new dictionary:

% cp dictionary /etc/raddb	# copy dictionary to radius directory

% ps -aux | grep radiusd	# get the PID of the current radiusd daemon

% kill -9 PID			# PID is the PID of radiusd found above

% radiusd			# re-start radiusd using new dictionary

3.2.2 RADIUS Client File

The RADIUS client file defines the client machines that are allowed

to make requests to the RADIUS server.  An entry for each of the

Pipeline/Max systems that will be using this RADIUS server will need

to be added to this file.  An entry has the form:

pipeline                  testing123

where pipeline is a name of a Pipeline/Max and testing123 is a password

for the system named pipeline.  An entry should be added to the clients

file with your Pipeline/Max name, as defined in /etc/hosts or Yellow

Pages database, and a password for the system.  In other words the first

entry represents an IP address and the second entry represents a password.

On the corresponding Pipeline/Max the password assigned in the clients

file will need to be configured.  In order to update this go to the

Ethernet - Mod Config edit menu and for the entry 'Auth Key=' enter the

identical password value given to the entry in the clients file for

that system.

3.2.3 RADIUS Users File

The RADIUS users file will need to be updated with user names, passwords

and data attribute/value pairs.  The details of specific information

to update is defined for each object below.

3.3 Connection Profiles

-----------------------

When a call comes in a certain amount of information is required

to route inbound packets to the proper destination.  The required

information is collectively known as a connection profile.  This

data, or the largest portion of it, can be entered for a specific

inbound call or connection from the Ethernet Connections menus.  If a

local connection of this sort has been defined for the call a remote

validation will not be performed.  This allows user administrators to

define default connections that will always be a part of their system

independent of the remote host based configuration.  If a local connection

profile has not been defined and RADIUS has been enabled as described

in section 2.1.9 above a remote validation request will be sent to the

RADIUS server.  The server will respond with a message indicating

validation success or failure.  If it returned a success indication

the data defined for the user of the incoming call is transferred to

a Pipeline/Max as the RADIUS client.  This data is a connection profile.

A dynamic profile is constructed and installed for the specific call and

processing and routing of information continues as it would normally.

In order for the remote validation and connection profile update to be

successful an entry in the RADIUS users file will need to be present.

It will need to contain the users name, password and connection profile

data.  The connection profile data is entered using the Ascend defined

attributes in a dictionary file as defined above in section 3.1.  An

example of an entry in a users file may look like this:

ascend	Password = "pipeline"

Framed-Address = 198.5.248.225,

Framed-Netmask = 255.255.255.0,

Ascend-Metric = 2,

Framed-Routing = None,

Framed-Route = "199.33.206.0 198.5.248.225 1",

Ascend-Idle-Limit = 30

Where "ascend" is the user name obtained from the remote caller and

"pipeline" is the password.  The subsequent lines define the connection

profile.  The first string is an attribute as defined in the dictionary

file.  The assigned value is the data itself.  So, in the example

above the netmask used for the incoming call will be 255.255.255.0.

The complete list of available attributes is found in the dictionary.

The details of data type, integer encoding and any aliased values for

Ascend attributes are defined in the dictionary.  The dictionary should be

the final authority in the accuracy of attribute descriptions.

4.  Dictionary Attributes

=========================

The common attributes and there encoding is defined in the internet draft

draft-ietf-nas-radius-00.txt.  That draft notes that attributes numbered

224 through 255 are reserved for implementation use.  Ascend uses these

attribute numbers for fields not specified in the internet draft.

Unrecognized attributes defined as a part of a connection profile will be

ignored.  The integer encodings defined in the Ascend dictionary MUST NOT

change.  The authentication procedure will fail if these values change from

those defined in the Ascend RADIUS dictionary.

4.1  Ascend-IPX-Address

-----------------------

Attribute	224

Length		8

Type		string

Reserved for future use

4.2  Ascend-Metric

------------------

Attribute:	225

Length		6

Type		integer

The metric assigned to the route created for this user.  If not present

a metric of 7 is assumed.

4.3  Ascend-PRI-Number-Type

---------------------------

Attribute	226

Length		6

Type		integer

Reserved for future use.

4.4  Ascend-Dial-Number

-----------------------

Attribute	227

Length		>= 3

Type		string

Values		Unknown-Number		0

Intl-Number		1

National-Number		2

Local-Number		4

Abbrev-Number		5

Reserved for future use.

4.5  Ascend-Route-IP

--------------------

Attribute	228

Length		6

Type		integer

Values		Route-IP-No		0

Route-IP-Yes		1

Set to Route-IP-Yes to enable IP routing on this link.  Only meaningful

when the caller is using PPP encapsulation; caused IPCP to be started.

4.6  Ascend-Route-IPX

Attribute	229

Length		6

Type		integer

Values		Route-IPX-No		0

Route-IPX-Yes		1

Reserved for future use.

4.6 Ascend-Bridge

-----------------

Attribute	230

Length		6

Type		integer

Values		Bridge-No		0

Bridge-Yes		1

Set to Bridge-Yes to enable PPP bridging on the link.  Only meaningful

when the caller is using PPP encapsulation; caused BCP to be started.

4.7  Ascend-Send-Auth

---------------------

Attribute	231

Length		6

Type		integer

Value		Send-Auth-None		0

Send-Auth-PAP		1

Send-Auth-CHAP		2

Reseverved for future use.

4.8  Ascend-Send-Passwd

-----------------------

Attribute	232

Length		>=3

Type		string

Reserved for future use

4.9  Ascend-Link-Compression

----------------------------

Attribute	233

Length		6

Type		integer

Value		Link-Comp-None		0

Link-Comp-Stac		1

Indicates the type of compression to use on the link.  When set to

Link-Comp-Stac CCP is used to negotiation Stac compression with the

far end.

4.10  Ascend-Minimum-Channels

-----------------------------

Attribute	234

Length		6

Type		integer

Reserved for future use.

4.11  Ascend-Maximum-Channels

-----------------------------

Attribute	235

Length		6

Type		integer

Reserved for future use.

4.12  Ascend-Inc-Channel-Count

------------------------------

Attribute	236

Length		6

Type		integer

Reserved for future use.

4.13  Ascend-Dec-Channel-Count

------------------------------

Attribute	237

Length		6

Type		integer

Reserved for future use.

4.14  Ascend-Seconds-Of-History

-------------------------------

Attribute	238

Length		6

Type		integer

Reserved for future use.

4.15  Ascend-History-Weigh-Type

-------------------------------

Attribute	238

Length		6

Type		integer

Values		History-Constant	0

History-Linear		1

History-Quadratic	2

Reserved for future use.

4.16  Ascend-Add-Seconds

------------------------

Attribute	240

Length		6

Type		integer

Reserved for future use.

4.17  Ascend-Remove-Seconds

---------------------------

Attribute	241

Length		6

Type		integer

Reserved for future use.

4.18  Ascend-Data-Filter

------------------------

Attribute	242

Length		>=3

Type		abinary

The Ascend-Data-Filter contains a filter element used to determine if

a filtered packet is forwarded or dropped.  There may be multiple

Ascend-Data-Filter definitions as part of a RADIUS entry.  The filter

elements are processed in the order coded.  Entries may exist for

input or output input are always from the point of view of the

Pipeline.

If there is at least one input entry any packet that does not match

any input entry is not forwarded.

If there is at least one output entry any packet that does not match

any output entry is not forwarded.

The Ascend-Data-Filter is an "abinary" field;  is coded as a "string" but

converted to binary form before being sent to the Pipeline.  The string

is coded as:

In the following definitions  [ ... ] indicates optional element.

keywords are NOT case sensitive

IP Filters:

"ip dir action [ dstip n.n.n.n/nn ] [ srcip n.n.n.n/nn ]

[ proto  [ dstport cmp value ] [ srcport cmp value ]  [ est ] ]"

where:

ip:		The keyword ip. This is used to indicate an IP filter.

dir:		Filter direction. Must be either IN or OUT.

IN filters packets coming IN to the Ascend box.

OUT filters packets going OUT of the Ascend box.

action:		What to do with a packet that matches the filter.

Must be either FORWARD or DROP.

dstip:		The optional destination IP.  If not present the

filter will match ANY IP addresses.  If a netmask

portion (/nn) of the address is present we will

only compare the masked bits. The keyword "dstip"

must proceed the IP address.

srcip:		The optional source IP.  If not present the

filter will match ANY IP addresses.  If a netmask

portion (/nn) of the address is present we will

only compare the masked bits. The keyword "srcip"

must proceed the IP address.

proto:		The optional protocol.  May be specified as either

a name or a number.  The supported names are

least icmp(1), tcp(6), udp(17), ospf(89).

dstport:	Only valid when proto is tcp(6) or udp(17).  'cmp'

can have the value '<', '=', '>', or '!='.  The

value can be entered as a number or a name.

Supported names are ftp-data(20), ftp(21),

telnet(23), smpt(25), nameserver(42), domain(53),

tftp(69), gopher(70), finger(79), www(80),

kerberos(88), hostname(101), nntp(119), ntp(123),

exec(512), login(513), cmd(514), and talk(517).

Matches ANY port when not present. The keyword

"dstport" must proceed 'cmp'.

srcport:	Only valid when proto is tcp(6) or udp(17).  'cmp'

can have the value '<', '=', '>', or '!='.  The

value can be entered as a number or a name.

Supported names are ftp-data(20), ftp(21),

telnet(23), smpt(25), nameserver(42), domain(53),

tftp(69), gopher(70), finger(79), www(80),

kerberos(88), hostname(101), nntp(119), ntp(123),

exec(512), login(513), cmd(514), and talk(517).

Matches ANY port when not present. The keyword

"srcport" must proceed 'cmp'.

est:		The optional keyword EST.  Only valid when the proto

field is tcp(6).

GENERIC Filters:

"generic dir action offset mask value [ more ]"

where:

generic:	The keyword "generic". This is used to indicate an

generic filter.

dir:		Filter direction. Must be either IN or OUT.

IN filters packets coming IN to the Ascend box.

OUT filters packets going OUT of the Ascend box.

action:		What to do with a packet that matches the filter.

Must be either FORWARD or DROP.

offset:		A number.  Specifies an offset into a frame to

start comparing.

mask:		A hexadecimal mask of bits to compare.  A one bit

in the mask indicates a bit to compare, zero bits

are ignored.  The length of the mask specifies the

length of the comparison.

value:		The value to compare with the masked data at offset

in packet.  Note:  The length of the value MUST

be the same as the mask or the entry will be

ignored.

more:		The optional keyword MORE.  If present specifies

that the next filter entry is to be applied to

the current packet.  The dir and action of the

next entry MUST be the same as the dir and action

of the current entry or the more flag will be

ignored.

Examples:

1) Drop all incoming ip packets

Ascend-Data-Filter  = "ip in drop"

2) Only tcp outgoing packets are forwarded.

Ascend-Data-Filter  = "ip out forward tcp"

3) Forward all non telnet packets to 198.5.248.25 from host 198.5.248.3

Ascend-Data-Filter =  "ip out forward tcp dstip 198.5.248.3/16 srcip \

198.5.248.25/16  dstport != telnet"

4) Let through only ip packets going from host1 port 2000 to host2 to port

telnet

Ascend-Data-Filter = "ip out forward 0 dstip 198.5.248.3/16 srcip \

198.5.248.25/16  dstport = telnet srcport = 2000"

5) Drop all incoming packets starting with 0080

Ascend-Data-Filter  = "generic in drop 0 ffff 0080"

6) Drop all incoming packets starting with 0080 and having aa at byte 16

Ascend-Data-Filter  = "generic in drop 0 ffff 0080 more"

Ascend-Data-Filter  = "generic in drop 16 ff aa"

4.19 Ascend-Call-Filter

-----------------------

Attribute	243

Length		>=3

Type		abinary

An Ascend-Call-Filter is used to filter items that will not be used to

place a call or to keep a link up.  Packets that fail this filter are

stipp forwarded, but will never be used to place a call and will not

mark the link as in use.  The format of this filter is identical to

Ascend-Data-Filter.

4.19  Ascend-Idle-Limit

-----------------------

Attribute	244

Length		6

Type		integer

The number of seconds of no activity required before the call is torn

down.  If set to zero the call is never terminated.

4.20  Ascend-Preempt-Limit

-----------------------

Attribute	245

Length		6

Type		integer

Reserved for future use.

4.21  Ascend-Callback

---------------------

Attribute	246

Length		6

Type		integer

Values		Callback-No		0

Callback-Yes		1

Reserved for future use.

4.22  Ascend-Data-Svc

---------------------

Attribute	247

Length		6

Type		integer

Values		Switched-Voice-Bearer	0

Switched-56KR		1

Switched-64K		2

Switched-64KR		3

Switched-56K		4

Switched-384KR		5

Switched-384K		6

Switched-1536K		7

Switched-1536KR		8

Switched-128K		9

Switched-192K		10

Switched-256K		11

Switched-320K		12

Switched-384K-MR	13

Switched-448K		14

Switched-512K		15

Switched-576K		16

Switched-640K		17

Switched-704K		18

Switched-768K		19

Switched-832K		20

Switched-896K		21

Switched-960K		22

Switched-1024K		23

Switched-1088K		24

Switched-1152K		25

Switched-1216K		26

Switched-1280K		27

Switched-1344K		28

Switched-1408K		29

Switched-1472K		30

Switched-1600K		31

Switched-1664K		32

Switched-1728K		33

Switched-1792K		34

Switched-1856K		35

Switched-1920K		36

Reserved for future use.

4.23  Ascend-Force-56

---------------------

Attribute	248

Length		6

Type		integer

Values		Force-56-No		0

Force-56-Yes		1

Reserved for future use.

4.24  Ascend-Billing-Number

---------------------------

Attribute	249

Length		>=3

Type		string

Reserved for future use.

4.25  Ascend-Call-By-Call

-------------------------

Attribute	250

Length		6

Type		integer

Reserved for future use.

4.26  Ascend-Transit-Number

---------------------------

Attribute	251

Length		>=3

Type		string

Reserved for future use.

4.27  Ascend-Host-Info

---------------------------

Attribute	252

Length		>=3

Type		string

Reserved for future use.

4.28  Ascend-PPP-Address

---------------------------

Attribute	253

Length		6

Type		ipaddr

The value of this field determines the IP address advertised to dial

in PPP users.  When the field is not specified, or specified as 0.0.0.0

the address of the ethernet interface is advertized at IPCP negotiation.

When the field has a valid IP address, the specified address is advertized

at IPCP netotiation.  When the field has the special value 255.255.255.255

IPCP negotiates with the address 0.0.0.0.