How to upgrade your CF100FX2-GSM or CF100FX2-CDMA CallFinder firmware.

Note: Firmware files need to be in the same directory where you are running flash wizard.

- Power on your CF100FX2.

- Connect the command cable “provided” to your com port and callfinder.

3. Check the voice/data switch and make sure it is in the voice position

4. Run the flash wizard utility.

5. Device is found, highlight CF100FX2 and click Next.

- Choose Hex file from drop down box, hightlight Port and click next.

1.xx is for GSM model CF100FX2-G

2.xx is for CDMA model CF100FX2-C.

7. The firmware will be sent to the CF100FX2, then click Next.

8. Programing completes, click Finish.

9. Connect with Hyperterminal and type ATI1<cr>.

1.xx firmware is for GSM model CF100FX2-G

2.xx firmware is for CDMA model CF100FX2-C

10. Program your CF100FX2 and test all call patterns.

For Multi-Tech Support call 800-972-2439, option2.