vs 2.04 incorporates changes made from 1.06 thru 1.07 of GSM.

Changed FXS incoming ring routine to generate precise 2 second on 4 second offring signals.

Change max digits to dial to 15 was supposed to be 15 but code had a mistake and it was 14.

increases ring amplitude on fxs port.

2.04 goes along with resistor change in FXS circuitry.

vs.2.04 needs resistor moved in FXS. Old code in a board with resistor moved will fail answer mode.

vs 2.04a Added auto dial feature.

If autodial turned on, when a call comes into FXO the Cf100 will answer

and immediately dial the auto dial number stored in memory.

New commands:

AT>AD=0 or 1<cr>. turns autodial feature on/off.

AT>AD?<cr>. report status of Autodial and the stored phone number.

ATDnnnnnA <cr> Stores autodial number to memory.

Fixed bug that an AT command would response "OK" but command would not really get executed.

Fixed problem of modem hanging up if no RTS present on comamnd port. (affected FXO mode only) .

Vs. 2.06  This version defaults audio mode to 4,(hands free mode).

The +vgr command was left at 3 (>spk=3)

Vgr=3 seems best for rev e and vgr=4 seems best for rev c.

Therefore, customers in the field with rev C boards will probably need to adjust their speaker volume to 4 via the AT command AT>SPK=4.