How to upgrade your CF100FX2-GSM CallFinder firmware.

Note: Firmware version 3.01 must only be installed on units that shipped from the factory with firmware version 3.00. For units running firmware version 1.xx, download and install version 1.07b instead. You can check the currently running firmware version by following the procedure in step 9 below.

Note: Firmware files need to be in the same directory where you are running flash wizard.

1. Power on your CF100FX2-G.

2. Connect the command cable “provided” to your com port and Callfinder.

3. Check the voice/data switch and make sure it is in the voice position

4. Run the flash wizard utility.

[image: image1.jpg]Type the name of a program, older, dacument, or
Internet resource, and Windows wil open t For you.

Open: | "CiFlashwizard|Flash Wizard.ex:





5. Device is found, highlight CF100FX and click Next. 

[image: image2.jpg]Devices have been ideniified.

Devices
Selectthe device pe to flash:

Click et to cortinue.

Neit> Cancel





6. Choose Hex file from drop down box, highlight Port and click next.

1.07B is for CF100FX2-G models currently running 1.xx firmware.

3.01 is for CF100FX2-G models currently running 3.00 firmware.

[image: image3.jpg]Firmware Up date Wiza

CFI00RX
Selectthe ports and hes fleto updste

Pot [ Version | Countip/Region
TEION 068 Unknown

HexFile
Notth America 1,078 -

<Back Net> Cancel





7. The firmware will be sent to the CF100FX2, then click Next.

[image: image4.jpg]Your devices are being updated.

Progress

Frogramming Complete:

Temmination cfthe program ding this updete:
mayrender your devices inoperable

it





8. Programming completes, click Finish.

[image: image5.jpg]Firmware Update Wizard

Resuls
1 0f 1 devices were updated sucoessfully

Pott__| Model Status
TOM1  CFIOOFX  Successhl

Firish





9. Connect with Hyperterminal and type ATI1<cr> to verify firmware version.

1.xx firmware is for early CF100FX2-G models.

3.xx firmware is for later CF100FX2-G models.

[image: image6.jpg]a =]
Fle Edt Vew Cal Transfer Hep

D@ 3 DB &

atIl

107b

0K

< |

Connected 0:00:53 ANST

&

115200 841

SO | Ches





2.xx firmware is for CDMA model CF100FX2-C

[image: image7.jpg]comm1 - HyperTerminal
Fie Edt Vew Cal Transfer Hep

D® 538 08 &
atIl ]
20ka
0K 1
< | =

Connected 0:00:06. Auto detect

115200 841




10. Program your CF100FX2 and test all call patterns. 

Technical Support is available at www.multitech.com 

