Pre release of version 1.02

Added code to watch for FXS to be back on hook after dialing, before it was simply a 70-second time out waiting for connect message.

Added code to make sure line reversal code wasn't getting executed.

Played with SlIC register settings to reduce idle noise.

Version 1.03

Added new line reversal commands for billing type of line reversal.

Added code for above as well.

Added code with for "disconnect on current loss" (FXO only).

Implemented disconnect on line reversal in FXO mode.

Both features above are based on >lr commands.

The LR commands are....

AT>LR=0 equals both line reversal features off.

AT>LR=1 equals billing line reversal feature on (FXS only)

AT>LR=2 equals disconnect on a line reversal on (FXO only)

Added ability to query the >FX commands and the >LR commands.

The command format is >fx? and >lr?.

Version 1.04

1. U.S. caller id added.

2. Dial tone frequencies from FSK port improved.

The command for CID is AT>CID=0 or 1

Version 1.05

Added code for FXS disconnect if wireless hangs up, tip and ring will open for 4 sec if wireless hangs up, command is AT>HFX=x (0 = off, 1 = on)

Version 1.07b

1. Added auto-dial feature on FXO port.

AT>AD=0 and AT>AD=1 to disable/enable.

AT>DnA<cr> will store the number to be dialed toward the cellular

network. n = number to dial.

2. Fixed AT command proccessing bug, would return OK but not set the value.