How to upgrade your CF100FX2-GSM CallFinder firmware.

Note: Firmware version 3.01 must only be installed on units that shipped from the factory with firmware version 3.00. For units running firmware version 1.xx, download and install version 1.07b instead. You can check the currently running firmware version by following the procedure in step 9 below.

Note: Firmware files need to be in the same directory where you are running flash wizard.

- Power on your CF100FX2-G.

- Connect the command cable “provided” to your com port and Callfinder.

3. Check the voice/data switch and make sure it is in the voice position

4. Run the flash wizard utility.

5. Device is found, highlight CF100FX and click Next.

- Choose Hex file from drop down box, highlight Port and click next.

1.07B is for CF100FX2-G models currently running 1.xx firmware.

3.01 is for CF100FX2-G models currently running 3.00 firmware.

7. The firmware will be sent to the CF100FX2, then click Next.

8. Programming completes, click Finish.

9. Connect with Hyperterminal and type ATI1<cr> to verify firmware version.

1.xx firmware is for early CF100FX2-G models.

3.xx firmware is for later CF100FX2-G models.

2.xx firmware is for CDMA model CF100FX2-C

10. Program your CF100FX2 and test all call patterns.

Technical Support is available at www.multitech.com