Release Notes for MT5692SMI-x patch code 1.01j to 1.01k.

- Fix for modem unable to connect when ATH1 then ATD or  ATH1 then ATA commands were issued.

- Fix for modem hanging up when switching from voice to data mode using AT+VNH=2. Previously it wasn’t a complete hang up and the modem would be in a confused state.

- Fix for Analog Loopback mode. AT&Q6 and AT&T1 now work with no garbage characters. Previously you would get CONNECT message then garbage.

- Fix AT-TTE command, so parameters 5 and 6 display correctly. Previously, parameter 6 would overwrite 5 and parameter 6 would be unchanged.

- Modify DTMF detection to display DTMF’s for valid digit presses. Previously, it would be just 1 digit no matter the length of press.

- Modified Transmit Level (S91) for Brazil from 10 to 8 for approval.