Release Notes for MT5634SMI-V92 firmware version from 1.32q to 1.32r.

- Modified default fax setting +FS1 from 14 to 32, how long to check for V.21 detection.

- Modified default fax setting +FS2 from 3 to 7, number of times to get a good V.21 detection.

- Fax fix when receiving from HP OfficeJet 4315 fax machine. On the last page the OfficeJet would glitch causing use to lose the primary channel and wait 60 seconds for the control channel. This is tested in fax regression test.

- Fax fix so we don’t reset in V.34 fax while waiting for data. Tested in fax regression test.

- Fax fix where the modem could get stuck off hook when making a fax call and only hearing answer tone. Tested in fax regression test.

- Fax fix in debug to remove CR-LF in debug string. No testing required.

- Fax fix to re-initialize frame count appropriately. Tested in fax regression test.

- Fax fix to fix issue where if we are waiting for a DCN and receive a RR after EOP/MCF sequence we hang up with fax error A2. Tested in fax regression test.

- Fax fix where if we don’t get high speed carrier waiting for a high speed block, we will wait long enough for the Post Page Message. Tested in fax regression test.

- Fixed issue where the modem hangs up with a +FHS:B7 when the caller sends a DCN after the modem gets into a RR-RNR loop and the T5 timer runs out.

- Fixed issue where the modem hangs when calling the other modem and answer tone is detected but no V.21 is detected.

- Fixed issue where the user gets an unknown +FHS:0 error on the FaxFinder.

- Fixed issue where the user gets a +FHS:A2 when a PPS-EOP is transmitted.

- Fixed issue where user gets a +FHS:B7 when a RR is sent after an MCF is sent by us.

- Fixed issue where the user gets a +FHS:B7 after a PPS-MPS was retransmitted instead of the next partial page.

- Fixed issue where the modem hangs off hook until the host hangs it up on receiving an error corrected fax. It would happen after the first CONNECT.

- Fixed issue where the customer gets a +FHS:A3 when receiving in ECM mode.

- Fixed issue where the call stays going forever because the modem never trains down in V.34 mode and no frames get through so the PPR is always the same.

- Fixed issue where the modem will disconnect when receiving a fax in error correction mode because of a +FHS:B7. This occurs because of a RR-RNR loop.

- Fixed issue which caused the modem to fail receiving a fax in V.34 mode where it would fail on the last page of the fax.

- Fixed issue where the modem will put out a NO CARRIER response when it doesn’t detect carrier on a non ECM page. On a FaxFinder, it would have an error message like: Unexpected response “ERROR’.