Release Notes for the MT9234ZBA-USB-CDC from beta 1.02j to 1.02l.

- Fix on hook flash testing to flash according to %T22,26 value.

- Add code so NO CARRIER is displayed if V.42 disconnect and not escaped from data mode.

- If +FCLASS greater than 0, add code to make sure ATZ and ATH actually hang up.

- Add code so between the USB interface and modem we always do hardware flow control. If &K4 then we do both hardware and software flow control.

- Fix V.42bis ftp download problem by modifying the follow values.

- NO_OF_LAPM_RECEIVE_FRAMES default from 16 to 8.

- lapm_t401 time from 2 seconds to 4 seconds.

- Lapm_sreg_negotiated default to FALSE.

- Maximum number of bytes in I-frame from 128 to 64.