Release Notes for MT9234ZBAV from 1.02h to 1.02i

- Fix PnP locking at 1200 baud issue. Now baud gets reset back to $SB after ATZ.

- Modified code so a power up with an incoming RING will still detect incoming rings.

- Fix code so when in +VLS=1 TAM mode, the +VGT and +VGR commands now control gain settings when issuing +VTX and +VRX.

- Add fix so a double RING will display CID information with +VCID=1 or 2 setting.

- Fixed %S1 (115200 only commands) to function and not lock up at 300 baud.

- Fix %S1$SBxx&W combination to allow data to transmit after a power cycle.

- Modified code so char abort is disabled when %S1 and not 115200 after power up.

- Fix issue when calling into certain V.22bis modems at 2400, with a long Round Trip Delay, where the modem wouldn’t connect.

- Add code to transmit a break during a data connection. Receiving break worked.

- Fix callback security issue where the callback attempt wouldn’t connect on first callback but connected on the second callback.

- Fix callback security to callback after an aborted callback attempt, if within number of re-try attempts.

- Added initial +FDE fax debug options, for debug purposes only.

- Added fax modifications which are tested and verified in fax regression tests.