# MT9234MU and MT9234ZBA-USB Installation Procedure

Plug In MT9234MU or MT9234ZBA-USB.

Begin installation of the Multi-port Serial Adapters:

Welcome to the Found New Hardware Wizard, Can Windows connect to Windows Update to search for software? Select ‘No, not this time’. Select ‘Next’.

This wizard helps you install software for: TUSB3410 Serial Port. Select ‘Install from a list or specific location (Advanced)’. Select ‘Next’.

Browse for ‘Windows_2k_XP/USB Serial Driver’. Select ‘OK’. Select ‘Next’.

Message appears that states ‘The software has not passed Windows Logo testing …’. Select ‘Continue Anyway’. Wizard installs the files: umpusbxp.sys and umpfmt9234model.i51.

Completing the Found New Hardware Wizard, The wizard has finished installing the software for: MultiModel MT9234Model. Select ‘Finish’.

Automatically the next wizard starts.

Begin installation of the Serial Port:

Welcome to the Found New Hardware Wizard, Can Windows connect to Windows Update to search for software? Select ‘No, not this time’. Select ‘Next’.

This wizard helps you install software for: Texas Instruments UMP Serial Port. Select ‘Install from a list or specific location (Advanced)’. Select ‘Next’.

Browse for ‘Windows_2k_XP/USB Serial Driver’. Select ‘OK’. Select ‘Next’.

Message appears that states ‘The software has not passed Windows Logo testing …’. Select ‘Continue Anyway’. Wizard installs the file: serenum.sys.

Completing the Found New Hardware Wizard, The wizard has finished installing the software for: MultiModel – Serial Port. Select ‘Finish’.

Automatically the next wizard starts. If not, unplug modem and plug back in and continue.

Begin installation of the modem:

Welcome to the Found New Hardware Wizard, Can Windows connect to Windows Update to search for software? Select ‘No, not this time’. Select ‘Next’.

This wizard helps you install software for: MultiTech Systems MT9234Model. Select ‘Install from a list or specific location (Advanced)’. Select ‘Next’.

Browse for ‘Windows_2k_XP/Modem Driver’. Select ‘OK’. Select ‘Next’.

Message appears that states ‘The software has not passed Windows Logo testing …’. Select ‘Continue Anyway’. Wizard installs the file: 9234Model.INF.

Completing the Found New Hardware Wizard, The wizard has finished installing the software for: MultiModel MT9234Model. Select ‘Finish’.

Device Manager:

There should now be a Multi-Port serial adapters tab containing MultiMobile MT9234Model.

There should now be a MultiMobile – Serial Port (COM?) under the Ports tab.

There should now be a MultiMobile MT9234Model under the Modems tab.