MVP130-MVP130 – FXS to FXS Auto Call Setup

Local MVP130 Connected to Analog Phone

Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MVP130’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.

Configuration -> Interface

1.  Configure Interface Type as FXS.

2. Click OK on the Interface page to save this change.

Configuration -> Voice/Fax

1. Enable MultiVOIP to Auto Call the Remote MVP130 as follows:

a) Configure Auto Call / OffHook Alert to Auto Call and enter 201 as 

Phone Number.

2. Click OK on Voice/Fax page to save these changes.

Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entry:

#1  Destination Pattern = 201





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP130 

      Description = Whatever you want

      Protocol Type = H.323 

      Q.931 Port Number = 1720


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entry:

#1  Remove Prefix = 101 

      Add Prefix = Blank  

      Channel Number = Channel 1 

      Description = Whatever you want 

      Call Forward = Not enabled 


Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

Remote MVP130 Connected to Analog Telephone

Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MVP130’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.
Configuration -> Interface

1.  Configure Interface Type as FXS (Loop Start)

2. Click OK on the Interface page to save these changes.

Configuration -> Voice/Fax

2.  Enable Remote MVP130 to Auto Call the Local MVP130 as follows:

a) Configure Auto Call / OffHook Alert to Auto Call and enter 101 as 

Phone Number.

1. Click OK on Voice/Fax page to save these changes.

Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entry:

Destination Pattern = 101

Remove Prefix = Blank 

Add Prefix = Blank 

IP Address = IP Address of MVP130 at the Local site

Description = Whatever you want 

Protocol Type = H.323 

Q.931 Port Number = 1720


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entry:

Remove Prefix = 201 

Add Prefix = Blank  

Channel Number = Channel 1 

Description = Whatever you want 

Call Forward = Not enabled 

Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

