MVP210FX-MVP130 – FXO to FXS Auto Call Setup

Local MVP210FX Connected to PBX Extensions/Station Ports

Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MultiVOIP’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.

Configuration -> Interface

1.  Configure Interface Type as FXO for both channels.

2. Click OK on the Interface page to save this change.

Configuration -> Voice/Fax

1. Enable MultiVOIP channel 1 to Auto Call the Remote MVP130 as follows:

a) Channel 1 - Configure Auto Call / OffHook Alert to Auto Call and enter 201 as 

Phone Number.

2. Click OK on Voice/Fax page to save these changes.

Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entry:

#1  Destination Pattern = 201





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP130

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entry:

#1  Remove Prefix = 101 

      Add Prefix = Blank  

      Channel Number = Channel 1 

      Description = Whatever you want 

      Call Forward = Not enabled 

#2  Remove Prefix = 102 

      Add Prefix = Blank  

      Channel Number = Channel 2 

      Description = Whatever you want 

      Call Forward = Not enabled

Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

Remote MVP130 Connected to Analog Telephone

Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MultiVOIP’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.
Configuration -> Interface

1.  Configure Interface Type as FXS (Loop Start).

2. Click OK on the Interface page to save these changes.

Configuration -> Voice/Fax

2.  Enable Remote MVP130 to Auto Call channel 1 of the MVP210FX connected to the PBX as follows:

a) Channel 1 - Configure Auto Call / OffHook Alert to Auto Call and enter 101 as 

Phone Number.

1. Click OK on Voice/Fax page to save these changes.

Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entry:

#1  Destination Pattern = 101





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX
      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entry:

#1  Remove Prefix = 201 

      Add Prefix = Blank  

      Channel Number = Channel 1 

      Description = Whatever you want 

      Call Forward = Not enabled 

Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

