MVP810-MVP210FX – FXO to FXS Auto Call Setup for Faxing
(This example only covers the parameters you must change to get this working.  Leave any parameters I don’t mention at their default values)

Local MVP810 Connected to Analog PBX Station port

Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MultiVOIP’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.

Configuration -> Interface

1.  Configure Interface Type as FXO for all channels.

2. Click OK on the Interface page to save this change.

Configuration -> Voice/Fax

1. Set Selected Coder to “G.711 U-law” under Coder.

2.  Under Advanced Features, disable “Silence Compression” and enable “Forward Error  

      Correction”.

3.  Enable MVP810 channels 1 & 2 to Auto Call the Remote MVP210FX #1 as follows:

a) Channel 1 - Configure Auto Call / OffHook Alert to Auto Call and enter 201 as 

Phone Number.

b) Channel 2 - Configure Auto Call / OffHook Alert to Auto Call and enter 202 as 

Phone Number.

4.  Click OK on Voice/Fax page to save these changes.

**** Do the same for the remaining 6 channels, with the exception that channels 3 & 4 will Auto Call 301 & 302, chs. 5 & 6 will Auto Call 401 & 402, and chs. 7 & 8 will Auto Call 501 & 502 ****
Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entries:

#1  Destination Pattern = 201





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #1
      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#2  Destination Pattern = 202





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #1 

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#3  Destination Pattern = 301





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #2

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#4  Destination Pattern = 302





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #2

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#5  Destination Pattern = 401





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #3

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#6  Destination Pattern = 402





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #3

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#7  Destination Pattern = 501





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #4

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#8  Destination Pattern = 502





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP210FX #4

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entries:

#1  Remove Prefix = 101 

      Add Prefix = Blank  

      Channel Number = Channel 1 

      Description = Whatever you want 

      Call Forward = Not enabled 

#2  Remove Prefix = 102 

      Add Prefix = Blank  

      Channel Number = Channel 2 

      Description = Whatever you want 

      Call Forward = Not enabled

#3  Remove Prefix = 103

      Add Prefix = Blank  

      Channel Number = Channel 3 

      Description = Whatever you want 

      Call Forward = Not enabled

#4  Remove Prefix = 104 

      Add Prefix = Blank  

      Channel Number = Channel 4 

      Description = Whatever you want 

      Call Forward = Not enabled

#5  Remove Prefix = 105 

      Add Prefix = Blank  

      Channel Number = Channel 5 

      Description = Whatever you want 

      Call Forward = Not enabled 

#6  Remove Prefix = 106

      Add Prefix = Blank  

      Channel Number = Channel 6 

      Description = Whatever you want 

      Call Forward = Not enabled

#7  Remove Prefix = 107

      Add Prefix = Blank  

      Channel Number = Channel 7 

      Description = Whatever you want 

      Call Forward = Not enabled

#8  Remove Prefix = 108 

      Add Prefix = Blank  

      Channel Number = Channel 8 

      Description = Whatever you want 

      Call Forward = Not enabled

Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

Remote MVP210FX #1 Connected to Fax Machine 
Configuration -> Ethernet / IP


IP Parameters

1. Configure IP Address, IP Mask, and Gateway addresses (a Gateway address will not       be required if both MultiVOIP’s are on the same private IP subnet).

2.   Click OK on the Ethernet / IP page to save these changes.
Configuration -> Interface

1.  Configure Interface Type as FXS (Loop Start) for both channels.

2. Click OK on the Interface page to save these changes.

**** Configure Interface Parameters identically on the other 3 MVP210-FX units ****

Configuration -> Voice/Fax

1.   Set Selected Coder to “G.711 U-law” under Coder.

2.  Under Advanced Features, disable “Silence Compression” and enable “Forward Error  

      Correction”.

3.   Enable Remote MVP210FX #1 to Auto Call channels 1 & 2 of the MVP810 as follows:

a) Channel 1 - Configure Auto Call / OffHook Alert to Auto Call and enter 101 as 

Phone Number.

b) Channel 2 - Configure Auto Call / OffHook Alert to Auto Call and enter 102 as 

Phone Number.

4. Click OK on Voice/Fax page to save these changes.

**** Do the same on the remaining MVP210-FX’s, except MVP210FX #2 channels 1 & 2  will Auto Call 103 & 104, MVP210FX #3 will Auto Call 105 & 106, and MVP210FX #4 will auto call 107 & 108 ****

Phone Book -> Phone Book Modify -> Outbound Phone Book

Create the following Outbound Phone Book entry:

#1  Destination Pattern = 101





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP810

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP 

#2  Destination Pattern = 102





      Remove Prefix = Blank 

      Add Prefix = Blank 

      IP Address = IP Address of Remote MVP810 

      Description = Whatever you want (or leave blank)

      Protocol Type = SPP
**** Create similar Outbound entries in the remaining MVP210-FX’s, except use 103 & 104 as destination patterns in MVP210FX #2, 105 & 106 as destination patterns in MVP210FX #3, and 107 & 108 as destination patterns in MVP210FX #4 ****


Phone Book -> Phone Book Modify -> Inbound Phone Book

Create the following Inbound Phone Book entry:

#1  Remove Prefix = 201 

      Add Prefix = Blank  

      Channel Number = Channel 1 

      Description = Whatever you want 

      Call Forward = Not enabled 

#2  Remove Prefix = 202 

      Add Prefix = Blank  

      Channel Number = Channel 2 

      Description = Whatever you want 

      Call Forward = Not enabled

**** Create similar Inbound entries in the remaining MVP210-FX units, except use 301 & 302 as the Remove Prefixes in MVP210FX #2, 401 & 402 as Remove Prefix in MVP210-FX #3, and 501 & 502 as Remove Prefixes in MVP210FX #4 ****
Save Configuration

1.  Click Save Setup -> Save and Reboot to save these configuration changes to the MultiVOIP.

