-------------------------------------------------------------------------

>XVIEW [opt #1] <file.in> <file.out> <alignment> <audit elem size> [opt #2]

>opt #1: -BIN: binary input file, -N: no address column

>opt #2: -M: Motorola-like 'shorts', -N: no system info, -Tnn: tick value in ms

Example:

if test.lst contains an ASCII dump, use the follwing command:

XVIEW test.lst test.vie 2 64 -M

if test.bin contains a binary dump, use the follwing command:

XVIEW -BIN test.bin test.vie 2 64 -M

-------------------------------------------------------------------------

XVIEW.BAT uses the 2 following tools:

XLATE.EXE is used to translate an ASCII-form msg_audit dump into a binary

form usable by DUMPVIEW.EXE.

Usage:   XLATE [options] <file.in> <file.out> <alignment> <audit elem size>

Options: -BIN  : Binary input file (default is ASCII)

-Snnn : Skip the nnn first bytes of the file (Binary mode)

-Lnnn : Skip the nnn first lines of the file (ASCII mode)

-H-   : Do not add the binary header for DUMPVIEW

-nn   : Force nn bytes per line

-N    : No address column at beginning of line

Example :

if test.lst contains an ASCII dump, use the follwing command:

XLATE  test.lst  test.dmp  2  64

if test.bin contains a binary dump, use the follwing command:

XLATE  -BIN test.bin  test.dmp  2  64

Then, to interprete the created binary dump file, type:

DUMPVIEW  test.dmp  -M  >test.vie

Usage: [-N] [-M] [-Tnn] [-F] [Qn] filename

-N   : No System Info (only messages are dumped)

-M   : Motorola-type source file (default is Intel-type)

-I   : Intel-type source file (default)

-Tnn : system Tick value in milliseconds

-F   : Fixed length display for the Audit buffer

-D   : Display Delta between alloc counters and it_stacks

-Qn  : Detail level for Q.931 decoding (0-2, default is 2)

filename : if not listed, "OMNITEL.DMP" is taken.