RTRWARE code  changes for integrating DHCP are as follows:

----------------------------------------------------------

Under APPLICAT directory

------------------------

1. One new DHCP directory is created for Dynamic Host Configuration

Protocol (DHCP) appication

Under PPP directory

-------------------

pppinit.c :

1. After LOWER_DEVICE_DRIVER_UP call to PPP, DHCP DISCOVER packet has sent

thru lsl_control for dynamic IP address for that perticular port (client).

2. After LOWER_DEVICE_DRIVER_DOWN call to PPP, DHCP RELEASE packet has sent

thru lsl_control for relesing the assigned dynamic IP address for that

perticular port (client).

3. In initialize_ppp_ports, ppp.port[real_port_number].dhcp_status = DHCP_STATUS_NO_OPERATION

is added.

pppconfg.c:

1. ppp_dhcp_control_function : This function is added for getting dhcp information

i.e. IP_ADDRESS_OBTAINED, DHCP_SERVER_NOT_FOUND and LEASE_TIME_EXPIRED etc.

2. ip_bring_down_ncp : This function is added to disable IP for the perticular

port (client), immidiatly after the LEASE_TIME expires .

ppprx.c	  :

1. Drops the IPCP PROTOCOL (NCP) packet till the dhcp_status set to IP_

ADRESS_OBTAINED by dhcp protocol and also after LEASE TIME expires.

vpppstr.h :

1. dhcp_status member is added to PPP_PORT_CLASS with which PPP keep tracks

the dhcp processing.

Under IP directory

------------------

ipinit.c:

1. is_dhcp_enabled : This function is added which checks bootp_enbled and remote access

enabled parameters in IP PORT configuration to proceed with dhcp processing.

udptx.c

1. In send_udp function , the tx_port_number is made to zero,if local_ip_address

is equal to INTERNET_ADDRESS_ANY (i.e. NULL).

rip.c

1.	In rip_is_it_ok_to_announce_this_route(). Changed the way netmask

is computed.

Under LSL directory

-------------------

vlslctrl.h:

1. The function "lsl_pass_parameters_to_application()" is added in the LSL

FUNCTION TABLE corresponding to the DHCP_CONTROL_OPERATION of enum DHCP_

CONTROL_OPERATION.

Under INCLUDE directory

-----------------------

lslproto.h:

1. ISSUE_DHCP_BROADCAST and GIVE_BACK_IP_ADDRESS are added to the APPLICATION_

CONTROL_OPERATION enumeration.

2. DHCP_CONTROL_OPERATION is added to the LSL_CONTROL_OPERATION for calling

dhcp_control function thru lsl_control function.

3. External declaration for initialize_dhcp() function

4. Some enumerations related to dhcp commands are added here. Because these

are also used by other modules (like PPP).

5. DHCP_APPLICATION is added to the APPLICATION_TYPE enumeration

vnvrmstr.h:

1. The external declaration for the get_dhcp_server_ip_address is included.

Under STORE directory

---------------------

vrtrwstr.h:

1. initialize_dhcp is added to router_protocol_software array of type

ROUTERWARE_MODULE_DEFINITION structure.

Under NVRAM directory

---------------------

vnvram.h:

1. dhcp_configuration_table is added in this file.

nvram.c:

1. get_dhcp_server_ip_address() function is added for configuring dhcp server

address.

Under MTROUTER or ROUTEFIN directory

------------------------------------

roucon.ini:

1. In IP_ROUTING section BOOT_ENABLED parameter is to be enabled

2. In DHCP section DHCP Relaying and DHCP Server IP Address paremeters are

added.