Virtual Server loopback Problem

Author : Ravi Ganesh V

Date     : 24 January 2000

Contents of the Document :

1. Pictorial Representation of the Problem

2. Problem Definition

3. Recommended Solution

4. Points to Ponder

5. Structures Used

6. Algorithm/Code Affected

7. Effort Estimate

8. Miscellaneous Note (Important)

Pictorial Representation of the Problem

Figure Above illustrates the pictorial representation, for the below problem definition.

Problem Definition :

1. If a client from the Internet Domain,  tries to FTP to X.COM, first resolves  into IP address

202.54.39.98, then tries to contact the FTP server. Since there is a virtual mapping to

192.168.3.2 does a network address translation and forwards the packet to 192.168.3.2

workstation. The response for the  packet from workstation 192.168.3.2 is sent to the firewall

and inturn proper address translation is  made on the response packet and the packet is

forwarded onto the Internet Domain.

FTP Request from 202.100.100.100

202.100.100.100  ---> 	202.54.39.98

202.100.100.100  ---> 	192.168.3.2 (After Address translation at Firewall).

This packet is forwarded to the Secure Lan.

FTP Response from 192.168.3.2

192.168.3.2          --->	202.100.100.100

202.54.39.98	 --->	202.100.100.100 (After Address translation at Firewall)

This packet is forwarded to Internet Domain.

This implementation is taken care in Firewall and termed it to be Virtual Server functionality.

2. If the same is tried from within a secure lan client i,e FTP X.COM, first resolves the IP address

to 202.54.39.98, then tries to contact to the FTP Server located at IP Address 202.54.39.98.

Since this FTP Server is found in the Same segment as that of client, calls for a special handling

at firewall as the packet is not forwarded to the Internet Domain, instead forwarded to the

Secure Lan. Hence we term this problem as Virtual Server Loopback problem.

Recommended Solution.

1. if a FTP client from within a secure lan, tires to contact FTP X.COM, the following happens.

Resolves into IP Address 202.54.39.98

FTP Request from the FTP Client 192.168.3.3

192.168.3.3  (1024)		--->	202.54.39.98 (21) 	Secure LAN

2. If a match found in the Virtual Server Table, then ...

Maintain a table which will store all the information about the source packet. Change the

source IP packet to Secure LAN IP Address of Firewall. Change the source Port to a dynamic

port. Change the Destination IP Address to the local IP address mapped in the Virtual Server

table. Forwarded the modified packet in the Secure LAN. Hence the packet look like

192.168.3.1  (dynamic port say 2000) --->   192.168.3.2(21) (At Firewall)

3. Response packet from FTP Server

FTP Response from FTP Server 192.168.3.2

192.168.3.2 (21)		--->	192.168.3.1 (dynamic port 2000)

Check in the information table store, and if there is a match found in the information table,

change the source IP address, destination IP address, and Destination port appropriately from the

information table and forwarded the packet onto Secure LAN. Hence the packet looks like this.

202.54.39.98 (21)	---> 	192.168.3.3 (1024) (At Firewall)

Points to Ponder :

1. Create the Information table whenever appropriate. These created descriptors should be

maintained separately and referred to when required.

2. Age these entries in a regular interval.

Structures  used :

1. We can use the same structure as that of  PROXY_SERVER_INFO. We should adopt the same

technique of Searching the entries in the table. Maintain two descriptor list, one for searching the

client descriptor list when a request is sent from the client and searching the server descriptor

when the response from the FTP Server  is reached at firewall.

Note : We can claim this operation as proxying partially. Since we change the destination

address also in this operation, we need to make sure we store these details in the RB Tree.

2. The keys for the descriptors i,e Client & Server are requirement specific. This document does

not enforce any standard usage of the keys.

3. Assumed to use RB Trees for the above operation.

Algorithm/ Code affected

IPRX.C

if  ( check if the packet is destined to us == TRUE)

{

if (check if received port is our Secure LAN port  == TRUE)

{

if (check if the destination is our Secure LAN IP Address == TRUE)

{

/* Response - Virtual Server Loopback */

Check the RB tree maintained using a RB Search of Server

Descriptor. Unique keys are used for this search. One key that

should participate in the search is the mapped port number. Can

have more than one key. Its requirement specific.

if (an entry found in the RB tree)

{

process the packet, change the required details in the

packet and  forward the same on to the secure LAN.

return (IP_PACKET_RECEIVED_AND_FORWARDED)

}

}

else

{

/* Request - Virtual Server Loopback */

Check for the destination address in the Virtual Server Table.

If (an entry found)

{

Store  the required information in the RB tree and change the

packet with the details  mention in the recommended solution,

and forward the packet onto the secure LAN.

return (IP_PACKET_RECEIVED_AND_FORWARDED)

}

}

}

Perform the normal operation. i,e pass_ip_packet_to_upper_layer.

return (IP_PACKET_RECEIVED_AND_FORWARDED)

}

else

{

/* Packet not Destined to us */

if (the packet is from Secure LAN == TRUE)

{

Check for the destination address in the Virtual Server Table.

If (an entry found)

{

Store  the required information in the RB tree and change the

packet with the details  mention in the recommended solution,

and forward the packet onto the secure LAN.

return (IP_PACKET_RECEIVED_AND_FORWARDED)

}

}

/* Not from secure LAN */

Perform Normal Operation

}

PR_TIMER.C

Age these entries at a regular interval. If the aging_timer is set to Zero, then delete the

descriptor.

Effort Estimate

Assumption made : An engineer is well worse with code, and has worked on the Proxy

Server before.

Effort                    : 100 Man hours.

50 Man hours for testing the feature.

Milestone  	     : The effort estimated, includes unit level testing and Final integration

testing.

Miscellaneous Note (Important)

Since Firewall , “echoes” every packet, this solution creates 2 packets for every packet generated from the client. One packet from Client to Firewall and another packet from Firewall to the final Destination.