For SLIP and CSLIP testing see sliptst.pln

SLIP CODE Integation and the changes to Routerware

--------------------------------------------------

slip.c

------

A new file with slip Rx and Tx function defintions.

pppinit.c

---------

1. In intialize_ppp_ports(), slip_on flag is set to TRUE if the wan port is

in dailing mode.

2. LOWER_DEVICE_DRIVER_SLIP_UP is added as another case to ppp_device__driver_control()

and will be called with command whenever the lower device driver comes up.

Here ip_control() is called with OPEN_SLIP_VIRTUAL_PORT to inform IP that port is up.

3. In the case LOWER_DEVICE_DRIVER_DOWN from the lower level driver and slip_on is TRUE,

ip_control is called with CLOSE_VIRTUAL_PORT to inform IP that port is going down.

4. In the case OPEN_DEVICE_DRIVER_PORT from LLDD, slip tx buffer list are built.

5. The function check_if_slip_on_is_set_to_true() is added.

ppprx.c

-------

In ppp_receive() (if slip_on is TRUE), slip receive processing functions are added.

ppptx.c

-------

In send_ppp_packet() (if slip_on is TRUE), slip transmit processing functions are added.

ppptimer.c

----------

If slip_on is TRUE, PPP periodic tranmits (lcp and ncp packets) are not to be sent.

ipctrl.c

--------

Another OPEN_SLIP_VIRTUAL_PORT case is added to the ip_control().

include\wanmgr.h

-----------------

OWNED_BY_SLIP is added to WAN_PORT_OWNER enumeration.

lslproto.h

----------

1. LOWER_DEVICE_DRIVER_SLIP_UP  is added to the DEVICE_CONTROL_OPERATION

enumeration.

2. OPEN_SLIP_VIRTUAL_PORT  is added to the PROTOCOL_CONTROL_OPERATION enumeration.

waninit.c

---------

A fourth parameter (int mode) is added InitSCCX() for initialization of SCC

to receive either SLIP packets only, PPP packets only or both.

wantx.c

-------

1. If slip_on is TRUE and wan tranmit fails for any case slip_packet_tranmitted

is called.

2. The tx_descriptor is put into schedule list or current tx list depend upon

sptr_wan_port_class->sptr_current_tx_descriptor is NULL or not.

3. In wan_tx_isr_complete, sptr_wan_port_class->sptr_current_tx_descriptor is put

into txmitted_list and it is set to NULL.

wanrx.c

-------

In wan_packet_received(), SLIP packet analysis is also added.

wanconf.c

---------

OWNED_BY_SLIP is added to wan config whereever necessary.

asyncscc.68k

------------

1. Array Wanfrmdelim is changed to PPPfrmdelim and SLIPfrmdelim is also added

for Tx Descriptors initialization.

2. SCC initialization is slightly modified and is done according to the mode of the operation.

3. SLIP packet recognition is added in the function check_and_verify_async_packet_received.

devdrvrs\include\driver.inc

---------------------------

SLIP_DELIMETER EQU is added.

Sachin 07 April :

-----------------

.	Why have a different command for ip_control. Ip need not

be aware of the type of HLDD.