DESIGN DOCUMENT - 68360 MTROUTER UI MODULE

------------------------------------------

Packet structure between SMC and PC COM port

--------------------------------------------

----------------------------------------------------------------------

7e	| BridgeNumber | Data ..... | CRC | 7e | Brid..

----------------------------------------------------------------------

7e		: Frame Delimiters

BridgeNumber	: Last 4 bytes of card address	- 4 bytes

Data		: Actual Packet			- maximum 512 bytes

CRC 		: Frame check sequence		- 2 bytes

MAX_PKT_LEN		-	512 bytes

SMC Frame Header	-	  4 bytes

I	Serial Management Controller

1. Configration

i. Interrupt on every character received (SMC does not support

interrupt on Control characters).

ii. 19200 baud, 8 bits, No parity, 1 stop bit

iii. 1 TxBD and 1 RxBD

2. Buffers

i . New Buffers used :

- Circular Buffer 		- 256 bytes

- Request Buffer 		- 530 bytes (512 + 12 + 4 + xtra)

- Response Buffer 		- 530 bytes	(512 + 12 + 4 + xtra)

- SCCQuote Buffer  		- 3200 bytes

- SMCTx Buffer  		- 1060 bytes (2 * ResponseBuffer)

ii. Existing Buffers used -

- RcvNetHanger from the Bridge's pool

3. Interrupt Handler

Read the SMCE to see the sources causing the interrupt.

i. Receive Interrupt

- Copy data from RXBD data ptr into CircularBuffer

- Resubmit RXBD to receive by setting E bit.

ii. Transmit Interrupt

- Do nothing

iii. Clear SMC1 bit in the CISR and return.

II	Bridge Foreground

1. Process the packet received from the SMC - call GetPacket.

III	GetPacket

1. Go through the receive state machine.

i. 	Read the next character from the SMC receive circular buffer.

Unquote characters that have been quoted and put into the Request

buffer. Repeat till the end of packet is reached(7eh).

ii.	If endofpkt is reached, check if minimum length has come.

minimum length = SMC header + SCC header + Request type packet.

iii. 	Check for packet integrity by calculating CRC for the received

pkt - call CheckCRC.

iv. 	- Process the request - call ProcessPkt.

- Quote the packet into SMC quote buffer - call Quote.

- Schedule the packet to go on the SMC - call SendPktOnSMC.

- Exit.

V	ProcessPkt

1.	Process the packet and put the response into the Response Buffer

at an offset of SMC header .

2.	Add the SMC header - the Bridge Number.

3.	Calculate CRC for the SMC pkt starting from SMC header and append the

CRC - call AddCRC.

4.	Exit

VI	SendPktOnSMC - Length of pkt to transmit.

1.  SMC quote buffer is the packet to go on the SMC. Update TXBD with

a pointer to SMC quote buffer.

2.	Length of buffer to transmit = 1.

3.	Set the status of TXBD as R.

VII	CallbackFromSCC

1. Update structure indicating that a bridge configuration pkt has come

Set the flag and copy the pointer of the RcvNetHanger.

/////////////////////////////////////////////////////////////////////////////////

*********************************************************************************

/////////////////////////////////////////////////////////////////////////////////

The first packet from the UI is QUERY_BRIDGE_NAME.

If (MTROUTER is not up)

Response =  ERR_NO_BRD_GET_PASSWORD or

ERR_NO_BRD_NO_PASSWORD depending on password present.

else if(Router is up)

Response = ERR_ROUTER_UP;

DOWNLOAD MODE:

The UI can :

1.Read, change and download the configuration.

2.Download new code.

3.Run Hardware Diagnostics.

ROUTING MODE:

The UI can:

1.Read and change the configuration.

2.Reboot the target to

- Download a new code or configuration.

- Just Reboot.