THIS DOCUMENT IS A GUIDELINE FOR DEVLOPMENT OF LOWER LEVEL DEVICE DRIVERS.

THIS IS BASED ON THE SAMPLE ASYNC DEVICE DRIVER FROM ROUTER WARE.

INIT ROUTINE

initialize_(network_type )_controller

where network_type be

a. async

b. sync

c. ethernet

d. token_ring

etc

This routine will be called from rtrware.c in the main() program loop.

This it self is to be initialized in \rtrware\store\vrtrwstr.h in the

ROUTERWARE_MODULE_DEFINITION routerware_device_drivers[] .

Each of the network_types is to have a (network)_CLASS data structure like

ASYNC_CLASS

SYNC_CLASS

ETHERNET_CLASS

TOKEN_RING_CLASS

etc

These are typically to have a enum BOOLEAN enabled to depict whether alive

or not along with that it is to have a (network_type)_PORT_CLASS so it becomes

for an ethernet port

typedef	struct	ETHERNET_CLASS

{

enum BOOLEAN			enabled;

ETHERNET_PORT_CLASS		port;

} ETHERNET_CLASS;

In each of the (network)_PORT_CLASS the following are to be present

along with any other port specific information.

(network)_TX_DESCRIPTOR			tx_descriptor[NUMBER_OF_TX_BUFFERS];

(network)_RX_DESCRIPTOR			rx_descriptor[NUMBER_OF_RX_BUFFERS];

/* this is only for static allocation of descriptors */

(network)_TX_DESCRIPTOR			*sptr_current_tx_descriptor;

/* this contains the pointer to the transmit descriptor in progress */

LINK							free_tx_list;

LINK							current_tx_list;

(network)_RX_DESCRIPTOR			*sptr_current_rx_descriptor;

LINK							free_rx_list;

LINK							current_rx_list;

USHORT							number_of_rx_buffers;

/* this value obtained from LSL_CONTROL routine */

LINK							rx_descriptor_need_buffer_list;

/* this contains the list of rx_descriptors for which an buffer

could not be allocated because malloc failed */

The (network)_TX_DESCRIPTOR itself is to have all the fields depicted below

and anything that is required.

typedef	struct	(network)_TX_DESCRIPTOR

{

LINK			links;

USHORT			length;

BUFFER	*sptr_tx_packet;

void 			(*fptr_tx_complete)

(USHORT port_number,SERIAL_BUFFER *sptr_ethernet_buffer);

enum BOOLEAN	in_use;

enum BOOLEAN	device_driver_buffer;

( this will be TRUE from upper layers if the buffer was allocated in

the device driver. In this case fptr_tx_complete will be NULL

so lsl routine packet_forward_device_driver_free (...) is called for

freeing up the buffer.)

USHORT			protocol_stack_virtual_port_number;

void			*vptr_async_buffer;

} (network)_TX_DESCRIPTOR;

The (network)_RX_DESCRIPTOR itself is to have all the fields depicted below

and anything that is required.

typedef	struct	(network)_RX_DESCRIPTOR

{

LINK			links;

BUFFER			*sptr_rx_buffer;

USHORT			buffer_size;

USHORT			number_of_bytes_rxed;

} (network)_RX_DESCRIPTOR;

The LINK is defined as

typedef	struct	LINK

{

struct	LINK	*sptr_forward_link;

struct	LINK	*sptr_backward_link;

} LINK;

in initialize_(network_type )_controller routine the following are to be

carried in addition to the specific procedure to be followed for the

initialization of the  port.

a.  Call lsl_control's  LSL_REGISTER_SERIAL_DEVICE_DRIVER or the

LSL_REGISTER_DEVICE_DRIVER.

In this call

fptr_tx_routine is to be send_(network)_packet.

fptr_tx_complete is to be (network)_packet_transmitted.

fptr_rx_routine is to be (network)_packet_received.

fptr_control_routine is to be (network)_control.

fptr_rx_buffer_return is to be (network)_return_buffer_to_device_driver

b.  call initialize_(network)_tx_descriptors()

In this routine (network)_port_class.tx_descriptor is to be

initialized. (in_use = FALSE,device_driver_buffer = FALSE,

sptr_tx_packet = NULL,fptr_tx_complete = NULL).

All the tx_descriptors are put in the free_tx_list by

calling add_entry_to_list(..).

b.  call initialize_(network)_rx_descriptors()

Call lsl_control(LSL_GET_BUFFER_SIZE,.) to get the buffer size for

the port. (network)_port_class.rx_descriptor is to be initialized.

device_driver_malloc() is to be called to get a buffer for each of

the rx_descriptor.

All the rx_descriptors are put in the free_tx_list by

calling add_entry_to_list(..).

A brief descripton and functionality of each of the different routines passed

to lsl in call to lsl_control(REGISTER_DEVICE_DRIVER....) is given below.

A seperate file containing all the routines connected with transmit of the form

(network)tx.c is to be created with the following routines.

send_(network)_packet this routine is called when ever there is a

packet to be transmitted. In this call get a free tx descriptor by calling

get_entry_from_list(free_tx_list). Update all the statistics. If the

tx_descriptor can be scheduled for transmission immediately it is done else

add_entry_to_list(current_tx_list,..) is called to put the packet in queue.

(network)_packet_transmitted is typically the tx complete interrupt service

routine. If rx_descriptor_need_buffer_list is not null meaning there is a

rx_descriptor for which malloc failed call device_driver_malloc(..) to get a

buffer.  For all of the packet transmitted call tx_complete_routine if it is

not null.  If current_tx_list is not null call

get_entry_from_list(current_tx_list) and schedule it for transmission.

A seperate file containing all the routines connected with receive of the form

(network)rx.c is to be created with the following routines.

(network)_packet_received is called periodically from lsl to check if there are

any packets. If there are any packets fill in the length and the ptr to buffer

field. If rx_descriptor_need_buffer_list is not null meaning there is a

rx_descriptor for which malloc failed call get_a_new_(network)_buffer to get

a buffer.

get_a_new_(network)_buffer is called for every rx_descriptor. In here

device_driver_malloc is called if it is successfull the rx_descriptor is put

in free_rx_list by calling add_entry_to_list(free_rx_list..) else it is put in

rx_descriptor_need_buffer_list by calling

add_entry_to_list (rx_descriptor_need_buffer_list..).

(network)_return_buffer_to_device_driver is called to free up a buffer

allocated from the device driver. In this routine call device_driver_free(...)

to free up the allocated buffer. If rx_descriptor_need_buffer_list is not null

meaning there is a rx_descriptor for which malloc failed call

get_a_new_(network)_buffer to get a buffer.