DIAL ON DEMAND

Dial on demand is a feature where the line is hungup automatically if there

is no traffic for a configurable amount of time.

|-------|              |------|

| MTR1  |------^v------| MTR2 |

|-------|              |------|

Consider two routers MTR1 & MTR2 connected as shown above. Let MTR1 WAN be

configured as answering and MTR2 WAN be configured as dialling. In the Router

configuration for MTR2 Dial-On-Demand is to enabled. When this is done the

following parameters are  automatically configured

1. Periodic IP RIP responses are disabled.

2. Spanning tree for this port is disabled.

3. Periodic IPX RIP & SAP are disabled for this port.

4. IP route Aging timer is disabled.

5. IPX RIP & SAP Age timers are disabled.

Dialling side will hangup the line when there is no traffic from this side to

the answering side. Traffic from the answering side is not considered for

hanging up the line. 1,2 & 3 assures that all periodic activity from dialling

side to the answering side is curtailed. After the line is hungup if any

packet is to go across to the answering side line is brought up again. This requries

that the routes present on the answering side be remembered. For this the 4 & 5

takes care as routes learnt will not be forgotten.

On the Answering side Dial-On-Demand is not enabled as it will not be able to

connect even if there is a demand. As Dialling side doesn't broadcast periodic

RIP's & SAP Answering side router will forget the route eventually. This can be

controlled by making the Aging timers for IPX RIP & SAP & IP Routes zero.