This document contains RouterWare, Inc. confidential information that is not to be disclosed without prior written authorization.

RouterWare, Inc.

3961 MacArthur Blvd., Suite 212

Newport Beach, CA 92660

(714) 442-0770

IP Router Software Documentation

March 4, 1996

Copyright Ó 1996, RouterWare, Inc.

Unauthorized use, duplication or distribution of this document is strictly prohibited.

RouterWare, Inc. makes no warranty for the use of this document and assumes no responsibility for any omissions and or errors which may be present.

RouterWare, Inc. retains the right to make changes to this document at any time, without notice.

All Rights Reserved

Permission to use, copy, modify, and distribute this documentation for any purpose is not permitted without the prior consent of RouterWare, Inc.

| Revision | Date | Revision Comments |
| --- | --- | --- |
| 1.30 | 1/5/95 | First pass of new style documentation. Applies to IP Router v1.30A |
| 1.31 | 3/4/96 | Added SNMP information, clean up. Applies to IP Router v1.31 |

Table of Contents

RouterWare Software Architecture Overview

Control Flow

Data Flow

Protocol Stack Interface to the LSL

initialize_ip()

ip_router_timer()

send_packet()

ip_router_rx_packet()

Ip_control()

IP Router Control Command Table

Native Interface to the IP Layer

ip_register_upper_layer_protocol()

send_ip_packet_from_upper_layer()

IP_UPPER_LAYER_PARAMETERS{}

ip_get_mtu_for_outgoing_port()

Native Interface to the UDP Layer

open_udp()

IP_SOCKET{}

Prototype for UDP user’s receive routine

IP_PARAMETERS{}

send_udp()

UDP_PACKET{}

Socket Datagram Interface to the UPD Protocol

socket()

bind()

recvfrom()

closesocket()

associate_application_with_socket()

Error reporting for all Socket Interface Functions

SNMP Interface to the IP Router

enum ASN_TYPES{}

IP Router MIB Access Functions

Sample section of RouterWare's SNMP Agent table of (MIB) VARIABLEs.

Other External Interfaces/Functions

C Preprocessor Definitions

Functions used from NVRAM library

Other Functions

Functions from the Standard C libraries

IP Router Configuration

IP

ARP

RARP

ICMP

RIP

IP Router Message Printing Control

Reserved

IP Router Configuration Rules and Notes:

# RouterWare Software Architecture Overview

All RouterWare software products conform to a common architecture the ensures consistent levels of portability and functionality across protocol stacks, device drivers, and applications.  For the purpose of this document, the RouterWare Software Architecture (“RSA”) is composed of three distinct layers: Protocol Stacks, Link Support Layer (LSL), and Device Drivers.

Figure 1

By utilizing a common set of function calls (i.e. a common API), the device drivers and protocol stacks are able to be interchanged with little or no impact to adjacent modules or other layers.  As will be seen shortly, the API between the protocol stacks and the LSL are the fundamental interface points that must be supported when putting a RouterWare protocol stack into an embedded system not providing the LSL functionality.

The Link Support Layer (LSL) is a middle layer of software separating the protocol stacks from the device drivers.  Its primary functions are to abstract the device drivers to a common port based model, multiplex data between multiple device drivers and protocol stacks, and to provide centralized control flow for the components of the RSA. For systems utilizing not utilizing the LSL, the equivalent set of services defined below are expected to be available so that the RouterWare protocol stack may function correctly.

## Control Flow

The flow of control in an RSA based system starts at the moment the system passes control the RSA’s main() C function.  From that point forward, the LSL can assume full control of the system resources, distributing processing to each of the components as is necessary, thus making the LSL the de facto RSA operating system.  Control of processing to the individual RSA components occurs through the timer and protocol_stack_receive_function functions.

The protocol_stack_receive_packet function is called by the layer below the protocol stack (i.e. LSL) upon receipt of a data packet by a device driver. As all the components of the RSA are natively run-to-completion in design, the received packet will be processed to its natural conclusion, whether that is discarding the packet, forwarding it through a protocol stack to a device driver for transmission, or sending it to an application resident above the protocol stack.  Once the packet receive processing is complete, control returns to the layer (i.e. LSL) that called the receive packet function.

The timer function provides a means of passing periodic control to each RSA component.  This control, once acquired by a protocol stack, is likely to result in processing periodic protocol functions such as table entry aging, periodic message delivery, etc.  As with the protocol_stack_receive_function, control returns to the caller of the timer function.

It is only through the use of both control entry points that full and correct operation of RSA components can be assured.

Though not directly supported, the use of multiple tasks in an RSA based system can be reasonably implemented by ensuring that individual components are treated as critical code sections that can not be preempted or re-entered.  Further optimization of semaphoring code sections in the presence of multiple tasks is left as an exercise for those porting RouterWare products to the final target platform.

## Data Flow

In the RSA, there are two principal flows of data: received traffic and traffic originating from within the RSA system itself. Received traffic is further divided into those items that are forwarded, those that are destined to the RSA itself, and those that are discarded.  Since the principal purpose of RSA components is related to forwarding of traffic from one network to another, this type of traffic is predominant.  To support the nominal flow of data, the RSA makes use of pre-allocated receive buffers, each of which is sized to hold a complete packet of data plus additional buffer management information. These buffers circulate through the RSA and are returned to the buffer pool once they have been completely processed.  For traffic originating in the protocol stacks, buffers are dynamically allocated using standard C functions.  Upon the completion of their transmission, a call back is made to the originator of the packet so that it can de-allocated (freed) and the memory returned to the system.

Protocol Stack

LSL

Receive

Buffer Pool

Device Driver

Device Driver

Protocol Stack

Protocol Stack

LSL

Receive

Buffer Pool

Device Driver

Device Driver

Protocol Stack

Protocol Stack

LSL

Receive

Buffer Pool

Device Driver

Device Driver

Protocol Stack

Forwarded Traffic

Data Flow

Traffic Addressed to System and Discarded Traffic Flow

Protocol Stack Generated Traffic Flow

?

# Protocol Stack Interface to the LSL

Each RSA protocol stack has been engineered to interface directly to the LSL component.  As the LSL is not required, the following information describes the interface between the a protocol stack and the LSL to illustrate the requirements for interfacing an RSA protocol stack to a non LSL based system.

All protocol stacks, at a minimum, interface to the LSL.  This interface, or API, is composed of five functions and the associated data structures.  These functions, generically, are:  initialize, send packet, receive packet, timer, and control.  As seen below, the principal functions used by a protocol stack are the send and receive packet functions.

Protocol Stack

LSL

ip_router_timer()

initialize_ip()

ip_control()

ip_router_rx_packet()

send_packet()

## initialize_ip()

The function initialize_ip() is the first protocol stack function to be called after configuration. A return code of PASS indicates a successful initialization, meaning that the protocol stack is fully operational.  Internal to this function, IP Router subsystems such as ARP, ICMP, UDP, and RIP are individually initialized.  If any of these subsystems fails to initialize properly, initialize_ip() will return with the FAIL code.

The IP Router maintains second and minute timers which are updated at the rate specified by the parameter passed.

| Prototype | Comments |
| --- | --- |
| enum TEST initialize_ip | enum TEST - PASS or FAIL. |
| ( |  |
| ULONG clock_ticks_per_second | RouterWare typically operates with this value set to 18 |
| ) |  |

## ip_router_timer()

The ip_router_timer() function is called from the LSL or its equivalent to pass periodic control from the LSL (or equivalent system kernel) to the IP Router.

| Prototype | Comments |
| --- | --- |
| void ip_router_timer |  |
| ( |  |
| void ) |  |

Additional Comments:	If the IP timer enabled is false, return.  If the IP timer is true, increment the IP one-second timer, is greater than the IP clock ticks per second, one second has elapsed, get the pointer to the reassembly block.  If the pointer to the reassembly block is not equal to null, increment the reassembly timer in seconds.  If it's greater than the reassembly timer time-out, set the reassembly timer in seconds to zero.  Store the pointer of the time-out reassembly block to the pointer to the reassembly block.  Set the pointer to the reassembly block to the next reassembly block in the list and time-out the current reassembly.  If the ARP timer enabled is true, increment the ARP one-minute counter.  If it's greater than 60, call the ARP one-minute timer function and set the one-minute counter back to zero.  If IP RIP enable RIP routing is set to true and RP time enabled is set to true, increment the RIP periodic timer counter.  If the need update and the periodic timer counter is greater than the next trigger broadcast time, send out a broadcast RIP update.  If the periodic timer is greater than the broadcast time interval in seconds, call the RIP periodic timer.  Set the periodic time counter back to zero.  Set the IP one-second counter to zero.

## send_packet()

The function send_packet() is called by the IP Router to transmit an IP datagram out a device driver.  This function, part of the LSL, accepts the packet, performs any necessary virtual to real port conversion and header conversion and sends it to the appropriate device driver for transmission.  Because the IP Router incorporates the ARP protocol, the packet being sent includes a data link header.  As with receiving, the packet must be contained in a single, contiguous, buffer.

Upon completion of the transmission, the device driver will call the transmit complete function specified.  The call back is used to de-allocate the packet buffer.

| Prototype | Comments |
| --- | --- |
| enum TEST send_packet | Returns PASS or FAIL. |
| ( |  |
| USHORT port_number_with_protocol_id, | Virtual IP Port number with IP Router protocol ID OR'ed into the upper byte |
| void *vptr_tx_buffer, | Pointer to buffer to be sent |
| USHORT number_of_bytes, | Number of bytes to be sent |
| enum BOOLEAN do_not_calculate_new_crc, | Set to FALSE |
| enum BOOLEAN device_driver_buffer, | Set to TRUE if packet is a device driver buffer (forwarded buffer), otherwise set to FALSE |
| void (*fptr_tx_completion) | Pointer to transmit completion routine. |
| ) |  |

Additional Comments:	Set the return value to PASS.  Get the virtual port number.  If the LSL is performing the header translation then get the protocol stack ID and real port number.  Otherwise, set the virtual port number to the real port number (it was converted at the protocol stack level.  Get a pointer to the device driver.  If the operational status is O.K. then check for odd pointers (debug utility for devices that can't handle an odd buffer pointer).  If the header translation is on, convert the header from the normalized header passed to the proper outgoing link layer header.  If the packet is a unicast or broadcast, then increment the respective counter.  Increment the number of bytes.  If the protocol ID pass through is set to FALSE (normal LAN drivers) then call the device driver send routine.  If the protocol ID pass through is set to TRUE, put the protocol stack ID back into the real port number's upper byte and call the device driver send routine.  Otherwise, increment the ifOutDiscards and call the transmit completion routine if the function pointer is not NULL.  Return the return_value.  Returns PASS if packet sent or queued, otherwise returns FAIL.

## ip_router_rx_packet()

The ip_router_rx_packet() is called by the LSL to pass a received packet to the IP Router.  Each RSA protocol stack has its own decoding logic to determine if the packet is of the interest.  If it is, the protocol stack will determine if the packet is intended to be delivered to the protocol stack itself, an application resident above the protocol stack, or to be forwarded.  After having completed all processing required on the packet, the function returns to the caller with a code indicating the nature of processing performed.  The possible results are:  packet not recognized and not forwarded, packet recognized and not forwarded, packet recognized and forwarded. This return code is used by the LSL to determine if the buffer passed to the protocol stack is ready to be reclaimed to the buffer pool or if the packet should be passed to another protocol stack for processing, as might be the case in a multiprotocol environment where the protocol stack decoding is done by threading the received packet in sequence to each stack until it is either accepted by one or rejected by all.

| Prototype | Comments |
| --- | --- |
| enum RX_PACKET_STATE ip_router_rx_packet | Receive packet state. Forwarded, not forwarded, or unknown |
| ( |  |
| USHORT port_number, | Virtual IP Port number |
| UNION_IP_PACKET *uptr_ip_rx_packet, | Pointer to IP receive packet. |
| USHORT size_of_packet | Size of packet received |
| ) |  |

Additional Comments:	Set the pointer to the LSL packet to the pointer to the receive packet minus the size of the protocol ID.  Check the LSL packet type.  If it's Ethernet 802.3 “Raw” or IEEE 802.2/LLC, return “packet not recognized”.  If it's Ethernet Type II or SNAP, check to see that the port is enabled.  If it's not, increment the number of receive packets while in disabled port and return “packet not recognized”.  Check the protocol type.  If it's an IP packet, increment the number of IP packets received and call route_ip_received_packet().  If it's an IP ARP packet, increment the number of ARP packets received.  Call the ip_arp_packet_receiv().  Otherwise increment the number of non-IP packets received and return “packet not recognized”.  The return code is equal to IP data packet received and was forwarded, return; packet recognized and forwarded.  Otherwise increment the number of packets not forwarded and return “packet recognized but not forwarded”.

## Ip_control()

ip_control() is called by the LSL to perform various run time operations on the IP Router.  The function prototype is:

| Prototype | Comments |
| --- | --- |
| enum TEST ip_control |  |
| ( |  |
| enum PROTOCOL_CONTROL_OPERATION command, | See IP Router Control Command Table |
| ULONG parameter_0, |  |
| LONG parameter_1 |  |
| ) |  |

## IP Router Control Command Table

The following table describes the control commands supported by the IP Router

| IP Router Control Command | Control Command Description |
| --- | --- |
| OPEN_PROTOCOL_STACK |  |
| CLOSE_PROTOCOL_STACK | Shuts down IP Router and returns PASS or FAIL |
| REINITIALIZE_PROTOCOL_STACK | Use when IP Router must be re-initialized after BOOTP returns the desired configuration parameter set provided by a BOOTP server |
| OPEN_VIRTUAL_PORT | This command notifies the IP Router to open a specific port and also passes the IP address of the WAN link peer. As most WAN links operate with a data link protocol (e.g. PPP and Frame Relay) that must undergo a negotiation process before the link is available for transport of IP datagrams, notification of the availability of the WAN port must occur dynamically |
| CLOSE_VIRTUAL_PORT | This command is most frequently used to a shut down an IP Port that is interfaced to a WAN link no longer available. |
| CLEAR_CACHED_ROUTE | Flushes the IP route cache |
| REGISTER_TO_IP_FROM_UPPER_LAYER | Registers a ULP |

# Native Interface to the IP Layer

The IP Router provides a native IP interface which can be used by Upper Layer Protocols (ULPs) in lieu of using the services provided by the Berkeley Style Sockets Interface.  To make use of this interface, the program(s) wishing to communicate to the IP layer must complete a registration call which communicates information to the IP Router that needed to properly send and receive IP datagrams.

The function ip_register_upper_layer_protocol() is used to carry out this registration.  The send_ip_packet_from_upper_layer() function is used by the ULP to transmit an IP datagram, while ip_get_mtu_for_outgoing_port() is used to determine the maximum size datagram that the ULP can generate without causing fragmentation. These functions, and their associated data structures are discussed below.

## ip_register_upper_layer_protocol()

Registration consists of each ULP calling the ip_register_upper_layer_protocol() function.  Transport ULPs, like TCP, are to register their “ULP Receive” functions while Routing ULPs, like IGRP, register both “ULP receive packet” and “ULP find route and update cache” functions.  The function prototype for ip_register_upper_layer_protocol() is as follows:

| Prototype | Comments |
| --- | --- |
| enum TEST ip_register_upper_layer_protocol | enum TEST is defined in rtrware\include\kstart.h. If registration of a ULP fails, FAIL is returned. Otherwise a PASS is returned. |
| ( |  |
| enum TRANSPORT_TYPE protocol_type, | TRANSPORT_TYPE is defined in rtrware\include\lslproto.h. protocol_type of the ULP is passed, making each registration distinct |
| void (*fptr_receive_packet) | Pointer to the “ULP Receive” function defined in the ULP that is to be called when IP receives a packet bound for each particular protocol. The prototype for this function is: void (*) (IP_PARAMETERS *sptr_ip_parameters, void *vptr_rx_packet, USHORT packet_size); The “ULP Receive” function returns void. |
| ( |  |
| IP_PARAMETERS *sptr_ip_parameters, | Pointer to the IP_PARAMETERS structure (see below), which is associated with the packet pointed to by vptr_rx_packet (cast to a void pointer), and the size of the packet. |
| void *vptr_rx_packet, |  |
| USHORT packet_size |  |
| ), |  |
| enum TEST (*fptr_find_route_and_update_cache) | Pointer to the “ULP find route and update cache” function defined in the ULP that is to be called when IP needs routing information to send a packet. The prototype for this function is: enum TEST (*) (ULONG type_of_service, ULONG destination_ip_address, void **ptr_to_vptr_route_entry); If the protocol does not have a route to the destination, it returns a FAIL.. Otherwise, the Routing ULP is expected to allocate an IP_ROUTE_ENTRY structure, assign this pointer to *ptr_to_vptr_route_entry, and enter the route information to the destination in this IP_ROUTE_ENTRY structure. |
| ( |  |
| ULONG type_of_service, | The type of IP service to be provided |
| ULONG destination_ip_address, | The IP address of the destination |
| void **ptr_to_vptr_route_entry | Pointer to a void pointer (i.e. a double pointer) to an IP_ROUTE_ENTRY structure |
| ) |  |
| ); |  |

## send_ip_packet_from_upper_layer()

This is the “Transmit” function that is called by ULPs when they need to send an IP packet.

| Prototype | Comments |
| --- | --- |
| void send_ip_packet_from_upper_layer | There is no return value |
| ( |  |
| IP_UPPER_LAYER_PARAMETERS *sptr_ip_upper_layer_parameters, |  |
| enum BOOLEAN forwarded_packet, | Boolean which specifies whether this IP packet is actually a received packet that is being forwarded or if it is a locally generated packet. If forwarded_packet is set to TRUE, it means that the memory for the packet was initially allocated by the device driver. If set to FALSE, it means that the IP packet was allocated by the upper layer protocol and the upper layer PDU (protocol data unit) is contained in the data portion of the IP packet. |
| IP_PACKET *sptr_ip_packet, | Pointer to the IP packet. It includes the MAC header, IP header and IP data (or upper layer PDU) |
| USHORT number_of_bytes, | Total size of the allocated IP packet (i.e. the MAC header, the IP header and the upper layer PDU.) |
| void (*fptr_tx_completion) | This function returns void |
| ( |  |
| USHORT port_number, | Pointer to the function defined in the upper layer protocol, that is called to free the memory associated with the current packet. The prototype for this function pointer is as follows: void (*) (USHORT port_number, IP_PACKET *sptr_ip_packet); This function is called with the port_number that the packet went out on or was to go out on and the pointer to the IP packet. |
| IP_PACKET *sptr_ip_packet | typedef struct IP_PACKET { UNION_MAC_HEADER mac_header; IP_HEADER header; UNION_OPTIONS_OR_DATA options_or_data; } IP_PACKET; |
| ) |  |
| ); |  |

## IP_UPPER_LAYER_PARAMETERS{}

The upper layer uses the structure pointed to by sptr_ip_upper_layer_parameters to hold all the parameters pertaining to the IP packet.

| Structure | Comments |
| --- | --- |
| typedef struct IP_UPPER_LAYER_PARAMETERS |  |
| { |  |
| ULONG source_address; | Source IP address to be used; if 0.0.0.0 then the IP address of the outgoing port will be used |
| ULONG destination_address; | IP address of the destination host |
| BYTE_ENUM (BOOLEAN) do_not_fragment_flag; | Used to set the do_not_fragment_flag in the IP header |
| SERVICE_TYPE_BIT_STRUCTURE type_of_service; | Used to set the type_of_service or TOS field in the IP header |
| USHORT sequence_id; | If greater than zero, it is used to set the packet identifier field in the IP header |
| BYTE time_to_live; | If non-zero, this value is used to set the time_to_live field in the IP header |
| BYTE protocol; | Used to set the protocol field in the IP header |
| BYTE option_length; | This is for the internal use of the IP stack |
| UNION_USER_OPTIONS user_options; |  |
| void *vptr_cached_route; | The address of the route cached on the last send by the ULP for the same session. this pointer was saved by the ULP after control returned from the last call to send_ip_packet_from_upper_layer |
| USHORT virtual_port_number; | If not NO_SUCH_PORT, do not bother to find route, send packet out through this port |
| ULONG gateway; | The tx_port_number has to be other than NO_SUCH_PORT for this field to be used. Then, if gateway is not 0.0.0.0, do not bother to find a route; send packet directly to this gateway |
| } IP_UPPER_LAYER_PARAMETERS; |  |

## ip_get_mtu_for_outgoing_port()

This function is called by ULPs, like TCP, to determine the MTU of the outgoing port to the destination host.  ULPs are required to call this function to ensure that they do not request that IP send a packet that is larger than the MTU and thus cause IP to fragment (even at the source). Note that the IP router stack supports fragmentation at intermediate hops to the destination, but it is to be avoided at the source host.

| Prototype | Comments |
| --- | --- |
| USHORT ip_get_mtu_for_outgoing_port | The MTU value returned will be no greater than 65535, the maximum value for a USHORT |
| ( |  |
| ULONG destination_address |  |
| ); |  |

# Native Interface to the UDP Layer

The IP Router provides a native UDP interface which can be used by Upper Layer Protocols (ULP’s) in lieu of using the Socket Datagram services provided by the Berkeley Style Sockets Interface.  To make use of this interface, the program(s) wishing to communicate to the IP layer must complete a registration call which communicates information to the IP Router that needed to properly send and receive IP datagrams

The function open_udp() is used to carry out this registration.  The send_udp() function is used by the ULP to transmit an UDP datagram.  UDP receives are handled by the function registered by the ULP.

## open_udp()

This function is called to create a UDP control block and register the UDP user (upper layer) protocol's receive routine. Upper layer protocols (i.e. UDP users like SNMP and BOOTP) need to register their receive function and their protocol port with UDP, otherwise UDP will not be able to deliver packets bound to those protocols.  open_udp() should be called by upper layer protocols before they can receive packets or before they try to send packets.

| Prototype | Comments |
| --- | --- |
| void *open_udp |  |
| ( |  |
| IP_SOCKET *sptr_local_socket, | The structure pointed to by sptr_local_socket informs UDP what protocol port is to be opened and with which IP address |
| void (*fptr_call_udp_user) | UDP user’s receive function |
| ( |  |
| USHORT rx_port_number, |  |
| IP_SOCKET *sptr_remote_socket, |  |
| void *vptr_packet_received, |  |
| USHORT rx_user_data_size, | The size of the user protocol data unit (i.e. UDP data). |
| IP_PARAMETERS *sptr_ip_parameters |  |
| ), |  |
| enum USER_TYPE user_type | The user_type is to be usually set to NATIVE_UDP_USER, unless one is building another interface like a socket interface on top of UDP |
| ); |  |

## IP_SOCKET{}

The IP_SOCKET structure pointed to by sptr_local_socket must be saved by the UDP user in its class structure.	Because this is to be used by the UDP user when it calls send_udp().

| Structure | Comments |
| --- | --- |
| typedef struct IP_SOCKET |  |
| { |  |
| USHORT port; | A protocol port like the BOOTP port, SNMP port etc. There is a protocol port number assigned for each upper layer protocol like BOOTP, SNMP etc. Refer to the "Assigned Numbers" RFC for protocol port numbers. |
| ULONG ip_address; | The IP address. It is usually set to 0.0.0.0 (meaning this host). |
| USHORT virtual_port_number; | This is a virtual IP (interface) port, but is usually unused by the udp_user protocols and so is usually set to NO_SUCH_PORT |
| BYTE hardware_address[16]; | The hardware_address associated with the port. This field is only used with BOOTP, DHCP or RARP |
| } IP_SOCKET; |  |

## Prototype for UDP user’s receive routine

| Prototype | Comments |
| --- | --- |
| void (*fptr_call_udp_user) | This function pointer is passed as a parameter of open_udp() returns void |
| ( |  |
| USHORT rx_port_number, | The interface port on which the packet was received |
| IP_SOCKET *sptr_remote_socket, | Pointer to the IP_SOCKET structure which contains the IP address and protocol port of the remote user. This is information obtained from the UDP header and IP header |
| void *vptr_packet_received, | Void pointer to the UDP packet (including MAC header, IP header, UDP header and data) |
| USHORT rx_user_data_size, |  |
| IP_PARAMETERS *sptr_ip_parameters | Holds all information pertaining to the IP packet |
| ), |  |

## IP_PARAMETERS{}

Though most of these fields are extracted from the IP header, they are all in host order these fields are not to be manipulated by the UDP user receive routine.

| Structure | Comments |
| --- | --- |
| typedef struct IP_PARAMETERS |  |
| { |  |
| ULONG source_address; | The source IP address in the IP header |
| ULONG destination_address; | The IP address of the destination, again from the IP header |
| USHORT offset; | Fragment offset field in the IP header |
| USHORT header_length; | Length of the IP header in bytes |
| SERVICE_TYPE_BIT_STRUCTURE type_of_service; | type_of_service or TOS field in the IP header |
| USHORT total_length; | Total_length field in the IP header |
| USHORT id; | The packet identifier field in the IP header |
| BYTE time_to_live; | The time_to_live field in the IP header |
| BYTE_ENUM (IP_PROTOCOL_VALUE) | The protocol field in the IP header |
| BYTE options_length; | The length, in bytes, of the IP options field in the IP packet |
| BYTE_ENUM (BOOLEAN) do_not_fragment_flag; | The do_not_fragment_flag in the IP header |
| BYTE_ENUM (BOOLEAN) more_fragment_flag; | The more_fragment_flag in the IP header |
| BYTE version; | The version field in the IP header |
| USHORT checksum; | The checksum field in the IP header |
| USHORT rx_port_number; | The virtual IP Port on which the packet was received |
| ULONG gateway; | This field is reserved for internal use of the IP stack |
| BYTE_ENUM (BOOLEAN) union_ip_packet_is_to_be_freed; | This field is reserved for internal use of the IP stack |
| } IP_PARAMETERS; |  |

If UDP succeeds in opening the protocol port and registering the receive routine, it returns the pointer to the UDP control block cast to a void pointer.

## send_udp()

This function is called when the UDP user (upper layer) protocol wants to send a UDP packet. The upper layer PDU is contained in the data portion of the UDP packet.  When the packet is to be sent on a specific port, the outgoing port is specified through the parameter tx_port_number.  Otherwise it is set to NO_SUCH_PORT.

| Prototype | Comments |
| --- | --- |
| USHORT send_udp | This function always returns 0. The return value has no significance. |
| ( |  |
| USHORT tx_port_number, | Outgoing port |
| IP_SOCKET *sptr_local_socket, | Pointer to the IP_SOCKET structure that was passed to the corresponding open_udp() call |
| IP_SOCKET *sptr_remote_socket, | Pointer to the IP_SOCKET structure that informs UDP the destination of the packet |
| SERVICE_TYPE_BIT_STRUCTURE type_of_service, | The type of service to be provided by IP for this packet. The IP stack does not implement any type of service, so this structure is usually zeroed |
| BYTE time_to_live, | The time_to_live field in the IP header of the packet. If this is zero, the default IP TTL is used instead. The default IP TTL is configurable. If not configured it defaults to 64 |
| void *vptr_udp_packet, | Pointer to the UDP packet cast to a void pointer. UDP assumes that the whole packet is pre-allocated by the upper layer protocol. As a result UDP does not have to do any copying of data (i.e. upper layer PDU). The structure of the UDP packet is as shown below |
| USHORT sequence_identifier, | Value to be used for the packet_identifier field of the IP header. |
| enum BOOLEAN do_not_fragment_flag, | Value to be used for the do_not_fragment_flag field of the IP header |
| USHORT number_of_bytes, | Total size of the allocated UDP packet (i.e. the MAC header, the IP header, UDP header and the upper layer PDU.) |
| void (*fptr_tx_completion) | Pointer to the function, defined in the upper layer protocol, that is called to free the memory associated with the current packet. The prototype for this function pointer is: void (*) (USHORT port_number, void *vptr_tx_packet); This function returns void |
| ( |  |
| USHORT port_number, | Port number that the packet went out on or was to go out on |
| void *vptr_tx_packet | Pointer to the packet cast to a void pointer |
| ), |  |
| void *vptr_control_block | Void pointer to the UDP_CONTROL_BLOCK that was returned by open_udp(). |
| ); |  |

## UDP_PACKET{}

| Structure | Comments |
| --- | --- |
| typedef struct UDP_PACKET |  |
| { |  |
| UNION_MAC_HEADER mac_header; |  |
| IP_HEADER ip_header; |  |
| UDP_HEADER header; |  |
| BYTE data[VARIABLE_NUMBER_OF_BYTES]; |  |
| } UDP_PACKET; |  |

# Socket Datagram Interface to the UPD Protocol

The functions socket(), bind(), recvfrom(), sendto(), closesocket(), associate_application_with_socket() form the socket (datagram) interface to UDP layer within the IP Router. Common to these functions are the SOCKADDR and SOCKADDR_IN data structures (located in \rtrware\socket\vsockadr.h) which are defined below.  SOCKADDR is a generic socket address structure while SOCKADDR_IN is specific to the Internet address family (ie. AF_INET).

#define SOCKADDR_DATA_SIZE	0x0e 		/* 15 */

| Structure | Comments |
| --- | --- |
| typedef struct sockaddr | Structure to hold an address |
| { |  |
| short sa_family; | Type of address e.g. AF_INET |
| char sa_data[SOCKADDR_DATA_SIZE]; | Value of address |
| } SOCKADDR; |  |

#define SIN_ZERO_SIZE		0x08		/* 8 */

| Structure | Comments |
| --- | --- |
| typedef struct sockaddr_in | Structure to hold an address |
| { |  |
| USHORT sin_family; | Type of address |
| USHORT sin_port; | Protocol port number |
| IN_ADDR sin_addr; | IP address |
| char sin_zero[SIN_ZERO_SIZE]; | Usually unused, set to zero |
| } SOCKADDR_IN; |  |

Note that SOCKADDR_IN and SOCKADDR have the same size and that SOCKADDR_IN fits into the SOCKADDR structure.  Therefore,

any pointer to a SOCKADDR_IN structure can be cast to a SOCKADDR structure pointer.

## socket()

socket() creates a socket used for network communication and returns an integer descriptor for that socket.

| Prototype | Comments |
| --- | --- |
| int socket | Returns the descriptor for the socket created or -1 to indicate that an error has occurred (and so failed to create the socket). |
| ( |  |
| int address_family, | Protocol or address family (AF_INET for TCP/IP family, AF_NETWARE for Novell's SPX and IPX etc.) |
| int socket_type_of_service, | Type of service (SOCK_STREAM for TCP or SOCK_DGRAM for UDP) |
| int protocol | Protocol number to use or 0 to request for the default for a given family and type. |
| ); |  |

## bind()

bind() specifies a local IP address and protocol port number for a socket. Bind is primarily used by servers, which need to specify a well-known protocol port.

| Prototype | Comments |
| --- | --- |
| int bind | bind() returns 0 if successful and -1 to indicate that an error has occurred |
| ( |  |
| int socket_descriptor, | A socket descriptor created by the "socket" call |
| SOCKADDR *sptr_sockaddr, | The address of a structure that specifies an IP address and protocol port number |
| int sockaddr_length | The size of address structure in bytes |
| ); |  |

## recvfrom()

recvfrom() extracts the next message that arrives at a socket and records the sender's address (enabling the caller (application) to send a reply).

| Prototype | Comments |
| --- | --- |
| int recvfrom | Returns the number of bytes in the message if successful or a -1 to indicate that an error has occurred. |
| ( |  |
| int socket_descriptor, | A socket descriptor created by the "socket" function. |
| char *cptr_data, | The address of a buffer to hold the message. |
| int length, | The length of the buffer. |
| int flags, | Control bits that specify out-of-band data or message look-ahead. Currently not supported and hence ignored |
| SOCKADDR *sptr_sockaddr, | The address of a structure to hold the sender's address |
| int *iptr_sockaddr_length | The address of an integer that holds the length of the address structure and the size of the sender's address is returned at that location |
| ); |  |

## sendto()

sendto() sends a message by taking the destination address from a structure.

| Prototype | Comments |
| --- | --- |
| int sendto | Returns the number of bytes sent if successful and -1 to indicate that an error has occurred. |
| ( |  |
| int socket_descriptor, | Socket descriptor created by the "socket" function. |
| char *cptr_data, | Pointer to the message. |
| int length, | The length of the message in bytes. |
| int flags, | control bits that specify out-of-band data or message look-ahead. Currently not supported and hence ignored. |
| SOCKADDR *sptr_sockaddr, | Pointer to the address structure. |
| int sockaddr_length | The length of the address structure in bytes. |
| ); |  |

## closesocket()

closesocket() terminates communication gracefully and removes the socket.  Any unread data waiting at the socket will be discarded. An application calls closesocket() after it finishes using a socket.

| Prototype | Comments |
| --- | --- |
| int closesocket | Returns 0 if successful and -1 to indicate that an error has occurred |
| ( |  |
| int socket_descriptor | The descriptor of a socket to be closed |
| ); |  |

## associate_application_with_socket()

associate_application_with_socket() is to be called by every application immediately after it creates a socket with the "socket" call. This function is specific to RouterWare's implementation of the socket library.

| Prototype | Comments |
| --- | --- |
| int associate_application_with_socket | Returns 0 if successful and -1 to indicate that an error has occurred. |
| ( |  |
| int socket_descriptor, | Socket descriptor created by the "socket" call |
| ULONG application_id | This is the id that the application obtains when it registers itself (with say LSL in the RouterWare architecture) |
| ); |  |

## Error reporting for all Socket Interface Functions

When an error occurs, the variable error_number in the socket_class structure, defined in the socket library, contains the code that specifies the cause of the error.

Socket Error Codes (refer to \rtrware\socket\ksocket.h)

WOULD_BLOCK_ERROR

NOT_CONNECTED_ERROR

SOCKET_NOT_SUPPORTED_ERROR

ADDRESS_FAMILY_NOT_SUPPORTED_ERROR

IS_CONNECTED_ERROR

OPERATION_NOT_SUPPORTED_ERROR

ALARM_ERROR

ABORT_ERROR

INTR

CONNECTION_REFUSED_ERROR

MESSAGE_SIZE_ERROR

ADDRESS_IN_USE_ERROR

MAXIMUM_ERROR

SHUTDOWN_RECEIVE_CHANNEL_ERROR

SOCKET_INTERFACE_NOT_ENABLED

TOO_MANY_SOCKETS_OPEN

BAD_SOCKET_HANDLE

BAD_DATA

NULL_LOCAL_ADDRESS

NULL_PEER_ADDRESS

NULL_SOCKET_ADDRESS

ALREADY_BOUND_TO_ADDRESS

NOT_BOUND_TO_ADDRESS

INVALID_SOCKET_VECTOR

BAD_LENGTH

BAD_NEW_SOCKET_HANDLE

END_OF_FILE_ERROR

# SNMP Interface to the IP Router

MIB’s are accessed by an SNMP Agent which parses SNMP requests to a specific MIB and then calls the appropriate access function. The RouterWare SNMP Agent library keeps an array of VARIABLE data structures, one for each MIB variable that is supported.

| Structure | Comments |
| --- | --- |
| typedef struct VARIABLE |  |
| { |  |
| OBJECT_ID object_id [ MAXIMUM_NUMBER_OF_SUB_IDS_IN_OBJECT_ID ]; | The object ID of the MIB variable. The definition of OBJECT_ID is: typedef BYTE OBJECT_ID; |
| BYTE object_id_length; | The length of the variable's object id in bytes. |
| BYTE_ENUM (ASN_TYPES) type; | The type field specifies the type of the MIB variable. It is defined in \rtrware\applicat\snmp\ksnmp.h. See table below. |
| char *mib_name; | The character string name of the MIB variable. |
| USHORT access_control_list; | Specifies whether variable is a READ_ONLY or a READ/WRITE |
| enum BOOLEAN is_table_entry; | Set to TRUE for a table MIB variable and set to FALSE otherwise. |
| enum TEST (*fptr_find_mib_variable) | The MIB access function for the variable(s). See table below. |
| ( |  |
| enum MIB_OPERATION mib_operation, | MIB operation to be performed (SET or GET). Defined in \rtrware\include\visnpstr.h. |
| char *cptr_mib_string, | The name of the MIB variable. |
| BYTE *bptr_variable_value, | The pointer to a location that is to hold (for a GET operation) or already holds (for a SET operation) the MIB variable value. |
| USHORT *usptr_size_of_variable_value, | The pointer to a location that is to hold (for a GET operation) or already holds (for a SET operation) the size (in bytes) of the MIB variable value. |
| USHORT *usptr_number_of_bytes_in_table_indices, | The pointer to a location that holds the number of bytes in the table indices array that is passed. |
| BYTE *bptr_table_indices, | The pointer to the table of indices. Once the MIB variable (i.e. a table MIB variable) has been recognized, the table indices are necessary to uniquely identify an instance of the variable in the table. |
| enum BOOLEAN *eptr_end_of_table, | This pointer is used to set a boolean variable to TRUE if the there are no more instances of the variable to be read from the MIB table. |
| char **cptr_next_variable_name | Pointer to the next variablefor a table MIB variable, if there are no more instances of the variable in the MIB table, this double character pointer is made to point to the name of the next variable (a scalar variable or table variable) in the protocol's MIB. for a scalar MIB variable, if there are no more variables in the protocol's MIB, it points to NULL. |
| ); |  |
| } VARIABLE; |  |

## enum ASN_TYPES{}

enum ASN_TYPES is defined in \rtrware\applicat\snmp\ksnmp.h and is currently set to:

enum ASN_TYPES

{

ASN_BOOLEAN = 0x01,

ASN_INTEGER = 0x02,

ASN_BIT_STRING = 0x03,

ASN_OCTET_STRING = 0x04,

ASN_NULL = 0x05,

ASN_OBJECT_ID = 0x06,

ASN_SEQUENCE = 0x10,

ASN_SET = 0x11,

ASN_UNIVERSAL = 0x00,

ASN_APPLICATION = 0x40,

IP_ADDRESS = (ASN_APPLICATION ¦ 0),

COUNTER = (ASN_APPLICATION ¦ 1),

GAUGE = (ASN_APPLICATION ¦ 2),

TIMETICKS = (ASN_APPLICATION ¦ 3),

OPAQUE = (ASN_APPLICATION ¦ 4),

_PHYSICAL_ADDRESS = (ASN_APPLICATION ¦ 5),

ASN_CONTEXT = 0x80,

ASN_PRIVATE = 0xC0,

ASN_PRIMITIVE = 0x00,

ASN_CONSTRUCTOR = 0x20

};

## IP Router MIB Access Functions

| get_rfc1213_arp_table_mib_variable() | Used to access the ARP MIB variables. |
| --- | --- |
| get_rfc1213_ip_mib_variable() | Used to access the IP MIB variables. |
| get_rfc1213_icmp_mib_variable() | Used to access the ICMP MIB variables. |
| get_rfc1213_udp_mib_variable() | Used to access the UDP MIB variables. |

## Sample section of RouterWare's SNMP Agent table of (MIB) VARIABLEs.

The three lines of the table shown below are the declarations of the VARIABLE structures for three "interface MIB" variables.

{{MIB,2,1,0}, 9, INTEGER, "ifNumber", READ_ONLY, FALSE, get_lsl_mib_variable},

{{MIB,2,2,1,1,0xff,0xff,0xff,0xff}, 14, INTEGER, "ifIndex", READ_ONLY, TRUE, get_lsl_mib_variable},

{{MIB,2,2,1,2,0xff,0xff,0xff,0xff}, 14, STRING, "ifDescr", READ_ONLY, TRUE, get_lsl_mib_variable}

The preprocessor definition for MIB is in file "ksnmp.h" of the SNMP library and is shown below:

#define MIB 1, 3, 6, 1, 2, 1

The three MIB variables are "ifNumber", "ifIndex" and "ifDescr". All the three are READ_ONLY variables. While "ifNumber" is a scalar variable (i.e. not a table variable), the other two are table variables. The index for the table variables is "ifIndex". Hence the object id has four 0xff terms in it that specify the index of the MIB variable instance. The function "get_lsl_mib_variable" is what is called to access the "interface MIB" variables defined in the LSL library.

# Other External Interfaces/Functions

## C Preprocessor Definitions

| __BIG_ENUMS__ | Required. |
| --- | --- |
| __IP_ALARM_DEBUG__ | To be defined if only alarm printfs are required for debugging. |
| __IP_BREAKPOINT__ | Reserved for RouterWare's internal use. |
| __IP_DEBUG__ | To be defined if full debugging using printfs is required. |
| __LSL__ | Required when used with RouterWare's LSL library. |
| __NEW_CONFIG__ | Reserved for RouterWare's internal use. |
| NUMBER_OF_IP_PORTS=16 | Maximum number of ports to be defined. This definition is required |

Note that if __IP_DEBUG__ is defined, __IP_ALARM_DEBUG__ need not be specifically defined because it gets defined in \rtrware\stacks\ip\ip.h

## Functions used from NVRAM library

set_ulong_decimal_value()

set_variable_port_and_enable()

set_variable_port_and_ip_address()

set_variable_port_and_ushort_decimal_value()

set_variable_string()

set_ushort_decimal_value()

set_byte_decimal_value()

## Other Functions

## Functions from the Standard C libraries

| stdlib library: | stdio library: | string library: | stdarg library: |
| --- | --- | --- | --- |
| srand() | sscanf() | strcmp() | va_start() |
| rand() | printf() | strchr() | va_arg() |
|  | sprintf() | strtok() | va_end() |
|  | vprintf() | memcpy() |  |
|  |  | memset() |  |

The macro offsetof() is defined by Microsoft C++ in the file \mvsc20\include\stddef.h.  Its definition is:

#define offsetof(s,m)	(size_t)&(((s *)0)->m)

# IP Router Configuration

The IP Router offers many capabilities, some of which, ultimately, pertain to the end user operating the router, other portions which may only be relevant to the development engineer tasked with porting the IP Router to a target system.  In the RouterWare model for configuration, these two different configuration requirements are met through a single process that allows for rapid re-configuration during prototype development and serves well as a system upon which run time (i.e. end user) configuration can be layered upon.

This single process, common to all RouterWare products, is referred to as NVRAM.  NVRAM defines, for each protocol stack, a set of configuration parameters and a method to process the configuration information and deliver it to the protocol stacks at the time of initialization of the protocol stacks.  As each protocol stack receives configuration information, it initializes the appropriate variables and data structures.  The configuration parameters are defined as ASCII text strings that have been engineered to be very readable by man and machine alike through the use of a hierarchical naming scheme and a consistent set of data type parameters.

The general form of an NVRAM configuration text string is:

Configuration Parameter Name = {VV}, value A1{, value A2,…value AN; value B1, value B2,…value BN;…}

where the items that are in bold text are mandatory elements, the items within the curly braces are optional depending upon the specific parameter, the comma separated values are the members of a set, which is delineated from additional sets by semicolon(s).

The Configuration Parameter Name for all IP Router configuration strings begin with the text “IP”, which refers to the protocol stack that the configuration parameter will be delivered to.  For values that are designate for each IP Port, the “Port” keyword immediately follows the initial “IP” keyword, thus distinguishing the global versus per port configuration parameters.  The text that follows these keywords uniquely identifies configurable parameter.  For example,

IP Reassembly Timeout = 30

defines the global IP parameter for Reassembly Timeouts as being 30 (seconds).

For per port configurable parameters, the “VV” item refers to the IP Port number to which the configuration parameter is to be applied to.  As the IP Port number serves as the basis for most of the IP Router operation, it is well known throughout the code and IP Router architecture.  The configuration strings

IP Port Address = 00, 128.1.2.3

IP Port Address = 01, 192.5.4.3

define the IP Address for two IP Ports, numbered 00 and 01.

Throughout the descriptions of the IP Router Configuration Parameters, the following syntax is used:

Left and right square brackets, “[“ and “]”, are used to indicate the position of configuration value(s).

The vertical bar, “|”, is used to delineate the two or more supported keywords (e.g. Enabled|Disabled = Enabled or Disabled)

[IP Address] refers to a four byte dotted decimal IP address (e.g. 128.1.2.3)

[IP Address Mask] refers to a four byte dotted decimal IP Address Mask (e.g. 255.255.255.0)

[…] refers to a repetition of the previous definition

Parameter values specified with a number immediately followed by a “+” (e.g. “1+”) indicate a range of supported values starting from the specified value and continuing to the maximum value supported by the C data type used to hold the associated variable (e.g. 255 for unsigned byte storage).

Parameter values specified with a question mark (e.g. “?-255”) indicate a value that must be determined

For those parameter values that don’t have supported values specified, the type of data to be coded is specified.

To help first time configuration, the suggested minimum set of configuration parameters needed to assess IP Router operation in an Ethernet LAN environment are shaded as follows:

## IP

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP = | Master enable/disable for the IP Router | [Enabled¦Disabled] | Enabled |
| - IP Default Route = | Specifies the IP address of an adjacent router, and the decimal metric (i.e. hops or “distance”) to that router, to send traffic to when a route to a destination is not otherwise known | [IP Address], [Metric] |  |
| - IP Default TTL = | Specifies the IP Time-To-Live (TTL), in decimal hops, to be set in all locally generated IP packets EXCEPT those for RIP messages | [1-255] | 64 |
| - IP Forwarding = | Master control for forwarding of IP datagrams | [Enabled¦Disabled] | Enabled |
| - IP Host Name = | ASCII Text string name of the IP Router for use by the RouterWare BOOTP | [ASCII Text] |  |
| - IP Maximum Number of Routes Cached in Each Hash Table Bucket = | IP Route cache hash table adjustment parameter | [Decimal Value] | 2 |
| - IP Number of Ports = | Number of IP ports to be configured for use by the IP Router | [Decimal Value] |  |
| - IP Number of Route Cache Hash Table Buckets = | IP Route cache hash table adjustment parameter. | [Decimal Value] | 4 |
| - IP Port = | Enable/disable control on a per port basis | VV,[Enabled¦Disabled] |  |
| - IP Port Address = | 32 bit dotted decimal (e.g. 1.2.3.4) IP address to be assigned to each port | VV,[IP Address] |  |
| - IP Port Address Mask = | 32 bit dotted decimal (e.g. 255.255.255.0) IP Address Mask to be assigned to each port | VV,[IP Address Mask] |  |
| - IP Port BOOTP = | Enable/disable processing of any BOOTP/DHCP related messages for the specified port | VV,[Enabled¦Disabled] | Disabled |
| - IP Port Default Gateway = | IP Address of the default gateway to be used when IP Router is operating as an IP client | VV,[IP Address] |  |
| - IP Port Frame Type = | Data link frame type | [TYPE_II¦SNAP] | TYPE_II |

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Port Maximum Reassembly Size = | Maximum permissible IP packet size, in decimal bytes, after reassembly | [?-65535] | 65535 |
| - IP Port Metric = | Logical RIP “distance” associated with the network on the specified port | VV,[1-255] | 1 |
| - IP Port MTU = | IP Maximum Transmission Unit value, in decimal bytes, for the specified port | VV,[64+] | 1518 |
| - IP Port Point-to-Point Link = | Defines if the specified IP port is attached to a point to point (WAN) link | VV,[Enabled¦Disabled] | Disabled |
| - IP Port Remote Address = | IP Address of the peer at the opposite end of the point to point link associated with the specified port | VV,[IP Address] | Disabled |
| - IP Port Token Ring = | Declares if the specified IP port is attached to an IEEE 802.5 Token Ring | VV,[Enabled¦Disabled] | Disabled |
| - IP Port Token Ring Largest Frame Size = | Specifies the value, in decimal bytes, for the “Largest-Size Information Field” for Source Routed IP datagrams sent out the associated port | VV,[516¦1500¦2052¦4472¦8144¦11407¦17800] |  |
| - IP Reassembly Timeout = | Time, in decimal seconds, that IP will wait for the all the fragments of an IP datagram to be reassembled before dumping the partial result | [1+] | 30 |
| - IP Set Default Values = | Enables use of default configuration values. This configuration parameter must be specified before all other IP configuration parameters in order to work. | [Enabled¦Disabled] |  |
| - IP Static Route Table Entries = | Define static routes to hosts or networks | [IP Address, Next Hop Gateway Address, IP Address Mask, Next Hop Gateway Metric]; […] |  |

## ARP

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Port ARP = | Enable/disable control for ARP on the specified port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port ARP Aging Timeout = | Defines the time, in decimal seconds, that an ARP table entry will persist since the time it was first learned | VV,[1+] | 600 |
| - IP Port ARP Number of Retransmissions = | Defines the decimal number of attempts to be made at resolving an address via ARP. Each attempt will occur after the preceding request times out | VV,[1+] | 3 |
| - IP Port ARP Reply Timeout = | Defines the time, in decimal seconds, to wait for an ARP reply to a request previously transmitted out the specified port | VV,[ 1+] | 3 |
| - IP Port RFC1042 = | Defines if Source Routed Token Ring ARP procedures are to be followed per RFC1042. If enabled, IP Router will ARP make use of Source Routed ARP messages when attempting to resolve an address on a Token Ring LAN. If disabled, non source routed operation will be used | VV,[Enabled¦Disabled] | Disabled |
| - IP Port Remote Access = | Enable/disable for including the specified IP port into the remote access mode of the IP Router | VV,[Enabled¦Disabled] | Disabled |
| - IP Port Proxy ARP = | Enables use of Proxy ARP based forwarding between this and one other IP Port configured for Proxy ARP operation | VV,[Enabled¦Disabled] | Disabled |

## RARP

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Port RARP Client = | Enable/disable control for RARP Client operation on the specified port. This value is mutually exclusive to the “IP Port RARP Server” parameter | VV,[Enabled¦Disabled] | Disabled |
| - IP Port RARP Reply Timeout = | Defines the time, in decimal seconds, to wait for an RARP reply to a request previously transmitted out the specified port | VV,[ 1+] | 3 |
| - IP Port RARP Server = | Enable/disable control for RARP Server operation on the specified IP Port. This value is mutually exclusive to the IP Port RARP Client | VV,[Enabled¦Disabled] | Disabled |
| - IP Port RARP Server Table Entries = | Defines IP address to hardware address translations to be used by the RARP Server | VV,[IP Address1, Hexadecimal Hardware Address1]; […] |  |

## ICMP

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP ICMP Address Mask Request Timeout = | Time, in decimal seconds, that the IP Router will wait for a reply to an ICMP Address Mask Request before terminating the operation | [1+] | 10 |
| - IP Port ICMP Mask Reply = | Enable/disable control for generating and processing ICMP Mask Reply messages | VV,[Enabled¦Disabled] | Enabled |
| - IP Port ICMP Mask Request = | Enable/disable control for generating and processing ICMP Mask Request messages | VV,[Enabled¦Disabled] | Enabled |
| - IP Port ICMP Redirect = | Enable/disable control for generating and processing ICMP Redirect messages | VV,[Enabled¦Disabled] | Enabled |

## RIP

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP RIP = | Master control for operation of RIP v1 based routing | VV,[Enabled¦Disabled] | Disabled |
| - IP RIP Neighbor List = | Enable/disable use of the RIP Neighbor List | VV,[Enabled¦Disabled] | Disabled |
| - IP RIP Neighbor List Entries = | The list of RIP neighbors that RIP will accept routing information from | [IP Address1];[IP Address2];[…] |  |
| - IP RIP Response Interval = | The time, in decimal seconds, between successive periodic RIP Response messages | [1+] | 30 |
| - IP RIP Route Aging Timeout = | The time, in decimal seconds, from learning a route via RIP, to the time it is marked as Aged (metric set to 16 a.k.a. unreachable) | [1+] | 180 |
| - IP RIP Route Garbage Collection Timeout = | The time, in decimal seconds, from a RIP based route being set to Aged to its being deleted from the route table | [1+] | 120 |
| - IP RIP TTL = | The Time To Live, in decimal hops, to be used when sending RIP messages. | [1-255] | 1 |
| - IP Port RIP Advertise List Entries = | List of destination hosts or networks that are permitted to be announced by RIP out the specified port | VV,[IP Address1];[IP Address2];[…] |  |
| - IP Port RIP Announce Default Route = | Enable/disable announcing the RIP Default Route out the specified port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Announce Host Routes = | Enable/disable announcing IP Host routes out the specified port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Announce Static Routes = | Enable/disable announcing static IP routes out the specified port | VV,[Enabled¦Disabled] | Disabled |
| - IP Port RIP Poison Reverse = | Enable/disable applying the poison reverse algorithm to RIP routes being advertised out the specified port | VV,[Enabled¦Disabled] | Enabled |

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Port RIP Reject List Entries = | List of destination hosts or networks that are not permitted to be learned by RIP from RIP messages received on the specified port | VV,[IP Address1];[IP Address2];[…] |  |
| - IP Port RIP Route Holddown = | Enable/disable for “holding down” (i.e. locking) aged routes learned from RIP messages on the specified port. If enabled, an aged route cannot be “refreshed” to a non-aged status but must instead deleted and then relearned, thus enhancing the stability of the RIP topology in the presence of transients. | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Send Requests = | Enable/disable sending of RIP requests on the specified IP Port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Send Responses = | Enable/disable sending of RIP responses on the specified IP Port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Split Horizon = | Enable/disable control for using the split horizon algorithm when sending RIP responses on the specified IP Port | VV,[Enabled¦Disabled] | Enabled |
| - IP Port RIP Summarize Routes = | Enable/disable of generating route summaries in RIP responses sent out the specified IP Port. Route Summarization consists of announcing only the parent network address of IP subnets to other IP networks. | VV,[Enabled¦Disabled] | Enabled |

## IP Router Message Printing Control

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Printf = | Global enable/disable for all IP console printing | [Enabled¦Disabled] | Disabled |
| - IP Printf Alarm = | Enable/disable printing of critical events that need immediate attention | [Enabled¦Disabled] | Disabled |
| - IP Printf ARP = | Enable/disable printing of all ARP packets and related events (Rx, Tx, table add, entry aging, etc.) | [Enabled¦Disabled] | Disabled |
| - IP Printf CACHE = | Enable/disable printing of all IP route cache related events such as adds, deletes | [Enabled¦Disabled] | Disabled |
| - IP Printf Data = | Enable/disable printing of packet data for all datagrams headers sent and received by IP Router | [Enabled¦Disabled] | Disabled |
| - IP Printf Diagnostic = | Enable/disable printing of special events | [Enabled¦Disabled] | Disabled |
| - IP Printf FRAGMENT = | Enable/disable printing of all IP fragmentation related events | [Enabled¦Disabled] | Disabled |
| - IP Printf ICMP = | Enable/disable printing of all ICMP packets and related events | [Enabled¦Disabled] | Disabled |
| - IP Printf IP = | Enable/disable printing of all IP protocol datagram headers on sent and received | [Enabled¦Disabled] | Disabled |
| - IP Printf Memory = | Enable/disable printing of memory allocation/freeing events | [Enabled¦Disabled] | Disabled |
| - IP Printf RARP = | Enable/disable printing of all RARP packets and related events | [Enabled¦Disabled] | Disabled |
| - IP Printf REASSEMBLY = | Enable/disable printing of all IP datagram reassembly related events | [Enabled¦Disabled] | Disabled |
| - IP Printf RECEIVE = | Enable/disable printing of all IP headers for IP datagrams received | [Enabled¦Disabled] | Disabled |
| - IP Printf RIP = | Enable/disable printing of all RIP related datagrams (including route information) and events | [Enabled¦Disabled] | Disabled |
| - IP Printf ROUTE = | Enable/disable printing of the IP routing table and related events (e.g. route add, delete, and aged) | [Enabled¦Disabled] | Disabled |
| - IP Printf TRANSMIT = | Enable/disable printing of all IP headers for IP datagrams transmitted | [Enabled¦Disabled] | Disabled |
| - IP Printf UDP = | Enable/disable printing of all UDP related datagram headers and events | [Enabled¦Disabled] | Disabled |

## Reserved

| Configuration Parameter | Description | Supported Values | Default |
| --- | --- | --- | --- |
| - IP Configuration Breakpoint = | Reserved for RouterWare Internal Use | [Enabled¦Disabled] |  |
| - IP Initialization Breakpoint = | Reserved for RouterWare Internal Use | [Enabled¦Disabled] |  |
| - IP Runtime Breakpoint = | Reserved for RouterWare Internal Use | [Enabled¦Disabled] |  |
| - IP Shutdown Breakpoint = | Reserved for RouterWare Internal Use | [Enabled¦Disabled] |  |

## IP Router Configuration Rules and Notes:

- All configuration parameters MUST be correct in syntax and MUST specify a supported value that does not conflict with any of the rules specified herein

- The “IP Set Default Values” parameter, if used, MUST be the first configuration parameter processed by the IP Router

- Configuration parameters processed after those set by the “IP Set Default Values” parameter will override the default values

- Configuring parameters during run time operation of the IP Router is not directly supported.

- The minimum configuration parameter set for IP Router is: “IP Set Default=enabled”, “IP=enabled”, “IP Number of Ports=2”, and the per port parameters: “IP Port =”, “IP Port Address=”, “IP Port Address Mask=”.  This configuration set will provide IP routing between locally attached LANs and will perform ICMP and ARP operations as is appropriate.

- To add RIP based routing to the minimum configuration parameter set, specify “IP RIP=enabled”

- IP Port numbers shall start at the value 00 and commence sequentially to the value defined by the “IP Number of Ports=” string

- “IP Port Default Gateway” is only for use when the IP Router is being used as an IP client

- An IP Port associated with a WAN interface MUST have “IP Port Point to Point Link” set to enabled, “IP Port ARP” set to disabled, “IP Port Remote Address” specified, and “IP Port Frame Type” set to TYPE_II.

- When configuring static route and/or default route information, the IP port configuration parameters that pertain to initializing the IP Route table MUST be processed before the static and default route parameters.  A simpler version of this rule is to specify the static and default route configuration information after all other IP configuration parameters have been processed to avoid of any inappropriate conflicts within the routing table.

- When configuring a port for IP Remote Access, there must be at least one IP Port (with a lower IP Port number than the first Remote Access port) which is interfaced to a network that supports ARP.  Use of Remote Access Mode is mutually exclusive to the use of Proxy ARP. The intended configuration for Remote Access is one 802 LAN and one or more WAN ports, one or more of which are configured for Remote Access.

- Proxy ARP only supports operation with two IP Ports, both of which must support ARP operation. Proxy ARP MUST be operated to the mutual exclusion of all other forms of IP routing (i.e. RIP, static, and default based routing must be disabled) on that port

- Static routes may be to hosts, nets, or subnets.

- Distribution of a net or subnet across multiple interfaces is only supported under configurations where resolution of the appropriate port to route data through can be determined from the destination IP address only.  At present, the only known configuration that is supported is a collection of point to point links with hosts or routers located at the distant end. Host addresses are typically defined by static route entries though they may be learned dynamically too.

- When utilizing RouterWare’s BOOTP or DCHP Client software, to configure the IP Router, the IP Router must be configured with the following configuration: “IP Set Default=enabled”, “IP=enabled”, “IP Number of Ports=”, and for each of the IP ports, “IP Port =Enabled”.

If values are not specified, a Default Route will not be announced. The metric specifies the routing “distance” to the adjacent router and is typically set to one.

Total number of entries in the Route Cache is IP Maximum Number of Routes Cached in Each Hash Table Bucket multiplied by IP Number of Route Cache Hash Table Buckets

The minimum value supported is equal to the maximum value of all the link MTUs encountered for any IP datagram routed via the specified port

Includes the data link header, IP header, and IP data field

When set, the IP Router knows to qualify routing table information related to the specified port with the state of the point to point link (i.e. up/down).

IP Router establishes a host route to the end point of each point to point link, the IP address of which comes from this parameter.

IP Port MTU and all buffering services below the associated IP Port (i.e. O/S and device driver) must support a frame size greater than or equal to the configured value

Must follow all the “IP Port…” parameters to ensure that the static routes are not over written by the per port configuration procedures.

Example: IP Static Route Table Entries = 128.2.30.0, 128.1.30.95, 255.255.255.0, 2; 128.2.40.0, 128.1.30.94, 255.255.255.0, 3

This mode is a hybrid of Proxy-ARP geared to support remote access clients that (typically) connect to IP Ports via point to point links.  Each of the WAN ports to be included in this mode is identified by this parameter

Example: IP Port RARP Server Table Entries = 00, 10.0.0.0, aabbccddeeff; 11.0.0.0, bbccddeeffaa