*******Memory Map of the Router*******

The memory map of the router is as follows.

0x0 - 1Mb  is Flash Prom.

1Mb - 3Mb(for RouteFinder) or 5Mb(for MTRouter) is the DRam area.

The space between 0x100000 & 0x102000 is reserved for debug monitor and

for the interrupt vector handlers. The S-records for the router code

are generated from 0x102000. Look at loc.cmd file.

At 0x102000 contains the version number of the router.

At 0x102008 contains the release date.

At 0x102020 contains the signature.

At 0x102040 is the actual program beginning always.

At 0x102080 is the AsyncHDLC micro code for 68360 VerA.2.

At 0x102880 is the AsyncHDLC micro code for 68360 VerB.2.

At 0x103080 is the AsyncHDLC micro code for 68360 VerC.1.

After this the actual code starts.

0x200000 is the heap start always.

The stack pointer is located 0x1ffff0. The stack grows below this. So

code, data and stack  are located in both RouteFinder and MTRouter in

between 0x100000 & 0x200000 DRam area.

*******MAKEFILE*******

The C compiler options & the assembler options are defined in the

\rtrware\include\glblmkdef file. A batch file called cat is called in from

the makefile in \rtrware\store directory. In here the s-records of the

micro code of AsyncHDLC of all the known versions of 68360 is concatenated

along with the S-Records of the router code before the bin is created.

\rtrware\makeall.bat goes down to each and every sub-directory and runs nmake.

\rtrware\frcmkall goes down to each and every sub-directory and runs nmake -a.

***********Microcode initialization***********

The AsyncHDLC micro code from motorola is always from 0x20000. This

has to be relocated at the appropriate memory location. There is a simple

utility srec.exe (from srec.c) which takes the S-Records file and the start

address as the input and converts it into S-Records for any memory

location. The HEX files in the \rtrware\360ppp\*.hex are the converted hex

records for the different versions of the 68360 chip. \rtrware\store\*.srx

are the original s-records from motorola located at 0x20000. The S-Records

of the micro code for different versions of 68360 are re-located at the

following addresses.

For VerA.2 is at 102080

For VerB.2 is at 102880

For VerC.1 is at 103080

This is done using srec.exe. Depending upon the version of the 68360 the

appropiate micro code is initialized.

For any newer version of 68360 convert the S-Records to be placed at

0x103080 + 2k ( see pmain.68k).

******Initialization for the library*******

There is single bin for the 2-port as well as 4-port router. The amount of

DRAM present is read in from the flash. The location from which the heap

is supposed to start are sent as parameters to StdLibInit() in

\rtrware\stdlib\libinit.c where the initialization is done. In either case

the heap always starts from 0x200000. The DRAM size is 3Mb or 1Mb depending

on whether it is MTRouter or RouteFinder.