# $ Firewall Configuration

You can setup various Firewall protocol stack and device driver parameters through this application. Also basic hardware testing and running the console terminal is possible from here. The various options are listed below.

Certain options are enabled depending on the state of the target. If no Firewall (target) is found, run the Port Setup application, specify the right local port parameters and retry running this application. You can also connect to the target through a dialup modem.

Firewall configuration program also has a console terminal for displaying various debugging messages output from the target. You can control the display of these messages using this  application. Normally you should turn off display of all messages for better performance of the server.

Stacks and Drivers

IPHIDD_IP_CONFIG

This button selects the Internet Protocol Configuration.

PPPHIDD_PPP_CONFIG

This button selects the Point-to-Point ProtocolPPP Configuration.

WANHIDD_WAN_CONFIG

This button selects the Wide Area Network link Configuration

Other Configuration

Proxy ServerHIDD_DIALOG_PROXY_IP

This button selects the ProxyServer Configuration

DHCP  ServerHIDD_DIALOGDHCP

This button selects the DHCPDHCP Server Configuration

ApplicationsHIDD_DIALOG1

This button selects the Configuration for Telnet ServerTelnet,  TFTP ServerTFTP and WEB browser.

Virtual ServersHIDD_NAT_SETUP

This button selects the Virtual Servers Configuration.

HIDD_DIALOG_IP_FILTERING_TYPE

DiagnosticsHIDD_DIAGNOSTIC

A limited set of hardware diagnostics are available through this button. It is also possible to test the communication link.

If  the Firewall is running, it will have to be rebooted before diagnostics can be performed. (You will be queried before doing so).

Print Console

Selecting this brings up the console terminal that displays the print messages   received from target.

Download Setup

You can update the Firewall Configuration on the target.

This will bring the Firewall down for updating the Configuration and will then, reboot the Firewall.

Other buttons and information

Firewall Version

The current version number of Firewall firmware is displayed in this field.

Firewall Status

The current status of the Firewall is displayed in this box, if Configuration program was able to communicate with the target. When Configuration program comes up and no “Dial String” is specified in the “Port Setup” section, it will automatically check for the presence of a target.

Retry

This button is shown only if the Configuration  program has come up but was not able to locate the target (Firewall). In this situation, most buttons are disabled. You could then select this button to try and locate the target again. If the target is found, then this button is hidden and you can use the Configuration  program normally.

Help

Selecting this will bring up Firewall Configuration program help.

StatisticsHIDD_DIALOG_PORT_STATISTICS

This button leads to the Real-time statistics window for the Firewall WAN Ports. Details like present state of modem and data transfer are displayed.

This button is enabled only if the configuration program is connected to the Firewall through the command port (serial communication line).

Exit

Selecting this button will end your session with Firewall Configuration .

# K $ IP (Internet Protocol) Configuration

This dialog allows Configuration  of IP parameters that is generally applicable to IP Proxying on all ports. The following parameters can be set.

### IP Port Configuration

Secured LAN Port Parameters

IP Address= [Dotted decimal IP address]

In case it is a LAN port you must enter a valid IP address in this field. It must be a unique host IP address which falls in the LAN IP network.

In cases of WAN ports whose IP Addresses are dynamically assigned by the ISP this field is not required and hence disabled.

Net Mask= [Dotted decimal IP address mask]

The IP subnetwork maskIP_Mask of the selected port is to be entered in this field.

### Gateway Address = [Dotted decimal IP address]

In case of Secured LAN port this parameter defines the Gateway address (default router)for the Firewall. The dotted decimal IP addressIPAddress  defines the full 32 bit IP router address of the next hop, in case the destination IP Address is not the directly connected LAN network.

Internet LAN Port Parameters

DHCP Client

In case of Internet LAN, IP Address can be obtained from DHCP Server. If this is not the case you need to uncheck this control and assign a valid Internet Address in the IP Address Field and a valid Subnet Mask in the Net Mask field.

DHCP Relay Agent and DHCP Server Address

Hosts on the Internet LAN can obtain IP Address from DHCP Server and if the DHCP Server is reachable through the WAN then you need to check this control and assign DHCP Server IP Address in DHCP Server Address Field.

ISP Assigns Dynamic address

On a WAN port normally the ISP assigns a dynamic address when the port comes up. If this is not the case you need to uncheck this control and assign a valid Internet address in the IP Address Field.

For the LAN port you must assign a valid IP address to suit your network setup and this field is disabled

Internet Gateway Parameters

Firewall can sent the internet traffic from the secure LAN to internet through    the internet LAN (typically in situations were cable modems are used) or WAN (when normal dialup ISP accounts are used). Selection of this parameter thus depends on the internet connectivity.

If LAN is the internet port, the gateway address (like the cable modems LAN port address) has to be entered in the address field. Alternatively, if DHCP is used for getting the internet LAN IP address, DHCP itself will fill the gateway field also.

In case if no DHCP client is enabled and Internet LAN is the port connecting to internet, you must provide the internet gateway address manually.

### IP Static Routes HIDD_IP_HOST_ROUTES

Selecting this button brings up a dialog that lets you view, add, edit or delete static routes to a target IP address (which can be the address of an IP host, IP network or IP subnetwork).

# K $ Static Routes

The administrator can define Static Routes to hosts, networks and subnetworks in an IP internet. Using these entries, the IP addressIPAddress of an immediate gateway to reach the specified host, network or subnetwork can be specified. There can be many static route specifications at the same time. Each entry is specified as a quadruple.

Destination IP address - Immediate Gateway address - Address Mask - Metric

The Destination IP address and gateway IP addressesIPAddress  are specified in dotted decimal form.  The gateway IP address is the IP address of the router that is the next hop towards the Destination IP address.  The metric is the hop count to the Destination IP address. The address mask is the subnet mask of the Destination IP address.

The static routes currently configured are displayed in the list. Multiple entries can be selected for deletion. Individual entries can be selected for editing. Selecting the Add button brings up a dialog that lets you add items to the list.

# K $ Add/Edit Static Route

This dialog lets you Add/Edit a static route entry.

IP Address

Specify the IP addressIPAddress  of the Destination (i.e. host, network or subnetwork IP address)  to which you want to define a static route, in this field.

Gateway Address

Specify the IP addressIPAddress  of the immediate gateway, to which Firewall can route all the packets to the Destination IP Address specified in other fields, in this field.

Address Mask

Specify the subnet mask of the Destination IP address here.

Metric

This is the hopHops count to the Destination IP address (this can be 0 or 1).

# K $ PPP Advanced Configuration

This setup lets you configure various general parameters for a PPPPPP connection.

### Message Printing Control

Check items in this group to generate messages on various conditions. Normally all items should be disabled to avoid degradation of server performance.

### PPP Echo Requests Maximum Number = [decimal value]

Defines the maximum number of unacknowledged LCP Echo Requests that will trigger terminating the link to the peer.

Recommended value: 20

### PPP Echo Request Send Interval Maximum = [decimal seconds]

Defines the interval, in seconds, that LCP will insert between two consecutive LCP Echo Requests.

Recommended value: 5

Important: Certain PPP clients do not support Echo request option. If you see that the link is coming up and going down then you may set this value to zero and checkout.

### PPP Termination Requests Maximum Number = [decimal value]

Defines the maximum number of LCP Termination Requests that are to be issued after completion of a backoff interval.  Once the maximum is reached with no response from the peer, LCP will commence issuing LCP Configuration Requests.

Recommended value: 10

### PPP Termination Request Send Interval Maximum = [decimal seconds]

Defines interval, in seconds, that LCP will insert between two consecutive LCP Termination Requests.

Recommended value:	5

### PPP Configuration Requests Maximum Number = [decimal value]

Defines the number of LCP Configuration Requests to be sent before backing off due to no response from the requests.

Recommended value: 20

### PPP Configuration Request Send Interval Maximum = [decimal seconds]

Defines the interval, in seconds, that LCP will insert between two consecutive LCP Configuration Requests.

Recommended value: 5

### PPP Configuration Request Backoff Interval Maximum = [decimal seconds]

Defines the interval, in seconds, that LCP will insert once the unanswered PPPPPP Maximum Number Of Configuration Requests has been reached.  Once this interval has transpired, PPPPPP will commence termination of the link.  See PPPPPP Maximum Number Of Termination Requests.

Recommended value: 10

See Also PPP Port SetupHIDD_PPP_PORT_CONFIG

# K $ PPP Port Configuration

This dialog box lets you setup the PPPPPP options for each WAN port.

PPP Enable

Firewall supports Point-to-point protocol on its WAN links. If you are planning to use this, you must check this option.

On a dialing WAN port if you have enabled SLIPSLIP, this option will be grayed as only either of PPP or SLIPSLIP can be used. However on answering WAN ports, Firewall is capable of detecting the type of connection (i.e. SLIPSLIP or PPP).

### PPP Port Data Compression = [enabled | disabled]

Enable or disable compression of packets that will be sent through the PPPPPP interface associated with the specified WAN port.

If you are using a Data Communication equipment which is capable of  hardware data compression (like a modem) it is better to turn this option off.

PPP Port VJC  = [enabled | disabled]

Enable this parameter to have VJC compression of TCP/IP headers on the selected PPP link.

Authentication

Firewall supports the use of two user authentication protocols on PPPPPP connections. One is the Password Authentication Protocol (PAPPAP) and the other is the Challenge Handshake Authentication Protocol (CHAPCHAP). The PPPPPP software in Firewall internally negotiates the use of a suitable protocol with the remote router or remote access server software.

### PPP Port User Name = [ASCII text string]

Defines the user name to be used by PAPPAP or CHAPCHAP when attempting to establish a link to the peer.

The text string size is limited to 30 characters.

### PPP Port Password = [ASCII text string]

Defines the password to be used by PAPPAP or CHAPCHAP when attempting to establish a link to the peer.

The text string size is limited to 20 characters.

PPP Port Periodic Timer = [decimal seconds]

This is the interval with which the authentication process will periodically be invoked.

Recommended value: 60 seconds

PPP Port Number of Retries = [decimal value]

The number of retries during each authentication using the PAPPAP or CHAPCHAP protocols.

Recommended value: 5

SLIP Enable = [enabled | disabled]

Firewall can use SLIPSLIP(Serial Line IP) for communication on the WAN links. If you are planning to use SLIP, you must check this option.

On a dialing WAN port if you have enabled PPP, this option will be grayed as only either of PPP or SLIPSLIP can be used. However on answering WAN ports, Firewall is capable of detecting the type of connection (i.e. SLIPSLIP or PPP).

SLIP Port MTU = [decimal value]

Firewall will restrict the maximum size of data packets transmitted to this number. The Maximum value for this parameter is 1006.

Recommended : 1006

CSLIP (Van Jacobson Compression) = [enabled | disabled]

TCP header transmitted over a serial link can be compressed using VJC compression. Such SLIPSLIP packets are called CSLIPCSLIP packets. Firewall will transmit and receive CSLIPCSLIP packets if you check this option.

### Port Dial-On-Demand = [enabled | disabled]

Firewall can be setup to bring down the connection on the WAN link when there is no real data traffic on the line. Firewall will then automatically bring up the WAN link when there is a need for data to get across.

This option is not applicable for Direct connect links and Synchronous links.

Port Hang-up After  = [decimal integer in minutes]

When a PPPPPP port is setup for " Dial on DemandDialOnDemand " Firewall will bring the WAN communication device down (sets the DTR signal low) when no data is to travel across this link. Firewall waits for this amount of time before it brings down the connection.

Recommended value : 3 minutes

See Also Advanced SetupHIDD_PPP_CONFIG.

# K $ WAN Advanced Configuration

This dialog lets you configure some parameters related to the WAN ports.

### WAN Message Printing Control

Check items in this section that you want should display console messages. Normally all items should be disabled to avoid degradation of server performance.

Modem Control

You may use this command to set up the command and response strings of connectable modems. Firewall stores the information about all types of modems in a common modem configuration file, called  modems.cnf.  The configuration file contains the commands that you want sent to the modem before dialing out.

You will notice that the modem highlighted in this dialog box is the modem you specified on the current communication link.  Additionally, you will see the selected modem's communication strings.

You may program the Dial Prefix and Suffix, the Ring Message, the Hang-up String, and the Modem command responses.  Check your modem's owner manual for precise information on your modem's capabilities.

The Tilde, ~, signifies a delay of 0.5 seconds before transmission of the rest of the string.  Sum consecutive tildes to increase the delay, i.e., ~~ equals one second.  The caret, ^, signifies a control character, i.e., ^M means Control M, or carriage return.

The string <BREAK> signifies a break signal to be sent out to the current communication port.

See Also WAN Port SetupHIDD_WAN_PORT_CONFIG .

# K $ WAN Port Configuration

### Port Enable

Check this to enable participation of  the specific WAN port in routing or bridging.

Mode

Firewall WAN port can be setup to perform Asynchronous or Synchronous communication. Also in Synchronous mode it can accept clock from the DCE or it can supply clock to the DCE. Select these parameters after referring to the communication equipment setup.

WAN Port Send Idle Flags = [enabled | disabled]

Enabling this parameter results in the Firewall sending idle flags on the synchronous WAN link when it is idle.  This is the default Configuration .

Connection Method

In case of dialup links, you need to specify one end of the WAN link as the dialing end and the other end as the answering end. So you need to appropriately select the router at one end of a WAN link (usually the router where application or file servers are present) as the answering end (choose the Answering button) and the router at the other end (usually the router where client software will run -- because client’s usually initiate conversations) as the dialing end (choose the Dialing button).

Since Firewall dials into an ISP in most common situations, it will be the dialing end.

NOTE: The controls in the Mode group and the controls in the Connection Method group are related and interact in various ways. Those controls that are not applicable for the current choice of  Mode and Connection Method are grayed out (disabled).

See Also WAN SetupHIDD_WAN_CONFIG .

# ProxyServer Configuration

Administrator can configure the set of applications that can be proxied by the Firewall through this dialog. An application is described as a pair of <Protocol,Port> values. Firewall will look for these values in the packets coming from the client side to check if the packet needs to be proxied or not.

Administrator can add, delete or modify entries in the list shown by pressing the corresponding button.

Note : When an entry is added or modified in this list, the system may substitute appropriate port name in place of the Port number added. In such cases the Port number is displayed at the end of the port name in brackets.

# $ K DHCP Server Configuration

MultiTech Firewall has a built-in Dynamic Host Configuration Protocol (DHCPDHCP) server. This lets the workstation IP setup on the client side LAN to be very simple.

DHCP Server = enabled | disabled

Master control for DHCPDHCP server module.

IP Address from -- IP Address to = List of Address ranges

This list box lists the address ranges configured for management by the DHCPDHCP Server. You may add or delete address ranges using the  buttons provided in this group.

Address range include both upper and lower bounds.

Once defined, the address range boundaries are not movable.

There will always be a “Default range” in this list. When a new address range is defined, the options of this range is copied into the new range

Selection = index number

When there are many entries in the above list of managed address ranges, this field is useful in knowing the selection index. Certain error messages print the index of  these entries

Exclude From -- Exclude To

In order to manage subnets as separate address ranges, user can exclude some portion of the defined range. Exclusions are defined during the add address range operation.

Exclude range include both upper and lower bounds.

Once defined, the Exclude range boundaries are not movable.

Add

Use this button to add a new range of addresses to manage.

Remove

You can select an address range and press on this button to remove it. The default range cannot be removed.

View

The boundaries, exclusion and address mask can be viewed for the selected address range by selection this option. This option is not defined for “Default Range”.

Bind

Administrator can bind specific IP addresses to specific MAC AddressesMAC using this option. IP addresses in the selected range only will be available for binding. This option is not available for default range

Option Types and Values

Option Name List

This list displays a list of Options that the DHCPDHCP server can respond to. It also displays the Option Code before the name of each option in decimal. The selected Option’s Basic type and Value(s) will be displayed in the Value List.

Value = Basic type  and List of values

This text displays the basic type of  the option, like “Dot decimal IP address”, “Unsigned 32 bit integer” etc.

The list box displays the configured values in preferential order.

Note: The following are the DHCP options presently configurable. The number in bracket indicate the option code value.

Router Address (3)

The router option specifies an IP address for router on the client's subnet.

DNS Server (6)

The Domain Name Server (DNS)option specifies a Domain Name System (STD 13, RFC 1035 [8]) name server available to the client.

Domain Name (15)

This option specifies the domain name that client should use when resolving hostnames via the Domain Name System.

Reassembly size (22)

This option specifies the maximum size datagram that the client should be prepared to reassemble. The size is specified as a 16-bit unsigned integer.  The minimum value legal value is 576.

Default IP TTL (23)

This option specifies the default time-to-live that the client should use on outgoing datagrams.  The TTL is specified as an octet with a value between 1 and 255.

MTU (26)

This option specifies the Maximum Transfer Unit (MTU) to use on this interface.  The MTU is specified as a 16-bit unsigned integer.  The minimum legal value for the MTU is 68.

Default TCP TTL (37)

This option specifies the default TTL that the client should use when sending TCP segments.  The value is represented as an 8-bit unsigned integer.  The minimum value is 1.

Lease Time (51)

This option is used in a client request (DHCPDISCOVER or DHCPREQUEST) to allow the client to request a lease time for the IP address.  In a server reply (DHCPOFFER), a DHCP server uses this option to specify the lease time it is willing to offer. The unit is this parameter is seconds.

Note:Certain Options can have multiple values. Such options must be ordered in the decreasing order of priority while adding them

Add

Select this button to add a value for the selected option (of the selected range). This button is disabled, when no more values can be added for this option.

Note: DHCP Server configuration does not check for duplication or any other integrity of the values (except for basic type and some limits). It is upto the system administrator to makesure of the integrity of  values added.

Remove

When a value in the list is selected, you can remove it by pressing on this button. Some options may need at least one value and in such cases the last value cannot be removed.

Edit

The value selected at any time can be edited with this button.

# $ K DHCP Add/Edit Address Range

This dialog lets the adding of a new range or viewing parameters of an existing range.

IP Address From

Type in the IP address range lower value in this filed. This is to be in dot decimal notion.

Please note that the range includes this address.

IP Address To

Type in the IP address range upper value in this filed. This is to be in dot decimal notion.

Please note that the range includes this address. (This address must be in the same IP network as the above field).

IP Address Mask

Type in the address mask of the address range

Exclude Addresses From

In order to manage subnetworks separate (i.e. subnetworks have separate option parameters, like Router address), ranges can be defined with Exclusion ranges.

Type in the Exclusion range’s start address in this field. The exclusion range starts at this address  (i.e. this address is not in the defined range)

Exclude IP Addresses till

Type in the Exclusion range’s end address in this field. The exclusion range ends at this address. (i.e. this address is not in the defined range).

# $ K DHCP Address Bindings

Administrator can statically bind specific IP Addresses to Specific MAC AddressesMAC . IP addresses which are in a defined range only can be bound and such bindings will inherit the option values of the defined range.

IP Address - MAC Address - Length

This list box displays information regarding the existing bindings in this range. The length field indicates the length of MAC AddressMAC in Bytes.

The Address range and Exclusion of the current range is displayed for information.

Add

Use this button to define a binding.

Remove

Select an entry from the Bindings list and click on this button to remove this binding

# $ K DHCP Add IP Address Binding

This dialog adds an IP Address binding in the selected range.

The Selected address range and Exclusion range are displayed for information.

IP Address to Bind = Dot decimal IP address

Enter an IP address in the selected range in this field.

This IP Address must not fall in the exclusion range.

An IP address can be bound only to one MAC AddressMAC.

MAC Address length

Enter the length of the MAC AddressMAC to be bound in this field. The unit is number of bytes. For Ethernet it is 6.

MAC Address.

Enter the hexadecimal value of the MAC AddressMAC into this field. Numeric characters are case insensitive.

MAC AddressesMAC will be padded with leading zeros if necessary.

Since a byte is represented as 2 hexadecimal digits, the string length of this field can be as much as twice the value of the above field.

# K $ DHCP Add Option Value

A value can be added for the selected option of the selected address range using this dialog.

Option Name = Selected Option name

The Option name of the current selection is displayed in this field.

Option Type = Basic Option Type

The Basic type of the selected option is displayed in this field.

Option Value to Add = Value

Enter the value of  the new option into this field. The type of this value must confirm the above displayed basic type.

Add value at position

In case of  an option which supports more than one value choose the position of the new value from this list. By default all new values are appended at the end of the current list to of values.

Note: Options which support multiple values (like DNS server) must be ordered in the order of decreasing preference.

# ProxyServer Add or Edit Entry

Protocol

Firewall can proxy for either TCP Protocol packets or UDP Protocol Packets. First select either of these options.

Port

This is an editable list box. The list will contain most of the common ports known for the selected Protocol (i.e. TCP or UDP as selected in the Protocol field).

If the desired application is not available in the list, the Administrator can type in the decimal value of the Application Port Number instead, into this filed.

Description

When a protocol port is added by typing a number in to the Port field, administrator must add a description to identify the port easily. Upto a maximum of 25 characters can be used for this. This field will be disabled if the selected port is predefined.

Note 1: Before adding a <Protocol,Port> entry, the configuration program will check if it is going to be a duplicate. If the entry is already present it will display a message and will not add it into the list.

Note 2: A well known application like Telnet can be specified in the list by name like <TCP,Telnet> also this entry is same as <TCP, 23>. Attempt to add <TCP,23> will generate duplicate entry message if <TCP,Telnet> is already present in the application list.

Note 3: Since certain applications use more than one port, it is preferable to select the application Port from the list, if they are available in the list.

# K $ Applications Configuration

This dialog lets you setup support for Telnet ServerTelnet,  TFTP ServerTFTP (Trivial File Transfer Protocol) and the WEB Server built into the Firewall.

Server Password

Firewall can be accessed for configuring and monitoring through the Internet. In order to ensure security for such accesses, a password can be provided in this field.

Server IP Address

This field displays the IP address at which the clients can contact the Firewall. These are the Telnet client, TFTP client and WEB client. Since this address is the same as the LAN Port IP Address of the Firewall, it is not editable.

Enable Telnet Server

Firewall has a built-in Telnet ServerTelnet. Using any standard Telnet client, authorized users can access the server and perform the following

1. View Server configuration and statistics

Any one who has the Server password can access the server with any standard Telnet client to view the configuration and statistics.

2. Modify Server configuration

Any one who has the Server password can access the server with any standard Telnet client to modify the configuration.

3. WAN device configuration

Any communication device connected to a WAN port can be directly accessed using this option. This is useful in setting up devices like modems, ISDN Terminal Adapters etc. for routing operations. At the end of WAN device configuration the server reboots automatically.

A  WAN port will NOT be available for Device configuration if

It is  already used for routing

It is used for configuring a device connected on it

If it is not configured as asynchronous port

Enable TFTP Server

Firewall has a built-in TFTP ServerTFTP for configuration and firmware updates across the IP internetwork.

Check this option to enable the TFTP Server.

Important : For security reasons, this option should be disabled once your Firewall is setup completely since anyone on the internetwork can change the server’s configuration by performing a TFTP upload.

Enable WEB Browser Management

Server can be accessed with a standard WEB browser anywhere from the connected Internet for management and configuration. If you plan to use this feature, check this option.

Enable Dumb Terminal Management

Server Administrator can connect a dumb terminal to the Command port (direct or using modems and access the server for configuration and management. If you plan to use this feature, check this option.

# $ K Statistics ( of WAN ports)

Firewall can display the real-time WAN statistics in this dialog. A list of the available WAN ports, the Modem states and the baud rates are displayed in this screen. More details of a port can be seen by double clicking on the entry in the list, or by pressing the details button after selecting the port.

Calls

Total number of calls connected on all WAN ports since the Firewall last came up.

Uptime

Total duration of all calls on all WAN ports since Firewall last came up in Days : Hours : Minutes : Seconds format.

Packets

Received - Total number of packets received on all WAN ports since Firewall last came up

Sent - Total number of packets sent on all WAN ports since Firewall last came up

Bytes

Received - Total number of  bytes received on all WAN ports since Firewall last came up

Sent - Total number of  bytes sent on all WAN ports since Firewall last came up

# $ K Port Statistics

The details of  a selected port are displayed in this dialog.

Port Number

Selected port number of which information is displayed

Baud rate

The Asynchronous speed of the port

User Name

User name used to login to the internet

Modem Type

Type of modem being used for connection

Current Connection Information

Modem Connect Message

Modem reported this message during connection (not implemented)

Port Status

Status of the port like,

Idle

Off-line

WAN Link Up

LCP Layer Up

PPP Client Up

Sending Init Command

Waiting For OK

Dialing Number

Waiting For Connection

IP Address

The IP parameters of the current connection are displayed in this group.

Local - IP Address of the WAN port

Remote - IP address to which the WAN port connects

Mask - IP address mask for the WAN port addresses

Up time

Indicates the time since the current connection is up in Days : Hours : Minutes : Seconds format.

Packets

The packet traffic details for the current connection are displayed here

Received - Number of packets received since current connection came up

Sent - Number of packets transmitted since current connection came up

Bytes

The bytes traffic details for the current connection are displayed here

Received - Number of bytes received since current connection came up

Sent - Number of bytes transmitted since current connection came up

Total Information

Details for the selected port since the Firewall started running (after the last boot) are displayed in this group.

Up time

Indicates the aggregate connect duration on this port since the Firewall last came up in Days : Hours : Minutes : Seconds format.

Calls

Total Number of calls made on this port since the Firewall came up.

Packets

The packet traffic details since the Firewall came up are displayed here

Received - Number of packets received

Sent - Number of packets transmitted

Bytes

The bytes traffic details since the Firewall came up are displayed here

Received - Number of bytes received

Sent - Number of bytes transmitted

# K $ Diagnostics

Firewall Configuration  program lets you perform various hardware tests on the LAN and WAN links.

When this dialog is closed, the server will reboot.

Server Version and CRC

This field displays the version of  Firewall Firmware that is running. It also displays the checksum (CRC) of  stored firmware.

Boot Version

This field displays the version of  boot code that is present in the target

Ethernet Address

This is the Ethernet network interface address of Firewall's Ethernet port. This field cannot  be modified.

Ethernet Port Test

You can do a loop back test on the twisted pair (RJ45) Ethernet connector. To perform this test you need a special RJ45 loop-back cableRJ45loopback connected to your Firewall.

WAN Port Test

WAN ports can be tested in synchronous or asynchronous mode.

To test any WAN port in Synchronous mode, you must connect a synchronous Data Communication Equipment (DCE), like a DSU, to that port. Power off the DSU and set it to loop-back mode. (Consult your DSU user manual for doing this). Select the port and then press the Perform Tests button to initiate the test. Results of the test will be displayed after a while.

NOTE:  In case of  a error message during test, make sure that you have connected a DSU in loop back mode to the correct WAN port. You will also be able to see the Send Data and Receive Data indicators blink during the test. Since Firewall expects an external clocking device during synchronous operation, it is not possible to conduct this test without any synchronous DCE.

To test WAN port in Asynchronous mode either an Asynchronous RS-232 loop-back cableRS232loopback or a modem having loop-back mode is needed. Also in this mode, you need to choose the baud rate at which the test has to be performed. If you are using a modem in loop-back mode, refer to the modem user manual for detail on setting it up. Modems support various loop-back modes like local digital, local analog, remote digital and remote analog. You may choose local digital for testing the WAN port.

Once the loop-back is setup, you should choose the WAN port and baud rate and then press Perform Test.  If an error occurs, check your loop-back setup once again.

WAN Link Test

Since the Configuration  software does a loop-back test (sends a pattern and expects to receive the same pattern), it can be used to test the Firewall hardware ports and also the actual communication links used for routing.

Also, on leased lines it will be possible to test the entire link and the remote DCE by putting remote DCE in loop back mode. Consult the user manual of the DCE for doing this.

Certain modems do support remote analog and digital loop back modes. If you are using these types of modems you can conduct link tests on Dial up lines. Consult your modem manual for this.

# About Command (Help Menu)

Use this command to display the version number of your copy of Firewall Configuration .

# RJ45 Loop-back Cable

This is a special cable for looping the transmitted signals back to the receive line on the same port. The wiring scheme is as follows for an RJ45 loop-back cable connector.

Pin 1  to Pin 3

Pin 2  to Pin 6

# RS232 Loop-back Cable

This is a special cable for looping the transmitted signals back to the receive line of an RS-232 port. You need a female 25-Pin D sub-connector for this. The wiring scheme is as follows

Pin 2 to Pin 3   (Transmit to Receive)

Pin 4 to Pin 5   (RTS to CTS)

# IP Address and Mask

IP address specify a host on an IP internetwork. Each IP address is a combination of a network number and a host number (host ID). IP Address is a 32 bit value which is represented as 4 decimal integers separated by 3 dots. Each decimal integer corresponds to the respective byte position in the 32 bit address. Firewall expects all IP addresses to be provided in dotted decimal notation.

# IP Address Mask

IP address mask is used to specify the network or subnet portion of an IP address. IP address mask is a 32-bit value. It is provided in what is known as the dotted decimal notation. In this notation, each 8-bit portion is expressed as a decimal quantity separated by a dot. For example. 198.2.10.1 . Firewall expects all IP address masks to be provided in dotted decimal notation.

Firewall doesn’t accept Masks which have “holes”. i.e. when the mask is represented as a 32 bit binary number, the 1s and 0s should not be mixed up. After a group of consecutive 1s at the Most Significant Digits, all the remaining digits will be 0s

# Internet Protocol (IP) Default Configuration

This dialog lets you setup minimal set of IP parameters of your Firewall. The various IP addressesIPAddress and subnet masksIPAddress are setup through this dialog.

You can also select the LAN frame type to be used by the IP.

# WAN Ports Default Configuration

This dialog lets you setup the WAN link options on the Firewall. Only the minimal set of parameters that are required to be changed in the normal cases are shown here. If you need to change more parameters, first finish the installation and later bring up the "Firewall Configuration" program. The WAN section there will let you control more parameters.

Enable

Check this box if you are planning to use the corresponding WAN port for routing.

Modem Type

If you have connected a modem in Asynchronous mode to this WAN port, you should select the type of your modem from the list. (If you don't see your modem in the list, you may add your modem type into the list after the installation is complete)

Speed

Select the Asynchronous baud rate that you want to use to communicate with the Data Communication Equipment connect to this WAN port.

Dial Number

You should enter the phone number of the peer end to this field if you are a planning to dial out from this WAN port.

User Name

Type in the  User Name corresponding to this link (Provided by the Internet Service Provider) in this field.

Password

Type in the Pass word corresponding to this link (Provided by the Internet Service Provider) in this field.

# #K $ IP Filtering

This dialog has tabs that let you configure so that IP packets that are received by the server can be selectively filtered or forwarded based on their addresses or protocol ports to which they are destined.

There are three types of IP filtering supported in Firewall.

- IP Protocol Port based filtering

In this method, IP packets can be filtered based on their specific purposes. e.g. Telnet packets (TCP based) or TFTP (UDP based) can be filtered or forwarded.

- IP Address based filtering

In this method, filtering is based on the source and destination IP addresses in the packet.

- ICMP filtering

Separate filtering support is provided for specific kinds of received ICMPICMP packets.

IP Protocol Port based filtering

Select the tab labeled Port Filtering to perform protocol port based filtering. This tab of the dialog lets you setup filtering of IP packets based on the higher level protocol supported. The controls that appear in this tab are explained below.

Default Action

A received IP packet whose IP protocol portIPProtocolPort is not found in the filtering database will be either filtered or forwarded as set in this field.

Default Setting for this option is : forward.

Filtering Entries List

This area lists the currently established IP protocol port filters in the server database. The first column shows the type of the filter (like filter on source address being equal or forward on destination address being equal etc.). The second column list the physical port (LAN or WAN) on which the filter is applicable. The third column shows the IP protocol (TCP or UDP) for which we need the filtering. The fourth column shows the protocol port number (like for Telnet, TFTP).

Add

Choose this option to establish a new IP protocol port based filter in the IP filtering database.

Remove

The current selection in the filtering database list can be deleted with this option.

IP Address based filtering

Select the tab labeled Address Filtering to perform IP address based filtering. This tab of the dialog lets you setup so that filtering of IP packets based on source or destination IP address takes place. The controls that are present in this tab are explained below.

Default Action

When an IP packet is received, of which neither the source IP Address nor the destination IP address is found in the filtering database, it will be filtered or forwarded as configured in this field

Default Setting for this option is : forward.

Filtering Entries list

This area lists the currently established IP filters in the server database. The first column shows the type of the filter (like filter on source address being equal or forward on destination address being equal etc.). The second column list the physical port on which the filter is applicable. The third column shows the IP address that will be acted upon for filtering or forwarding.

Add

Choose this option to establish a new IP address based filter in the IP filtering database.

Remove

The current selection in the filtering database list can be deleted with this option.

ICMP Filtering

Select the tab labeled ICMPICMP Filtering to setup filters for received ICMPICMP packets. This controls in this tab let you choose the kind of ICMPICMP packet to filter for each of the IP ports (LAN and WAN). Check the ICMPICMP packet type to filter the packet received on a port. Use the tabs on the left to choose the port to perform the filtering action on. Select OK once all your choices have been made.

#IP Filtering - Protocol PortIPProtocolPort Based - Add Entry

This dialog allows adding a new entry into the server IP filtering database for protocol port based filtering.

Protocol

Select the IP Protocol TCP or UDP.

Physical Port

Select the WAN or LAN port.

Protocol Port

Enter the Protocol port number in this field. Some well-known ports can be selected from the pre-defined list.

Filter Type

Choose the type of filtering you want in this field. Four options are available

1. Filter on source address

Drop all IP packets whose source IP protocol portIPProtocolPort matches with the IP protocol port entered in the above field.

2. Forward on source address

Forward all IP packets whose source IP protocol portIPProtocolPort matches with the IP protocol portIPProtocolPort entered in the above field.

3. Filter on destination address

Drop all IP packets whose destination IP protocol portIPProtocolPort matches with the IP protocol portIPProtocolPort entered in the above field.

4. Forward on destination address

Forward all IP packets whose destination IP protocol portIPProtocolPort matches with the IP protocol portIPProtocolPort entered in the above field.

# IP Filtering - IP Address Based - Add Entry

This dialog allows adding a new entry into the Server IP Filtering database.

IP Address

Enter the IP address to be checked for filtering or forwarding

Port

Choose the physical port on which the filtering is to be performed

Filter Type

Choose the type of filtering you want in this field. Four options are available

1. Filter on source address

Drop all IP packets whose source IP address matches with the IP address entered in the above field.

2. Forward on source address

Forward all IP packets whose source IP address matches with the IP address entered in the above field.

3. Filter on destination address

Drop all IP packets whose destination IP address matches with the IP address entered in the above field.

4. Forward on destination address

Forward all IP packets whose destination IP address matches with the IP address entered in the above field.

#$ K Dial On Demand

Firewall can be instructed to hang-up a WAN connection if there is no actual data to get across. Later "On Demand” the link is brought up automatically.

# IP Protocol Port

Internet Protocol (IP) packets have certain specific values in some fields for some types of well known services like Telnet, TFTP etc. These fields are called "Ports".

$ K K # PPP

Point-to-Point Protocol. This is a protocol commonly used in serial links (asynchronous or synchronous) for the purpose of transfer of packets between two end-points. This protocol basically allows the end-points to negotiate use of link and protocol parameters in a standardized way and also allows for standardized encapsulation of the packets being transferred between the end-points. The advantage of PPP is that it allows for inter-operability between end -points (say routers) using PPP for their serial communication.

$ K K # ARP

Address Resolution Protocol. ARP is used by nodes in IP networks to map IP addresses to physical addresses.

$ K # RARP

Reverse Address Resolution Protocol. RARP is used in IP networks to map physical node addresses to IP addresses. i.e. nodes that want to learn their IP address will usually use RARP to it out by providing their physical node address.

$ K # Proxy ARP

Some nodes (like routers) in an IP network can usually behave as proxies for distant (reachable only through routers or bridges) nodes replying to ARPARP requests on behalf of the distant nodes. Firewall can behave in this manner and provide a Proxy ARP service. The idea is to reduce link traffic on slow links through which the distant node can be reached.

$ K K # ICMP

Internet Control Message Protocol. ICMP is used in IP networks as an internal protocol for nodes to exchange control and diagnostic information. Applications normally will not use ICMP for any purpose.

$ K K K K # DSU/CSU

Data Service Unit. DSU’s are used to connect data terminal equipment (DTE) such as a router to a digital transmission service

Channel Service Unit. CSU’s are digital interface devices that connect end user equipment to the local digital telephone loop.

$ K # Telnet

Telnet is an application level protocol commonly found in IP based networks that allows terminal emulation at a remote workstation. To do this, there is usually a server at a central point that allows multiple clients to connect and request to use the service. The service could be anything. Usually it is terminal emulation. Firewall has a Telnet server that allows multiple clients to view statistics information from the box. Telnet is usually implemented over TCP.

You can also reconfigure the Firewall using this service

$ K K # TFTP

Trivial File Transfer Protocol. TFTP is a file transfer protocol that uses UDP and provides a simple method for transfer of files between two nodes (one the ‘server’ and the other the ‘client’). Firewall has a built-in TFTP server that allows firmware updates and setup upload/download over the internetwork.

$ K K # CHAP

Challenge Handshake Authentication Protocol. This protocol works over links using the PPPPPP encapsulation method. This is a security feature used by devices that need to allow only authorized access to the device. In Firewall, CHAP is usually setup for use in case Remote Access clients need to access the router. CHAP is more secure than PAPPAP. PAPPAP is another authentication protocol supported by older PPP based clients.

$ K K # PAP

Password Authentication Protocol. This protocol works over links using the PPPPPP encapsulation method. This is a security feature used by devices that need to allow only authorized access to the device. In Firewall, PAP is usually setup for use in case Remote Access clients need to access the router.

# $ Modem Information

This Dialog lets you specify various Modem Commands

#K K$ SLIP

Serial Line IP protocol - for communication between point-to-point links using TCP/IP.

#KK$ CSLIP

Van Jacobson Compressed Serial Line IP protocol - for communication between point-to-point links using TCP/IP.

#$ K DHCP

Dynamic Host Configuration Protocol. This is a protocol used to configure IP parameters of a workstation from a central server. Parameters like IP address, Default gateway, TTL etc. are some of the IP parameters needed for an IP stack.

#K$ MAC Address

Media Access Control Address. This is a number assigned to the network interface so that it has a unique identity. For Ethernet networks, it is a 6 bytes long integer.

# Virtual Server Setup

This dialog box allows you to configure various parameters related to virtual servers.

Global IP Address

Enter the globally unique addresses that can be mapped to the local IP addresses.

The configuration program does not allow duplicate entries for the global addresses.

Add

Click this button to add a new IP address. You will be prompted to enter an IP address in the Address field. Once you have done this, click on Add again to add the new address to the list of Global IP Addresses.

Delete

Click this button to remove the selected IP address from the Global IP Addresses list.

Address

To add a new IP address to the Global IP Addresses list, enter the new address in this field and then click on the Add button.

IP Address Mapping Details

Add

Click the Add button to add a static or dynamic mapping to the list.

Delete

To delete an existing mapping entry, select the entry in the list box and click the Delete button.

Edit

To make changes to an existing mapping, select the entry from the list box and click the Edit button.

# Virtual Server Mappings

Mapping Type This dialog box allows you to configure the static and dynamic mapping entries.

This can be either static or dynamic.

If the mapping is static, then a given unique global IP address can be mapped to only one local IP address.

If the mapping is dynamic, then a given unique global IP address can be mapped to any number of unique <local address, protocol, port> triplets.

Global Address

From the list, select a global address to which you want to map a local address.

In case of dynamic mapping, only one global address is allowed to be mapped.

Mapping Details (Local)

Local Address

Enter a valid local IP address to be mapped with the selected global address.

Protocol

Either TCP or UDP can be selected. This option is grayed out in case of static mappings.

Port

This is an editable list box. The list will contain most of the common ports known for the selected Protocol (i.e. TCP or UDP as selected in the Protocol field). This option is grayed out if the selected mapping is static.

If the desired application is not available in the list, the Administrator can type in the decimal value of the Application Port Number instead.

Map

selected global The address is mapped with the local address only after the MAP button is pressed.

Note: Before adding a <Local address,Protocol,Port> entry for dynamic mapping, or a <Global address, Local Address> entry for static mapping, the configuration program will check if it is a duplicate entry. If the entry is already present it will display a message and will not add it into the list.

# HIDD_DIALOG_MCI

$ Contents

# HIDD_IP_CONFIG

K IP

$ IP Setup

# HIDD_IP_HOST_ROUTES

K STAIC ROUTE

$ STAIC ROUTE SETUP

# HIDD_IP_ADD_ROUTE

K

$

# HIDD_PPP_CONFIG

K

$

# HIDD_PPP_PORT_CONFIG

K PPP

$ PPP Port Setup

# HIDD_WAN_CONFIG

K WAN

$ WAN Setup

# HIDD_WAN_PORT_CONFIG

K WAN

$ WAN Port Setup

# HIDD_DIALOG_PROXY_IP

# HIDD_DIALOGDHCP

$ DHCP

K DHCP

# HIDD_DIALOG_DHCP_RANGE

$ Range

K Address Range

# HIDD_DIALOG_BIND

$ Bindings

K Address Bindings

# HIDD_DIALOG_BIND_ADD

$ Add Bindings

K Add Bindings

# HIDD_DIALOG_DHCP_ADD_OPTN

K Option value

$ Option value

# HIDD_DIALOG_PROXY_ADD_EDIT

# HIDD_DIALOG1

K Applications

$ Applications Setup

# HIDD_DIALOG_PORT_STATISTICS

$ statistics

K Real-time statistics

# HIDD_DIALOG_PORT_INFO

$ Port Statistics

K Port Statistics

# HIDD_DIAGNOSTIC

K Diagnostics

$ Diagnostics

# HID_APP_ABOUT

# RJ45loopBack

# RS232loopback

# IPAddress

# IP_Mask

# HIDD_IPDEFS

# HIDD_WANDEFS

# HIDD_DIALOG_ICMP_FILTERING

# HIDD_DIALOG_IP_FILTERING_TYPE

K Filtering - IP Port

$ Filtering IP - Port based

# HIDD_DIALOG_NEW_IP_PORT_FILTER

# HIDD_DIALOG_NEW_IP_ADDRESS_FILTER

# DialOnDemand

$ Dial On Demand

K Dial On Demand

# IPProtocolPort

$ PPP

K Point-to-Point Protocol

K PPP

# PPP

$ ARP

K Address Resolution Protocol

K ARP

# ARP

$ RARP

K RARP

# RARP

$ Proxy ARP

K Proxy ARP

# ProxyARP

$ ICMP

K Internet Control Message Protocol

K ICMP

# ICMP

$ DSU

K Data Service Unit

K Channel Service Unit

K CSU

K DSU

# DSUCSU

$ Telnet

K Telnet

# Telnet

$ TFTP

K Trivial File Transfer Protocol

K TFTP

# TFTP

$ CHAP

K Challenge Handshake Authentication Protocol

K CHAP

# CHAP

$ PAP

K Password Authentication Protocol

K CHAP

# PAP

# HIDD_DIALOG_MODEM_TYPES

$ Contents

# SLIP

K Serial Line IP

K SLIP

$ SLIP

# CSLIP

K CSLIP

K Compressed Serial Line IP

$ CSLIP

# DHCP

$ DHCP Definition

K DHCP

# MAC

K MAC Address

$ MAC Address definition

# HIDD_NAT_SETUP

# HIDD_NAT_MAPPING