-----------------------------------------------------------------------------

PPP Dial Backup

-----------------------------------------------------------------------------

Author			: Chidanand

Date written		: 3rd Feb 1997

-----------------------------------------------------------------------------

Modified By		:

Modification Date	:

-----------------------------------------------------------------------------

Overview

--------

1. Introduction

2. Terminologies

3. New configuration strings

4. Changes in ROUCON

5. Source Modification Notes

6. Test Plan

7. Known Bugs

Note : You need to read this document immaterial of whether you want to

use Dial Backup or not.

1. Introduction

When two routers are connected thru' a PPP link, if the connection

were to go down because of any reason, Dial Backup feature enables DIALING

router to establish an alternate connection(a backup connection) via a

Dial-up line. After the primary connection gets right, backup dial connection

would be withdrawn. This whole process of switching between connections goes

without the knowledge of higher layers.

2. Terminologies

Few terms introduced while incorporating this feature are as follows :

Station, Link, Primary Link, Backup Link, Station Priority, etc.

Station : refers to a Connection Destination or a physical station such as

another router to which this router is connected.

For example, on MTASR3, you can define and have a maximum

of 3 stations. If you want to use Dial Backup feature, then you can

only have a maximum of 2 stations.

Link	: refers to a connection between this router and the Station.

Note that to get connected to a station, you have to define and enable

atleast one link, even if you do not want to use Dial Backup.

However to use Dial backup feature, you should have defined atleast

two links. (Ofcourse, the current version supports only 2 links,

i.e., only one backup link).

ROUCON allows you to define a maximum of 2 links for every station.

With every link you have to associate one and only one WAN port.

Primary Link : is the link which you want the router to use normally.

You have to associate an unique WAN port with a primary link, and

the same should not be associated with any other link of either

this station or links of other stations.

Backup Link : is the link which you want the router to use only when

Primary Link fails.

You have to associate backup link with one WAN port; but two

backup links of two different stations can be associated with

the same WAN port.

Station Priority :

In case you want to associate the same WAN port with backup links

of two different stations, then if by chance, primary links of both

stations go down, then using Station Priority router decides which

station to be backed up.

You can give a priority for a station. It can be a number in the

range 1 - 10, with 10 corresponding to highest priority.

3. New configuration strings

Here are the new configuration strings introduced to support Dial

Backup.

**(1) PPP Port Station=[disabled/enabled]

Specifies whether specified station is defined or not.

Station no. are 1, 2 and 3 as seen by the user, and are 0, 1 and 2

as seen by the programmer(We shall maintain only the later notation throughout).

But there is a small crux here. In order to avoid major modifications,

Station No. and Link No. had to be together accomodated

in a single number. Hence to have brevity as well as simplicity, right now,

the combined number represents (Station No. * 10 + Link No). This notation is

maintained throughout all the new configuration strings.

Ex:- To undef Station 0, and to define the stations 1 and 2, we have strings.

PPP Port0 Station=disabled

PPP Port10 Station=enabled

PPP Port20 Station=enabled

**(2) PPP Port10 Station Priority=5

defines priority of station 1. Valid values are from 1 to 10, with

10 corresponding to highest level of priority.

**(3) PPP Port10 Link Id List=0

This is an internal string to be filled up by ROUCON. ROUCON allows

the user to define any of either Link 0 or Link 1 as primary link and

the other as backup link.

But ROUCON sets : PPP Port20 Link Id List=1, if station 2's

primary link is link 1.

Similarly if station 2's backup link is link 0, then it puts string

PPP Port21 Link Id List=0

If backup link is undefined for station 2, then ROUCON puts string

PPP Port21 Link Id List=3

since 3 is an invalid link no.

PPP Port[StnNo][0-primary/1-backup] Link Id List=[Link No]

**(4) PPP Port10 Link=enabled

Specifies whether Link 0 of Station 1 is enabled or not

**(5) PPP Port10 Link Type=0

Type of Link 0 of Station 1.

0 - PRIMARY	1 - BACKUP

Right now this field is not used by BIN. Only ROUCON needs to keep

track of it.

**(6) PPP Port10 Link Wan Port=2

WAN port associated with Link 0 of Station 1 is WAN Port 2

**(7) PPP Port10 Link IP=disabled

PPP Port10 Link IPX=enabled

PPP Port10 Link NetBIOS=disabled

PPP Port10 Link STP=disabled

specify which protocol is permitted on Link 0 of Station 1.

**(8) PPP Port10 Link Auto Answer=disabled

PPP Port10 Link Phone Number=406

specify whether Link 0 of Stn 1 is dialing end or answering end.

If dialing, then phone number to be dialled.

In the case of DOD(symmetric), even on answering end, phone number of

other end is to be accepted.

**(9) 	PPP Port10 Link Connection Dial On Demand=disabled

specifies whether on Link 0 of Stn 1, DOD is enabled or not.

**(10) 	PPP Port10 Station Maximum Dial Attempts=3

specifies that on Station 1, after dialing 3 times for primary link

unsuccessfully, backup link can be brought up.

Other strings which are not tested

(11)	"PPP Port10 Link User Name =",

"PPP Port10 Link Remote User Name =",

"PPP Port10 Link User Password =",

"PPP Port10 Link Remote User Password =",

"PPP Port10 Link Periodic Authentication Timer =",

"PPP Port10 Link Maximum Number of Authentication Retries =",

"PPP Port10 Link Connection Idle Timer =",

4. Changes in ROUCON

One important change in the perspective. A PPP port now no more maps

to a corresponding WAN port.

IP, IPX,  (Spanning Tree)

---------

Don't visualize as WAN ports. Here afterwards they have to be seen

as Stations.

WAN Dialog Box

---------------

Remove Answering/Dialing and Phone Number from this dialog box.

Also remove the corresponding strings

PPP Dialog Box

---------------

Have 3 Tab Dialog Boxes, each for Station 1, 2 and 3

In each Station Dialog Box, have

(i)	Enable - check box

(ii)	Priority - pop-down menu

(iii) 	Maximum Number of Dial Attempts (0 - 10)

(iv) 	2 Tab Dialog Boxes, each for Link 1 and 2.

In each Link Dialog Box, have

(a)	Enable	 - check box

PPP Port10 Link=enabled

(b)	Type	 - pop-down menu  (PRIMARY, BACKUP)

PPP Port10 Link Type=0

(c) 	WAN Port - pop-down menu

PPP Port10 Link Wan Port=2

(d)	Link's PPP parameters

IP - check box

PPP Port10 Link IP=disabled

IPX - check box

PPP Port10 Link IPX=enabled

NetBIOS - check box

PPP Port10 Link NetBIOS=disabled

Spanning Tree - check box

PPP Port10 Link STP=disabled

If we want flexibility of enabling/disabling specific

protocols on specific links, we can have these; otherwise

we can remove them.

Authentication

User Name

PPP Port10 Link User Name = Chidanand

PPP Port10 Link Password = CMR

PPP Port10 Link Periodic Timer = 20

PPP Port10 Link Number of Retries = 5

PAP 	- check box	(Not incorporated in code)

PPP Port10 Link Option%LCP%Authentication (PAP)=3,Auto,Negotiation Required,Not Negotiable,H2,c023

CHAP	- check box	(Not incorporated in code)

PPP Port10 Link Option%LCP%Authentication (CHAP)=3,Auto,Negotiation Required,Not Negotiable,H2,c025

Data Compression - check box (not incorporated in code)

Same as before

Header Compression - cluster (VJC & CIPX) (not incorporated in code)

Same as before

Maximum Receive Unit  (not incorporated in code)

Same as before

Maximum Remote Receive (not incorporated in code)

Same as before

(e) WAN parameters

Dial On Demand - check box

PPP Port10 Link Connection Dial On Demand=disabled

Hang up After

PPP Port10 Link Connection Idle Timer=180

Connection Method

Answering - check box

PPP Port10 Link Auto Answer=disabled

Phone Number

PPP Port10 Link Phone Number=406

Remote Port Setup (not thought out)

Integrity constraints to be enforced

------------------------------------

1. Primary links should be allotted a unique WAN port

Two or more Backup links can share a WAN port which is not allotted to

primary link

**2. Every enabled station should have one and only one enabled primary link

3. For a backup link, we can copy the parameters of the primary link; but

allow the user to edit them

**4. WAN port associated with a BACKUP link has to be asynchronous.

**5. If primary link is dialing type, backup should also be of dialing type.

If primary link is answering type, backup should also be of answering type.

**6. If a link has DOD enabled, then even on the answering end, phone number is

required. Just a warning message needs to be issued to the user, if he

has not given. But he need not necessarily provide it.

**7. A warning message can be issued, if both primary link and backup link

are defined with same phone numbers.

5. Source Modification Notes

a) PPP Module

Major changes in code are in PPP module.

VPPPSTR.H has been modified to declare STATIONs and LINKs.

PPP Ports have been treated analogous to Stations.

Some fields of PPP_PORT_CLASS such as serial_driver, device_driver_id, etc.

(given comments in VPPPSTR.H) are maintained with 1-1 correspondence with WAN

ports. In such cases, mapping functions are used to get the correct parameters.

Station to WAN port, and vice-versa mapping functions are defined in PPPDBKUP.C.

Link switching is handled in PPPINIT.C

Only WAN ports of primary links of enabled stations would be INITed. Other

WAN ports are INITed if and when they are required for backing up.

Almost all files (about 20) in PPP are modified to reflect mapping.

b) WAN Module

Changes are for having

- facility to suspend & resume WAN ports so that initially

only PRIMARY ports can be resumed, and all others

suspended.

6. Test Plan

a) With Only one Station

Requires 2 routers and 4 modems

- Check whether primary link comes up

- Test stability of primary link

- If primary goes down, check whether back up link comes up

- Test stability of backup link

- If primary comes up again, whether backup goes down ?

- Repeat this switching back & forth

- If backup link also goes off, what happens ?

If brought back does it come back ?

- Test the above after enabling each of the features of MTASR3.

Test the above with Primary Link as a Leased Line.

b) With Two Stations

Requires 3 routers and 7 modems

- Share a backup link between two stations

- Test part (a) for both stations seperately

- If a backup link is occupied by a lesser priority station,

does the one with the higher one can take it ?

- After it is taken, and if primary link of higher priority station

comes up, does the backup port released & occupied by low

priority station ?

In the meanwhile, what happpens if primary link of low

priority station comes up ?

- If a backup link is occupied by a higher priority station, if other

station also wants it, what happens ?

- Test the above after enabling each of the features of MTASR3.

Test the above with Primary Link as a Leased Line.

7. Known Bugs/Limitations