Overview of the AG code ported to the Router Box

------------------------------------------------

24, September, 1996

-------------------

The AG code was not directly ported. Instead it was re-written. But

all the actions as performed by the DOS AG code on packet reception

and session maintainenace have been reproduced exactly. In addition,

most constants used in the DOS AG have been maintained.

Notable differences are

(1) The network interaction code is new. In the box, we use

RouterWare's SPX code. This code differs from the way DOS

based IPX stacks work in that receives can be issued on a

per session basis rather than without any session assoication like

in DOS. The XON/XOFF flow control between the AG server and the

MCSI clients that exists in the DOS environment has been

discarded because here we can have receives on a per session

basis.

In addition, the RouterWare SPX supports only a limited API,

uses different parameters in its API calls and assumes proper use

of the interface. In particular, calls are not verified to see if

they are made on valid sessions.

(2) The serial driver interface is different in this case. A new

serial driver has been written that in many ways works like the

network interface in the sense that we send and receive packets

rather than like in PC based UART's where we need to manage

bytes.

(3) Advantage has been taken of the fact that there is no

re-entrancy in any portion of the RouterWare code.

(4) The DOS AG had an elaborate scheme of queueing up packets that

were received and that needed to be sent in two separate queues

and then having separate processes manage these queues (to handle

received packets and to send) carefully.

In the box, since the Router is the only code around and most

things are done in the foreground, this scheme has been

discarded. Instead packets are handled as and when they are

received and transmitted when the need arises.

Terminology

-----------

Line 	- A WAN link.

Session - A SPX connection between the AG server and a MCSI client.

A session may or may not have a line associated with it.

Connection - A WAN link in use by a session

Data Structures

---------------

There are 3 main structures (in VAGSTR.H).

(1) The AG class structure. This has all globals related to working

of AG. It has configuration information, the other 2 structures

and other globals embedded in it. AG_CLASS.

(2) The Session Information structure. This contains information on

network sessions. SESSION_TABLE_ENTRY. Forms a linked list.

(3) The Line Information structure. This contains configured and

dynamic information related to each of the WAN ports.

LINE_INFO_TYPE. Is an array of known size.

When a connection is established, the line information structure will

have a pointer to the session that caused the connection.

Sessions can be in various states. Lines too can be in various

states. Handling of incoming packets depends on the states of the

session and the line.

Code

----

The entire code can be viewed as operating in two phases

- Initialization

- Event handling

Initialization:

---------------

The init code takes care of setting up the data structures as per

the configuration. In addition, it arranges for IPX to make

periodic SAP broadcasts of the presence of the AG server. This is

done by directly making a call to the IPX router code to add a

permanent SAP entry to its SAP table.

Event Handling:

---------------

Normal events are

- Packet reception from the network

- Packet reception on the serial side

- Timer

Packet reception from the network

---------------------------------

During the init phase, the code arranges to have a IPX listen

pending. Clients who want to use the server, try to establish a SPX

session. On doing so, the listen succeeds, a session entry is created

and receives are kept pending on the session. In addition another

listen is issued for future sessions with other clients. Once a

session is established, clients can send MCSI packets, the pending

receives will post, the packets are analysed by request type and are

handled accordingly.

Certain types of packet receptions can result in sends on the serial

side. When a receive packet has to be sent out on the serial side,

the received packet is passed onto the serial side for transmission.

The serial driver will queue this up and on completely sending the

buffer will call the server back to return the buffer. Then the

server can re-use the buffer for future network receives.

Packets received on the serial side

-----------------------------------

The serial driver will receive packets only when enabled. Sessions

being setup on the network side can enable the serial driver on

specific ports to receive and send packets. Depending on the phase of

init of the server, serially received packets may be used internally

by the server or (mostly) will be sent out on the network. For the

sake of efficiency, when the serial driver passes a buffer to the

server and the server needs to transmit the buffer on the network, it

uses the serial buffer as a network buffer (the driver reserves

enough space ahead of the buffer to form network headers) and sends

it out on the network. When the network send is complete, the buffer

is returned back to the serial driver.

Timer

-----

The timer keeps track of sessions and lines and results in internal

events. In the timer code, receives are issued on sessions that need

them, sessions that are safe to cleanup are cleaned up, lines that

are inbound and checked for the presence of CD and the inbound state

machine is activated.

As a result of the timer working, the server may generate disconnect

packets to be sent to clients.

Session Behaviour

-----------------

Normally the server will be used by MCSI clients in the DOS, Windows

and Win95 environments.

The way client applications use the server is first they run a TSR

(in DOS called MCSI.EXE) or a Windows program (COMMAP.EXE) and

perform a logon operation.

In the MCSI way of doing things, a session is established with a

server for

1. Logging on

2. Verifying logon password

3. Getting status of ports

4. Acquiring a WAN port for dialout

5. Transmitting a packet

6. Awaiting a dialin request

7. Terminating a established connection

In the first three cases and the last two cases, the session does the

job and is immediately terminated. In the case of (4) above, the

session does not terminate. Instead, the session waits to receive

packets of type (5) where the received packets are put out onto the

serial ports.

So you run the MCSI client program and perform the logon operation.

Then the application that supports a MCSI interface can be run. Within

this application, you need to choose MCSI for the serial interface.

From then on, the client program will ensure that packets are sent

and received to and from the server.

Inbound Call Handling

---------------------

The server supports calls from clients that register awaiting a

inbound call. Inbound calls can occur on any of the available WAN

ports. Since the box also has a router and by default all calls are

directed to the router, some code modification had to be done to the

original router code to demultiplex incoming packets and assign ports

to either the dialout module or the router module depending on what

packet was received. See the file PORTS.DOC for details on this

mechanism.

So if there are clients that have registered for a inbound call and a

inbound call occurs, the server will inform the client and will

complete the connection. From then on the client can use the WAN

port and the behaviour is the same as that of a dialout call. The

exception is that for dial-in calls, if the CD drops, then the port

is made available again (though the session with the client is not

terminated).

Other Points

------------

1. There are 16 receives per session (configurable)

2. There is a system of callbacks where buffers are passed between

the network and serial driver during file transfers. Callbacks

ensure that the buffers are returned to the owner.

3. Default configuration of 8 sessions.

4. The MCSI emulator is on intel processor based PC's. So when

MCSI packets are sent across the network, the header portion is

in intel (little-endian) format. In our code, before looking into

the header, we do a motorola (big-endian) conversion. Similary

before sending a MCSI packet, we do a little-endian conversion.

Changes :

---------

14/11/1996 :

Sachin made changes in AGRESP.C (process_status_request()) .