DIAL-on-DEMAND (DOD)

--------------------

A port configured with DOD enabled, will bring down the

link if no PROTOCOL packets are transmitted on that port for some given time.

The connect_state field indicates whether the port is being brought down

due to no traffic or due to link failure.

This is, to avoid the port being brought up again as it is done in case

of link failure.

Following are the files where changes have been made to take care of DOD.

1. pppdod.c

-----------

a) ppp_link_idle : Changes the connect_state to indicate that the link is

being brought down and tells the WAN driver to do so.

b) ppp_link_active : Changes the connect_state to indicate that the link has

to be brought up again since some higher layer protocol packet has to

be transmitted. Indicates the WAN driver to do so.

c) reset_ppp_idle_timer : Every time a higher layer protocol packet is

transmitted on the link, the DOD idle timer is reset.

2. pppncptx.c

-------------

a) signal_down_event_to_ncp - If a link is brought down due to no traffic,

the higher layer protocol stacks must not be informed about the link being

down. DOD is transparent to the higher layers.

3. ppptimer.c

-------------

a) ppp_timer_routine - Now that PPP controls the WAN links, the WAN links

are brought up when PPP calls the appropriate functions to INIT_SERIAL_PORT

and OPEN_SERIAL_PORT. INIT_SERIAL_PORT needs to be done only ONCE for

a port and this is done after the serial port registers with PPP.

The idle timer for the DOD port is decremented and the link is brought

down if the count is 0.

4. pppinit.c

------------

a) ppp_device_driver_control - PPP connect_state is changed according to

the control functions called for the PPP driver.

5. ppptx.c

----------

a) send_ppp_packet - Since, send_ppp_packet is called from the higher layer

protocols, if the link state is DOWN_BY_DEMAND, the link is brought up again

(though the packet to be transmitted is lost). Otherwise, the

packet is transmitted on the link and the idle timer reset.

b) send_native_ppp_packet - The idle timer is reset for all LCP or NCP

packets transmitted on the port except the periodic ECHO/LQR/Authentication

packets.

------------------------------------------------------------------------

DOD with MLPPP - Naveen

1. Bringing Down the Bundle By Demand

- To bring down the bundle , take each link in the bundle

bring down them by calling CLOSE_SERIAL_PORT (for each of the links in

the bundle). And now it is required to note the bundle as 'Bundle

DOWN By DEMAND'. And it is also required to save the port numbers of

the links which are included in the bundle before bringing down the

whole bundle by demand.

2. Bringing Up the Bundle By Deamnd

- When some module try to transmit something on the bundle which is

down by demand, then it is reqd to bring the whole bundle up.

- The function ('ppp_link_active()') to bring up the bundle by DEMAND

is called from  send_mlppp_packet in the file mppptx.c

- When there is a DEMAND to bring up a bundle it is reqd to call

OPEN_SERIAL_PORT for each of the port numbers which were saved

while bringing down the whole bundle(These ports were the links

of this bundle before bringing down this bundle).

3. Mapping real port to virtual port

- While Mapping a real port to virtual port (i.e. while mapping a port

number to Bundle number) initially it was reqd to just check whether

that bundle is already mapped or not. If it is not already mapped then

it can be used, otherwise it is reqd to go to next Bundle number.

But now(for DOD support), the above check is reqd, additionaly it is reqd

to check whether the unused bundle was brought down by demand (i.e.

once the bundle brought down by demand it will be free but it

should not be given for others to use because after sometime that bundle

might come up by DEMAND).

------------------------------------------------------------------------