DTR_DIALING

-----------

DTR_dialing is enabled on a port if :

a) Port is synchronous.

b) Port is asynchronous, modem connected, dialing and has no dial string

Following files are changed to support DTR dialing :

1. waninit.c

------------

a) wan_control - If DTR_dialing is disabled on a port the DTR is brought up

in case of direct_connect. Other wise, the DTR is brought up in the modem

state machine.

If DTR dialing is enabled the port is put into a new modem state

MDM_STATE_DTR_DIAL_DTR_DN.

2. modem.c

----------

4 new modem states have been added to take care of DTR dialing.

MdmStateDTRDialLinkDn

Sets DTR down and informs PPP that the port is down.

MdmStateDTRDialDTRDn

Sets DTR down for some time.

MdmStateDTRDialDTRUp

Brings up DTR

MdmStateDTRDialChkOnline

Checks for CD. If CD is not up even after 1.5 min,

the DTR is toggled so as to redial.

MdmStateOnLine - If CD is lost, depending on port being sync or DTR_dialing

enabled, the modem state is changed to MDM_STATE_DTR_DIAL_DTR_DN.