This doc is regarding the ASYNC driver portion.

The async driver used for waiting for a call on the UART is not interrupt

driven.

There are 8 Buffer descriptors provided to the receive side.

Buffers are closed on MRBLR (700h)

Buffers are closed on reception of 7E

Buffers are not closed on MAXIDL (ie it is 0)

During modem init, before sending the last portion

of the command the Receive buffers currently programmed are

reclaimed and SCC is inited fresh and new buffers allocated

So we can assume that the modem responses will land in the

first buffer always as MAXIDL= 0;

For checking of PPP or AG inbound packets, ie the routine,

_check_and_verify_async_packet_received we will check all the buffers

including the current buffer (which is not closed) for any receive

characters