Some Documentation for RouterWare's IPX API Interface Support

-------------------------------------------------------------

The following functions are detected to be compatible to the IPX

interface normally found in DOS machines.

(For the set of SPX interface functions supported, look in SPX.DOC in

SPX directory)

IPX Open Socket			- ipx_open_socket(), ipxendst.c

IPX Close Socket		- ipx_close_socket(), ipxendst.c

IPX Send Packet			- ipx_end_station_send_packet(), ipxendst.c -- Extra parameter required for this

IPX Listen For Packet		- ipx_listen_for_packet(), ipxendst.c -- Extra parameter

IPX Get Internetwork Address	- ipx_get_internetwork_address(), ipxendst.c

NOTE:

1. The IPXOpenSocket call is different. You need to pass only one

parameter. On entry, this should be 0 or a well-known socket. On

return it will have a socket number. See implementation in IPXENDST.C

before using it.

2. Note that ECB structure has similar fields, but sizes of internal

fields are not the same as in DOS.

3. Note that the ECB structure defined here is different from the

ECB structure as defined for SPX.

4. When you need a buffer to send, you need to use a buffer that has

space for MAC, IPX and data portions. ECB fragment should point to

the MAC header.

When post routine is called too, the packet pointer points to the

MAC header.

5. To avoid copying and considering the mechanism of buffer usage (as

mentioned above), only single fragments are supported.

6. To receive, you need to issue an ECB. For this receive ECB also,

use a large enough buffer (MRU size). Into this will be copied the

MAC_HEADER, IPX_HEADER and data as received, but the

size will refer to only the size of data as received. Make sure

you offset properly to the data portion before using the buffer.