WAN Port Ownership and Handling of Inbound Calls

------------------------------------------------

23, August, 1996

----------------

Sanjay S.

---------

Since there are multiple modules that can gain ownership of a WAN

port, a scheme has been devised so that individual modules can find

out if a WAN port is available for use or is owned by another module.

The support code for this (WAN Manager code) is in the WANDRV module

in the file WANCONF.C. Direct calls can be made to the support functions

there after including the header file WANMGR.H. WANMGR.H is in the INCLUDE

directory.

The modules that currently can use WAN ports are

- PPP (for routing or supporting remote access clients)

- Telnet for dialout

- AG for dialin or dialout

Before using any WAN port, each module should check with the WAN

Manager code to see if the WAN port is in use by another module. Use

the call get_wan_port_owner() for this. If port owner is

OWNED_BY_NONE, then the port is free. Else it is in use by another

module. If a port is free, then the module can use it after

registering with tne WAN Manager code through the call

set_wan_port_owner(). In addition, the module can check for other

features like async, port enabled, etc. through support calls in the

WANDRV section. Currently the owners PPP, Telnet and AG are

supported. Other modules can be added easily by modifying the

WANMGR.H list of owner enumerations. The module should make sure

that no race condition occurs by acquiring the port as soon as it

is finds a port is free and decides to use it.

Any new module created should follow this method.

Dial-on-Demand

--------------

At the answering end, it is not possible to say if a port is free or

in use if DoD is configured. PPP will claim permanent ownership of

ports configured as DoD.

Callback

--------

Callback occurs in two cases.

One is if the port is owned by the PPP module. In this case PPP

will not release ownership of port until the callback is done.

Callback can also occur in the AG module on inbound calls. The AG

module will not release ownership of the port during this time.

Direct Connect Links

--------------------

In direct connect links, the WANDRV will usually straightaway allot

the port to PPP and PPP will claim ownership. Now because of the

ownership mechanism and the support for inbound calls, direct connect

links will not the alloted to PPP (at the answering end only).

Incoming packets are checked and based on this modules are given the

port with a direct-connect link. Then it is the responsibility of the

module owning the direct-connect link to release ownership when it

thinks it is reasonable to do so.

Releasing ownership

-------------------

Each module should take care to release ownership of a WAN port by

making the call set_wan_port_owner() with the OWNED_BY_NONE parameter

when it no longer needs the port.

----------------------------------------------------------------------

Inbound Calls

=============

Support for inbound calls complicates the regular WAN driver.

When a calls comes in (CD goes high), the WANDRV in its original

behaviour informs PPP. PPP thens starts off its state machine and

starts using the port.

This has been modified as below. (Only for Async or even for Sync?)

When CD goes high, if the port end is the dialing end, then

the WANDRV will immediately inform PPP. PPP will take ownership

of the port. This is because at the dialing end, CD going high is

assumed to be because of dialing out to a known router at the other

end.

If the port end is the answering end (where the AG runs is usually

the answering end), PPP is NOT informed straightaway. Instead the

WANDRV waits for the first packet to come in. This is because the

call could be from another router or a remote access client or a

inbound call for a mcsi or similar client. So we need to analyse whom

to give the call to.

If the first packet does not come in within say 10 seconds, the call

is given to the AG inbound module. If the AG inbound module does not

want it, we continue waiting this way for 10 seconds and repeating

the process.

In case of direct connect links at the answering end, this timer

is not allowed. Instead for a direct connect link, either the PPP or

the AG module will be called to give ownership based on what kind of

packet comes in (the first time it comes in). Until then no one owns

a direct connect link. So dialout modules can use the direct connect

links until ownership is given to someone else.

If the first packet comes in the packet could be either a PPP packet

or a non-PPP packet. We can check this by looking for PPP delimiters

in the packet. If it is a PPP packet, which is the case if the other

end is a router or a remote access client, then the packet is DROPPED

and the port is registered as in use by PPP. Dropping the packet will

cause no harm as the other end will retry. If it is not a PPP packet,

the packet is dropped and the call is passed on to the AG inbound module.

If the module does not want the port, then the process is repeated.

Currently incoming packets are queued up and checked in the

foreground loop. This loop should not be allowed to see WAN packets

until the WANDRV has determined who should be given the call.

To be able to check if a packet is a PPP packet, we need to have the

HDLC frame-delimiter. Which means that the HDLC firmware should not

be immediately be enabled when CD goes high. This is because the

firmware will strip out the HDLC frame-delimiters and give us the

stripped packet.