Some Documentation for RouterWare's SPX API Interface Support

-------------------------------------------------------------

The following functions are detected to be compatible to the SPX

interface normally found in DOS machines. Also read the NOTES

carefully.

(For the set of IPX interface functions supported, look in IPX.DOC in

ENDSTATN directory)

SPX Initialize			- ** not available **

SPX Listen For Connection	- ??spx_listen_for_connection(), spxrx.c -- this seems to require extra param

SPX Terminate Connection	- spx_terminate_connection(), spxconn.c

SPX Send Sequenced Packet	- spx_send_sequenced_packet(), spxtx.c

SPX Listen For Sequenced Packet - ??spx_listen_for_sequenced_packets(), spxrx.c -- seems to require extra param

The SPX API

-----------

24, July, 1996

This is documentation for the SPX API provided by the RouterWare SPX

code version 1.03. This API is a lot different from the one provided in

the DOS environment. The API is mostly for a server sort of program

that does not establish connections by itself. Only 4 calls are

supported.

SPX Listen For Connection

SPX Terminate Connection

SPX Send Sequenced Packet

SPX Listen For Sequenced Packet

SPX Establish Connection -- This is not documented properly

as we did not require it currently.

NOTES:

1. In all ECB's fragment count should be 1. i.e. number of

fragments supported is always 1.

2. The ECB fragment buffer should point to the MAC header of

the packet to send. So the packet should have space for MAC

header, IPX header, SPX header and the actual data. In addition

there should be some reserved space (about 64 bytes) before the

MAC header (though pointer points to the MAC header).

Note that the fragment length should include the length of the

MAC and IPX/SPX headers.

Preferably for buffers you can use the documented functions

spx_get_a_send_packet() and spx_free_a_send_packet().

These are provided by the stack code.

3. Different calls return different error codes. You need to include

header files from this section to gain access to the error codes.

In addition ESR's are called on errors in the original call.

4. The ECB format is different from that available in DOS.

5. The in-use flag of the ECB is not managed properly. Don't depend

on it.

6. There is watchdog timer support.

7. Once a listen is posted, we need to immediately provide a ECB

known as the disconnect ECB for use by the SPX layer when it

receives a disconnect packet. Else if the SPX layer receives

a disconnect packet, then it does not have a ECB to use to

receive the disconnect packet in. So there is a disconnect

ECB provided as a parameter to the "listen" function.

1. spx_listen_for_connection()

Prototype:

enum SPX_RETURN_CODE spx_listen_for_connection(

BYTE number_of_connection_retries, enum BOOLEAN watchdog,

ULONG *ulptr_session_handle,

EVENT_CONTROL_BLOCK *sptr_listen_connect_ecb,

EVENT_CONTROL_BLOCK *sptr_disconnect_ecb)

Parameters:

number_of_connection_retries - Can be 0 in which case a

default is used. Default is set via NVRAM

configuration (20 currently).

watchdog - TRUE - enable watchog; FALSE - disable watchdog

ulptr_session_handle - Pointer to a ULONG to receive the

connection ID. Unlike in DOS, here connection ID

is returned immediately. Should be used in all

future calls (including "listen for sequenced

packets").

sptr_listen_connect_ecb - Event control block for the listen

operation. In this ECB you need to fill

socket number, (ALLOCATE_DYNAMIC_SOCKET if

you want a dynamic socket)

fragment count, (always 1)

fragment 0 length,

fragment 0 buffer pointer, (needs a minimum

IPX/SPX header space)

sptr_disconnect_ecb - ECB that will be used when a disconnect

packet is received. This ECB will never post. So take

care to free up the ECB at a suitable time.

Errors: (Immediate return codes)

On errors, the ESR is NOT called.

SPX_SUCCESSFUL		- if successfully waiting for connect

requests.

SPX_SOCKET_NOT_FOUND

SPX_OUT_OF_RESOURCES

SPX_CONNECT_TABLE_FULL

ESR:

Not called on errors in the original call.

Completion code is always SPX_CONNECTION_STARTED if called

properly.

If there are no data receive ECB's available, the SPX module

may call the post routine with SPX_OUT_OF_RESOURCES error.

(See captialised statement below).

The ECB has the 'destination_ipx_address' field filled with the

source address of the client opening the connection.

The fragment 0 length field is modified not to inculde the

MAC,IPX and SPX headers.

THE ECB AND ITS BUFFER SHOULD NOT BE FREED IN THE ESR. THIS

IS BECAUSE THE EXISTING CODE USES THIS ECB **AFTER THE ESR

IS CALLED** TO SEND THE CONNECTION ACK. So you should have

a single never-freed constant ECB probably called the

"listen_ecb" for EACH SESSION along with a constant buffer.

2. spx_terminate_connection() -- This was re-done.

Prototype:

void spx_terminate_connection(ULONG connection_handle,

EVENT_CONTROL_BLOCK *sptr_terminate_ecb)

Parameters:

connection_handle - Connection ID returned during the

spx_listen_for_connection() call.

sptr_terminate_ECB - ECB to use for the terminate call.

In this ECB, fill the following fields.

session_handle -- this is not there in DOS

type ECB's; fill this with the

connection ID returned from the

original listen call; this can be

used when the post occurs to

distinguish the connection on which

the post occured

Errors:

No immediate errors are returned.

ESR:

Is called. In the app, look for nothing and terminate the app session.

3. spx_listen_for_sequenced_packets()

Prototype:

void spx_listen_for_sequenced_packets(ULONG connection_handle,

EVENT_CONTROL_BLOCK *sptr_listen_ecb)

Parameters:

connection_handle - Connection ID acquired during the listen

call. This better be correct, no validation is done.

sptr_listen_ecb - ECB to use when packet received. In this

fill the following fields.

ESR address,

socket number,

fragment count, (always 1)

fragment 0 length, (as large as possible for

for the MRU)

fragmnet 0 buffer, (as large as possible for

for the MRU)

session_handle -- this is not there in DOS

type ECB's; fill this with the

connection ID returned from the

original listen call; this can be

used when the post occurs to

distinguish the connection on which

the post occured

Errors:

There are no immediate return codes.

ESR:

If the buffer has valid data, the ESR is called with

SPX_SUCCESSFUL in the ECB completion code. In this case,

the fragment 0 buffer is filled (MAC header, IPX header,

SPX header followed by data) and the fragment 0 buffer

length field gives length of data alone excluding the

MAC, IPX and SPX headers.

The ESR may also be called with SPX_CONNECT_TERMINATE

or SPX_CONNECT_ABORT completion code in the ECB if the

connection was terminated by any means.

NOTE: Preferably, make the receive data buffer large enough

to receive the max of the MRU's on all ports. (About 1518).

4. spx_send_sequenced_packet()

Prototype:

void spx_send_sequenced_packet(ULONG connection_handle,

EVENT_CONTROL_BLOCK *sptr_ecb)

Parameters:

connection_handle - Connection ID obtained during the

"listen for connection" call. This is not validated,

so it should be correct.

sptr_ecb - Pointer to ECB for the send request. In this ECB,

the following fields need to be filled.

ESR address,

fragment count, (always 1)

fragment 0 length, (include MAC, IPX, SPX

header)

fragment 0 buffer,

session_handle -- this is not there in DOS

type ECB's; fill this with the

connection ID returned from the

original listen call; this can be

used when the post occurs to

distinguish the connection on which

the post occured

Errors:

There are no immediate error codes.

ESR:

ESR is usually called with SPX_SUCCESSFUL completion code in

the ECB. Only when normal termination occurs, the error

SPX_CONNECT_TERMINATED is returned else on abnormal

termination, SPX_CONNECT_ABORT is returned.

The ECB fragment 0 buffer pointer points to rx-ed packet

including MAC, IPX, SPX headers. The fragment 0 length

field does not include these headers in the length.

Other useful functions

----------------------

1. spx_get_a_send_packet()

Prototype:

SPX_PACKET *spx_get_a_send_packet (ULONG size_of_packet)

Parameters:

size_of_packet - size of data to send (or receive) +

sizeof(MAC_HEADER) + sizeof(IPX_HEADER) +

sizeof(SPX_HEADER)

Notes:

You can use this same function in case you need receive

buffers also.

2. spx_free_a_send_packet()

Prototype:

void spx_free_a_send_packet (SPX_PACKET *sptr_txed_packet)

Parameters:

sptr_txed_packet - Pointer to the original packet acquired

thro' the get call.

NOTES:

1. SPXListenForConnection and SPXListenForSequencedPacket need an

additional parameter. This is because the listen process is

different from how it is in DOS. In DOS you need to issue a couple

of SPXListenForSequencedPacket calls before you can issue

SPXListenForConnection. Consequently, the

SPXListenForSequencedPacket call does not require a ConnectionID

parameter. But in this code, (the way things are coded -- ask

RouterWare why) you need not issue SPXListenForSequencedPacket

calls before the other call. Instead you issue

SPXListenForConnection first. This will return a ConnectionID as

a return parameter (pass by reference parameter). Subseqently on

listen completion, you can issue SPXListenForSequencedPacket with

this ConnectionID as an additional parameter.

2. You don't need to do an IPXOpenSocket call to get (or register)

a socket number that you can use in the SPXListenForConnection

call. Instead, simply pass a Socket number (well-known or 0) in

the socket number field of the ECB and the

spx_listen_for_connection() call will get or register the socket

number for you. Likewise, you need not do an IPXCloseSocket call.

Internally the SPX connection state machine will do the job once

you call spx_terminate_connection().

3. Note that ECB structure has similar fields, but sizes of internal

fields are not the same as in DOS.

4. Note that the ECB structure defined here is different from the

ECB structure as defined for IPX. So you cannot mix and match

IPX ECB's with SPX ECB's. Make sure that you use the right

header file depending on whether you call the IPX functions or

the SPX functions.

5. In the ECB passed, ONLY the socket number field needs to be in

hi-lo format. Fragment count is not used at all (internally only

single fragment is assumed always).

6. For a send to work properly, you need to pass a pointer to a

single buffer that includes the SPX header in the beginning. This

same buffer should have space for MAC header before the SPX

header.

7. When you need a buffer to send, call the SPX code provided function

spx_get_a_send_packet(). This function needs a size parameter.

Give this as sizeof(UNION_MAC_HEADER) + sizeof(IPX_HEADER) +

sizeof(SPX_HEADER) + data_length. (SPX_HEADER does not include IPX_HEADER).

So you will get a pointer to a buffer pointing to the MAC_HEADER.

Set the ECB's FragmentDescriptor[0].address to point to this

buffer. Fill in the IPX, SPX headers and data in the right places

by offseting into the buffer.

8. To avoid copying and considering the mechanism of buffer usage (as

mentioned above), only single fragments are supported.

9. To receive, you need to issue an ECB. For this receive ECB also,

use a large enough buffer (MRU size). Into this will be copied the

MAC_HEADER, IPX_HEADER, SPX_HEADER and data as received, but the

size will refer to only the size of data as received. Make sure

you offset properly to the data portion before using the buffer.

Sanjay's Notes

--------------

1. Modified the terminate behaviour (original code did not work

properly when the SPX API was used). In the process may have screwed

up the socket support. So beware. Also only the paths we require

were modified. Unusual paths of execution may still present a

problem.

2. Added a new function setup_spx_disconnect_ecb() for use by app's

that use the spx API, so that disconnect packets are received into

this ECB. Note that I think the socket support is defnitely

screwed because of this change, so beware.

3. When receiving packets, if there are no ECB's pending, the SPX

module will use the listen ECB to call up to the app with a

OUT_OF_RESOURCES error AND THE LISTEN WILL STILL BE LEFT PENDING.

So if a listen is posted with this error, either issue receive

calls, or ignore the error and do nothing.