TCP Configuration for Multi-Router

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

The RouterWare configuration file should have a section with the

heading and strings as below.

NOTE:

1. The RHS of all strings are the default values.

2. Each of the following strings MUST be present or the software will

not work.

3. All times (on the RHS) MUST be in milli-seconds.

4. Among the printf's, only the ALARM printf is enabled.

5. Initially configuration may be provided only for a few of the

strings as explained below in the "Allow Config" section, but

definitely put the other strings into the config file with the

default specified values.

6. All numbers are in decimals.

7. Since the MTR router box has a timer of 50ms resolution only, the

minimum time value should be atleast 50ms and other values should

preferably store in 50ms increments.

=======================================================================

[[ TCP Transport = Section Start ]]

TCP = enabled

TCP Maximum Number Of Connections = 8

TCP Maximum Segment Lifetime = 120000

TCP Maximum Retransmit Retries = 10

TCP Maximum Retransmit Timeout = 120000

TCP Minimum Retransmit Timeout = 50

TCP Connection Idle Timeout = 900000

TCP ACK Delay Period = 250

TCP Maximum Segment Size = 1500

TCP Send Window Size = 1536

TCP Receive Window Size = 1536

TCP Printf = enabled

TCP Printf TCP = disabled

TCP Alarm Printf = enabled

TCP Data Printf = disabled

TCP Diagnostic Printf = disabled

TCP States Printf = disabled

======================================================================

Details

-------

----------------------------------------------------------------------

TCP = enabled

Defaults: enabled

Range: enabled or disabled

Allow Config: yes

----------------------------------------------------------------------

TCP Maximum Number Of Connections = 8

Defaults: 8

Range: 2 - 65535

Allow Config: yes

Meaning:	Number of people who can establish a tcp session

concurrently.

----------------------------------------------------------------------

TCP Maximum Segment Lifetime = 120000

Defaults: 120 ms = 2 min

Range: 30 secs, 1 min or 2 min

Allow Config: no

Meaning:	This is the maximum amount of time a segment can

exist in the network before being discarded. Affects

how long it takes to close connections.

----------------------------------------------------------------------

TCP Maximum Retransmit Retries = 10

Defaults: 10

Range: 3 to 25

Allow Config: no

Meaning:	How many times to retry a transmit before aborting

the session.

----------------------------------------------------------------------

TCP Maximum Retransmit Timeout = 120000

Defaults: 2 min

Range: 500ms to 240000

Allow Config: no

Meaning:	Maximum delay between retransmissions of the same

packet.

----------------------------------------------------------------------

TCP Minimum Retransmit Timeout = 40

Defaults: 50 ms

Range: 50 ms to 200 ms

Allow Config: no

Meaning:	Minimum delay between retransmissions of the same

packet.

----------------------------------------------------------------------

TCP Connection Idle Timeout = 900000

Defaults: 15 minutes

Range: 10 min to 2 hours, special case value of 0xFFFFFFFF (dword max)

to indicate no timeout.

Allow Config: YES

Meaning:	If there is no activity on a connection for this much

time, the connection is terminated. Meant to detect

ends that have crashed.

----------------------------------------------------------------------

TCP ACK Delay Period = 150

Defaults: 250 ms

Range: 50 ms to 500 ms

Allow Config: no

Meaning:	How long ACKnowledgements can be delayed.

----------------------------------------------------------------------

TCP Maximum Segment Size = 1500

Defaults: 1500 bytes

Range: 536 to 1500 bytes

Allow Config: no

Meaning:	Largest single packet size. Usually equal to MTU for

the network or = 536 (set internally) if the packet

will go across a internetwork.

----------------------------------------------------------------------

TCP Send Window Size = 1536

Defaults: 1536 bytes

Range: 1024 to 4096 bytes

Allow Config: no

Meaning:	Window size for the sliding-window protocol

implementation on the send side.

----------------------------------------------------------------------

TCP Receive Window Size = 1536

Defaults: 1536 bytes

Range: 1024 to 4096 bytes

Allow Config: no

Meaning:	Window size for the sliding-window protocol

implementation on the receive side.

----------------------------------------------------------------------

TCP Printf = enabled

Defaults: enabled

Range: enabled or disabled

Allow Config: yes

----------------------------------------------------------------------

TCP Printf TCP = disabled

Defaults: disabled

Range: enabled or disabled

Allow Config: yes

Meaning:	General information regarding TCP progress.

----------------------------------------------------------------------

TCP Alarm Printf = enabled

Defaults: enabled

Range: enabled or disabled

Allow Config: yes

Meaning:	Warning messages like out of buffer temporarily,

etc,.

----------------------------------------------------------------------

TCP Data Printf = disabled

Defaults: disabled

Range: enabled or disabled

Allow Config: no

Meaning:	Dumps of critical TCP data fields

----------------------------------------------------------------------

TCP Diagnostic Printf = disabled

Defaults: disabled

Range: enabled or disabled

Allow Config: no

Meaning:	Debug statements showing function call progress

----------------------------------------------------------------------

TCP States Printf = disabled

Defaults: disabled

Range: enabled or disabled

Allow Config: no

Meaning:	State transition of TCP state machine on a per-

connection basis.

----------------------------------------------------------------------