Scope   :   Testing Lowcost proxy

By    :   Selvaraj

On    :   November 17 ‘th ,1998.

Functionality testing :

1.   LAN :

Pinging the LAN interface from client LAN up to 1500 pac. size:

The LAN interface is pinging up to 750 packet

size.So have a check on this.

2.  WAN :

Dial on Demand:

The setup which was analysed for the DOD is as follows.

--------------

| FTP Server |

---------------

|           [ISOLATED LAN]

|

--------------------------------------------------------------------

|

|

-----------

|    RAS    |

-----------

|

|

|

--------------

|   PROXY    |

--------------

|

|

----------------------------------------------------------------------

|

|           [ISOLATED LAN]

------------

|  CLIENT  |

------------

After the PPP link is established between the RAS &

Proxy,if there is no request fron the client side the PPP link would

come down after the idle time which has been set in the proxy.But for

this proxy the link come down after the preferred value.

For ex:if the idle time is 1 min,it takes 3 mins to come down.similarly

for 2 it takes 6 mins.

3. DHCP Server:

The setup is nothing but the clients should be in the same

LAN as that of the proxy.

The exclude ranges in the DHCP server does not work for any

clients.But the dns server & domain name is perfect.(i.e)whatever we are

configuring in DNS Server (IP Setup) and domain name will be displaced

in  the client configuration.

4. Virtual server:

The setup performed for the NAT is follows.

---- ----------                        ------------

| INTERNET |                      | FTP client |

---------------                        -------------

|                                         |        [MAIN LAN]

-------------------------------------------------------------------

|

---------

|   RAS   |

----------

|

|

----------

|  PROXY  |

-----------

|

|

-------------------------------------------------------------------

|

-------------           [ISOLATED LAN]

| FTP Server |

--------------

If we map the Proxy's wan address to the server,we can't able

to ping the server and mapping is impossible .So transfer is regretted.

5. Domain filter:

The PPP link between the router & proxy can't be estabished

and so filtering in domain names is impossible.There should be a check on

the link establishment between router & proxy.

6. Thruput test:

[a] Direct connect:

1.  19200 baud  : a. The file size of 1,358,627 bytes transfered with

(single link)         the thruput of 17.54 kbps.

b. The file size of 3,882,581 bytes tranfered with

the thruput of 17.52 kbps.

2.115200 baud :  a. The file size of 1,358,627 bytes transfered with

(single link)         the thruput of 101.98kbps.

b. The file size of 3,882,581 bytes transfered with

the thruput of 101.84 kbps.

3.230400 baud :    The link establishment using this baud is

impossible.So have a check on this.

[b] Modem connect :

1. 19200  baud:  a. The file size of 1,892,547 bytes tranfered with

(single Link)         the thruput of 16.95 kbps.

b. The file size of 1,358,627 bytes tranfered with

the thruput of 16.92 kbps.

2. 115200 baud:  a. The file size of 3,882,581 bytes tranfered with

(Single Link)             the thruput of 31.40 kbps.

b. The file size of 1,892,547 bytes tranfered with

the thruput of 31.47 kbps.

7.  Reboot:

After configuring any datas in proxy,if we want to download

the datas,rebooting setup will be displayed but there will be no message

in meterm.Then we want to cold boot the proxy,so have a check on this.

***********