Scope  :  Testing Serial ProxyServer

By    :  Selva

From  :  September 13’th ,1999.

Bugs Which were traced for Serial ProxyServer MTPSR 3 -100  			     Version 2.00 Software.

Software :

1.   WAN Link using Script Language :

Two of the three Ports in the Proxy will  come up using the

Script language of ATDT commands but a remaining port atleast should

have two ATDT commands for the link to come up.If else we didn’t support

it,only two links will come up and most probably the other will be down

correspondingly.We should have a note of the commands which is to be

passed.

2.   Hangup and Redial:

Hangup and Redial works in two ports ( in 2 and 3 ).But in the

first Port the link will come down after the Redial but link won’t come up

instead it will be waiting on the ONLINE and the WAN port won’t comeup

forever.

3.   Unability to do the Data Xfers:

Since the first port wasn’t comeup,any data xfer doesn’t take

thru the first port. But the xfer will be going on 2 ‘nd and 3’rd port ,which

indeed doesn’t bringup the first link also.Meaning to say,the link wasn’t up

and mainly problem with Hangup and Redial Property.

4.   MLPPP link using script:

After the link has come down,the demand given by the client

will bring up the one port as a MLPPP client and the other two won’t be up

due to the send init commands in the modems.The same thing happens when

we try to have Need to dial out initially was unchecked.So we should have a

note of it.

5.   Link Control using Script :

When we try to use the script for the link up after some no. of

connections for the 2’nd and 3’rd port,the link of the first port should be

loaded and after the second port starts to trigger up,some bad FCS packets

gets xfered in the first port and it stops all the data xfer in the corresponding

port,this happens only if we try to use the script.Passing of raw data was also

happened in this link which stops the data control.

6.   Link down in the third port:

Always the link corrsponding to this port will come down after

sometime with out any idle time set in the answering end,normally it will be

RAS Server in our cases.The printfs came in the TTY was

“ PPP  LCP : Terminate request received on port 0002

PPP NCP :  Port down 0002

Port 0003 Down”

7.   Remote try to contact the Proxy:

Any remote client can dial in to the Proxy to access the Internet

using the modem / Direct Connection.It works fine for the modem connection

but the same doesn’t work fine using direct connection.Using direct

connection, the link won’t come up and there is no printfs in the console but

say if disable the same check and try to connect,the link has come up and the

remote ( WANDERLINK) got his local address from our Server.So we should

make a minor change regarding this.

8.   Need to dial out initially:

Say  if  Need to dial out initially was unchecked,the system after gets

booted won’t bringup the link,any FTP / WEB demand bringsup all the ports

which doesn’t happen if we have some no of connections in the corresponding

port (this should be applicable only for the ICMP requests).So the

corresponding check was to be made in all cases.

9.   Call back for the Remote Clients:

Say if there was no users in the Userdatabase and say if any

remote client try to dial in to the Proxy,a callback was inuntiated by the client

with out any callback property,Same thing works fine if we try for aparticular

user who has the entry in  the Userdatabase.So we should have a check on

this.

10.  WAN Device Configuration:

Port types should be significant say if any port was choosed

as RAS or Proxy / RAS, port shouldn’t available for the corresponding port

types and it should be supported only if the port type was Proxy only.

11.  DOD for WAN Device Conf:

If  DOD was Enabled for a Particular Port, the corresponding

port shouldn’t be available for WAN Device Configuration as our

consideration and most probably this case was to be supported.

UI  Configuration and Management

ROUCON

1.  Wizard Setup:

IP address Reconfiguration Menu should be displayed in the middle

of the Window for a Nicelook,instead it was displayed at the bottom.

2.  IP Wizard Setup:

In IP Wizard setup Menu, Selected Port shows always WAN 1 for

all the WAN ports( namely 2 and 3,while doing the wizard setup).

3.  IP setup:

In the above dialog box , IP address shouldn’t accept as that of the

subnet mask.Have a check for the above case.

4.  WAN IP address:

Same IP address shouldn’t be accepted by all the three WAN

Ports ( ISP asskighned address).

5.  DHCP Client:

DHCP Client’s  Server IP address shouldn’t accept IP of the same

net as that of the Proxy’s address itself.

6.  Help for Staticroutes:

Help in this section should be implemented to Robohelp.( this to be

taken care in US)

7.  Static Route:

Once a Net was allocated for the port to have a staticroute,the same

shouldn’t be supported by some other port types( either it may be a LAN /

WAN ports.

8.  RAS:

Once a RAS port type was enabled, WAN port IP address should be

grayed out in IP section of the Configuration stuff.

9.  Dial Number:

If Dialno., Username and Password  was given the max characters in

all three ports and if we try to download to the stuff,the Proxy doesn’t get

booted and it takes sometime to be booted.So have a check on this.The same

thing doesn’t happen if we remove the dial no. in any one of the ports.

10.  Need to dial out initially:

Hotkey for Need to dial out initially should be incuded.

11.  Internet Setup:

Tabkeys should be allighned properly in the advanced section of

Internet parameters.( say for printfs and MLPPP).

12.  Proxy application List:

Say if we delete all the applications in the List , the tabkey of the

Menu gets disappeared and we can’t add any ports.So have a check on this.

13.  Port No. :

Hotkey for the Port Name / Number doesn’t work properly.

14.  Proxy Applications Configuration:

Tabkeys allignment should be done in  Add / Edit entry of

Proxy applications configuration.

15.  Adding Ports:

Hotkey can be included for Range Enabled in this menu of add /edit

entry table.

16.  WAN Setup:

Hotkey for Port type should be included for the easy validation.

17.  WAN setup:

Once RAS was enabled in any port - Script ,Restart Script and Link

Usage Control buttons should be grayed out.

18.   DHCP Server:

In adding the address ranges for the clients,the address range of the

following type shouldn’t be accepted.

Say as  1.1.1.1 to 1.1.1.1

19.   MAC Address:

Say if we try to bind some MAC address for the machines using 10

bits, some junks were displayed in the Binding table.

20.   Options:

If we try to add the Options in the DHCP Server,the tab key gets

disappeared as in the Proxy application List.

21.   Virtual Server:

Hotkey for Address,Mapping details table in virtual server setup

should be included.

22.   Map Button:

BMP should be incorporated in the MAP button of Virtual Server

Mappings.

23.   Global Address:

Same address shouldn’t be accepted for Local and Global addresses.

24.   Proxy Server Setup:

When we try to exit from the ProxyServer Setup, a Check for the

exit should be displayed for the User confirmation.

25.   Help in DHCP & Virtual Server:

Delete button in the IP address ranges shows for the add function.

Similarly add button in the Virtual server Menu goes to a new page.

26.   UserDatabase:

UserName shouldn’t accept some unassigned integers say % ,$, _.

DUMB TTY

1.   DHCP Client :

DHCP Client can’t be diabled in TTY terminal.

2.   Script:

If we make any changes in TTY and try to save the changes,

script written in roucon will get deleted (since we didn’t support

Script in TTY).

3.   Subnet Mask:

Subnet Mask doesn’t accepts any values via TTY,the value

was something like editable.

4.   Direct Connection:

Once we’ve chosen the direct connection,choice to Linkusage

control should be avoided.

5.  Password:

Password display only two characters. UserName accepts only 25

Characters and password accepts only 15.So take a note of it.

6.   Userdefined Port:

In adding the Userdefined port,it shouldn’t accept one value say

(12,12) , meaning to say the range no. shouldn’t be same.

7.   Reassembly Size:

Reassembly Size doesn’t accept any values in TTY.

8.   Lease Time:

Lease time should accept the values from 600 itself.

DHCP Server Functionality

1.   DHCP Server :

Exclusion of IP addresses doesn’t work in IP address ranges

as  a Functional cases.

2.   Bindings:

Binding of addresses doesn’t work properly and any client if

he try to renew the address,it doesn’t get renewd up.In print console,

“ DHCP request Packet was received “ was displayed.The client will

get the correct binded address only if we try to reboot our Server

once more.