Scope  :   Testing  Lowcost  proxy

By   :   Selva

On  :   December  3’rd ,1998.

1.  Dial On Demand: [two links]

If we set idletime as 1 min,it will take 1.5 min to come down

by "PORT FOR NO DEMAND" and similarly for every min. it takes 1/2 min

extra to comedown.so have a check on this.

2.  Link control for two links :

If we set 4 connections to comeup for the second link but

the link will comeup within 2 connections.

for ex:

SET                             TAKEN

|

4 to 7           |                  2

8 to 11          |                  4

12 to 15          |                  6

3.  The  Linkcontrol will be good enough for the first time,then the link will

comeup with out any demand & respectively will come down.

4.  Link control for three links:

The Linkcontrol for three links does not work.All the

links will comeup when we give the demand for the first link itself.

so there is no effect of linkcontrol.

5.  During File transfer,pptp messages like

"PPTP :RECEIVED PACKET TYPE IS     "

exists in meterm.so have a check on this.

6.  File Tranfer:

After the file transfer is complete and when the connection

gets closed,

" WAN 0 WITH NO FREE RX BUFFERS " and

" DISCARDING FRAGMENTS DUE TO ILLEGAL SEQ.NO "

is displayed.

7.  Thruput:

MODEM CONNECT :

[a]  115200 baud:

1. Three links : The Filesize of 6885174 bytes transfered with  the

(MLPPP)    thruput of 81.35 kbps.

2. Two links  :  The file size of 6924095 bytes transfered with the

(MLPPP)     thruput of 61.80 kbps.

3. One link    :  The file size of 3589724 bytes transfered with the

thruput of 31.35 kbps.

Enabling MLPPP: The Filesize of 3427543 bytes tranfered with the

thruput of 28.90 kbps.

upload:

1.Three links : The filesize of 3562284 bytes transfered with the

(MLPPP)       thruput of 17.82 kbps.

2. Two links  : The filesize of 3562284 bytes transfered with the

(MLPPP)       thruput of 15.82 kbps.

3. single link :  The filesize of 3562284 bytes transfered with the

thruput of 15.40 kbps.

Enabling MLPPP The Filesize of 3562284 bytes transfered with the

thruput of 15.40 kbps.

[b] 19200 baud :

1. Three Links  : The filesize of 1959598 bytes transfered with the

thruput of 48.13 kbps.

2. Two Links    : The filesize of 1959598 bytes transfered with the

th

3. Single Link  : The Filesize of 1959598 bytes transfered with the

thruput of 17.40 kbps.

DIRECT CONNECT:

[a] 115200 baud :

1. Three Links : The filesize of 1,892,547 bytes transfered with the

(MLPPP)    thruput of 252.60 kbps.

2. Two links   : The Filesize of 1,892,547 bytes transfered with the

(MLPPP)    thruput of 197.92 kbps.

3. Single Link : The filesize of 1,892,547 bytes transfered with the

thruput of 101.97 kbps.

[b] 19200 baud :

1. Three Links : The Filesize of 1,892,547 butes transfered with the

(MLPPP)      thruput of 49.19 kbps.

2. Two Links   : The filesize of 1,892,547 bytes transfered with the

(MLPPP)      thruput of 33.70 kbps.

3. Single Link : The filesize of 1,892,547 bytes tranfered with the

thruput of 17.40 kbps.

8.    During Uploading,the message one like

"DELETED 0001 DESCRIPTORS FROM NAT TABLE" is displayed in printfs.

9.   Link control for hosts & connections :

Link control is perfect but some printf like

" PORT 3 IS BLACK LISTED"  is displayed.

So have a check on this.

****************