Scope  :  Testing Low cost proxy

By     :  Selva

On    :  December 30’th,1999

DUMB TTY

1.    In IP Menu - IP Port Information - Ethernet , if we give an valid address

for the subnet mask,its accepting.

2.    In PPP port 1 setup,if we give an invalid choice in password,then if we

give an valid choice,the characters itself will be displayed.It's perfect

to say that its not recognising the characters as the password.(this is

possible only for the large number).

3.    Periodic timers unit should not be displayed in milliseconds.It's perfect

to say that it should be changed to seconds.

4.    Periodic Timers accepts 0 and also any type of values 1 to 4.After 65535

its restarting from 0. So have a check on this.

For ex :66000 ----->  464.

5.   WAN Ports:

when WAN ports are disabled,Internet factors like Port enable,

Dial no. ,authentication,periodic timer should not changes its values.

6.   DNS Server :

Same IP addresses are accepted by the primary,secondary

server.so have a check on this.

7.   Dial On Demand:

DOD is not displaying its states.But I think so,he's recognising

the change.so have a check.

8.   DOD is accepting the values beyond its ranges,(max/min that range).

9.   The Possibilty of changing the connection is not included.It's pefect to

say that we can't include the direct connection instead  of modem

connection.In this condition script,restart script should not changes its

states.

10.  Restricted host entry database does not add the host address after

confirming it also.

11.  Applications:

If we give an invalid choice in "OTHERS",router gets

crashed.so have a check on this.

12.  Host Restriction:

When DOD is disabled,we shouldn't support the  Host

restriction.Its perfect to say that adding the hosts should be

impossible.

13.  Applications:

Same description are accepted by the user defined ports.

14.  After adding 2/4 userdefined ports,then if we add the ports with

different description,it does not get addedup.

15.  In adding userdefined ports,"RANGE ENABLED" should be included for

changing its state.

16.  In filters,if we add any address/ports it doesn't get added.so have

a check on this.

17.  In IP Proxy server configuration,IP menu,IP Proxyserver information,

if we give an invalid choice first & then a valid choice,he's not

accepting.

18.  When PPP is disabled,we shouldn't change the values of user name,

password.

19.  DHCP Server :

In DHCP Server - bind section,if we MAC address give as 6,it

will accepts up to 8 digits.

for ex :

~~~

1.   7 ----> 9

2.   8 ----> 9

3.   9 ----> 9

4. 10 ----> 9.

20.  In DHCP Options,router address accepts 0.0.0.0 also.so have a check on

this.

21.  Reassembly choice doesn't accepts the values after we've configured.

It's perfect to say that the added entry is not displayed in the

table.

22.  Default IP TTL accepts zero also as an value.

23.  MTU accepts the value ranging from 68 - 9999.But the prescribed value

is 68 to 65535.so have a check on this.

24.  Default TCP TTL accepts zero also.

25.  If we give an invalid choice in DHCP Options table like (a),it shouldn't

accept.

26.  Virtual Server :

In virtual server,we can't see the global addresses which

are added.If so we add,there is some jung addresses in mapping.If I

want to delete,the router gets crashed.

27.  When I want to save changes,i used to reboot.At that time a message one

like

"   CALLOC FAILS

MEMORY ALLOCATION FAILED FOR PRIVILAGE USERS LIST

NVRAM : ERROR PROCESSING SECTION 0*5887 "

and it won't get rebooted.so have a check on this.

28.  Sometimes if i want to save changes in configuration,the bus error

comes.so have a check on this.

29.  In DHCP Server address list,if we give "d" for deleting the range,

he's telling that its an invalid choice.so have a check on this.

SUGGESTIONS

1.   In proxy server management menu,if we give an invalid choice,

" Enter your choice 1 to 2 " is displayed instead of " 1 to 3".

2.    In proxy server management- proxy statistics- proxy server statistics-

port information,there should be comma between Refresh & Esc.

3.    In proxy statistics-TCP/IP Statistics ,if we give an invalid choice

"Enter your choice 1 to 6 " is displayed instead of "1 to 3 ".

4.    In proxy sever configuration - internet menu (PPP),if we give an invalid

choice " 1 to 2 " is displayed instead of " 1 "

5.    In proxyserver application menu,if wegive an invalid choice " 1 to 0"

is displayed instead of " 1 to 2 ".

6.    In proxyserver application(standard) table,there should be a comma

between EDIT & NEXT.

7.    In proxyserver while adding protocols,there should be a comma between

EDIT & C:CONFIRM CHANGES.

8.    In proxyserver application (userdefined)table ,there should be a comma

between EDIT & EXIT.

9.    In proxyserver application filter table,there should be a comma between

EXIT & C:CONFIRM CHANGES.

10.  In NAT Static mapping table database,there should be a comma between

EDIT & NEXT.

------*------*--------