-----------------------------------------------------------------------

Functionality Testing By selva for small proxy on feb 8,99

-----------------------------------------------------------------------

DHCP Server:

The DHCP Server in proxy doesn't works fine.The problem is that the

clients didn't get the IP addresses from the proxy.A message one like

" DHCP Server Unavailable"

is displayed in the screen.so we should have a check on this.Only the

client is sending the broadcast to the server, the proxy is not responding

to the clients.I think so there is some problem in the default bin .

Need to dial out intially :  /* 9/2/99 */

The link comes up automatically if we make a check on this

property.If we made uncheck,then the link will comeup only when we give the

demand and there is problem only in the link down.

Commonly the link will not come down if we make uncheck

on the link control(DOD).But the problem we'd now is the link comes down after

10 minutes with out configuring anything in the DOD section.(This happens if

we make check\uncheck on the dial out property).I think so the link comes down

by taking 10 as the default time.so we should have a check on this.The

samething happens for MLPPP also.

Configuration Testing : /*12.2.99*/

1. DHCP Server:

If we bind any address for 10 bytes and then if we save changes

by download the setup.Then if we open the configuration to read,a message

one like

"Roucon performed an illegal operation".

Its nothing but the roucon does not works for moreever and we can able to

read the configuration only after the default setup.so we should have a

check on this.

2.  In filters,MAC Addresses accepts the type like 0.0.0.0