Scope : Testing Serial Proxy

By   :  Selva

On  :  August 7'th,1999.

Bugs  faced  while undergoing Functionality Testings :

1.   Any Remote client try to contacting the Proxy: ( Needs to be discussed)

The Property of the RAS feature is included with the idea of

providing the access to any remote client to contact the Proxy.Now if the

remote client try to transfer the files from any server which is in public

segment.The thruput was moderate and it is very low as 17.40 kbps even

it gets connected at the speed rate 28.8 kbps.So we should have a note in

making the thruput very appropriate and in sequence.

2.   Proxy loosing its buffers after long transfer of Packets: ( should be 	 		     implement)

Usually the Feature of  RAS was implemented with the view to

give the access to any remote client those who contacting the Proxy.As usual

when the remote client establish the link between the Proxy,the remote will

become the part of the Proxy’s net and the remote if want can access the

Internet from any remote area.Now in our case when any remote contacting 		      the Proxy for accessing the Internet and if say any server which is in same  		      Private segment of the Proxy and try to tranfer any file from any FTP server 		      leads to the buffer allocating Problems.When I start to tranfer the file from the 	       	      Server (which is in Internet) to the remote client and correspndingly some 		      other servers which is in the Private LAN segment,I got the buffer allocating 		      Problem noted from the  Dumb terminal  as “ in  need  buffer “. Usually this 		      problem will come in to existence when we put in to congestion.

3.   Problem in choosing the application Ports: *

Say when we transfer the file from the server to the client,will be

shown by some application Ports in the Dumb terminal,the application Ports

are sometimes shown by the large values say 51400.So we should have a note

of it.The same case can be occured in the demands given by any clients. 		      Usually the link will come down after DOD time set in the WAN  Parameters 		      and correspondingly the link will come up only if there is a demand from the 		      client.The demands will give some application ports which founded to be a 		      junk values.

4.   A case of Proxy / RAS :  * (bringing up the port wasn’t done for the 	  	     demands)

Consider the following setup for the configuration stuffs.

Let a Port be configured as  Proxy / RAS and the need to dial out

initially was unchecked,now when the proxy gets booted the modem

shouldn’t be dialed but the case was opposed by the Proxy that it

brings up the link with out check of  Need to dial out initially.So we

should have a check for both the cases as our need.

5.   Link control along with RAS :  * (it was fixed according to the concept

as that any one proxy port should trigger with out looking at their no. of 	      	       conns) *

If say  Port 1 is used for the RAS client to contact the proxy  and 	         	      Port 2 and Port 3 for Proxying with DOD as one min and max no. of

connections as 5 and 10 for Port 2 and 3.Now after the link has come down

after the DOD time set and any demand given by the clients won’t bring up

the link.So we should think little bit about it.

6.   NAT in Static routes : *

When static routes was included in the Proxy for the clients which

is in different net from that of the Proxy  to access any servers which is in the 		      insecured Part as good as saying Public LAN segment.In this case if any

Internet client want to access any Private servers,we should support  NAT

which found to be not working in the Serial Proxy only for the Server which

is in the different from that of the Proxy.So have a note of it.(I think so this

problem was fixed immediately).

7.   Static routes: *

Static routes which have been newly implemented  doesn’t work for

the router client to contact the Proxy and say any Proxy client if try to contact

the client which was in different net doesn’t get the reply from the same.So 		      we should  a right implementations in this record.This can be explained from

the following setup.

8.   User database: * (its ok)

Usually some remote clients want to access the Internet will dial in 		      to the Proxy and say they can proceed only if we have a check in User Entries.

when there was no user entries in the Userdata base and the client can contact 	       	      the Proxy for any users and passwords, this shouldn’t  happen for the User

Entries.The same problem was noted in Big Proxy also.

9.   Link control for the host restrictions: * (works acc to the concept)

Say if wegive some restriction for the 2 and 3 ports by some hosts 		      and now if we try to give the requests from an unknown proxy client,it brings 	        	      up the link for both the ports but the requests also going thru it.It doesn’t

take the account of  host restriction,so have a check on this.

10.  Demands given by the clients( should differ by ICMP ,WEB and            	       	      FTP requests): (fixed)

Say if link control was enabled with some no. of connections and

Need to dial out initially was unchecked,the port will come up only if

he exceeds the max no. of connections.Now consider the case - if the link

control was not given by some no. of connections,the ports should trigger the

modem according to the normal conditions( such that the First FTP/WEB

demand should trrigger the first port and correspondingly the others).

11. No. of  connections and hosts: * (works fine acc to the code)

Consider the following setup.

If the two ports (2 and 3) was configured to come up after 5  and 10   		        	      connections and possibly after 2 and 3 IP host machines.

Now the ports will come up after exceeding the max.

no. of connections and the transfer will go through the ports after it satisfies 		      both the conditions,now we should have a check for the above configuration.

12.  DOD for the Proxy/RAS: ( it can happen)

Say if we choose the above port type and try to check for DOD ,

the link doesn’t come down after the specified time and some rip packets

gets tranfered at both the ends.So we should have a check on this.

This happens only if we choose Client / LAN at the answering end.

13. RAS client: (Buffer - Need to be decided)

As the normal case when the remote client tries to tranfer the file 		      from the Internet ,the packets which are going thru leads to the “in need 		      buffer”  Problem to crash the Proxy.(The crashed file was noted as ‘malloc’ 		      in the same destination).I think so this is mainly with the malloc fails.

14. RAS with Link control: * ( Fixed as the true case)

Consider the following configuration.

Say if the three WAN ports are configured as Proxy only type, and

let the two ports(2 and 3) come up after 5 and 10 no. of connections.Now 		      after the Proxy gets booted, any FTP / Telnet requests won’t go thru the 		      ports. - This bug has to be checked again.

Now say if you change the First Port as RAS only port

with the same configuartion as the above.Now say the requests from the RAS

port won’t bringup any ports and the printfs like “No Internet Ports to Proxy 	         	      on”.So we should take care of this problem ( I’ve told jessi also the same,so 		      consult with her)

15.  Host restriction: * (it was fine)

Say when we are trying to restrict some client for a particlular port,

the requests given by him should bring up the port.But now say if any RAS 		       client tries to dial in to the proxy using the first port and say if the second 		       port was resticted using the same RAS client IP address,the requests given by 	        	       any Proxy client brings up the second port.So have a note of it.I think so the    	       same problem doesn’t exist for the normal Proxy clients.

16.  Bus Error:

Say whenever any Remote client contacts the Proxy and after the 		       remote becomes the part of the Proxy’s LAN,the IP Local broadcast Putting 		       on the ports leading to the malloc failure and the bus error was approached in 	        	       the PC 11bo8 (The file was put in the same destination as busmal ~1)

17.  Proxy Crashes:( same buffer probem only)

Say if the first two ports are configured as RAS type and now say if

we try to xfer the packets from the RAS clients to the router clients,the proxy

gets crashed due to the same “DET  TIMER : Packet Malloc Failed “.So we 		       should have a check that with out any packet was to be proxied to the 		        	       Internet the proxy crashes.The proxy crashes even if we have three RAS 		       ports.(have a note of the file xfer also)

18.   Static route with RAS:

Say if the WAN  ports 1 and 2 are of RAS type / let it be 		        disabled and a route was statically configured for the host 200.200.125.25 to  	        go through the port1, Now say the demands for the corresponding host goes  	        thru the third port which  is the only proxying port.So the same problem was 		        reflected if we try to disable both 1’st and 2‘nd port with out choosing them  	     	        as RAS ports.

19.   Linkup / Proxying of the Ports for netbios broadcasts:

+This Problem was faced when any clients(not the Proxy 		         clients)which are in the same net as that of the Proxy will bring up the

ports for the netbios broadcasts also.(the net was 192.168.4.10 and

200.2.19.2).If you want,u can consult with jessi or else we will see this

when I come back. I think so,the proxy was proxying the broadcasts sent

by the same clients.He’s even freeing up the descriptor for 137 and 138

ports(netbios).

20.   A case of Proxy / RAS:  * ( It was fixed)

Say when the first port was choosed as Proxy / RAS type

and the rest as Proxy only with the 2’nd and 3’rd port to comeup after

5 and 10 connections normally.Now the requests from the RAS client 	  		        bringsup the second and third port with looking out the no. of connections.	 	        Mainly there is a buffer problem and even for the RAS port IP broadcasts,

the malloc gets failed and the Proxy gets booted.

--------------------------------

Bugs after sept 6’th

--------------------------------

21.   Portup doesn’t executed : ( yet to be seen )

Consider the following case in which it has the configuration stuff 		        as the foll one.

Link control was Enabled,

2 ‘ nd link should comeup after 2 machines

3 ‘ rd link should come up after 3 machines.

Now in the above setup if  any remote client gets connected

in the second port ,the third port won’t come up forever (even the requests

was given by more than 3 machines).

22.   Link control :

Link control wasn’t supported any Proxy / RAS cases.We are 		        having the Problem when any remote client gets connected to the Proxy.

We should handle the code properly for the RAS Clients as same as that of 		        the Proxy clients.

23.   Check for the foll conf stuff: * (It was Fixed)

Say if the port types of the three ports were configured as Proxy / 		        RAS, Proxy , Proxy .

Now after the link has come down, any remote client which gets 		        connected to the first port, should bringup the other two ports when the

demands was given by the RAS ports.Sametime if we connect one more 		        remote client to the second port(if we permitted) and the demands from the

same bringsup the third port.So we should handle the code properly for the

first port.

24.   Freeing up the descriptor:

Proxy freeingup the descriptor for the port 137 must be taken in to 		        account and the corresponding port is for the netbios broadcasts.This 			        happens only if we try to enable Proxy any application.

25.   Bringsup the port:

Say whenever any machine( which is in different net from that of 		        the Proxy but physically he was in the same net)sents a netbios broadcasts		        and the router which is in the same net in Proxy pickup the broadcasts and

put it in the Proxy( which we‘d specified in the router) and our serial proxy

proxies that packets in which he triggers the port and freeing it the same.

26.   Same cases of Proxy and RAS:

Say if the port types are Proxy / RAS , RAS, RAS . Now

the requests from the clients which connected in the 2 and 3 ‘rd port

doesn’t trigger the first port. So have a check on the  RAS clients for

Proxying the datas.

27.   Static route with Proxy / RAS combinations: (fixed)

Say if the port types are  RAS, Proxy / RAS ,Proxy with IP

static route as 200.200.125.1   to go thru 1’st WAN port

200.200.125.25 to go thru 2‘nd WAN port.

200.202.1.1 to go thru the third port.

Now if the RAS client gets connected to the first port ,the demand 		         from any proxy client bringsup the second port even if we send the requests

to 200.200.125.1, and correspondingly if wetry to send the requests to

200.202.1.1 brings up the third port. We should clear this idea.I think so he’s

bringing up the port by Selecting statically configured port 3.

28.    xfer of datas with out checking static route: (fixed)

Say after remote clients gets connected in the first two ports,

the packets destined to 200.200.125.1 and 25 are going thru 3’rd port

only, so have a chek on static routing for destined IP address.

29.    No. of connections: (fixed)

In the above setup itself,if we try to make the proxying  port to 		         comeup after 5 no. of connections and shouldn’t bringup the third port after 		         a single requests itself from the Proxy clients.(should comeup after 5 cnns).

Bugs related to Configuration Stuff :

ROUCON:

1.   Gateway Address:

When we try to give the Gateway address as the address which is in

different net from that of  the Proxy,a menu will be displayed for the check

and the menu was found to be unfair and we should allighned properly.

2.   DNS Server :

Same address shouldn’t be accepted by both primary and secondary 	        DNS server and most probably we should have a check for both the cases.

3.   Static Routes:

In Static route table,if we try to add the path - the hotkey doesn’t 		        work for the IP address and metric.

4.   Link Usage control:

If we try to enable direct connection,the corresponding Link control

should be grayed out for each and every ports.

5.    DOD Time:

Once we use the RAS for a single port,the corresponding Link 		         control Property should be grayed out.(DOD shold be grayed out).

6.    Static Routes:

If we try to add the static route for the LAN in which the subnet

was 255.255.255.255(to access for the particular hosts),the dialog tells us

to give the Gateway address as that of the Proxy Local IP eventhough if we 		        provide the same net IP address itself.

7.    WAN  IP address:

If we try to use the RAS property in any of the port,the    			        corresponding ISP assign should be grayed out.But the Proxy/RAS choice

shouldn’t support this case.

8.   RAS only:

If we try to choose RAS only option for a particular port,the

link control of the corresponding port should be grayed out.The same

should be reproduced in TTY.

9.    Hotkey for Port type in WAN Section:

The hotkeys for the port type should be choosed say

for RAS,Proxy and Proxy/RAS.

10.   Script while RAS was Enabled:

When a Particular WAN port was enabled for the RAS 			       combination,the script section should be grayed out.

11.   LAN and Remote IP addresses:

When we try to give the same IP addresses for the WAN Remote and 		       LAN  IP and say OK,it accepts but at the same time if we try to give the same

address and trying to say OK in WAN 1 section,it wont accept.We should 	  	       have a check for both the conditions.

.

DUMB TTY:

1.    DHCP Client:

DHCP client can’t be disabled in TTY and it is always enabled,so its

not accepting the address also.

2.    Subnet Mask in Port type :

Subnet mask of type 255.255.255.240 wasn’t accepted in all cases

of IP and  all WAN  Ports.

3.    DNS Server:

Primary and Secondary DNS Server should accept 0.0.0.0 because 		        if the user wants to unuse the same,he should be capable to do it in the TTY 		        itself.

4.    TFTP:

In applications,the TFTP choice should be enabled as the default, 		        since we support both TTY and TFTP applcations.

5.    Proxy and RAS:

If we enable MLPPP and if we try to change the port as Proxy/RAS,

he should remained that MLPPP was enabled as when we try to choose the

RAS to be one of the port type.

6.    Static routes:

Same Entries shouldn’t be accepted for the LAN or any Port types.

Meaning to say duplicate entries shouldn’t be accepted.

7.    Proxy / RAS :

When Proxy / RAS was Enabled,if we try to disable DOD,it 		        shouldn’t be supported.

8.    RAS only:

If a Particular WAN port was chosed as RAS only port type,

the link control property shouldn’t be altered.

Bugs which were found in Serial Proxyserver Version 2.00

1.   Configuration  Port Setup:

In the top of the Menu PSR3-100.200 should be changed to Serial

Proxy Server 2.00.

2.   Userdata Base:

If wetry to add the user and press Enter button,the user Database 	  	       doesn’t  get downloaded and instead a menu like ” User Database Transfer” 		       will be displayed and it won’t get downloaded.

3.   Password:

If we try to add the Pasword in the TTY,only the first two 			       characters gets stored which leads to an unauthentication case for the

check for Passwords.