# Detailed QA Plan for Serial ProxyServer MTPSR3_100 version 2.00 Software

Preliminary copy: dated 1st June 1999

Author: Vidyasagaran P, MTSS India

# Product has the following hardware and software features

# Hardware

1 Ethernet LAN port and 3 WAN ports

LAN      : 10Mbps Ethernet LAN functioning as Secure LAN of the ProxyServer

WAN    : These are RJ45 Jack interface.    Can be Internet access port when connected to the ISP. These are asynchronous l\only ports. Max speed = 230400 bps.

Software

- Can act as a ProxyServer with LAN as secure LAN and WAN as the internet accessing port .

- Can proxy a list of applications including FTP, WEB etc.

- Can act as a RAS (remote access server) with WAN as the remote accessing port.

- Can function as a DHCP server to workstations on the LAN.

- Supports DHCP client functionality for RAS clients.

- Supports access to host in side from Internet using the virtual server (NAT) feature.

- Can be detected and configured over the client LAN automatically with a Windows95/98/NT machine connected to the same IP physical segment.

- Can be configured using a dumb terminal connected to the command port.

- Firmware can be upgraded over command port.

- WAN links support PPP in async mode.

- Script Language support on WAN links to make connections and reconnections.

- WAN links support Dial-on-demand.

- PAP and CHAP authentication.

- Sharing of load on the available links.

- Link control - Wan links 2 and 3 support link control by which bringing up of WAN2 and WAN3 for proxying are controlled by number of connections from clients and number of clients.

- Restricted hosts - WAN links 2 and 3 support restricted hosts by which the link is restricted to specific hosts, thus giving special priviledges to certain hosts.

- Can reflect the PPP negotiated DNS address to the DHCP server.

- Initial need to dial out - Brings up the the configured link for proxying only when there is a demand.

- MultiLinkPPP (MLPPP) - Through which a throughput which equals the sum of the 3 individual link can be obtained.

- Can verify the remote user trying to call in from WAN link.

- Support of Proxy or RAS or a combination of Proxy and RAS functionality on the WAN links.

- Dynamic switching between the configured functionalities for that particular link, as and when the respective clients come in.

- WAN device configuration.

- Diagnostics of communication interfaces.

- On line Statistics – General and per WAN link based.

- Log View - Logging all the events happening in each WAN link.

- Monitoring Program - UDP based event logger on Windows 3.1x/95/98/NT client.

- Proxy any application.

- Filtering of packets based on IP address, mac address, applications, domain names.

- Upgrading setup as well as firmware together.

- Support for static routing on the Client LAN side

- Support for length based prioritization of WAN transmitted packet.

Test plan

The test plan is organized in different sections as below.

During functionality tests memory leakage tests can be conducted by extra commands from the command port. For this boot the system and wait for some time so that all the initialization is over. Now type”mem dbg” (lower case). System will respond with the previous memory debug status. During the course of testing the testing person can type “alloc table” and log the response into a text file.

UI configuration and managment

- Every option that appears on the UI (via command port and via TFTP) needs to be tested.

- Every option that appears on the Dumb TTY needs to be tested.

Configure through Roucon and check using Dumb TTY and vice versa.

## Option Testing

Each option in the dialog, menu or the WEB page has to be checked out

- Minimum Boundary Value

- Maxmimum Boundary Value

- Less than minimum

- Greater than maximum

- Special Values

- Special meanings (like 0.0.0.0 may stand for any IP Address)

- Numeric/Alphanumeric/signed/unsigned inputs

# Configuration Parameters, Value types and details

In order to check out the UI the following definitions, clarifications and explanations are provided

IP addresses

Stands for a doted decimal IP address of the format a.b.c.d where a, b, c and d are integers from 0 to 255. Depending on situations there are values which are not leagal.

IP Network masks

This is a 32 bit unsinged integer most often noted in doted decimal form, like 255.255.255.0. Nework masks when represented in binary will have all 1’s to the left and all 0s to the right. They can’t have intermediate 0s or 1s.

# Gateway address

Defined as the address of the next hop IP address where to a packet is to be sent. In the following diagram, the gateway at the router is configured as the ProxyServer LAN interface. A gateway configured on any IP device must belong to an IP network of one of the connected segments on the device.

Router						ProxyServer		Internet

Gateway = ProxyLAN			            192.168.0.1

= 192.168.0.1

#### Metric

This is a positive integer which reflects the cost of a route. A value of 16 is maximum and indicates unreachability.

#### Asynchronous mode

This is a mode of transfering data between to equipments where there is no clocking is provided on the communicaiton line. The start and stop bits are part of the bit stream. TxData, RxData, CTS/RTS(handshake signals) DCD (indicating modem on-line) and ground are generally needed to be wired

#### Synchronous mode

This is a mode of serial communication in which the data is send along with the clock. Device on one end of the wire generally provides transmit and reveice clocks to the other end.

#### Baud rate

An integer value selected from a list. Indicates the speed at which an asyncronous communication port will transfer data. The highest value selectable from all UI must be same. Communication and data transfer has to be checked out with the higest baud for minimum of 10 minutes for error free and sustained xfer at the configured speed.

#### Internal/External Clock

A Synchronous interface, configured for internal clock will generate a clock at the specified clock speed and will trnsmit data at that speed. It also outputs the transmit and receive clocks for the other end to achieve synchronism with this end.

#### Clock speed

Synchronous interfaces configured for internal clocks will transmit and receive data at this speed

#### Send Idle Flags

A synchronous interace can either transmit nothing or character 0xFF when it is idle.

#### Direct connect/Leased line

This indicates if the interface is connected to a leased line where no dialingup a peer id required.

#### Script

A set of commands installed on an interface to be executed as soon as the interface intialization is over. The set of commands are edited, compiled and downloaded into the system. Details of script commands, grammar and examples are provided lated

#### User name / Password

A pair of ascii printable character strings used for getting authenticated by the system called. Length is limited to 40 characters for each

#### Dial on Demand

A feature by which the toll charges incurred on a dialup line can be saved. If there is no real data transfer on a connected link for a user specifiable duration of time, the connection is automatically braught down. System brings the link up back again when there is a packet to get across.

#### DHCP Server

Dynamic Host configuration Protocol Server. A protocol used for automatically configuring IP workstations

#### DHCP Server - Manage address range

A range of IP address (possibly with a block of addresses excluded)

#### DHCP Server – Address exclude range

A range of IP address excluded form a managed range of IP addresses

DHCP Server    - Option types and values

A DHCP Server can supply values of various options to a workstation which is DHCP confgiured. They include options like IP address, network mask, DNS primary and secondary server addresses, default gateway etc.

#### Proxy Any Application

ProxyServer can let various applications like FTP, WEB, telnet etc to access servers on the internet by clients on the private (intranet LAN). Normally the applications listed in the ProxyApplications list are allowed to go through the ProxyServer. By enabling this option any application which doesn’t require any special handling can be effectively proxied by the ProxyServer.

#### Virtual Server – NAT

This is a feature by which an address on the internet side can represent a host on the intranet. It is also called Network Address Translation. When a global address is mapped to a local host IP address on the intranet, all packets coming to that global address are forwarded to the intranet host after the address in the packet is modified accordingly.

For more flexibility Different UDP and TCP ports of a global address can be mapped to different internal hosts. Like WEB on to the internal WEB server, FTP on the internal FTP server etc. This is called dynamic mapping.

#### Telnet Server

By connecting a telnet terminal over the IP network to the ProxyServer, administrator can manage and view the state of the system.

WEB server

By connecting a standard WEB browser to the system over the IP network, administrator can manage and view the state of the system

#### TFTP server

An embedded TFTP server is used to configure the system over the IP network. This also takes care of updating the firmware over IP network, with new releases

#### Management Dumb terminal

By connecting a standard terminal like VT100 or ANSI administrator can manage and view the state of the ProxyServer.

#### Built-in Tests

A set of simple tests to check out the interfaces (ethernet and WAN) are provided. It requires some special cables to carryout the tests.

Make sure firmware has the current date and time stamp.

Check all the icons in the program group and make sure they work for

both command port and TFTP.

## Command Port based

# Connect a tty to command port – login and logout N number of times (check “alloc table”).

- Configure all the parameters from the menu.

- Connect Windows 95/98/3.11 PCs and configure the ProxyServer.

- Read and write configuration, User Database, script through COM port.

- Firmware can be downloaded only over COM port.

- Dialog based testing (to be done for each dialog).

- Each time makesure the configuration is saved and redisplayed properly.

- Check out 640 X 480 display (needed only for big dialogs).

- Help and dialog correlation – check for each field – Since Multitech US creates the help files this might have implications on the elapsed time.

- Check all fields for out of bound values, illegal values.

- Check out the tab stops in dialogs.

- Checkout for any “missing focus rectangle” during tab operation (Focus goes to an invisible but not disabled control).

- Checkout proper fuctioning of all accelerator keys.

- Operate with no mouse connected.

- Open the statistics window and check for the details like total system up time, total calls, total connect time, total packets and total bytes transmitted and received etc.

- For each WAN link check local and remote ip address, connect speed, connect string, Modem type, total packets and total bytes transmitted and received on that particular link.

## UDP Based Link Event Logger

Open up the event logger and check for events being logged.

Check for time stamps.

Check for accumulation of events in disk file.

## LAN based

- Ping from PCTCP/Win95 workstations for long time.

Connect workstations and ping the ProxyServer for hours.

Use PCTCP client as the pings are much faster in this case

- Configure over IP.

Should be able to read and reconfigure the ProxyServer over the Client LAN across at least one router

- Read and write configuration, User Database, Script over IP.

- Connect to Windows 3.1x/95/98/NT and autodetect over IP.

- DHCP Server functionality

### Aquire address from different workstations.

Connect different clients (95/98/NT, Mac, Linux) and get IP parameters from DHCP server of the ProxyServer.

DHCP server test:

Internet is LAN/WAN:

Internet is static IP/DHCP of ISP:

Bindings:

| Mac Address | IP Address |
| --- | --- |
| 000080001234 | 1.1.1.1 |

| Client | IP address got | Gateway Got | DNS Primary | DNSSec | Remarks |
| --- | --- | --- | --- | --- | --- |
| 95 |  |  |  |  | Internet Down |
| Mac |  |  |  |  | Internet Up |
| Linux |  |  |  |  | Internet Up |

- Release addresses.

- Re-aquire addresses from workstations.

When a workstation re-requests DHCP must give the same IP address (as far as possible) to it.

- Binding of MAC and physical addresses.

Create Physical to IP address bindings and checkout

- Gateways, DNS server information and such information given to clients.

Check for all parameters received from the DHCP server like gateways, TTL, lease time, DNS information etc.

- Getting DNS information from PPP negotiated values.

Make sure the DNS primary and secondary negotiated on the WAN link is reflected in the DHCP offer packets (when the internet link is WAN)

## Proxy function testing

- Check all common aplications like FTP, WEB, telnet etc.

Connect workstations on the Client LAN

Ping for existing internet servers using PCTCP stations

Ping for non-existing internet servers using PCTCP stations

Get “alloc table” print

# Connect CuteFTP Clients

Using Scripting do large number(say 1000) FTP transfers of small files (10 bytes)

Get “alloc table”

Do File Transfer of big files (like 30MB)

Get “alloc table”

- Check PPTP.

Connect Windows95/98 clients to PPTP servers on the internet side

Check for sustained connection for 20 minutes with idle connection

Do large file transfers (30MB plus)

Try IPX stack

Have multiple clients connecting to multiple servers

Have same client connecting and disconnecting

Get “alloc table”

- Check FTP in Passive mode and active mode for ordinary PPP and MLPPP.

Use the Netscape Browser and connect to the FTP server on the internet side.

Check small size file transfer (1K)

Check big file transfer (30M)

- Check FTP with large files (like 30 MB) and look for proper termination of transfer.

Different FTP clients like that of Netscape, Internet Explorer, Windows95/98/NT, CuteFTP, WSFTP, Linux, Macintosh NetSacpe

The termination of the transfer is to be noted.

The connection should be able to do another transfer without the need to reconnect

- Check form different workstations like win95, mac and linux.

- Long duration testing with FTP’s and WEB sessions (print “alloc table”).

Need to perform Lot of FTP access and WEB access for some time and get the “alloc table”.

- Throughput during FTP. Must be close to the WAN speed.

Connect FTP client to server. Try PUT and GET of files (say 1MB size) and note the thruputs

Thruput Test: Internet Interface = LAN/WAN If WAN = Modem connect speed = Serial Connect Speed = Did the modem drop speed during test? (If yes, repeat test)

| Client | FileSize and Type (MB/Zip) | Direction | TimeTaken Seconds | Thruput Bytes/Sec | Result OK? (Y/N) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Win95 | X MB/Text | Put |  |  |  |  |
| Mac Netscape | X MB/Zip | Get |  |  |  |  |

- Remove some ordinary applications form the proxy list and check proxy “any-application” feature.

List applications removed and checked

Removing FTP (both 20 and 21 ports) should still get FTP trhu if this option is enabled

- Access internal servers with DNS name. (translating internet addresses into internal address – feature yet to be implemented).

- Proxy user defined ports.

- Proxy a range of ports by adding a range of port numbers.

## Filtering Module testing

Creation of filters based on DNS address, IP address, MAC address, Applications etc.

Basic testing of filtering.

## User database

- Add users.

- Delete users.

- Edit users.

- Dial out, dial in permissions to users.

- Callback enabled, disabled.

## RAS Function

- Connect RAS clients thru DUN-95 and check whether its calls are answered

only if that function type is supported for that particular link.

2.      Check callback function.

3.      While disconnecting, check for RTS being retained.

WAN based Async mode

- Async modem based connection.

- Async direct connection.

- PPP call back.

- PPP authentication – PAP and CHAP.

- Dial-on-demand. Connect reconnect N times. Get “alloc tables”

- Check for reconnects during demand.

- Check for authentication failures during reconnection.

- Check for initial need to dial out control.

- Script based login.

- Restart Script on communication failure.

- Restart script on demand.

- Check if PPP negotiated DNS information is passed to DHCP server module.

- Check load sharing without link control.

- Check link control with number of connections, number of hosts and both.

- Check hosts restricted to a specific WAN link.

- Check Initial need to dial out is triggering the links iff there there is a demand.

- Connect the WAN to ISP and ping.

- Subnetting.

- Using WAN link as the internet link, try accessing our telnet and web server if no NAT mapping is present for those link interface addresses.

- Try accessing our telnet and web server using RAS clients.

- Virtual Server

- Check Dynamic mapping of global addresses.

Create some global addresses.

Should be able to map different ports of the same address to different internal hosts.

Access the different intenal hosts using different ports

Ping for internal hosts will be responed by one host.

Internet Cloud

<Global IP address,Port>

<Local host IP address>

Check for (GlobalIPAddress,Port)->LocalIPAddress Mapping

- Check static mapping of global addresses.

Check for one Global IP address to one Local host IP address mapping.

An address mapped this way can’t be mapped more than once nor is it available for dyanmic mapping

Ping to the internal host must be echoed from the internal host itself

- Access internal FTP server in Passive mode and active mode. Multiple clients at the same time. FTP transfer of very big files upto 30 MB.

- FTP thruput testing

Connect FTP client to server. Try PUT and GET of files (say 1MB size) and note the thruputs

Thruput Test: Internet Interface = LAN/WAN If WAN = Modem connect speed = Serial Connect Speed = Did the modem drop speed during test? (If yes, repeat test)

| Client | FileSize and Type (MB/Zip) | Direction | TimeTaken Seconds | Thruput Bytes/Sec | Result OK? (Y/N) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Win95 | X MB/Text | Put |  |  |  |  |
| Mac Netscape | X MB/Zip | Get |  |  |  |  |

- Access internal WEB servers – multiple clients at the same time.

f)      Mapping of 0.0.0.0 for internet interface address with DHCP client.

When the internet link IP address is assigned dynamically by the ISP (be it LAN or WAN) use 0.0.0.0 to represent this address for virtual server mappings

Checkout dynamic and static mappings with 0.0.0.0

## Attacks

- Try accessing without virtual server mappings.

- Try various port numbers.

- Try directing packets.

- Try broadcasts and see if they get in.

- Try socket based attacks.

- Find out from WEB on standard attacks and test suits.

## Test for Static Routes

Setup

RAS Client

m/c 5

RAS Client

m/c 4

RAS Client

m/c 3

Intranet

Internet

Intranet

RAS

RAS

RAS

Remote Client

m/c 6

Master

ProxyServer

Slave

ProxyServer

Proxy Client

m/c 1

Router

Router

Router Client

m/c    2

- All local Clients and local Routers (on Proxy’s LAN) have Master ProxyServer as

their Gateway.

- All connections can be direct connect or modem connect.

Tests to be performed

- All remote LAN’s should be able to access internet and servers on local LAN :

m/c 2 should be able to access m/c 4.

m/c 2 should be able to access    m/c 1.

- All local clients should be able to access internet and servers on remote LAN :

m/c 1 should be able to access m/c 4.

m/c 1 should be able to access m/c 2 (Check for ICMP Redirect).

- All remote LAN’s should be able to access each other :

m/c 1 should be able to access m/c 2 (Check for ICMP Redirect).

m/c 2 should be able to access m/c 1.

- All remote LAN’s should be able to access ALL intranets  :

m/c 2 should be able to access m/c 3.

m/c 2 should be able to access    m/c 5 (Check for ICMP Redirect).

- All local clients should be able to access ALL intranets :

m/c 1 should be able to access m/c 3.

m/c 1 should be able to access m/c 5 (Check for ICMP Redirect).

- All Remote Clients should be able to access Internet, ALL Intranets, local LAN and ALL remote LAN’S :

m/c 6 should be able to access m/c 4.

m/c 6 should be able to access m/c 3.

m/c 6 should be able to access m/c 5.

m/c 6 should be able to access m/c 1.

m/c 6 should be able to access m/c 2.

## Installation tests

## Group name should reflect the release version.

# Destination directory should reflect the release version.

- Communication setup should default to IP for ethernet autodetection and configuration.

- Checkout upgrade for setup and firmware.

- Checkout Uninstall – cleanup the group and directory.

## Other General Tests

Keep enabling and disabling various options and check for consistency.