The Target operates in 2 modes.

1.DOWNLOAD MODE:

The UI can :

1.Read, change and download the configuration.

2.Download new code.

3.Run Hardware Diagnostics.

4.Reboot the target.

2.ROUTING MODE:

The UI can:

1.Read and change the configuration.

2.Reboot the target to

- go to DOWNLOAD MODE.

- remain in the same state as before Reboot.

Packet structure between SMC and UI port

--------------------------------------------

----------------------------------------------------------------------

7e	| BridgeNumber | Data ..... | CRC | 7e | Bridge Number..

----------------------------------------------------------------------

7e		: Frame Delimiters

BridgeNumber	: Last 4 bytes of card address	- 4 bytes

Data		: Actual Packet			- maximum 512 bytes

CRC 		: Frame check sequence		- 2 bytes

The first packet from the UI is QUERY_BRIDGE_NAME.

If (TargetMode == DOWNLOAD MODE)

Response =  ERR_NO_BRD_GET_PASSWORD or

ERR_NO_BRD_NO_PASSWORD depending on password present.

else if(TargetMode == ROUTING MODE)

Response = ERR_ROUTER_UP;

else

No response.

/*******************************************************************************/

Data : Different Pkt Types

/*******************************************************************************/

1. PASSWORD_TYPE

2. TRGT_CMND_TYPE

SUBTYPES:

A. DNLOAD_CODE_TYPE

B. HW_DIAG_TYPE

1. SCC1

2. SCC2

3. SCC3

4. SCC4

C. DNLOAD_CFG_TYPE

3. READ_REQ_TYPE

4. WRITE_REQ_TYPE	(ONLY FOR PRODUCTION)

5. REBOOT_TYPE

DownLoader

----------

This is a module in the booter to download the Express bridge code and

update the Bridge code header.

This module when invoked will accept compressed code packets

or normal configuration packets under a specific protocol and update the

Flash PROM area and Code/Config Header. It supports Abort and Timeout so that

the remote UI can get control back in the event of unexpected xmission

troubles.

///////////////////////////////////////////////////////////

NOTE: Download packets do not have a 4byte Bridge Id.

///////////////////////////////////////////////////////////

Packet structure for DOWNLOAD

-----------------------------

------+-----------+--------------+-----+...--------+---------+--------+-----

7e  | Pkt Type  | Sequence Num | Packet Data ... |   CRC   |  7e    |

------+-----------+--------------+-----+...--------+---------+--------+-----

7e		: Frame Delimiters		- 1 byte

PktType		: Type of download packet	- 1 byte

SeqNum		: Sequence Number of pkt	- 1 byte

: SeqNum = 0 for header pkt. All subsequent pkts ( including

: download and eof pkts) have the next incremented value of SeqNum.

PacketData	: Compressed Code or Normal Configuration pkt - Max 2048 bytes

CRC 		: Frame check sequence		- 2 bytes

/* Packet types */

PT_UNKNOWN	0

PT_HEADER	1		/* download header type - from UI */

PT_DATA		2		/* download data pkt - from UI */

PT_ABORT	3		/* abort download pkt - from UI and Target */

PT_EOF		4		/* endof download packet - from UI */

PT_NACK		5		/* negative ack - from Target */

PT_ACK		6		/* positive ack - from Target */

PT_TMOUT	7		/* Timeout packet - from Target */

PT_INFO		8		/* Info req./response - from UI and Target */

The following is the protocol

Rcvr State           Received event        Rcvr Action

----------           --------------        -----------

1. Wait for          1a. Bad CRC.          ProcessErrorRcv

HEADER | ABORT |   1b. HEADER rcvd.      ProcessHeader.SendACK. Goto 2

TMOUT              1c. ABORT rcvd.       StopDownLoad

1d. TMOUT occured.    if (GblTimeOut++ <= DnLdMaxTries)

Send NACK

else

AbortDownLoad

1e. Previous SeqNum   Send ACK;

1f. Wrong SeqNum      AbortDownLoad

2. Wait for          2a. Bad CRC           ProcessErrorRcv

DATA | EOF |       2b. DATA rcvd.        ProcessData.Send ACK.

ABORT | TMOUT      2c. EOF rcvd.         ProcessEOF

2d. ABORT rcvd.       AbortDownLoad

2e. TMOUT occured.    if (GblTimeOut++ <= DnLdMaxTries)

Send LACK

else

AbortDownLoad

2f. Previous SeqNum   Send ACK;

2g. Wrong SeqNum      AbortDownLoad

Sender State         Received Event        Sender Action

------------         --------------        -------------

The Console TMOUT interval is = Target's ((DnLdMaxRetries+1) * DnLdTmOut)

1. Send Header		1a. NACK came		Resend Header

1b. ACK came		Goto 2

1c. TMOUT		Abort

1d. User ABORT		Send ABORT and abort download

1e. Target ABORT	Abort download. Flash MSG

1f. Bad CRC		Send same packet.

2. Send code		2a. NACK/LACK came	Resend Data

2b. ACK came		Send Code if end goto 3

2c. TMOUT		Abort

2d. User ABORT		Send ABORT and abort download

2e. Target ABORT	Abort download. Flash Msg

2f. Bad CRC		Send same packet.

3. Send EOF 		3a. NACK/LACK came	Resend Data

3b. ACK came		Send Code if end goto 3

3c. TMOUT		Abort

3d. Target ABORT	Abort download. Flash Msg

3e. Bad CRC		Send same packet.

ProcessErrorRcv()

{

TotErrorCnt++;

if(TotErrorCnt >= MAX_ERROR_CNT)

AbortDownload();

else

SendNACK();

}

ProcessHeader()

{

If SeqNum != CurSeqNum

AbortDownload();

return;

If HeaderTypePkt

ProcessPkt;

SendACK();

return;

else if AbortPkt

StopDownLoad();

return;

SendACK();

}

StopDownLoad()

{

StopTimer;

return STOP_DNLD;

}

AbortDownload()

{

StopTimer;

SendErrorPkt();

return STOP_DNLD;

}