Telnet Server Implementation

----------------------------

A TELNET server accepts key-strokes from a telnet client and returns screen

images in response to the key-strokes. The MultiRouter Telnet server asks

for the Password (if it exists) as soon as the client connects and after verifying the

password, it presents the Telnet client with a 'menu' .

The menu contains items like :

1. Protocol Stacks

2. High Level Device Drivers

3. Low Level Device Drivers

4. System Information

Under Protocol Stacks, the configuration and statistics corresponding to IPX,

IP, SNMP etc. can be seen. This Telnet session cn be used only to view the

MultiRouter setup. It does not allow the user to change/modify the config-

uration of MultiRouter.

Options Negotiation

-------------------

MultiRouter Telnet Server will operate in the following mode :

1. Terminal-Type : Network Virtual Terminal (NVT)

Since not much screen manipulation is needed to present a menu,

the Terminal-Type option is not negotiated. Later, certain well-known/common

terminals may be supported.

2. Echo

The Telnet server will NOT echo each character that arrives from the client,

to avoid delays and reduce processing. Echoing of characters will be done

locally by the client.

3. Suppress Go-Ahead

The Telnet server will NOT send a Go-Ahead character to the client after every

processing. Since the connection is effectively full-duplex, the Go-Ahead

signal is not required. The prompt sent by the server will be an indication

to the users that they can enter the next choice. However, if this option is not

negotiated (clients refuses to accept WILL SGA), an 'IAC GA' is sent after every

packet sent from the server. (This may not be OK since we might be sending

more than 1 packet at a time and so the 'IAC GA' must be sent after all these

packets are sent).

4. Transmit-Binary

Since the Telnet server will use NVT, binary transmission will NOT be supported.

Later, when different Terminal-Types are negotiated, binary transmission will

automatically have to be negotiated.

Processing Input

----------------

Since the server refuses to Echo characters, clients are expected to locally

echo the characters that are typed by the user and also take care of editing

character (BS) . These clients send the string as soon as <ENTER> is pressed

or a TELNET command has to be sent (IP, AO, AYT, EC etc).

The clients are expected to map their end-of-line character to <CR><LF> NVT

sequence. Some clients send this sequence and others send only <CR>.

The TELNET server processes the input as soon as a <CR> is encountered without

waiting for the next character to be <LF>. Any <LF>/<NULL> characters are discarded.

Menu Handling

-------------

Each menu is identified by an index.

Main Menu -> TMS_MAINMENU

IPX Port Setup ->TMS_IPXPORTSETUP etc

Corresponding to each of these values is a structure in the menufsmhdlr array.

The structure contains:

1. Menu handler - Function to be called to process the input for this menu.

2. Menu         - Pointer to the menu to be displayed

3. Parent Menu  - Index of this menu's parent menu.

Parent menu is the menu that was displayed prior to

displaying this menu.

Parent of IPX Port Setup Menu (TMS_IPXPORTSETUP) is

IPX Port Information Menu (TMS_IPXPORTINFO).

4. Items        - Number of items in the menu.

The structure for a menu can be identified by its index value.

Chat Mode

----------

Chat Mode can be initiated on a WAN port by selecting the port from the

menu. If any WAN port is already in that mode an error is indicated.

Once a Chat session is initiated by a client, all other connections

are closed. The Chat session is stopped when :

a) Client closes the connection by exiting (System Reboot).

b) Client sends an Abort Output (AO) command (Chat -> Menu Mode).

c) Client sends an Interrupt Process (IP) to close connection  (System Reboot).

Print

-----

In the stdlib.lib 'printf' call, a request for a Print Descriptor is made.

This request must not be made in case of 'sprintf' which also calls 'printf'

but with the argument 'bufmode' set to TRUE.

Adding Menus

------------

1. In menufsm.c, menufsmhdlr structure, add the entry corresponding to the

menu replacing the NULL structure. (If there is no NULL structure, u need insert

the new structure and move all structures down by 1.

Add the actual menu also in the file.

2. Declare the Menu as extern in menufsm.h

3. Give the proper index to the structure, index to the

elements in the menu and the prototype of the menu in menu.h

4. Write the menuhandler.