The following verbs are supported for the router script :

1.RGETC

2.RGETS

3.GETDCD

4.HANGUP

5.BAUDRATE

6.STOPBITS

7.PARITY

8.DEC

9.INC

10.EXIT

11.ATOI

12.ITOA

13.STRLEN

14.STRCAT

15.STRCMP

16.STRCPY

17.STRFMT

18.TOUPPER

19.TOLOWER

20.TRANSMIT

21.WAITFOR

22.WAIT

23.BREAK

24.RXFLUSH

25.TXFLUSH

26.THISLAYERUP

The total size of each script (i.e compressed SCR and uncompressed COD)

cannot exceed 8 KB

For each Wan Port 'n' (where n = 1, 2,...) :-

The source is named PORTn.SCR

The compiled code is named PORTn.COD

The compressed source is named PORTn.SC_

PORTn.SC_ + PORTn.COD ==> PORTn.FIL

The Script is executed only for an ASYNC - Modem connected port

The Script can be run only in the beginning or can also be run

upon Communication failure.

This is based on a string in the Configuration file

WAN Port Restart Script On Communication Failure = enabled/disabled

For PORTn.SCR to be compiled, the date of modification of PORTn.SCR

should be later than that of PORTn.COD

Unless a new script is generated it cannot be downloaded again

Each of the verbs has to be tested