24, July, 1996

Sanjay

MSM (Modem State Machine) Module Documentation

----------------------------------------------

This module provides three simple state machines. One is for

initializing a modem with an init string. Another is for

hanging up a modem connection. And a third is for sending a

break sequence. The support provided by this module can be used

by any "app".

The module library is found in DEVDRVRS\MSM directory.

Three functions are provided to start off these state

machines. Once started, each of the state machines reaches a

final state and in the final state, calls a callback

function provided as parameters to the start off function.

The functions are

1.

Prototype:

void (*)(USHORT, BYTE *, USHORT) start_modem_init_state_machine(

USHORT port_number,

char **modem_init_string,

void (*fptr_write_port)(USHORT, BYTE *, USHORT),

void (*fptr_modem_init_complete)(USHORT, enum BOOLEAN))

Parameters:

port_number - port on which modem is found; 0 based; 0 =

SCC2 and so on.

modem_init_string - this is a pointer to an array of 5

character pointers; this should not be null, but

each element can be null; if non-null, each element

is a pointer to a init string for the modem; each

init string can be atmost 40 characters + 1 for null.

fptr_write_port - pointer to a serial write function; this

will be called with the port number to write on, the

pointer to buffer to write and number of bytes in

buffer to write as parameter; this function should

allocate a separate buffer, copy the data to write to

that buffer and use the new buffer for send purposes,

suitably de-allocating it when the send is done

fptr_modem_init_complete - can be null; function to call when

final state is reached; this function is called with

the port number and TRUE if init was successful, else

port number and FALSE as parameters; init is

successful if the string OK was received from the

modem

Returns:

A pointer to a function (or null) that the app must call

during this init phase (until the callback is called) on

every packet received from the SCC.

This function requires as parameters the port on which

the packet was received (0 based), a pointer to the

received data and the length of data received.

NOTES:

1. Due to the way the QUICC SCC's operate, only the app will

receive data from the SCC. So during the init phase, the

app should call the returned function with a pointer to

the received data packet.

2. start_modem_hangup_state_machine()

Prototype:

void start_modem_hangup_state_machine(USHORT port_number,

char *modem_hangup_string,

void (*fptr_write_port)(USHORT, BYTE *, USHORT),

void (*fptr_modem_hangup_complete)(USHORT))

Parameters:

port_number - port on which hangup is desired; 0 based

modem_hangup_string - can be null; can have <BREAK> or '~'

in the string

fptr_write_port - pointer to a serial write function; this

will be called with the port number to write on, the

pointer to buffer to write and number of bytes in

buffer to write as parameter

fptr_modem_hangup_complete - called when all of hangup string

has been sent; called with port number on which

hangup was attempted as parameter.

Returns:

Nothing

NOTES:

1. This does not actually guarantee that hangup will be

successful. It first attempts to hangup by bringing down the

DTR. If that fails, it sends the hangup string, waits for

some amount of time and then returns without again verifying

if the hangup actually occured.

3. send_break()

Prototype:

void send_break(USHORT port_number)

Parameters:

port_number - 0 based port on which the break sequence is to

be sent

Returns:

Nothing

Notes:

Break sequence is pre-determined and number of characters are

also pre-programmed in the BOOT code.

Caller should make sure that the other state machines are

not called for atleast one timer tick (the time this state

machine spends sending the break sequence)

General Notes

-------------

1. Include the header file MSM.H at calling end. This is in the

\rtrware\include directory.

2. Only 3 WAN ports are currently supported SCC2, SCC3 and SCC4.

3. Since app control read and write to the serial channels, it

should provide a serial write function that accepts following

parameters. This function is provided as parameters to the

init and hangup interface functions.

USHORT port_number - port number to write on,

BYTE *sptr_buffer - pointer to buffer to write

USHORT bytes_to_write - number of bytes in buffer to write

4. The hangup state machine works properly only if the Port C

is programmed with CD pins as general purpose input pins.

Also the ULONG variable LocalIOToMove is used.

5. Both the init and hangup state machines will finish in finite

time. Init may take atmost 4 seconds. Hangup may take atmost

4 seconds.